Cisco BTS 10200 Softswitch Software Upgrade for Release

3.2.1 V00 to 4.2.0 V10

July 7, 2004

Corporate Headquarters

Cisco Systems, Inc.

170 West Tasman Drive

San Jose, CA 95134-1706

USA

http://www.cisco.com

Tel: 	408 526-4000

800 553-NETS (6387)

Fax:	408 526-4100

THE SPECIFICATIONS AND INFORMATION REGARDING THE PRODUCTS IN THIS MANUAL ARE SUBJECT TO CHANGE WITHOUT NOTICE. ALL STATEMENTS, INFORMATION, AND RECOMMENDATIONS IN THIS MANUAL ARE BELIEVED TO BE ACCURATE BUT ARE PRESENTED WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED. USERS MUST TAKE FULL RESPONSIBILITY FOR THEIR APPLICATION OF ANY PRODUCTS.

THE SOFTWARE LICENSE AND LIMITED WARRANTY FOR THE ACCOMPANYING PRODUCT ARE SET FORTH IN THE INFORMATION PACKET THAT SHIPPED WITH THE PRODUCT AND ARE INCORPORATED HEREIN BY THIS REFERENCE. IF YOU ARE UNABLE TO LOCATE THE SOFTWARE LICENSE OR LIMITED WARRANTY, CONTACT YOUR CISCO REPRESENTATIVE FOR A COPY.

The Cisco implementation of TCP header compression is an adaptation of a program developed by the University of California, Berkeley (UCB) as part of UCB’s public domain version of the UNIX operating system. All rights reserved. Copyright © 1981, Regents of the University of California.

NOTWITHSTANDING ANY OTHER WARRANTY HEREIN, ALL DOCUMENT FILES AND SOFTWARE OF THESE SUPPLIERS ARE PROVIDED “AS IS” WITH ALL FAULTS. CISCO AND THE ABOVE-NAMED SUPPLIERS DISCLAIM ALL WARRANTIES, EXPRESSED OR IMPLIED, INCLUDING, WITHOUT LIMITATION, THOSE OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT OR ARISING FROM A COURSE OF DEALING, USAGE, OR TRADE PRACTICE.

IN NO EVENT SHALL CISCO OR ITS SUPPLIERS BE LIABLE FOR ANY INDIRECT, SPECIAL, CONSEQUENTIAL, OR INCIDENTAL DAMAGES, INCLUDING, WITHOUT LIMITATION, LOST PROFITS OR LOSS OR DAMAGE TO DATA ARISING OUT OF THE USE OR INABILITY TO USE THIS MANUAL, EVEN IF CISCO OR ITS SUPPLIERS HAVE BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES.

CCIP, CCSP, the Cisco Arrow logo, the Cisco Powered Network mark, the Cisco Systems Verified logo, Cisco Unity, Follow Me Browsing, FormShare, iQ Breakthrough, iQ FastTrack, the iQ Logo, iQ Net Readiness Scorecard, Networking Academy, ScriptShare, SMARTnet, TransPath, and Voice LAN are trademarks of Cisco Systems, Inc.; Changing the Way We Work, Live, Play, and Learn, The Fastest Way to Increase Your Internet Quotient, and iQuick Study are service marks of Cisco Systems, Inc.; and Aironet, ASIST, BPX, Catalyst, CCDA, CCDP, CCIE, CCNA, CCNP, Cisco, the Cisco Certified Internetwork Expert logo, Cisco IOS, the Cisco IOS logo, Cisco Press, Cisco Systems, Cisco Systems Capital, the Cisco Systems logo, Empowering the Internet Generation, Enterprise/Solver, EtherChannel, EtherSwitch, Fast Step, GigaStack, Internet Quotient, IOS, IP/TV, iQ Expertise, LightStream, MGX, MICA, the Networkers logo, Network Registrar, Packet, PIX, Post-Routing, Pre-Routing, RateMUX, Registrar, SlideCast, StrataView Plus, Stratm, SwitchProbe, TeleRouter, and VCO are registered trademarks of Cisco Systems, Inc. and/or its affiliates in the U.S. and certain other countries.

All other trademarks mentioned in this document or Web site are the property of their respective owners. The use of the word partner does not imply a partnership relationship between Cisco and any other company. (0301R)

Cisco BTS 10200 Softswitch Software Upgrade

Copyright © 2004, Cisco Systems, Inc.

All rights reserved.

| Revision History |  |  |  |
| --- | --- | --- | --- |
| Date | Version | Revised By | Description |
| 6/24/2004 | 1.0 | Jack Daih | - Initial Version |
| 7/7/2004 | 1.0 | Jack Daih | - Added error handling instructions if SS7-CIC table fails during data migration. SS7-CIC is not suppose to have any provisioned data for China market. For North America market, OPC and NET-AP must be defined prior to data migration. |

Table of Contents

Table of Contents	4

Preface	7

Obtaining Documentation	7

World Wide Web	7

Documentation CD-ROM	7

Ordering Documentation	7

Documentation Feedback	8

Obtaining Technical Assistance	8

Cisco.com	8

Technical Assistance Center	9

Cisco TAC Web Site	9

Cisco TAC Escalation Center	10

Chapter 1	11

Upgrade Requirements	11

Introduction	11

Assumptions	12

Requirements	13

Important notes about this procedure	13

Chapter 2	15

Preparation	15

Prerequisites	15

Task 1: Prepare CLI scripts to Migrate Features from Release 3.2 to Release 4.2	15

Chapter 3	16

Check Release 3.2 System	16

Task 1: Verify there is no redundant TSAP addresses	16

From EMS Side A	16

Task 2: Check required billing information on release 3.2	17

From EMS Side A	17

Chapter 4	18

Check Release 4.2 System	18

Task 1: Verify system status	18

Chapter 5	19

Migrate Data from Release 3.2 to Release 4.2	19

You will be working from 4.2 systems for the entire chapter.	19

Task 1: Clean up database	19

From EMS side B	19

From EMS side A	20

From CA/FS side B	20

From CA/FS side A	21

Task 2: Change EMS side B host database information	22

From EMS side B	22

Task 3: Change EMS side B Oracle DB configuration	22

From EMS side B	22

Task 4: Migrating data from 3.2 systems to 4.2 systems	23

From EMS side B	23

Task 5: Restore EMS side B Oracle DB configuration	24

From EMS side B	24

Task 6: Restore EMS side B host database information	25

From EMS side B	25

Task 7: Bring up EMS side B applications	25

From EMS side B	25

Task 8: Populate EMS side A with new data	25

From EMS side A	25

Task 9: Start CA/FS applications	27

From CA/FS side A	27

From CA/FS side B	27

Task 10: Pushing data down to CA/FS	28

From EMS side B	28

Chapter 6	29

Finalizing Upgrade on 4.2 Systems	29

Task 1: Switchover activity from EMS side B to EMS side A	29

From EMS side B	29

Task 2: Restore the system to normal mode	29

From EMS side A	29

Task 3: Change TSAP addresses	30

From EMS side A	30

Task 4: Check SUP values	30

From EMS side A	30

Task 5: Provision DB license	31

From EMS side A	31

Task 6: Using prepared CLI scripts to Migrate Features from Release 3.2 to Release 4.2	31

From EMS side A	31

Task 7: Verify system status	32

Appendix A	34

Check System Status	34

From Active EMS side A	34

Appendix B	37

Check Call Processing	37

From EMS side A	37

Appendix C	40

Check Provisioning and Database	40

From EMS side A	40

Perform database audits	40

Check transaction queue	40

Appendix D	42

Check Alarm Status	42

From EMS side A	42

Appendix E	44

Check Oracle Database Replication and Error Correction	44

Check Oracle DB replication status	44

From EMS side A	44

Correct replication error	45

From EMS Side B	45

From EMS Side A	45

Appendix F	47

Add New Announcements	47

Task 1: Record Announcements	47

Task 2: Place Announcements	47

From Active EMS	47

Appendix G	49

Associate Announcements	49

Task 1: Associate announcement with route-guide	49

From Active EMS	49

Task 2: Associate announcement with release causes	50

From Active EMS	50

Appendix H	52

Create New Features	52

Task 1: Call Forwarding Unconditional (CFU)	52

From Active EMS	52

Task 2: Call Forwarding Busy (CFB)	53

From Active EMS	53

Task 3: Call Forward No-Answer (CFNA)	53

From Active EMS	54

Appendix I	55

Updating Existing Features	55

From EMS side A	55

Preface

# Obtaining Documentation

These sections explain how to obtain documentation from Cisco Systems.

### World Wide Web

You can access the most current Cisco documentation on the World Wide Web at this URL: http://www.cisco.com/

Translated documentation is available at this URL: http://www.cisco.com/public/countries_languages.shtml

### Documentation CD-ROM

Cisco documentation and additional literature are available in a Cisco Documentation CD-ROM package, which is shipped with your product. The Documentation CD-ROM is updated monthly and may be more current than printed documentation. The CD-ROM package is available as a single unit or through an annual subscription.

### Ordering Documentation

You can order Cisco documentation in these ways:

Registered Cisco.com users (Cisco direct customers) can order Cisco product documentation from the Networking Products MarketPlace: http://www.cisco.com/cgi-bin/order/order_root.pl

Registered Cisco.com users can order the Documentation CD-ROM through the online Subscription Store: http://www.cisco.com/go/subscription

Nonregistered Cisco.com users can order documentation through a local account representative by calling Cisco Systems Corporate Headquarters (California, U.S.A.) at 408 526-7208 or, elsewhere in North America, by calling 800 553-NETS (6387).

### Documentation Feedback

You can submit comments electronically on Cisco.com. In the Cisco Documentation home page, click the Fax or Email option in the “Leave Feedback” section at the bottom of the page.

You can e-mail your comments to mailto:bug-doc@cisco.com.

You can submit your comments by mail by using the response card behind the front cover of your document or by writing to the following address:

Cisco Systems, INC.

Attn: Document Resource Connection

170 West Tasman Drive

San Jose, CA 95134-9883

# Obtaining Technical Assistance

Cisco provides Cisco.com as a starting point for all technical assistance. Customers and partners can obtain online documentation, troubleshooting tips, and sample configurations from online tools by using the Cisco Technical Assistance Center (TAC) Web Site. Cisco.com registered users have complete access to the technical support resources on the Cisco TAC Web Site: http://www.cisco.com/tac

### Cisco.com

Cisco.com is the foundation of a suite of interactive, networked services that provides immediate, open access to Cisco information, networking solutions, services, programs, and resources at any time, from anywhere in the world.

Cisco.com is a highly integrated Internet application and a powerful, easy-to-use tool that provides a broad range of features and services to help you with these tasks:

Streamline business processes and improve productivity

Resolve technical issues with online support

Download and test software packages

Order Cisco learning materials and merchandise

Register for online skill assessment, training, and certification programs

If you want to obtain customized information and service, you can self-register on Cisco.com. To access Cisco.com, go to this URL: http://www.cisco.com/

### Technical Assistance Center

The Cisco Technical Assistance Center (TAC) is available to all customers who need technical assistance with a Cisco product, technology, or solution. Two levels of support are available: the Cisco TAC Web Site and the Cisco TAC Escalation Center.

Cisco TAC inquiries are categorized according to the urgency of the issue:

Priority level 4 (P4)—You need information or assistance concerning Cisco product capabilities, product installation, or basic product configuration.

Priority level 3 (P3)—Your network performance is degraded. Network functionality is noticeably impaired, but most business operations continue.

Priority level 2 (P2)—Your production network is severely degraded, affecting significant aspects of business operations. No workaround is available.

Priority level 1 (P1)—Your production network is down, and a critical impact to business operations will occur if service is not restored quickly. No workaround is available.

The Cisco TAC resource that you choose is based on the priority of the problem and the conditions of service contracts, when applicable.

### Cisco TAC Web Site

You can use the Cisco TAC Web Site to resolve P3 and P4 issues yourself, saving both cost and time. The site provides around-the-clock access to online tools, knowledge bases, and software. To access the Cisco TAC Web Site, go to this URL: http://www.cisco.com/tac

All customers, partners, and resellers who have a valid Cisco service contract have complete access to the technical support resources on the Cisco TAC Web Site. The Cisco TAC Web Site requires a Cisco.com Log in ID and password. If you have a valid service contract but do not have a Log in ID or password, go to this URL to register: http://www.cisco.com/register/

If you are a Cisco.com registered user, and you cannot resolve your technical issues by using the Cisco TAC Web Site, you can open a case online by using the TAC Case Open tool at this URL: http://www.cisco.com/tac/caseopen

If you have Internet access, we recommend that you open P3 and P4 cases through the Cisco TAC Web Site: http://www.cisco.com/tac

### Cisco TAC Escalation Center

The Cisco TAC Escalation Center addresses priority level 1 or priority level 2 issues. These classifications are assigned when severe network degradation significantly impacts business operations. When you contact the TAC Escalation Center with a P1 or P2 problem, a Cisco TAC engineer automatically opens a case.

To obtain a directory of toll-free Cisco TAC telephone numbers for your country, go to this URL: http://www.cisco.com/warp/public/687/Directory/DirTAC.shtml

Before calling, please check with your network operations center to determine the level of Cisco support services to which your company is entitled: for example, SMARTnet, SMARTnet Onsite, or Network Supported Accounts (NSA). When you call the center, please have available your service agreement number and your product serial number.

Chapter 1

Upgrade Requirements

# Introduction

Application software loads are designated as Release 900-aa.bb.cc.Vxx, where

- aa=major release number, for example, 01

- bb=minor release number, for example, 03

- cc=maintenance release, for example, 00

- Vxx=Version number, for example V04

This procedure can be used on an in-service system, but the steps must be followed as shown in this document in order to avoid traffic interruptions.

|  | Caution Performing the steps in this procedure will bring down and restart individual platforms in a specific sequence. Do not perform the steps out of sequence, as it could affect traffic. If you have questions, contact Cisco Support. |
| --- | --- |

This procedure should be performed during a maintenance window.

|  | Note In this document, the following designations are used: - EMS = Element Management System; - CA/FS = Call Agent / Feature Server - Primary is also referred to as "Side A" - Secondary is also referred to as "Side B" See Figure 1-1 for a front view of the Softswitch rack. |
| --- | --- |

Figure 1-1   Cisco BTS 10200 Softswitch—Rack Configuration

# Assumptions

The following assumptions are made.

- The installer has a basic understanding of UNIX and Oracle commands.

- The installer has the appropriate user name(s) and password(s) to log on to each EMS/CA/FS platform as root user, and as Command Line Interface (CLI) user on the EMS.

- The installer has a NETWORK INFORMATION DATA SHEET (NIDS) with the IP addresses of each EMS/CA/FS to be upgraded, and all the data for the opticall.cfg file.

- Confirm that all names in opticall.cfg are in the DNS server

- The CD-ROM disks for the correct software version is available to the installer, and is readable.

|  | Note Contact Cisco Support before you start if you have any questions. |
| --- | --- |

# Requirements

Verify that opticall.cfg has the correct information for each of the following machines.

- Side A EMS

- Side B EMS

- Side A CA/FS

- Side B CA/FS

Determine the oracle and root passwords for the systems you are upgrading. If you do not know these passwords, ask your system administrator.

# Important notes about this procedure

Throughout this procedure, each command is shown with the appropriate system prompt, followed by the command to be entered in bold. The prompt is generally one of the following:

- Host system prompt (<hostname>#)

- Oracle prompt (<hostname>$)

- SQL prompt (SQL>)

- CLI prompt (CLI>)

- FTP prompt (ftp>)

Note the following conventions used throughout the steps in this procedure:

- Enter commands as shown, as they are case sensitive (except for CLI commands).

- Press the Return (or Enter) key at the end of each command, as indicated by "<Return>".

It is recommended that you read through the entire procedure before performing any steps.

Chapter 2

Preparation

|  | Note CDR delimiter customization is not retained after software upgrade. The customer or Cisco engineer must manually customize again to keep the same customization. |
| --- | --- |

# Prerequisites

- Verified host names for the 3.2 system are correct.

- Verified host names for the 4.2 system are correct.

- Verified the 4.2 system hardware, OS, and application software are installed.

- Verified the 4.2 system provisioning and call-processing are working.

- Verified there is secure shell (ssh) access to the system.

# Task 1: Prepare CLI scripts to Migrate Features from Release 3.2 to Release 4.2

Complete one-two weeks before

- Create new announcements and place them to the proper media gateways using steps provided in Appendix F.

- Prepare CLI scripts for announcement association -- Cisco will help generate the CLI script. This requires each customer to provide current announcement information. Please refer to Appendix G for detailed steps.

File Name: ________________________________________

- Prepare CLI scripts for feature upgrades -- Cisco will help generate the CLI script. This requires each customer to provide current feature set information. Please refer to Appendix H and Appendix I for detailed steps.

File Name: ________________________________________

Chapter 3

# Check Release 3.2 System

This chapter applies only to the BTS 3.2 system.

# Task 1: Verify there is no redundant TSAP addresses

# From EMS Side A

Step 1   Log in as root.

Step 2   Log in as Oracle user

<hostname># su – oracle <Return>

Step 3   Connect to Oracle database

<hostname>$ sqlplus optiuser/optiuser <Return>

Step 4   Check the result for the following SQL queries. Look for multiple records for each table.

- For table AGGR:

SQL> select id, TSAP_ADDR from AGGR t1 where exists (select 1 from AGGR t2 where upper(t1.TSAP_ADDR) = upper(t2.TSAP_ADDR) and not t1.id = t2.id ) order by upper(TSAP_ADDR); <Return>

- For table MGW:

SQL> select id, TSAP_ADDR from MGW t1 where exists (select 1 from MGW t2 where upper(t1.TSAP_ADDR) = upper(t2.TSAP_ADDR) and not t1.id = t2.id ) order by upper(TSAP_ADDR); <Return>

- For table TRUNK_GRP:

SQL> select id, SOFTSW_TSAP_ADDR from TRUNK_GRP t1

where exists (select 1 from TRUNK_GRP t2 where upper(t1.SOFTSW_TSAP_ADDR)=upper(t2.SOFTSW_TSAP_ADDR)

and not t1.id = t2.id ) order by upper(SOFTSW_TSAP_ADDR); <Return>

Step 5   SQL> exit <Return>

Step 6   <hostname>$ exit <Return>

Step 7   Delete the redundant record from CLI using the report from Step 4 for each of the above tables. Customer should perform this step as they can determine which record should be deleted.

# Task 2: Check required billing information on release 3.2

# From EMS Side A

Step 1   Log in as root.

Step 2   Log in as CLI user

Step 3   CLI> show billing-acct-addr; <Return>

- Verify following fields are defined:

  - BILLINGSERVERDIRECTORY

  - BILLINGSERVERADDRESS

  - USERNAME

- If not, please use following sample CLI change command to add information for the above two fields:

CLI> change billing-acct-addr billing-server-directory=<target directory name>; billing-server-addr=<server qualified domain name>; user-name=<user name>; password=<pass word>; <Return>

|  | Warning Don’t continue without above fields being defined. |
| --- | --- |

Chapter 4

# Check Release 4.2 System

This chapter applies only to the BTS 4.2 system.  The 4.2 system must already have database provisioned and call-processing has been verified working.

# Task 1: Verify system status

Step 1   Verify that the side A systems are in the active state. Use Appendix A for this procedure.

Step 2   Verify that call processing is working without error. Use Appendix B for this procedure.

Step 3   Verify that provisioning is operational from CLI command line, and verify database. Use Appendix C for this procedure.

Step 4   Verify that there are no outstanding major or critical alarms. Use Appendix D for this procedure.

Step 5   Verify that Oracle database and replication functions are working properly. Use Appendix E for this procedure.

|  | Caution Do not continue until the above verifications have been made. Call Cisco Support if you need assistance. |
| --- | --- |

Chapter 5

Migrating Data from Release 3.2 to Release 4.2

This chapter requires to be working on 4.2 systems exclusively. Existing provisioned data will be migrated from the old 3.2 systems to the new 4.2 systems.

|  | Warning Please make absolute certain that no provisioning activities on release 3.2 systems. Also make sure there is no activity of any kind, either provisioning or live traffic, on release 4.2 systems during entire period of performing this upgrade procedure. |
| --- | --- |

# You will be working from 4.2 systems for the entire chapter.

# Task 1: Clean up database

This procedure will remove all existing provisioned data from 4.2 systems.

# From EMS side B

Step 1   Log in as root.

Step 2   Disable the cron daemon.

<hostname># /etc/rc2.d/S75cron stop <Return>

Step 3   Stop applications.

<hostname># platform stop all <Return>

Step 4   Disable platform auto start program.

<hostname># cd /etc/rc3.d <Return>

<hostname># mv S99platform _S99platform <Return>

Step 5   Clean up Oracle data.

<hostname># cd /opt/ems/utils <Return>

<hostname># install.sh -cleardb <Return>

  - Answer “y” when prompted for Continue

  - Answer “y” when prompted for Reboot

  - Wait for the system to boot up

# From EMS side A

Step 1   Log in as root.

Step 2   Disable the cron daemon.

<hostname># /etc/rc2.d/S75cron stop <Return>

Step 3   Stop applications.

<hostname># platform stop all <Return>

Step 4   Disable platform auto start program.

<hostname># cd /etc/rc3.d <Return>

<hostname># mv S99platform _S99platform <Return>

Step 5   Clean up Oracle data.

<hostname># cd /opt/ems/utils <Return>

<hostname># install.sh -cleardb <Return>

  - Answer “y” when prompted for Continue

  - Answer “y” when prompted for Reboot

  - Wait for the system to boot up

# From CA/FS side B

Step 1   Log in as root.

Step 2   Disable the cron daemon.

<hostname># /etc/rc2.d/S75cron stop <Return>

Step 3   Stop applications.

<hostname># platform stop all <Return>

Step 4   Disable platform auto start program.

<hostname># cd /etc/rc3.d <Return>

<hostname># mv S99platform _S99platform <Return>

Step 5   Clean up shared memory data.

<hostname># cd /opt/ems/utils <Return>

<hostname># install.sh -clearshm <Return>

  - Answer “y” when prompted for Continue

  - Answer “y” when prompted for Reboot

  - Wait for the system to boot up

# From CA/FS side A

Step 1   Log in as root.

Step 2   Disable the cron daemon.

<hostname># /etc/rc2.d/S75cron stop <Return>

Step 3   Stop applications.

<hostname># platform stop all <Return>

Step 4   Disable platform auto start program.

<hostname># cd /etc/rc3.d <Return>

<hostname># mv S99platform _S99platform <Return>

Step 5   Clean up shared memory data.

<hostname># cd /opt/ems/utils <Return>

<hostname># install.sh -clearshm <Return>

  - Answer “y” when prompted for Continue

  - Answer “y” when prompted for Reboot

  - Wait for the system to boot up

# Task 2: Change EMS side B host database information

# From EMS side B

Step 1   Log in as root.

Step 2   Disable the cron daemon.

<hostname># /etc/rc2.d/S75cron stop <Return>

Step 3   <hostname># cp /etc/hosts /etc/hosts.42 <Return>

Step 4   <hostname># vi /etc/hosts <Return>

- Add the 3.2 EMS side A (primary) hostname with the corresponding IP address for the host

- Save the file

# Task 3: Change EMS side B Oracle DB configuration

# From EMS side B

Step 1   Log in as root.

Step 2   <hostname># su - oracle <Return>

Step 3   <hostname>$ cd /opt/oracle/product/8.1.7/network/admin  <Return>

Step 4   <hostname>$ \rm tnsnames.ora <Return>

Step 5   <hostname>$ cp /opt/oracle/admin/etc/tnsnames.template tnsnames.ora <Return>

Step 6   <hostname>$ chmod 666 tnsnames.ora <Return>

Step 7   <hostname>$ vi tnsnames.ora <Return>

- Replace all instances of “HOST = primary” to “HOST = <3.2 EMS side A primary hostname>”

- Replace all instances of “HOST = secondary” to “HOST = <4.2 EMS side B secondary hostname>”

- Save the file.

Step 8   <hostname>$ cd /opt/oracle/admin/etc <Return>

Step 9   <hostname>$ cp dba.properties dba.properties.42 <Return>

Step 10   <hostname>$ vi dba.properties <Return>

- Replace all instances of “4.2 EMS side A hostname” to “3.2 EMS side A hostname”

- Save the file.

Step 11   <hostname>$ dbinit –LE –i start <Return>

# Task 4: Migrating data from 3.2 systems to 4.2 systems

# From EMS side B

Step 1   Migrate data.

<hostname>$ cd /opt/oracle/admin/upd <Return>

<hostname>$ java dba.upd.UPDMgr –loadconfig ./config/3.2.0_to_4.2.0.cfg <Return>

<hostname>$ java dba.upd.UPDMgr –skip reset upgrade <Return>

<hostname>$ java dba.upd.UPDMgr –upgrade all <Return>

Step 2   Verify the FAIL=0 is reported.

<hostname>$ grep "FAIL=" UPDMgr.log <Return>

- If the return result is “FAIL=1”, please ignore the error if the failure is on SS7_CIC table by running “grep –i ss7_cic UPDMgr.log | grep –i fail.”

Step 3   Verify there is no constraint warning reported.

<hostname>$ grep constraint UPDMgr.log | grep –i warning <Return>

- If here is constraint warning, sftp /opt/oracle/admin/upd/UPDMgr.log file off the system, call Cisco Support for immediate technical assistance.

Step 4   Reload EMS only static data:

<hostname>$ cd /opt/oracle/opticall/create <Return>

<hostname>$ make nsc2 <Return>

Step 5   <hostname>$ dbinit –LE –i stop <Return>

# Task 5: Restore EMS side B Oracle DB configuration

# From EMS side B

Step 1   <hostname>$ cd /opt/oracle/product/8.1.7/network/admin  <Return>

Step 2   <hostname>$ \rm tnsnames.ora <Return>

Step 3   <hostname>$ ln –s /opt/oracle/admin/etc/tnsnames.ora tnsnames.ora <Return>

Step 4   <hostname>$ cd /opt/oracle/admin/etc <Return>

Step 5   <hostname>$ mv dba.properties.42 dba.properties <Return>

Step 6   <hostname>$ exit <Return>

# Task 6: Restore EMS side B host database information

# From EMS side B

Step 1   Log in as root.

Step 2   <hostname># mv /etc/hosts.42 /etc/hosts <Return>

# Task 7: Bring up EMS side B applications

# From EMS side B

Step 1   Log in as root.

Step 2   <hostname># platform start <Return>

Step 3   <hostname># nodestat <Return>

- Verify applications are in service.

Step 4   <hostname># /etc/rc2.d/S75cron start <Return>

Step 5   Restore platform auto start program.

<hostname># cd /etc/rc3.d <Return>

<hostname># mv _S99platform S99platform <Return>

# Task 8: Populate EMS side A with new data

# From EMS side A

Step 1   Log in as root.

Step 2   Disable the cron daemon.

<hostname># /etc/rc2.d/S75cron stop <Return>

Step 3   <hostname># platform start –i oracle <Return>

Step 4   Migrate data.

<hostname># su - oracle <Return>

<hostname>$ cd /opt/oracle/admin/upd <Return>

<hostname>$ java dba.upd.UPDMgr –loadconfig <Return>

<hostname>$ java dba.upd.UPDMgr –skip reset copy <Return>

<hostname>$ java dba.upd.UPDMgr –copy all <Return>

Step 5   Verify the FAIL=0 is reported.

<hostname>$ grep "FAIL=" UPDMgr.log <Return>

Step 6   Verify there is no constraint warning reported.

<hostname>$ grep constraint UPDMgr.log | grep –i warning <Return>

Step 7   If FAIL count is not 0 on step 5 or there is constraint warning on step 6, sftp /opt/oracle/admin/upd/UPDMgr.log file off the system, call Cisco Support for immediate technical assistance.

Step 8   Reload EMS only static data:

<hostname>$ cd /opt/oracle/opticall/create <Return>

<hostname>$ make nsc1 <Return>

Step 9   <hostname>$ exit <Return>

Step 10   <hostname># platform start <Return>

Step 11   Verify applications are in service.

<hostname># nodestat <Return>

Step 12   <hostname># /etc/rc2.d/S75cron start <Return>

Step 13   Restore platform auto start program.

<hostname># cd /etc/rc3.d <Return>

<hostname># mv _S99platform S99platform <Return>

# Task 9: Start CA/FS applications

# From CA/FS side A

Step 1   Log in as root.

Step 2   <hostname># platform start <Return>

Step 3   <hostname># nodestat <Return>

- Verify applications are in service.

Step 4   Restore platform auto start program.

<hostname># cd /etc/rc3.d <Return>

<hostname># mv _S99platform S99platform <Return>

# From CA/FS side B

Step 1   Log in as root.

Step 2   <hostname># platform start <Return>

Step 3   <hostname># nodestat <Return>

- Verify applications are in service.

Step 4   Restore platform auto start program.

<hostname># cd /etc/rc3.d <Return>

<hostname># mv _S99platform S99platform <Return>

# Task 10: Pushing data down to CA/FS

# From EMS side B

Step 1   Log in as CLI user.

Step 2   CLI> download database target=CA; <Return>

Step 3   CLI> download database target=FSPTC; <Return>

Step 4   CLI> download database target=FSAIN; <Return>

Chapter 6

Finalizing Upgrade on 4.2 Systems

Execute steps in this chapter on 4.2 systems only.

# Task 1: Switchover activity from EMS side B to EMS side A

This task will force the active system activity from EMS side B to side A.

# From EMS side B

|  | Note In the commands below, "xxx", "yyy" or "zzz" is the instance for the process on your system. |
| --- | --- |

Step 1   Log in as CLI user.

Step 2   CLI> control bdms id=BDMS01; target-state=forced-active-standby; <Return>

Step 3   CLI> control element-manager id=EM01; target-state=forced-active-standby; <Return>

Step 4   CLI shell session should be terminated when last CLI commands completes.

# Task 2: Restore the system to normal mode

This task will remove the forced switch and restore the system to NORMAL state.

# From EMS side A

|  | Note In the commands below, "xxx", "yyy" or "zzz" is the instance for the process on your system. |
| --- | --- |

Step 1   Log in as CLI user.

Step 2   CLI> control bdms id=BDMS01; target-state=normal; <Return>

Step 3   CLI> control element-manager id=EM01; target-state=normal; <Return>

# Task 3: Change TSAP addresses

This task will change the TSAP addresses from the old 3.2 systems to the new 4.2 systems.

# From EMS side A

|  | Note In the commands below, "xxx", "yyy" or "zzz" is the instance for the process on your system. |
| --- | --- |

Step 1   CLI> change call-agent ID=CAxxx; TSAP_ADDR_SIDEA=<New domain name>; <Return>

Step 2   CLI> change feature-server TYPE=AIN; ID=FSAINyyy; TSAP_ADDR_SIDEA=<New domain name:port_number>; <Return>

Step 3   CLI> change feature-server TYPE=POTS; ID=FSPTCzzz; TSAP_ADDR_SIDEA=<New domain name:port_number>; <Return>

# Task 4: Check SUP values

In this task, you will check for SUP values.

# From EMS side A

Step 1   Verify the SUP configuration is set up correctly.

CLI> show sup-config; <return>

- Verify refresh rate is set to 86400.

- If not, run the following CLI command:

CLI> change sup-config type=refresh_rate; value=86400; <return>

Step 2   CLI> exit <return>

# Task 5: Provision DB license

# From EMS side A

Step 1   Log in as root

Step 2   <hostname># cd /opt/oracle/admin/upd/config <Return>

Step 3   <hostname># chown ciscouser:staff dbsize.cli <Return>

Step 4   <hostname># cp -p dbsize.cli /opt/ems/ftp/deposit <Return>

Step 5   Bring up Internet Browser and enter address “http://<Side A EMS hostname>:10200”, then click on “report” and find the file. Then check for provisioning status report to make sure the fail count is 0.

# Task 6: Using prepared CLI scripts to Migrate Features from Release 3.2 to Release 4.2

# From EMS side A

Step 1   Verify new announcements have been placed to the proper media gateways with instructions provided in Appendix F.

Step 2   Add announcement associations to the system using the CLI script prepared one week before with detailed steps provided in Appendix G.

- FTP the announcement association script file to the EMS side A /opt/ems/ftp directory

- <hostname># cd /opt/ems/ftp <Return>

- <hostname># chown optiuser:staff <script file name> <Return>

- <hostname># cp –p <script file name> deposit <Return>

- Bring up Internet Browser and enter address “http://<Side A EMS hostname>:10200”, then click on “report” and find the file. Then check for provisioning status report to make sure the fail count is 0.

Step 3   Create new features using the CLI script prepared one week before with detailed steps provided in Appendix H.

- FTP the create new features script file to the EMS side A /opt/ems/ftp directory

- <hostname># cd /opt/ems/ftp <Return>

- <hostname># chown optiuser:staff <script file name> <Return>

- <hostname># cp –p <script file name> deposit <Return>

- Bring up Internet Browser and enter address “http://<Side A EMS hostname>:10200”, then click on “report” and find the file. Then check for provisioning status report to make sure the fail count is 0.

Step 4   Update existing features using the CLI script prepared one week before with detailed steps provided in Appendix I.

- FTP the update existing features script file to the EMS side A /opt/ems/ftp directory

- <hostname># cd /opt/ems/ftp <Return>

- <hostname># chown optiuser:staff <script file name> <Return>

- <hostname># cp –p <script file name> deposit <Return>

- Bring up Internet Browser and enter address “http://<Side A EMS hostname>:10200”, then click on “report” and find the file. Then check for provisioning status report to make sure the fail count is 0.

# Task 7: Verify system status

Step 1   Verify that the side A systems are in the active state. Use Appendix A for this procedure.

Step 2   Verify that call processing is working without error. Use Appendix B for this procedure.

Step 3   Verify that provisioning is operational from CLI command line, and verify database. Use Appendix C for this procedure.

Step 4   Verify that there are no outstanding major or critical alarms. Use Appendix D for this procedure.

Step 5   Verify that Oracle database and replication functions are working properly. Use Appendix E for this procedure.

|  | Caution Do not continue until the above verifications have been made. Call Cisco Support if you need assistance. |
| --- | --- |

You have completed BTS 10200 system upgrade process successfully.

Appendix A

Check System Status

The purpose of this procedure is to verify the system is running in NORMAL mode, with the side A systems in ACTIVE state and the side B systems in STANDBY state. This condition is illustrated in Figure A-1.

Figure A-1   Side A ACTIVE_NORMAL and Side B STANDBY_NORMAL

|  | Note In the commands below, "xxx", "yyy" or "zzz" is the instance for the process on your system, and DomainName is your system domain name. |
| --- | --- |

# From Active EMS side A

Step 1   Log in as CLI user.

Step 2   CLI> status call-agent id=CAxxx; <Return>

System response:

Reply : Success: APPLICATION INSTANCE -> Call Agent [CAxxx] PRIMARY STATUS -> ACTIVE_NORMAL SECONDARY STATUS -> STANDBY_NORMAL

Step 3   CLI> status feature-server id=FSAINyyy; <Return>

System response:

Reply : Success: APPLICATION INSTANCE -> Feature Server [FSAINyyy] PRIMARY STATUS -> ACTIVE_NORMAL SECONDARY STATUS -> STANDBY_NORMAL

Step 4   CLI> status feature-server id=FSPTCzzz; <Return>

System response:

Reply : Success: APPLICATION INSTANCE -> Feature Server [FSPTCzzz] PRIMARY STATUS -> ACTIVE_NORMAL SECONDARY STATUS -> STANDBY_NORMAL

Step 5   CLI> status bdms id=BDMS01; <Return>

System response:

Reply : Success: BILLING SERVER STATUS IS... -> APPLICATION INSTANCE -> Bulk Data Management Server [BDMS01] PRIMARY STATUS -> ACTIVE_NORMAL SECONDARY STATUS -> STANDBY_NORMAL BILLING MYSQL STATUS IS... -> Daemon is running!

Step 6   CLI> status element-manager id=EM01; <Return>

System response:

Reply : Success: ELEMENT MANAGER STATUS IS... -> APPLICATION INSTANCE -> Element Manager [EM01] PRIMARY STATUS -> ACTIVE_NORMAL SECONDARY STATUS -> STANDBY_NORMAL EMS MYSQL STATUS IS ... -> Daemon is running! ORACLE STATUS IS... -> Daemon is running!

Appendix B

Check Call Processing

This procedure verifies that call processing is functioning without error. The billing record verification is accomplished by making a sample phone call and verify the billing record is collected correctly.

# From EMS side A

Step 1   Log in as CLI user.

Step 2   Make a new phone call on the system. Verify that you have two-way voice communication. Then hang up both phones.

Step 3   CLI> report billing-record tail=1; <Return>

Sample billing record:

Reply : Success: Request was successful. HOST=<hostname> SEQUENCENUMBER=15 CALLTYPE=EMG SIGSTARTTIME=2004-06-14 09:38:31 SIGSTOPTIME=2004-06-14 09:39:10 ICSTARTTIME=2004-06-14 09:38:31 ICSTOPTIME=2004-06-14 09:39:10 CALLCONNECTTIME=2004-06-14 09:38:47 CALLANSWERTIME=2004-06-14 09:38:47 CALLDISCONNECTTIME=2004-06-14 09:39:10 CALLELAPSEDTIME=00:00:23 INTERCONNECTELAPSEDTIME=00:00:39 ORIGNUMBER=9722550001 TERMNUMBER=911 CHARGENUMBER=9722550001 DIALEDDIGITS=911 FORWARDINGNUMBER= QUERYTIME2=0 QUERYTIME3=0 OFFHOOKIND=1 SHORTOFFHOOKIND=0 CALLTERMCAUSE=NORMAL_CALL_CLEARING OPERATORACTION=0 ORIGSIGNALINGTYPE=0 TERMSIGNALINGTYPE=3 ORIGTRUNKNUMBER=0 TERMTRUNKNUMBER=911 OUTGOINGTRUNKNUMBER=0 ORIGCIRCUITID=0 TERMCIRCUITID=1 PICSOURCE=2 ICINCIND=1 ICINCEVENTSTATUSIND=20 ICINCRTIND=0 ORIGQOSTIME=2004-06-14 09:39:10 ORIGQOSPACKETSSENT=1081 ORIGQOSPACKETSRECD=494 ORIGQOSOCTETSSENT=171448 ORIGQOSOCTETSRECD=78084 ORIGQOSPACKETSLOST=0 ORIGQOSJITTER=512 ORIGQOSAVGLATENCY=6 TERMQOSTIME=2004-06-14 09:39:10 TERMQOSPACKETSSENT=494 TERMQOSPACKETSRECD=1081 TERMQOSOCTETSSENT=78084 TERMQOSOCTETSRECD=171448 TERMQOSPACKETSLOST=0 TERMQOSJITTER=440 TERMQOSAVGLATENCY=1 PACKETIZATIONTIME=20 SILENCESUPPRESION=1 ECHOCANCELLATION=0 CODERTYPE=PCMU CONNECTIONTYPE=IP OPERATORINVOLVED=0 CASUALCALL=0 INTERSTATEINDICATOR=0 OVERALLCORRELATIONID=CA1461 TIMERINDICATOR=0 RECORDTYPE=NORMAL RECORD CALLAGENTID=CA146 POPTIMEZONE=CST ORIGTYPE=ON NET TERMTYPE=OFF NET NASERRORCODE=0 NASDLCXREASON=0 FAXINDICATOR=NOT A FAX FAXPAGESSENT=0 FAXPAGESRECEIVED=0 == 1 Record(s) retrieved

Step 4   Verify that the attributes in the CDR match the call just made.

Appendix C

Check Provisioning and Database

# From EMS side A

The purpose of this procedure is to verify that provisioning is functioning without error. The following commands will add a "dummy" carrier then delete it.

Step 1   Log in as CLI user.

Step 2   CLI> add carrier id=8080; <Return>

Step 3   CLI> show carrier id=8080; <Return>

Step 4   CLI> delete carrier id=8080; <Return>

Step 5   CLI> show carrier id=8080; <Return>

- Verify message is: Database is void of entries.

# Perform database audits

In this task, you will perform a full database audit and correct any errors, if necessary.

Step 1   CLI> audit database type=full; <Return>

Step 2   Check the audit report and verify there is no discrepancy or errors. If errors are found, please try to correct them. If you are unable to correct, please contact Cisco Support.

# Check transaction queue

In this task, you will verify that the OAMP transaction queue status. The queue should be empty.

Step 1   CLI> show transaction-queue; <Return>

- Verify there is no entry shown. You should get following reply back:

Reply : Success: Database is void of entries.

- If the queue is not empty, wait for the queue to empty. If the problem persists, contact Cisco Support.

Step 2   CLI> exit <Return>

Appendix D

Check Alarm Status

The purpose of this procedure is to verify that there are no outstanding major/critical alarms.

# From EMS side A

Step 1   Log in as CLI user.

Step 2  CLI> show alarm; <Return>

- The system responds with all current alarms, which must be verified or cleared before executing this upgrade procedure.

|  | Tip Use the following command information for reference material ONLY. |
| --- | --- |

Step 3   To monitor system alarm continuously.

CLI> subscribe alarm-report severity=all; type=all; <Return>

|  | Valid severity: MINOR, MAJOR, CRITICAL, ALL Valid types: CALLP, CONFIG, DATABASE, MAINTENANCE, OSS, SECURITY, SIGNALING, STATISTICS, BILLING, ALL, SYSTEM, AUDIT |
| --- | --- |

Step 4   System will display alarms if alarm is reported.

CLI><------------------- START ALARM REPORT --------------------> TIMESTAMP: Fri Mar 14 14:01:28 CST 2003 DESCRIPTION: Disk Partition Moderately Consumed TYPE & NUMBER: MAINTENANCE (90) SEVERITY: MINOR ALARM-STATUS: ON ORIGIN: <hostname> COMPONENT-ID: HMN Disk Partition: / Percentage Used: 54.88 <------------------- END ALARM REPORT ---------------------->

Step 5   To stop monitoring system alarm.

CLI> unsubscribe alarm-report severity=all; type=all; <Return>

Step 6   Exit CLI.

CLI> exit <Return>

Appendix E

Check Oracle Database Replication and Error Correction

Perform the following steps on the Active EMS side A to check the Oracle database and replication status.

# Check Oracle DB replication status

# From EMS side A

Step 1   Log in as root.

Step 2   Log in as oracle.

<hostname># su – oracle <Return>

Step 3   Enter the command to check replication status and compare contents of tables on the side A and side B EMS databases:

<hostname>$ dbadm –C rep <Return>

Step 4   Verify that “Deferror is empty?” is “YES”.

OPTICAL1::Deftrandest is empty?           YES

OPTICAL1::dba_repcatlog is empty?       YES

OPTICAL1::Deferror is empty?              YES Make sure it is “YES”

OPTICAL1::Deftran is empty?                  YES

OPTICAL1::Has no broken job?               YES

OPTICAL1::JQ Lock is empty?                YES

OPTICAL2::Deftrandest is empty?          YES

OPTICAL2::dba_repcatlog is empty?      YES

OPTICAL2::Deferror is empty?             YES Make sure it is “YES”

OPTICAL2::Deftran is empty?                YES

OPTICAL2::Has no broken job?              YES

OPTICAL2::JQ Lock is empty?               YES

Step 5   If the “Deferror is empty?” is “NO”, please try to correct the error using steps in “Correct replication error” below. If you are unable to clear the error, contact Cisco Support.

# Correct replication error

|  | Note You must run the following steps on standby EMS side B first, then on active EMS side A. |
| --- | --- |

# From EMS Side B

Step 1   Log in as root

Step 2   <hostname># su – oracle <Return>

Step 3   <hostname>$dbadm -A copy -o oamp -t ALARM_LOG <Return>

- Enter “y” to continue

Step 4   <hostname>$dbadm -A copy -o oamp -t EVENT_LOG <Return>

- Enter “y” to continue

Step 4   <hostname>$dbadm -A copy -o oamp -t CURRENT_ALARM <Return>

- Enter “y” to continue

Step 5   <hostname>$dbadm –A truncate_def <Return>

- Enter “y” to continue

# From EMS Side A

Step 1   <hostname>$dbadm –A truncate_def <Return>

- Enter “y” to continue

Step 2   Re-verify that “Deferror is empty?” is “YES”.

<hostname>$dbadm –C rep <Return>

OPTICAL1::Deftrandest is empty?           YES

OPTICAL1::dba_repcatlog is empty?       YES

OPTICAL1::Deferror is empty?              YES  Make sure it is “YES”

OPTICAL1::Deftran is empty?                  YES

OPTICAL1::Has no broken job?               YES

OPTICAL1::JQ Lock is empty?                YES

OPTICAL2::Deftrandest is empty?          YES

OPTICAL2::dba_repcatlog is empty?      YES

OPTICAL2::Deferror is empty?             YES  Make sure it is “YES”

OPTICAL2::Deftran is empty?                YES

OPTICAL2::Has no broken job?              YES

OPTICAL2::JQ Lock is empty?               YES

Appendix F

Add New Announcements

This procedure describes steps to record new announcements and place each announcement file onto proper media gateways.

# Task 1: Record Announcements

Step 1   "Your Anonymous Call Rejection service is now on. All incoming calls will be checked for number privacy before they are allowed to complete to your line". Set filename to "ann_id_540.au"

Step 2   "Your Anonymous Call Rejection service can not be successfully activated in your line. Please check with your service provider and try again.". Set filename to "ann_id_541.au"

Step 3   "Your Anonymous Call Rejection service is now off. Incoming calls will not be checked for number privacy status". Set filename to "ann_id_542.au"

Step 4   "Your Anonymous Call Rejection service can not be successfully deactivated in your line". Set filename to "ann_id_543.au"

# Task 2: Place Announcements

# From Active EMS

Step 1   Log in as CLI user.

Step 2   Show a list of announcement trunk groups.

CLI> show trunk-grp tg-type=annc; <Return>

Step 3   For each trunk group ID shown in step 2, show a list of Media Gateways being used for announcements. These media gateways are where you would place the announcement files.

CLI> show trunk tgn-id=[Trunk Group ID]; <Return>

Appendix G

Associate Announcements

Associate the announcement files created in Appendix F with announcement IDs.

# Task 1: Associate announcement with route-guide

The route-guide to set is the route-guide being used for other announcements on the existing system. Do a “show route-guide” to get the list of route-guides available on the system. There should be one being used by the system.

# From Active EMS

Step 1   Log in as CLI user.

Step 2   Show a list of announcement trunk groups.

CLI> show trunk-grp tg-type=annc; <Return>

Step 3   For each trunk group ID shown in step 2, show a list of routes.

CLI> show route tgn1-id=[Trunk Group ID]; <Return>

Step 4    For each route ID shown in step 3, show a list of route guides.

CLI> show route-guide policy-id=[Route ID]; <Return>

- The route-guide IDs listed above now can be used for announcements in the subsequent steps. If CLI commands specified from Step 5 to Step 8 fails to execute, please choose a higher announcement ID that will work.

Step 5   CLI> add annc; id=540; type=SYSTEM; ANNOUNCEMENT_FILE=ann_id_540.au;  ROUTE_GUIDE=[Route Guide ID]; <Return>

Step 6   CLI> add annc; id=541; type=SYSTEM; ANNOUNCEMENT_FILE=ann_id_541.au; ROUTE_GUIDE=[ Route Guide ID]; <Return>

Step 7   CLI> add annc; id=542; type=SYSTEM; ANNOUNCEMENT_FILE=ann_id_542.au; ROUTE_GUIDE=[Route Guide ID]; <Return>

Step 8   CLI> add annc; id=543; type=SYSTEM; ANNOUNCEMENT_FILE=ann_id_543.au; ROUTE_GUIDE=[Route Guide ID]; <Return>

# Task 2: Associate announcement with release causes

These release-causes should be provisioned as is and none of them should be changed.

# From Active EMS

Step 1   CLI> add release-cause; id=1161;  annc_id=540; <Return>

- If the above CLI command fails, execute following command:

CLI> change release-cause; id=1161;  annc_id=540; <Return>

Step 2   CLI> add release-cause; id=1162;  annc_id=541; <Return>

- If the above CLI command fails, execute following command:

CLI> change release-cause; id=1162;  annc_id=541; <Return>

Step 3   CLI> add release-cause; id=1163;  annc_id=542; <Return>

- If the above CLI command fails, execute following command:

CLI> change release-cause; id=1163;  annc_id=542; <Return>

Step 4   CLI> add release-cause; id=1164;  annc_id=543;  <Return>

- If the above CLI command fails, execute following command:

CLI> change release-cause; id=1164;  annc_id=543; <Return>

Step 5   CLI> exit <Return>

Appendix H

Create New Features

Create new features: Call Forwarding Unconditional (CFU), Call Forwarding Busy (CFB), and Call Forward No-Answer (CFNA).

# Task 1: Call Forwarding Unconditional (CFU)

# From Active EMS

Step 1   Log in as CLI user.

Step 2   Add following records thru CLI to the system:

CLI> add nod-restrict-list; fname=CFU; nod=700;

CLI> add nod-restrict-list; fname=CFU; nod=900;

CLI> add nod-restrict-list; fname=CFU; nod=976;

CLI> add nod-restrict-list; fname=CFU; nod=AMBULANCE;

CLI> add nod-restrict-list; fname=CFU; nod=BUSINESS;

CLI> add nod-restrict-list; fname=CFU; nod=CUT_THRU;

CLI> add nod-restrict-list; fname=CFU; nod=DA;

CLI> add nod-restrict-list; fname=CFU; nod=DA_TOLL;

CLI> add nod-restrict-list; fname=CFU; nod=EMG;

CLI> add nod-restrict-list; fname=CFU; nod=FIRE;

CLI> add nod-restrict-list; fname=CFU; nod=INFO;

CLI> add nod-restrict-list; fname=CFU; nod=INTL_OPR;

CLI> add nod-restrict-list; fname=CFU; nod=INTL_WZ1;

CLI> add nod-restrict-list; fname=CFU; nod=NAT_OPR;

CLI> add nod-restrict-list; fname=CFU; nod=NON_EMG;

CLI> add nod-restrict-list; fname=CFU; nod=OPERATOR;

CLI> add nod-restrict-list; fname=CFU; nod=POLICE;

CLI> add nod-restrict-list; fname=CFU; nod=PREMIUM;

CLI> add nod-restrict-list; fname=CFU; nod=RELAY;

CLI> add nod-restrict-list; fname=CFU; nod=REPAIR;

CLI> add nod-restrict-list; fname=CFU; nod=SUBSCRIBER;

CLI> add nod-restrict-list; fname=CFU; nod=TIME;

CLI> add nod-restrict-list; fname=CFU; nod=TRAFFIC;

CLI> add nod-restrict-list; fname=CFU; nod=TW;

# Task 2: Call Forwarding Busy (CFB)

# From Active EMS

Step 1   Log in as CLI user.

Step 2   Add following records thru CLI to the system:

CLI> add nod-restrict-list; fname=CFB; nod=700;

CLI> add nod-restrict-list; fname=CFB; nod=900;

CLI> add nod-restrict-list; fname=CFB; nod=976;

CLI> add nod-restrict-list; fname=CFB; nod=AMBULANCE;

CLI> add nod-restrict-list; fname=CFB; nod=BUSINESS;

CLI> add nod-restrict-list; fname=CFB; nod=CUT_THRU;

CLI> add nod-restrict-list; fname=CFB; nod=DA;

CLI> add nod-restrict-list; fname=CFB; nod=DA_TOLL;

CLI> add nod-restrict-list; fname=CFB; nod=EMG;

CLI> add nod-restrict-list; fname=CFB; nod=FIRE;

CLI> add nod-restrict-list; fname=CFB; nod=INFO;

CLI> add nod-restrict-list; fname=CFB; nod=INTL_OPR;

CLI> add nod-restrict-list; fname=CFB; nod=INTL_WZ1;

CLI> add nod-restrict-list; fname=CFB; nod=NAT_OPR;

CLI> add nod-restrict-list; fname=CFB; nod=NON_EMG;

CLI> add nod-restrict-list; fname=CFB; nod=OPERATOR;

CLI> add nod-restrict-list; fname=CFB; nod=POLICE;

CLI> add nod-restrict-list; fname=CFB; nod=PREMIUM;

CLI> add nod-restrict-list; fname=CFB; nod=RELAY;

CLI> add nod-restrict-list; fname=CFB; nod=REPAIR;

CLI> add nod-restrict-list; fname=CFB; nod=SUBSCRIBER;

CLI> add nod-restrict-list; fname=CFB; nod=TIME;

CLI> add nod-restrict-list; fname=CFB; nod=TRAFFIC;

CLI> add nod-restrict-list; fname=CFB; nod=TW;

# Task 3: Call Forward No-Answer (CFNA)

# From Active EMS

Step 1   Log in as CLI user.

Step 2   Add following records thru CLI to the system:

CLI> add nod-restrict-list; fname=CFNA; nod=700;

CLI> add nod-restrict-list; fname=CFNA; nod=900;

CLI> add nod-restrict-list; fname=CFNA; nod=976;

CLI> add nod-restrict-list; fname=CFNA; nod=AMBULANCE;

CLI> add nod-restrict-list; fname=CFNA; nod=BUSINESS;

CLI> add nod-restrict-list; fname=CFNA; nod=CUT_THRU;

CLI> add nod-restrict-list; fname=CFNA; nod=DA;

CLI> add nod-restrict-list; fname=CFNA; nod=DA_TOLL;

CLI> add nod-restrict-list; fname=CFNA; nod=EMG;

CLI> add nod-restrict-list; fname=CFNA; nod=FIRE;

CLI> add nod-restrict-list; fname=CFNA; nod=INFO;

CLI> add nod-restrict-list; fname=CFNA; nod=INTL_OPR;

CLI> add nod-restrict-list; fname=CFNA; nod=INTL_WZ1;

CLI> add nod-restrict-list; fname=CFNA; nod=NAT_OPR;

CLI> add nod-restrict-list; fname=CFNA; nod=NON_EMG;

CLI> add nod-restrict-list; fname=CFNA; nod=OPERATOR;

CLI> add nod-restrict-list; fname=CFNA; nod=POLICE;

CLI> add nod-restrict-list; fname=CFNA; nod=PREMIUM;

CLI> add nod-restrict-list; fname=CFNA; nod=RELAY;

CLI> add nod-restrict-list; fname=CFNA; nod=REPAIR;

CLI> add nod-restrict-list; fname=CFNA; nod=SUBSCRIBER;

CLI> add nod-restrict-list; fname=CFNA; nod=TIME;

CLI> add nod-restrict-list; fname=CFNA; nod=TRAFFIC;

CLI> add nod-restrict-list; fname=CFNA; nod=TW;

Appendix I

Updating Existing Features

you will migrate existing features tot eh new release.

# From EMS side A

Step 1   Log in CLI user

Step 2   CLI> show feature fname=CFUA;

- Record the value for INTL flag

- CLI> add feature FNAME=CFVABBG; FEATURE_SERVER_ID=[FSPTCzzz]; TYPE1=CC; VALUE1=N; TYPE2=FDT; VALUE2=Y; TYPE3=SDT; VALUE3=Y; TYPE4=INTL; VALUE4=[INTL value recorded]; DESCRIPTION=call forwarding activation for basic business group;

Step 3   CLI> show feature fname=CFU;

- Record the value for MCF flag and RR flag

- CLI> add feature; fname=CFVBBG; TDP1=TERMINATION_ATTEMPT_AUTHORIZED; TID1=TERMINATION_ATTEMPT_AUTHORIZED; TTYPE1=R; FNAME1=CFVABBG; FNAME2=CFUD; FNAME3=CFUI; FEATURE_SERVER_ID=[FSPTCzzz]; TYPE1=MCF; VALUE1=[MCF value recorded]; TYPE2=RR; VALUE2=[RR value recorded]; DESCRIPTION=call forwarding for basic business group; GRP_FEATURE=N;

Step 4   CLI> show service fname1=CFU;

- Record the service IDs

- For each service ID found, add the new feature CFVBBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME[x]=CFVBBG;

Step 5   CLI>show service fname2=CFU;

- Record the service IDs

- For each service ID found, add the new feature CFVBBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME[x]=CFVBBG;

Step 6   CLI>show service fname3=CFU;

- Record the service IDs

- For each service ID found, add the new feature CFVBBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME[x]=CFVBBG;

Step 7   CLI>show service fname4=CFU;

- Record the service IDs

- For each service ID found, add the new feature CFVBBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME[x]=CFVBBG;

Step 8  CLI>show service fname5=CFU;

- Record the service IDs

- For each service ID found, add the new feature CFVBBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME[x]=CFVBBG;

Step 9  CLI>show service fname6=CFU;

- Record the service IDs

- For each service ID found, add the new feature CFVBBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME[x]=CFVBBG;

Step 10 CLI>show service fname7=CFU;

- Record the service IDs

- For each service ID found, add the new feature CFVBBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME[x]=CFVBBG;

Step 11  CLI>show service fname8=CFU;

- Record the service IDs

- For each service ID found, add the new feature CFVBBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME[x]=CFVBBG;

Step 12  CLI>show service fname9=CFU;

- Record the service IDs

- For each service ID found, add the new feature CFVBBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME[x]=CFVBBG;

Step 13  CLI>show service fname10=CFU;

- Record the service IDs

- For each service ID found, add the new feature CFVBBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME[x]=CFVBBG;

Step 14  CLI>show service fname1=CFUA;

- Record the service IDs

- For each service ID found, add the new feature CFVBBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME[x]=CFVABBG;

Step 15  CLI>show service fname2=CFUA;

- Record the service IDs

- For each service ID found, add the new feature CFVBBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME[x]=CFVABBG;

Step 16 CLI>show service fname3=CFUA;

- Record the service IDs

- For each service ID found, add the new feature CFVBBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME[x]=CFVABBG;

Step 17  CLI>show service fname4=CFUA;

- Record the service IDs

- For each service ID found, add the new feature CFVBBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME[x]=CFVABBG;

Step 18 CLI>show service fname5=CFUA;

- Record the service IDs

- For each service ID found, add the new feature CFVBBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME[x]=CFVABBG;

Step 19 CLI>show service fname6=CFUA;

- Record the service IDs

- For each service ID found, add the new feature CFVBBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME[x]=CFVABBG;

Step 20  CLI>show service fname7=CFUA;

- Record the service IDs

- For each service ID found, add the new feature CFVBBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME[x]=CFVABBG;

Step 21  CLI>show service fname8=CFUA;

- Record the service IDs

- For each service ID found, add the new feature CFVBBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME[x]=CFVABBG;

Step 22  CLI>show service fname9=CFUA;

- Record the service IDs

- For each service ID found, add the new feature CFVBBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME[x]=CFVABBG;

Step 23  CLI>show service fname10=CFUA;

- Record the service IDs

- For each service ID found, add the new feature CFVBBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME[x]=CFVABBG;

Step 24  CLI>show cdp fname=CFUA;

- Record all cdp IDs and respective DIGIT_STRING for the cdp record shown

- For each cdp ID found, change the record using the sample CLI command below:

CLI>change cdp id=[cdp id]; DIGIT_STRING=[DIGIT_STRING value recorded]; FNAME=CFVABBG

Step 25   CLI>exit