Document Number 	EDCS-441247

Revision		25.0

Cisco BTS 10200 Softswitch Software Upgrade for Release

3.5.4.Vxx to 4.5.0.Vxx

July 2, 2006

Corporate Headquarters

Cisco Systems, Inc.

170 West Tasman Drive

San Jose, CA 95134-1706

USA

http://www.cisco.com

Tel: 	408 526-4000

800 553-NETS (6387)

Fax:	408 526-4100

THE SPECIFICATIONS AND INFORMATION REGARDING THE PRODUCTS IN THIS MANUAL ARE SUBJECT TO CHANGE WITHOUT NOTICE. ALL STATEMENTS, INFORMATION, AND RECOMMENDATIONS IN THIS MANUAL ARE BELIEVED TO BE ACCURATE BUT ARE PRESENTED WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED. USERS MUST TAKE FULL RESPONSIBILITY FOR THEIR APPLICATION OF ANY PRODUCTS.

THE SOFTWARE LICENSE AND LIMITED WARRANTY FOR THE ACCOMPANYING PRODUCT ARE SET FORTH IN THE INFORMATION PACKET THAT SHIPPED WITH THE PRODUCT AND ARE INCORPORATED HEREIN BY THIS REFERENCE. IF YOU ARE UNABLE TO LOCATE THE SOFTWARE LICENSE OR LIMITED WARRANTY, CONTACT YOUR CISCO REPRESENTATIVE FOR A COPY.

The Cisco implementation of TCP header compression is an adaptation of a program developed by the University of California, Berkeley (UCB) as part of UCB’s public domain version of the UNIX operating system. All rights reserved. Copyright © 1981, Regents of the University of California.

NOTWITHSTANDING ANY OTHER WARRANTY HEREIN, ALL DOCUMENT FILES AND SOFTWARE OF THESE SUPPLIERS ARE PROVIDED “AS IS” WITH ALL FAULTS. CISCO AND THE ABOVE-NAMED SUPPLIERS DISCLAIM ALL WARRANTIES, EXPRESSED OR IMPLIED, INCLUDING, WITHOUT LIMITATION, THOSE OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT OR ARISING FROM A COURSE OF DEALING, USAGE, OR TRADE PRACTICE.

IN NO EVENT SHALL CISCO OR ITS SUPPLIERS BE LIABLE FOR ANY INDIRECT, SPECIAL, CONSEQUENTIAL, OR INCIDENTAL DAMAGES, INCLUDING, WITHOUT LIMITATION, LOST PROFITS OR LOSS OR DAMAGE TO DATA ARISING OUT OF THE USE OR INABILITY TO USE THIS MANUAL, EVEN IF CISCO OR ITS SUPPLIERS HAVE BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES.

CCIP, CCSP, the Cisco Arrow logo, the Cisco Powered Network mark, the Cisco Systems Verified logo, Cisco Unity, Follow Me Browsing, FormShare, iQ Breakthrough, iQ FastTrack, the iQ Logo, iQ Net Readiness Scorecard, Networking Academy, ScriptShare, SMARTnet, TransPath, and Voice LAN are trademarks of Cisco Systems, Inc.; Changing the Way We Work, Live, Play, and Learn, The Fastest Way to Increase Your Internet Quotient, and iQuick Study are service marks of Cisco Systems, Inc.; and Aironet, ASIST, BPX, Catalyst, CCDA, CCDP, CCIE, CCNA, CCNP, Cisco, the Cisco Certified Internetwork Expert logo, Cisco IOS, the Cisco IOS logo, Cisco Press, Cisco Systems, Cisco Systems Capital, the Cisco Systems logo, Empowering the Internet Generation, Enterprise/Solver, EtherChannel, EtherSwitch, Fast Step, GigaStack, Internet Quotient, IOS, IP/TV, iQ Expertise, LightStream, MGX, MICA, the Networkers logo, Network Registrar, Packet, PIX, Post-Routing, Pre-Routing, RateMUX, Registrar, SlideCast, StrataView Plus, Stratm, SwitchProbe, TeleRouter, and VCO are registered trademarks of Cisco Systems, Inc. and/or its affiliates in the U.S. and certain other countries.

All other trademarks mentioned in this document or Web site are the property of their respective owners. The use of the word partner does not imply a partnership relationship between Cisco and any other company. (0301R)

Cisco BTS 10200 Softswitch Software Upgrade

Copyright © 2005, Cisco Systems, Inc.

All rights reserved.

| Revision History |  |  |
| --- | --- | --- |
| Date | Version | Description |
| 4/6/2005 | 1.0 | - Initial Version |
| 4/15/2005 | 2.0 | Removed “forced“ from control commands once the system is in 4.5.0 |
| 5/5/2005 | 3.0 | - Corrected typo in Appendix T. |
| 5/17/2005 | 4.0 | - Incorporated comments resulted from the document review |
| 5/26/3005 | 5.0 | - Moved the announcement and new feature provisioning from mid-upgrade to prost-upgrade. Also, removed steps where SS7 CICs are being controlled out before making 4.5.0 active and steps for controlling them into service after 4.5.0 becoming active. |
| 6/6/2005 | 6.0 | - Revised Chapter 7, Task 3 for better instructions and Chapter 7, task 4 for reusing the old domain name with logical IPs |
| 6/9/2005 | 7.0 | - Added steps in Chapter 2 to prepare ITP. Removed task 3 and 4 in Chapter 7. |
| 6/27/2005 | 8.0 | - Moved Chapter 6, Task 1 to Task 3. |
| 6/28/2005 |  | - This procedure used 3.5.4.V01 and 4.5.0 plus patches for verification. |
| 6/29/2005 | 9.0 | - Remove redundant task 8 to sync db usage in Chapter 7 |
| 7/1/2005 | 10.0 | - Added pre-check for table SLE |
| 7/6/2005 | 11.0 | - Corrected Appendix R step 3. Change trunk-termination to trunk-grp |
| 7/7/2005 | 12.0 | - Added more details to the SLE table check. |
| 7/18/2005 | 13.0 | - Added Task 7 and Task 8 in Appendix H to handle BDMS PC CHP version mismatches |
| 7/25/2005 | 14.0 | Added new comments in the Chapter 2: Each customer must purchase 8 disks with matching disk size to the existing system that is to be upgraded. |
| 7/28/2005 | 15.0 | Add new parameters introduced in opticall.cfg in the 450 Q17 load: - CAxxx_LAF_PARAMETER - FSPTCxxx_LAF_PARAMETER - FSAINxxx_LAF_PARAMETER - EMS_LAF_PARAMETER - BDMS_LAF_PARAMETER |
| 8/19/2005 | 16.0 | - Added appendix X to enable disk mirroring after completing upgrade |
| 11/10/2005 | 17.0 | - Revised Appendix X disk mirroring steps. |
| 11/28/2005 | 18.0 | - Corrected typo on Page 32, Task 7, Step 3 and added Task 9 in Chapter 4. Added Task 1 in Chapter 2. |
| 12/06/2005 | 19.0 | - Added an “exit” step in Chapter 8, Task 6, Step 3. And added Task 2 in Chapter 6 for checking ntp.conf file peer information. Modified disk mirroring steps to complete the process One side at a time. |
| 1/6/2006 | 20.0 | - Added pre-upgrade step to make sure there are terminations associated with ss7 trunk groups. |
| 1/30/2006 | 21 | - Correct the error in the SQL statement for pre-checking the SS7 trunk groups. Added Appendix Y to bounce SS7 trunks After Primary CA/FS and EMS Side A becoming active and in release 3.5.4. |
| 2/20/2006 | 22 | - Corrected typo in Page 72, Step 17 and removed Chapter 8, Task 5. |
| 7/20/2006 | 23 | - Added check for named.conf with correct DNS for customers with named daemon enabled. This is mainly for none-cable customers. Added Step 16 in Chapter 6, task 6 & 7; and Step 10 named.conf FTP in Chapter 8, Task 3 & 4. |
| 8/2/2006 | 23 | - Added step 7 and 8 in Chapter 9, Task 3 to correct data mismatches in mgw and cas-tg-profile tables. |
| 09/05/2006 | 24 | - Added CLI instructions to Step 7, Chapter 4 to associate valid carrier ID’s with the destinations. |
| 07/02/2007 | 25 | - Added DIAL-PLAN-ID check on chapter 4 task 10 to resolve CSCsi09503 |

Table of Contents

Table of Contents	6

Table of Contents	6

Preface	15

Obtaining Documentation	15

World Wide Web	15

Documentation CD-ROM	15

Ordering Documentation	15

Documentation Feedback	16

Obtaining Technical Assistance	16

Cisco.com	16

Technical Assistance Center	17

Cisco TAC Web Site	17

Cisco TAC Escalation Center	18

Chapter 1	19

Upgrade Requirements	19

Introduction	19

Assumptions	20

Requirements	20

Important notes about this procedure	21

Chapter 2	23

Preparation	23

Complete 4-6 weeks before the scheduled upgrade	23

Task 1: Remove second Quad Ethernet Card from CA/FS machines	23

Task 2: Install signaling gateways and links	23

Task 3: Setting up MTP and M3UA configuration on the signaling gateway	24

Task 4: Create links and linkset between BTS and ITP	24

Task 5: Prepare ITP SCCP configuration command files	24

Task 6: Purchase and Prepare Disks	24

Prerequisites	25

Chapter 3	27

Complete 2-4 weeks before the scheduled upgrade	27

Task 1: Scripts needed during upgrade in order to complete the SS7 upgrade from OMNI to Sigtran	27

Task 2: Scripts for post-upgrade new feature activation	28

Chapter 4	29

Complete one week before the scheduled upgrade	29

Task 1: Add new domain names to DNS	29

Task 2: Pre-construct opticall.cfg for the system to be upgraded to 4.5.0 release	31

Task 3: Check RUDP_BACKHAUL_SESSION port number assignment	32

From Active EMS	32

Task 4: Check mlhg_terminal table	33

From Active EMS	33

Task 5: Check mgw table	33

From Active EMS	33

Task 6: Check Termination table	34

From Active EMS	34

Task 7: Check Destination table	34

From Active EMS	34

Task 8: Check SLE table	35

From Active EMS	35

Task 9: Check Trunk_grp table	35

From Active EMS	35

Task 10: Check DIAL_PLAN_ID Table	36

From Active EMS	36

Task 11: Check call-agent-profile CDB generation Flag	36

From Active EMS	36

Task 12: Change SS7-CIC table for SS7 Upgrade	37

Task 13: Save customized cron jobs	37

Chapter 5	38

Prepare System for Upgrade	38

Task 1: Change MGCP Domain IP addresses from Domain Name servers	38

Task 2: Verify System Status	38

Task 3: Check required billing information	39

From EMS Side A	39

Task 4: Backup Billing DB	39

From EMS Side A	40

Task 5: Backup user account	40

From EMS Side A	40

Chapter 6	41

Upgrade Side B Systems	41

Task 1: Disable Oracle DB replication	41

From EMS side A	41

From EMS side A	41

Task 2: Force side A systems to active	42

From Active EMS Side B	42

Task 3: Inhibit EMS mate communication	42

From EMS side A	43

Task 4: Stop applications and shutdown EMS side B	43

From EMS side B	43

Task 5: Stop applications and shutdown CA/FS side B	43

From CA/FS side B	44

Task 6: Upgrade EMS side B to the new release	44

From EMS side B	45

Task 7: Upgrade CA/FS Side B to the new release	50

From CA/FS side B	50

Task 8: Migrate oracle data	56

From EMS side B	56

Task 9: Restore billing address and billing alarm	57

From EMS side B	57

Task 10: To install CORBA on EMS side B, please follow Appendix I.	58

Chapter 7	59

Prepare Side A Systems for Upgrade	59

Task 1: Control SS7 trunk groups out of service on release 3.5.4	59

From EMS side A	59

Task 2: Control SS7 links out of service	59

From CA/FS side A	60

Task 3: Force side A systems to standby	60

From EMS side A	60

Task 4: Provision 4.5.0 SS7 configuration	61

From EMS side B	61

Task 5: Control SS7 trunk groups in service	61

From EMS side B	62

Task 6: Sync Data from EMS side B to CA/FS side B	62

From EMS side B	62

Task 7: Validate release 4.5.0 software operation	63

From EMS side B	63

From EMS side A	64

Chapter 8	65

Upgrade Side A Systems	65

Task 1: Stop applications and shutdown EMS side A	65

From EMS side A	65

Task 2: Stop applications and shutdown CA/FS side A	65

From CA/FS side A	65

Task 3: Upgrade EMS side A to the new release	66

From EMS side A	66

Task 4: Upgrade CA/FS Side A to the new release	70

From CA/FS side A	70

Task 5: Restore EMS mate communication	74

From EMS side B	75

Task 6: Copying oracle data	75

From EMS side A	75

Task 7: To install CORBA on EMS side A, please follow Appendix I.	76

Chapter 9	77

Finalizing Upgrade	77

Task 1: Switchover activity from side B to side A	77

From EMS side B	77

Task 2: Enable Oracle DB replication on EMS side B	77

From EMS side B	77

Task 3: Synchronize handset provisioning data and check both mgw and cas-tg-profile tables	78

From EMS side A	78

Task 4: Restore customized cron jobs	80

Task 5: Synchronize the hosts and opticall.cfg files	80

From Primary CA/FS	80

Task 6: Verify system status	80

Chapter 10	82

Post Upgrade Tasks	82

Task 1: Change MGCP Domain IP addresses from Domain Name Servers	82

Task 2: Correct ntp.conf File	82

From EMS side B	82

From EMS side A	83

Task 3: Remove BTS (3.5.4) linksets	84

Task 4: Provisioning new features	84

From EMS side A	84

Task 5: Restore platform.cfg with logical IPs	86

From EMS side A	86

Task 6: Enable disk mirroring	86

Appendix A	87

Check System Status	87

From Active EMS side A	87

Appendix B	90

Check Call Processing	90

From EMS side A	90

Appendix C	92

Check Provisioning and Database	92

From EMS side A	92

Check transaction queue	92

Perform database audit	93

Appendix D	94

Check Alarm Status	94

From EMS side A	94

Appendix E	96

Check Oracle Database Replication and Error Correction	96

Check Oracle DB replication status	96

From EMS side A	96

Correct replication error	97

From EMS Side B	97

From EMS Side A	97

Appendix F	99

Flash Archive Steps	99

Task 1: Ensure side A systems are ACTIVE	99

Task 2: Perform a full database audit	99

From EMS Side A	100

Task 3: Perform shared memory integrity check	100

From CA/FS side A	100

From CA/FS side B	101

Task 4: Perform flash archive on EMS side B	102

From EMS side B	102

Task 5: Perform flash archive on CA/FS side B	104

From CA/FS side B	104

Task 6: Switch activity from side A to side B	106

From EMS side A	106

Task 7: Perform flash archive on EMS side A	106

From EMS side A	107

Task 8: Perform flash archive on CA/FS side A	108

From CA/FS side A	109

Task 9: Restore the system to normal mode	110

From EMS side B	110

This completes the flash archive process.	111

Appendix G	112

Backout Procedure for Side B Systems	112

Introduction	112

Task 1: Remove SCCP configuration from ITPs	113

Task 2: Restore OMNI SS7 link(s) and linkset from ITP	113

Task 3: Control SS7 trunk groups out of service on release 4.5.0	114

From EMS side B	114

Task 4: Force side A CA/FS to active	115

From EMS side B	115

Task 5: SFTP billing records to a mediation device	115

From EMS side B	115

Task 6: Control SS7 links in service	116

From CA/FS side A	116

Task 7: Control SS7 trunk groups in service on release 3.5.4 side	116

From EMS side A	116

Task 8: Bounce SS7 trunks on release 3.5.4	117

From EMS side A	117

118

Task 9: Sync DB usage	118

From EMS side A	118

Task 10: Stop applications and shutdown side B systems	118

From EMS side B	118

From CA/FS side B	118

Task 11: Restore side B systems to the old release	119

From CA/FS side B	119

From EMS side B	120

Task 12: Restore EMS mate communication	120

From EMS side A	120

Task 13: Switchover activity to EMS side B	120

From Active EMS side A	121

Task 14: Enable Oracle DB replication on EMS side A	121

From EMS side A	121

Task 15: Synchronize handset provisioning data	122

From EMS side B	122

Task 16: Switchover activity from EMS side B to EMS side A	122

From EMS side B	122

Task 17: Restore system to normal mode	123

From EMS side A	123

Task 18: Verify system status	123

Task 19: Change MGCP Domain IP addresses from Domain Name Server	124

Appendix H	125

System Backout Procedure	125

Introduction	125

Task 1: Check MGCP Domain IP addresses from Domain Name Servers	125

Task 2: Disable Oracle DB replication on EMS side B	126

From Active EMS	126

From EMS side B	126

Task 3: Inhibit EMS mate communication	127

From EMS side B	127

Task 4: Force side B systems to active	127

From EMS side A	127

Task 5: SFTP billing records to a mediation device	128

From EMS side A	128

Task 6: Stop applications	128

From EMS side A	128

From CA/FS side A	128

Task 7: Restore side A systems to the old release	129

From CA/FS side A	129

From EMS side A	129

Task 8: Stop EMS side B billing application	130

From EMS side B	131

Task 9: Start EMS side A applications	131

From EMS side A	131

Task 10: To continue fallback process, please follow Appendix G.	131

Appendix I	132

CORBA Installation	132

Task 1: Open Unix Shell on EMS	132

Task 2: Install OpenORB CORBA Application	132

Remove Installed OpenORB Application	132

Install OpenORB Packages	133

Appendix J	135

Add New Announcements	135

Task 1: Record Announcements	135

Task 2: Place Announcements	135

From Active EMS	135

Appendix K	137

Associate Announcements	137

Task 1: Associate announcement with route-guide	137

From Active EMS	137

Task 2: Associate announcement with release causes	138

From Active EMS	138

Appendix L	139

Created New Features	139

From Active EMS	139

Task 1: Call Forwarding Unconditional (CFU)	139

Task 2: Call Forwarding Busy (CFB)	140

Task 3: Call Forward No-Answer (CFNA)	140

Task 4: Call Forward Unconditional Interrogation (CFUI)	141

Appendix M	142

Updating Existing Features	142

From EMS side A	142

Appendix N	147

Setting up MTP and M3UA Configuration on the Signaling Gateway	147

Requirements and Prerequisites	147

Preparation	147

Task 1: Define MTP Variant	148

Task 2: Define Point Code	148

Task 3: Define Ethernet Configuration	148

Task 4: Define SGMP (When SG Mated Pair is used)	149

Task 5: Define M3UA Port Number	149

Task 6: Define Application Server Process (ASP)	150

Task 7: Define SS7 Links	150

Task 8: Define Linkset	151

Task 9: Define SS7 Route Sets	152

Task 10: Define Routing Key	153

Appendix O	155

Setting up SCCP configuration on the Signaling Gateway	155

Task 1: Define SUA Port Number	155

Task 2: Define ASP	155

Task 3: Define Routing Keys for Various Services	156

Task 4: Define GTT Configuration	157

Task 5: Saving the Configuration	158

Appendix P	159

Preparing for the SS7 Upgrade	159

Requirements and Prerequisites	159

Preparation	159

Task 1: Define OPC and NET-AP in SS7-CIC Table	159

From CA/FS side A	159

From Active EMS	160

Appendix Q	161

Controlling SS7 Trunk Groups Out of Service	161

Task 1: Control SS7 Trunk Groups Out of Service	161

From Active EMS	161

Appendix R	162

Controlling SS7 Trunk Groups in service	162

Task 1: Control SS7 Trunk Groups in Service	162

From Active EMS	162

Appendix S	163

Provisioning Release 4.5.0 Specific Configuration on the Cisco BTS 10200 Call Agent	163

Requirements and Prerequisites	163

Preparation	163

Task 1: Define Signaling Gateway Components	163

From Active EMS	163

Task 2: Define SCTP Association	164

Task 3: Define ISUP Variant	164

Task 4: Define OPC	165

Task 5: Define Routing Key	165

Task 6: Define DPC	165

Task 7: Define Call Control Routes	166

Appendix T	167

Provisioning Release 4.5.0 Specific Configuration on the Cisco BTS 10200 Feature Server	167

Requirements and Prerequisites	167

Task 1: Define SCTP Association	167

Task 2: Add SCCP Network	168

Task 3: Add OPC in POP table	168

Task 4: Define CNAM Service	168

Task 5: Define LNP Service	169

Task 6: Define IN1 Toll Free Service	170

Task 7: Define AIN Toll Free Service	171

Task 8: Define AC and AR Services	172

Appendix U	175

Testing the SS7 Upgrade	175

Requirements and Prerequisites	175

Task 1: Check the Status of SCTP Associations	175

From Active EMS	175

Task 2: Check the Status of the Signaling Gateway Processes	176

Task 3: Check the Status of DPCs	176

Task 4: Check the Status of Trunk Terminations	176

Task 5: Check the Status of Subsystems	176

Task 6: Make a SS7 Call	177

Appendix V	178

D-Link Configuration	178

Appendix W	180

Preparing Disks for Upgrade	180

Task 1: Locate and label the disks	180

Label disks for EMS Side A	180

Label disks for EMS Side B	180

Label disks for CA/FS Side A	180

Label disks for CA/FS Side B	181

Task 2: Disk slot lay out	181

Task 3: Construct opticall.cfg	182

Task 4: Disk preparation	182

For both EMS side A and B	182

For both CA/FS side A and B	183

Appendix X	185

Disk Mirroring after Upgrade	185

Task 1: Configuring the Secondary EMS Side B	185

Task 2: Configuring the Secondary CA/FS Side B	185

Task 3: Switchover Applications to the Secondary Side	186

From Active EMS Side A	186

Task 4: Configuring the Primary EMS Side A	186

Task 5: Configuring the Primary CA/FS Side A	187

Task 6: Restore the System to Normal Mode	187

From Active EMS Side A	187

Appendix Y	189

Bounce SS7 Trunks	189

Task 1: Bounce SS7 Trunks	189

From Active EMS	189

Preface

# Obtaining Documentation

These sections explain how to obtain documentation from Cisco Systems.

### World Wide Web

You can access the most current Cisco documentation on the World Wide Web at this URL: http://www.cisco.com/

Translated documentation is available at this URL: http://www.cisco.com/public/countries_languages.shtml

### Documentation CD-ROM

Cisco documentation and additional literature are available in a Cisco Documentation CD-ROM package, which is shipped with your product. The Documentation CD-ROM is updated monthly and may be more current than printed documentation. The CD-ROM package is available as a single unit or through an annual subscription.

### Ordering Documentation

You can order Cisco documentation in these ways:

Registered Cisco.com users (Cisco direct customers) can order Cisco product documentation from the Networking Products MarketPlace: http://www.cisco.com/cgi-bin/order/order_root.pl

Registered Cisco.com users can order the Documentation CD-ROM through the online Subscription Store: http://www.cisco.com/go/subscription

Nonregistered Cisco.com users can order documentation through a local account representative by calling Cisco Systems Corporate Headquarters (California, U.S.A.) at 408 526-7208 or, elsewhere in North America, by calling 800 553-NETS (6387).

### Documentation Feedback

You can submit comments electronically on Cisco.com. In the Cisco Documentation home page, click the Fax or Email option in the “Leave Feedback” section at the bottom of the page.

You can e-mail your comments to mailto:bug-doc@cisco.com.

You can submit your comments by mail by using the response card behind the front cover of your document or by writing to the following address:

Cisco Systems, INC.

Attn: Document Resource Connection

170 West Tasman Drive

San Jose, CA 95134-9883

# Obtaining Technical Assistance

Cisco provides Cisco.com as a starting point for all technical assistance. Customers and partners can obtain online documentation, troubleshooting tips, and sample configurations from online tools by using the Cisco Technical Assistance Center (TAC) Web Site. Cisco.com registered users have complete access to the technical support resources on the Cisco TAC Web Site: http://www.cisco.com/tac

### Cisco.com

Cisco.com is the foundation of a suite of interactive, networked services that provides immediate, open access to Cisco information, networking solutions, services, programs, and resources at any time, from anywhere in the world.

Cisco.com is a highly integrated Internet application and a powerful, easy-to-use tool that provides a broad range of features and services to help you with these tasks:

Streamline business processes and improve productivity

Resolve technical issues with online support

Download and test software packages

Order Cisco learning materials and merchandise

Register for online skill assessment, training, and certification programs

If you want to obtain customized information and service, you can self-register on Cisco.com. To access Cisco.com, go to this URL: http://www.cisco.com/

### Technical Assistance Center

The Cisco Technical Assistance Center (TAC) is available to all customers who need technical assistance with a Cisco product, technology, or solution. Two levels of support are available: the Cisco TAC Web Site and the Cisco TAC Escalation Center.

Cisco TAC inquiries are categorized according to the urgency of the issue:

Priority level 4 (P4)—You need information or assistance concerning Cisco product capabilities, product installation, or basic product configuration.

Priority level 3 (P3)—Your network performance is degraded. Network functionality is noticeably impaired, but most business operations continue.

Priority level 2 (P2)—Your production network is severely degraded, affecting significant aspects of business operations. No workaround is available.

Priority level 1 (P1)—Your production network is down, and a critical impact to business operations will occur if service is not restored quickly. No workaround is available.

The Cisco TAC resource that you choose is based on the priority of the problem and the conditions of service contracts, when applicable.

### Cisco TAC Web Site

You can use the Cisco TAC Web Site to resolve P3 and P4 issues yourself, saving both cost and time. The site provides around-the-clock access to online tools, knowledge bases, and software. To access the Cisco TAC Web Site, go to this URL: http://www.cisco.com/tac

All customers, partners, and resellers who have a valid Cisco service contract have complete access to the technical support resources on the Cisco TAC Web Site. The Cisco TAC Web Site requires a Cisco.com Log in ID and password. If you have a valid service contract but do not have a Log in ID or password, go to this URL to register: http://www.cisco.com/register/

If you are a Cisco.com registered user, and you cannot resolve your technical issues by using the Cisco TAC Web Site, you can open a case online by using the TAC Case Open tool at this URL: http://www.cisco.com/tac/caseopen

If you have Internet access, we recommend that you open P3 and P4 cases through the Cisco TAC Web Site: http://www.cisco.com/tac

### Cisco TAC Escalation Center

The Cisco TAC Escalation Center addresses priority level 1 or priority level 2 issues. These classifications are assigned when severe network degradation significantly impacts business operations. When you contact the TAC Escalation Center with a P1 or P2 problem, a Cisco TAC engineer automatically opens a case.

To obtain a directory of toll-free Cisco TAC telephone numbers for your country, go to this URL: http://www.cisco.com/warp/public/687/Directory/DirTAC.shtml

Before calling, please check with your network operations center to determine the level of Cisco support services to which your company is entitled: for example, SMARTnet, SMARTnet Onsite, or Network Supported Accounts (NSA). When you call the center, please have available your service agreement number and your product serial number.

Chapter 1

Upgrade Requirements

# Introduction

Application software loads are designated as Release 900-aa.bb.cc.Vxx, where

- aa=major release number, for example, 01

- bb=minor release number, for example, 03

- cc=maintenance release, for example, 00

- Vxx=Version number, for example V04

This procedure can be used on an in-service system, but the steps must be followed as shown in this document in order to avoid traffic interruptions.

|  | Caution Performing the steps in this procedure will bring down and restart individual platforms in a specific sequence. Do not perform the steps out of sequence, as it could affect traffic. If you have questions, contact Cisco Support. |
| --- | --- |

This procedure should be performed during a maintenance window.

Note   In this document, the following designations are used:

- EMS -- Element Management System

- CA/FS -- Call Agent / Feature Server

- Primary -- Also referred to as "Side A"

- Secondary -- Also referred to as "Side B"

# Assumptions

The following assumptions are made.

- The installer has a basic understanding of UNIX and Oracle commands.

- The installer has the appropriate user name(s) and password(s) to log on to each EMS/CA/FS platform as root user, and as Command Line Interface (CLI) user on the EMS.

|  | Note Contact Cisco Support before you start if you have any questions. |
| --- | --- |

# Requirements

Verify that opticall.cfg has the correct information for each of the following machines.

- Side A EMS

- Side B EMS

- Side A CA/FS

- Side B CA/FS

Determine the oracle and root passwords for the systems you are upgrading. If you do not know these passwords, ask your system administrator.

Refer to local documentation to determine if CORBA installation is required on this system. If unsure, ask your system administrator.

# Important notes about this procedure

Throughout this procedure, each command is shown with the appropriate system prompt, followed by the command to be entered in bold. The prompt is generally one of the following:

- Host system prompt (<hostname>#)

- Oracle prompt (<hostname>$)

- SQL prompt (SQL>)

- CLI prompt (CLI>)

- SFTP prompt (sftp>)

Note the following conventions used throughout the steps in this procedure:

- Enter commands as shown, as they are case sensitive (except for CLI commands).

Note 1:  It is recommended that you read through the entire procedure before performing any steps.

Note 2:  To shorten the upgrade window, Cisco recommends the full database audit be performed at the night before the upgrade if it is absolutely certain that no provisioning activities will occur during the next 24 hour period.

Note 3 -- It will take approximately between 5-9 hours to complete the entire upgrade process depending on the number of subscribers provisioned in the system. Please plan accordingly to minimize any negative service impacts.

Note 4:  CDR delimiter customization is not retained after software upgrade. The customer or Cisco engineer must manually customize again to keep the same customization.

Note 5:  The total SS7 outage time depends on the number of SS7 CICs provisioned in the system. Based on data collected from lab testing, for a system with 10K CICs, the time it took from the blocking to unblocking and calls being made was 10 minutes.

If fallback is absolutely required, Force Primary side CA/FS and EMS to be active, restore OMNI SS7 links. Then control the trunk groups into service. Then control the trunks out of service, unequip, equip, in service. This will register the CICs to OMNI stack.

Note 6:  There will be no CLI provisioning allowed during entire upgrade process.

Chapter 2

Preparation

# Complete 4-6 weeks before the scheduled upgrade

# Task 1: Remove second Quad Ethernet Card from CA/FS machines

It is critical to have the extra quad card physically removed from each CA/FS machine. Otherwise, when the disk is swapped out with Solaris 10, the Ethernet port will be mis-assigned, which will require manual correction of the Ethernet port by editing the /etc/path_to_inst file.

# Task 2: Install signaling gateways and links

- Install required hardware --This requires advance planning to acquire necessary ITP hardware

- .

- ITP software requirement -- When upgrading to 4.5.0, the ITP Signaling Gateway must be running IOS software version 12.2(25)SW1 or later.

- It is required for all customers to have redundant ITP deployment. ITP Signaling Gateways are configured as STPs. The fully redundant SG Mated Pair is the only topology considered in this upgrade. The connection from SG Mated Pair to the SS7 Service Provider is via D-links.

- If a customer have a SS7 network topology different from the one stated above, please contact Cisco support for immediate assistance.

- Please follow steps in Appendix G for introducing ITPs to an existing SS7 network and SS7 transitional states.

# Task 3: Setting up MTP and M3UA configuration on the signaling gateway

Please follow steps specified in Appendix N for configuring ITP signaling gateway.

# Task 4: Create links and linkset between BTS and ITP

- Make ITP acts as an STP -- from ITP, create link(s) and linkset to BTS with OPC x.x.x. Sample IOS commands are given below for a BTS 10200 with an OPC 7.7.7; and ITP port 5 for SLC 0, port 6 for SLC 1.

cs7 linkset to_bts10200 7.7.7

description linkset to bts10200

link 0 Serial1/1/5:0

link 1 Serial1/1/6:0

no shut

# Task 5: Prepare ITP SCCP configuration command files

Please use the information provided in Appendix O to prepare for the detailed SCCP configuration set up command file. There are two files needed:

- Command file 1: for setting up the SCCP configuration. This is to be performed right before switching over BTS CA/FS applications from Primary Side A in release 3.5.4 to Secondary Side B in release 4.5.0.

- Command file 2: for removing the SCCP configuration. This is to be performed right before falling back BTS CA/FS applications from Secondary Side B in release 4.5.0 to Primary Side A in release 3.5.4.

# Task 6: Purchase and Prepare Disks

Each customer must purchase 8 disks with matching disk size to the existing system that is to be upgraded.

# Prerequisites

- Two sets of four disk drives jumpstarted with Solaris 10. Each disk must be prepared in a hardware platform that matches the target system to be upgraded.  Please refer to Appendix J to prepare each disk.

- Two disk drives for EMS side A -- the first disk is normally used for the upgrade and the second disk will be used only when the first disk goes bad. Each disk should already have:

- Pre-jumpstarted with Solaris 10 OS.

- Pre-staged with BTS 10200 Software Release 4.5.0

- Pre-installed EMS application software and databases

- Two disk drives for EMS side B -- the first disk is normally used for the upgrade and the second disk will be used only when the first disk goes bad. Each disk should already have:

- Pre-jumpstarted with Solaris 10 OS

- Pre-staged with BTS 10200 Software Release 4.5.0

- Pre-installed EMS application software and databases

- Two disk drives for CA/FS side A -- the first disk is normally used for the upgrade and the second disk will be used only when the first disk goes bad. Each disk should already have:

- Pre-jumpstarted with Solaris 10 OS

- Pre-staged with BTS 10200 Software Release 4.5.0

- Two disk drives for CA/FS side B -- the first disk is normally used for the upgrade and the second disk will be used only when the first disk goes bad. Each disk should already have:

  - Pre-jumpstarted with Solaris 10 OS

  - Pre-staged with BTS 10200 Software Release 4.5.0

- Locate CD-ROM disc labeled as BTSAUTO.tar

- Completed Network Information Data Sheets for release 4.5.0.

- There is secure shell (ssh) access to the Cisco BTS 10200 system.

- There is console access to the Cisco BTS 10200 system.

- Network interface migration from 9/5 to 4/2 has been completed. The interface migration procedure can be found in the following link: http://lbj/push_targets1/ucdit/cc/td/doc/product/voice/bts10200/rel_3_5/upgrade/353/mig95_42.doc

- Verify the target system to be upgraded has the latest 3.5.4 release deployed and the most recent patches applied if any. Please contact Cisco support if you are not sure what patch level the system is in.

- A Network File Server (NFS) accessible from the Cisco BTS 10200 system.

Chapter 3

# Complete 2-4 weeks before the scheduled upgrade

This chapter describes the tasks a user must complete one week before the scheduled upgrade.

Note:  All the scripts being run in this chapter and other chapters will display CLI session output and other output from other operations. These are only for information/logging purpose.

# Task 1: Scripts needed during upgrade in order to complete the SS7 upgrade from OMNI to Sigtran

- Prepare one script to control all SS7 trunk groups out of service – The trunk groups list must be prioritized so the critical ones are controlled out of service last to minimize the impact. This script can be used for both 3.5.4 and 4.5.0 releases. Please refer to Appendix Q for more details.

File Name: ________________________________________

- Prepare one script to control all SS7 trunk groups in service -- The trunk group list must be prioritized so the critical ones are controlled in service first to minimize the impact. This script can be used for both 3.5.4 and 4.5.0 releases. Please refer to Appendix R for more details.

File Name: ________________________________________

- Prepare one CLI script to migrate SS7 configuration from OMNI based (3.5.4) to ITP based (4.5.0) – The CLI scripts need to be generated from SS7 network information configured in OMNI.

- This script must not contain any control commands to bring up the trunk groups in service. Please refer to Appendix S and Appendix T for more details.

File Name 1: ________________________________________

File Name 2: ________________________________________

- Prepare one script to bounce release 3.5.4 SS7 trunks -- The trunk list must be prioritized so the critical ones are controlled out of service last and controlled in service first to minimize service outage. This script is used only when fallback is absolutely needed to fallback the system from 4.5.0 to 3.5.4 release. Please refer to Appendix Y for more details.

File Name: ________________________________________

# Task 2: Scripts for post-upgrade new feature activation

Cisco recommends that following new features to be activated only when the upgrade to release 4.5.0 is successful and no fallback will be needed. A separate maintenance window maybe required to activate the new features. This should be done within 1 week after completing the release software upgrade.

- Create new announcements and place them to the proper media gateways using Appendix J.

- Prepare CLI scripts for announcement association – A CLI script should be generated. This requires each customer to provide current announcement information. Please refer to Appendix K.

File Name: ________________________________________

- Prepare CLI scripts for feature upgrades -- A CLI script should be generated. This requires each customer to provide current feature set information. Please refer to Appendix L and Appendix M.

File Name 1: ________________________________________

File Name 2: ________________________________________

Chapter 4

# Complete one week before the scheduled upgrade

This chapter describes the tasks a user must complete one week before the scheduled upgrade.

# Task 1: Add new domain names to DNS

This task must be performed on Domain Name Servers that are serving the Cisco BTS 10200 system.

Step 1   Log in to Domain Name Servers for Cisco BTS 10200

Step 2   Add domain names for the following opticall.cfg parameters to Domain Name Server database where xxx – is the application instance number specific to the site.

- DNS_FOR_CA_SIDE_A_BLG_LINK_MONITOR

Note This is a qualified domain name used by a LHM process in Call Agents for monitoring network interface status used by billing. This name should return 2 IP addresses of Primary Call Agent.

- DNS_FOR_CA_SIDE_B_BLG_LINK_MONITOR

Note This is a qualified domain name used by a LHM process in Call Agents for monitoring network interface status used by billing. This name should return 2 IP addresses of Secondary Call Agent.

- DNS_FOR_CAxxx_H3A_COM

Note This is a qualified domain name used by a H3A process in Call Agents for communication to external devices. This name should return 2 Logical IP addresses. CAxxx – Installed instance for Call Agent

- DNS_FOR_CA_SIDE_A_SGA_COM

Note This is a qualified domain name used by a SGA process in Call Agents for communication to external devices (ITP). This name should return 2 IP addresses of Primary Call Agent.

- DNS_FOR_CA_SIDE_B_SGA_COM

Note This is a qualified domain name used by a SGA process in Call Agents for communication to external devices (ITP). This name should return 2 IP addresses of Secondary Call Agent.

- DNS_FOR_CAxxx_SIM_COM

Note This is a qualified DNS name used by a SIM process in Call Agents for communication to external devices. Each name resolves to two logical IP addresses in the same subnet as primary and secondary interfaces respectively. Each instance must have a unique DNS name and two uniquely assocaited LOGICAL IP addresses. For a simplex, use the host name for this parameter.

- DNS_FOR_FSAIN_SIDE_A_SGW_COM

Note This is a qualified domain name used by a TSA process in AIN Feature Server for communication to external devices (ITP). This name should return 2 IP addresses of Primary AIN Feature Server.

- DNS_FOR_FSAIN_SIDE_B_SGW_COM

Note This is a qualified domain name used by a TSA process in AIN Feature Server for communication to external devices (ITP). This name should return 2 IP addresses of Secondary AIN Feature Server.

- DNS_FOR_FSAINxxx_ASM_COM

Note This is a qualified DNS name used by the AIN process in Feature Server FSAIN. Each name should return two logical IP addresses of a AIN Feature Server which match the subnet of its two physical interfaces. For a simplex system, use host name for this parameter.

- DNS_FOR_FSPTC_SIDE_A_SGW_COM

Note This is a qualified domain name used by a TSA process in POTS Feature Server for communication to external devices (ITP). This name should return 2 IP addresses of Primary POTS Feature Server.

- DNS_FOR_FSPTC_SIDE_B_SGW_COM

Note This is a qualified domain name used by a TSA process in POTS Feature Server for communication to external devices (ITP). This name should return 2 IP addresses of Secondary POTS Feature Server.

- DNS_FOR_FSPTCzzz_GFS_COM

Note This is a qualified domain name used by GFS module of the POTS process in POTS Feature Server for communication to external devices. This name should return 2 Logical IP addresses. FSPTCzzz – Installed instance for POTS feature server

- DNS_FOR_FSPTCxxx_POTS_COM

Note This is a qualified DNS name used by the POTS process in Feature Server FSPTC. Each name should return two logical IP addresses of a POTS/CENTRIX Feature Server which match the subnet of its two physical interfaces. For a simplex system, use host name for this parameter.

# Task 2: Pre-construct opticall.cfg for the system to be upgraded to 4.5.0 release

Step 1   Get a copy of the completed Network Information Data Sheets (NIDS)

Step 2   Get a copy of the new opticall.cfg file for release 4.5.0 from Cisco

Step 3   Fill in value for each parameter defined in the opticall.cfg using data from Network Information Data Sheets and then place the file on the Network File Server (NFS).

Note   New parameters added to the 4.5.0 release:

Where xxx – is the application instance number specific to the site.

- NAMED_ENABLED

- MARKET_TYPE

- SS7_ENABLED

- IPSEC_ENABLED

- MEM_CFG_SELECTION

- SGW_OPTION

- NTP_SERVERS

- CAxxx_LAF_PARAMETER

- FSPTCxxx_LAF_PARAMETER

- FSAINxxx_LAF_PARAMETER

- EMS_LAF_PARAMETER

- BDMS_LAF_PARAMETER

- DNS_FOR_CA_SIDE_A_BLG_LINK_MONITOR

- DNS_FOR_CA_SIDE_B_BLG_LINK_MONITOR

- DNS_FOR_CAxxx_MGA_COM

Note This is a qualified domain name used by a MGA process in Call Agents for communication to Media Gateways. This domain name should return 2 logical IP addresses when system is upgraded. Please define the domain name value same as the parameter DNS_FOR_CA_MGCP_COM. The IP addresses for the domain name DNS_FOR_CA_MGCP_COM will be changed from 4 physical to 2 logical during the upgrade process.

- DNS_FOR_CAxxx_H3A_COM

- DNS_FOR_CAxxx_SIM_COM

- DNS_FOR_CA_SIDE_A_SGA_COM

- DNS_FOR_CA_SIDE_B_SGA_COM

- DNS_FOR_FSAINxxx_ASM_COM

- DNS_FOR_FSAIN_SIDE_A_SGW_COM

- DNS_FOR_FSAIN_SIDE_B_SGW_COM

- DNS_FOR_FSPTCxxx_POTS_COM

- DNS_FOR_FSPTC_SIDE_A_SGW_COM

- DNS_FOR_FSPTC_SIDE_B_SGW_COM

# Task 3: Check RUDP_BACKHAUL_SESSION port number assignment

# From Active EMS

Step 1   Log in as CLI user

Step 2   CLI> show rudp_backhaul_session;

- Make sure the value assigned to CALL_AGENT_BACKHAUL_PORT and MGW_BACKHAUL_PORT is within range 1024-9999.

- If the value for either field is out of range 1024-9999, please update fields with appropriate value.

Step 3   CLI> exit

# Task 4: Check mlhg_terminal table

# From Active EMS

Step 1   <hostname># su – oracle

Step 2    <hostname>$ sqlplus optiuser/optiuser

Step 3    sql> select term_id, mlhg_id, mgw_id from mlhg_terminal group by term_id, mlhg_id, mgw_id having count(*) > 1;

Please check:

- Check for duplicated records with TERM_ID, MLHG_ID, MGW_ID

- If there is any record shown from the above query, remove the duplicated records from CLI. Failure to do so will result in an upgrade failure.

# Task 5: Check mgw table

# From Active EMS

This task checks for any mgw records without associated terminations or mgw type with NULL value. If mgw is provisioned without termination, the upgrade data migration process will be able to determine the type of mgw. This will cause upgrade to fail. So, it is critical to pre-check the MGW table.

Step 1    sql> select id from mgw where id not in (select unique mgw_id from termination);

Please check:

- If there is any record shown from the above query, meaning there is no corresponding termination for a given mgw. Then the type of mgw is not known.

- You must first delete the MGW record from CLI. If the mgw is valid, then added the mgw back once the system is upgraded to release 4.5.0 from CLI. Failure to do so will result in an upgrade failure.

# Task 6: Check Termination table

Query termination table to identify records with wrong MGCP package type.

# From Active EMS

Step 1    sql> select id, mgw_id, tgn_id, mgcp_pkg_type from termination where tgn_id in (select tgn_id from (select unique tgn_id, mgcp_pkg_type from termination where tgn_id is not null) group by tgn_id having count(*) > 1);

Please check:

- MGCP package type (mgcp_pkg_type) should be the same for all the terminations (id) within the same trunk_group (tgn_id).

- If there is any record shown from the above query, meaning MGCP package type is inconsistent with trunk_group. You must make the correction from CLI. Failure to do so will result in an upgrade failure.

# Task 7: Check Destination table

Query destination table to identify invalid carrier id.

# From Active EMS

Step 1    sql> select dest_id, carrier_id from destination where carrier_id not in (select id from carrier);

Please check:

- If the above query returns any record, please replace the invalid carrier ID with a valid carrier ID from CLI using the steps given below. Failure to do so will result in an upgrade failure.

- If the above query does not return any records, proceed to the next task.

Step 2

- Log in to CLI

- CLI> show carrier

  - Make  a note of all the valid carrier ID’s.

- CLI>change destination dest_id=”xxxx”, carrier_id=”yyyy”

  - Associate the valid carrier ID’s with the destinations. If the carrier ID is not valid, change it to another valid carrier ID. Note that “xxxx” and “yyyy” should be replaced with valid carrier and destination ID’s.

- CLI> exit

# Task 8: Check SLE table

# From Active EMS

Step 1    sql> select fname from sle where fname not in

('SCA','SCR','SCF','DRCW','NSA');

Please check:

- If the above query returns any record, you either have to delete the sle record or upgrade the sle record with a valid fname from CLI. Failure to do so will result in an upgrade failure.

# Task 9: Check Trunk_grp table

Query trunk_grp table to identify the trunk groups without associated terminations.

# From Active EMS

Step 1    sql> select id from trunk_grp where tg_type=’SS7’ and not exists (select 1 from termination where tgn_id=trunk_grp.id);

Please check:

- For any trunk group record shown above, you have to either delete the trunk-grp record or assign a proper termination. Failure to do so will result in total loss of SS7 call traffic.

# Task 10: Check DIAL_PLAN_ID Table

# From Active EMS

Step 1    SQL> col direction for a9;

Step 2    SQL> col TG_TYPE for a7;

Step 3    SQL> select id,dial_plan_id,tg_type,direction,main_sub_id from trunk_grp where TG_TYPE != 'ANNC' and DIRECTION != 'OUT'and MAIN_SUB_ID is null and dial_plan_id is null;

- If the above query returns a result with any row, then use CLI command to assign the missing DIAL_PLAN_ID. Failure to do so will result in an upgrade failure.

For example:

ID          DIAL_PLAN_ID         TG_TYPE     DIRECTION     MAIN_SUB_ID
-------------     -----------------------          --------------     -----------------     ----------------------80033                                          SOFTSW         BOTH

CLI example to fix> change trunk-group; ID=80033; dial-plan-id=<some entry>;

Step 2   sql> exit

Step 3   <hostname>$ exit

# Task 11: Check call-agent-profile CDB generation Flag

# From Active EMS

Step 1   Check billing flag in the call-agent-profile table

- Log in to CLI

- CLI> show call-agent-profile

  - If there is no record shown, you mush add a new record with “cdb-billing-supp” flag set to “y”.

  - If there is a record and the record shown with “cdb-billing-supp” flag set to “n”, you must change the value to “y.”

- CLI> exit

# Task 12: Change SS7-CIC table for SS7 Upgrade

Please execute steps specified in Appendix P. If this step is missed, SS7 call failure will result.

# Task 13: Save customized cron jobs

This upgrade process requires disk replacement. Because of this, all customized cron jobs in the system will be lost. Please save the cron jobs to your network file servers to be restored once the entire system is upgraded to the 4.5.0 release.

Chapter 5

Prepare System for Upgrade

Suspend all CLI provisioning activity during the entire upgrade process.

This chapter describes the steps a user must complete the morning or the night before the scheduled upgrade.

# Task 1: Change MGCP Domain IP addresses from Domain Name servers

From each Domain Name Server that is serving the BTS 10200 to be upgraded, change the IP addresses of MGCP domain name from 4 physical IPs (2 IPs from Primary and 2 IPs from secondary: 2P + 2S) to 2 physical plus 2 logical (2 IPs from primary plus 2 logical IPs:  2P + 2L).  So the IP for the mgcp domain name will be: 2P + 2S  2P + 2L.

# Task 2: Verify System Status

Step 1   Verify that the side A systems are in the active state. Use Appendix A for this procedure.

Step 2   Verify that call processing is working without error. Use Appendix B for this procedure.

Step 3   Verify that provisioning is operational from CLI command line, and verify database. Use Appendix C for this procedure.

Step 4   Verify that there are no outstanding major or critical alarms. Use Appendix D for this procedure.

Step 5   Use Appendix E to verify that Oracle database and replication functions are working properly.

|  | Caution Do not continue until the above verifications have been made. Call Cisco Support if you need assistance. |
| --- | --- |

# Task 3: Check required billing information

# From EMS Side A

Step 1   Log in as root.

Step 2   Log in as CLI user

Step 3   CLI> show billing-acct-addr;

- Verify following fields are defined:

  - BILLINGSERVERDIRECTORY

  - BILLINGSERVERADDRESS

  - USERNAME

- If not, please use following sample CLI change command to add information for the above two fields:

CLI> change billing-acct-addr billing-server-directory=<target directory name>; billing-server-addr=<server qualified domain name>; user-name=<user name>; password=<pass word>;

Step 4   CLI> exit

|  | Warning Don’t continue without above fields being defined. |
| --- | --- |

# Task 4: Backup Billing DB

The billing records saved in this task is to be used to convert billing records from Mysql to oracle once side B ems is upgrade to 4.5.0 release. Billing information is kept in Oracle DB in 4.5.0.

# From EMS Side A

Step 1   Log in as root

Step 2   <hostname># mkdir –p /opt/.upgrade

Step 3   <hostname># cd /opt/BTSmysql/bin

Step 4   <hostname># mysqldump --add-drop-table -u root -pmc68000 BILLING BillingAcctAddr > /opt/.upgrade/BillingAcctAddr.sql

Step 5   <hostname># mysqldump --add-drop-table -u root -pmc68000 BILLING BillingAlarm > /opt/.upgrade/BillingAlarm.sql

# Task 5: Backup user account

The user accounts saved in this task is to be restored to side B EMS once it is upgraded to 4.5.0 release.

# From EMS Side A

Step 1   Log in as root

Step 2   Save the /opt/ems/users directory:

<hostname># tar -cvf /opt/.upgrade/users.tar /opt/ems/users

Chapter 6

Upgrade Side B Systems

# Task 1: Disable Oracle DB replication

# From EMS side A

Step 1   Log in to Active EMS as CLI user.

Step 2   CLI> control bdms id=BDMS01; target-state=forced-standby-active;

Step 3   CLI> control element-manager id=EM01; target-state=forced-standby-active;

Step 4    CLI session will terminate when application platform switchover is complete.

# From EMS side A

Note Make sure there is no CLI session established before executing following steps.

Step 1   Log in as Oracle user:

<hostname># su – oracle

<hostname>$ cd /opt/oracle/admin/utl

Step 2   Load Oracle package.

<hostname>$ rep_toggle –s optical1 –t load_pkg

Answer “y” when prompt

Step 3   Add the toggle function:

<hostname>$ rep_toggle –s optical1 –t add_toggle

Answer “y” when prompt

Step 4   Set Oracle DB to simplex mode:

<hostname>$ rep_toggle –s optical1 –t set_simplex

Answer “y” when prompt

Answer “y” again when prompt

Step 5   <hostname>$ exit

Step 6   Restart applications to release Oracle connections:

<hostname># platform stop all

<hostname># platform start

# Task 2: Force side A systems to active

This procedure will force the side A systems to remain active.

|  | Note In the commands below, "xxx", "yyy" or "zzz" is the instance for the process on your system. |
| --- | --- |

# From Active EMS Side B

Step 1   Log in to Active EMS as CLI user.

Step 2   CLI> control bdms id=BDMS01; target-state=forced-active-standby;

Step 3   CLI> control element-manager id=EM01; target-state=forced-active-standby;

# Task 3: Inhibit EMS mate communication

In this task, you will isolate the OMS Hub on EMS side A from talking to EMS side B.

# From EMS side A

Step 1   Log in as root

Step 2   <hostname># /opt/ems/utils/updMgr.sh –split_hub

Step 3   <hostname># nodestat

- Verify there is no HUB communication from EMS side A to CA/FS side B.

- Verify OMS Hub mate port status: No communication between EMS

# Task 4: Stop applications and shutdown EMS side B

# From EMS side B

Step 1   Log in as root

Step 2   Record the IP address and netmask for the management interface of the system.

- For an example, if the “hme0” is used for management interface, then execute the following command:

<hostname># ifconfig hme0

- Record the IP address and netmask for the interface to be used in the next task.

IP: _____________ Netmask: ____________ Interface Name: ___________

Step 3   <hostname># mv /etc/rc3.d/S99platform /etc/rc3.d/_S99platform

Step 4   <hostname># platform stop all

Step 5   <hostname># sync; sync

Step 6   <hostname># shutdown –i5 –g0 –y

# Task 5: Stop applications and shutdown CA/FS side B

# From CA/FS side B

Step 1   Log in as root

Step 2   Record the IP address and netmask for the management interface of the system.

- For an example, if the “hme0” is used for management interface, then execute the following command:

<hostname># ifconfig hme0

- Record the IP address and netmask for the interface to be used in the next task.

IP: _____________ Netmask: ____________ Interface Name: ___________

Step 3   <hostname># mv /etc/rc3.d/S99platform /etc/rc3.d/_S99platform

Step 4   Deactivate SS7 Links on CA/FS side B.

- <hostname># termhandler -node a7n1

- OMNI [date] #1:deact-slk:slk=<link on CA/FS side B >;

- Enter y to continue.

- Repeat above 2 steps for each active link

- OMNI [date] #2:display-slk;

- Enter y to continue

- Verify that the state of each link is INACTIVE.

- OMNI[date]#3:quit

Step 5   <hostname># platform stop all

Step 6    <hostname># sync; sync

Step 7   <hostname># shutdown –i5 –g0 –y

# Task 6: Upgrade EMS side B to the new release

|  | Warning Do not proceed if you don’t have a pre-constructed opticall.cfg file for the system. The opticall.cfg file should already be constructed in Chapter 4, Task 2. |
| --- | --- |

# From EMS side B

Step 1   Power off the machine

Step 2   Remove disk0 from slot 0 off the machine and label it as “Release 3.5.4 EMS side B disk0”

- SunFire V120 disk slot lay out:

| CD-ROM | Disk 0 | Disk 1 |
| --- | --- | --- |

- SunFire V240 disk slot lay out:

| Disk 2 | Disk 3 |  |
| --- | --- | --- |
| Disk 0 | Disk 1 | DVD-ROM |

- SunFire V440 disk slot lay out:

| Disk 3 | DVD-ROM |
| --- | --- |
| Disk 2 |  |
| Disk 1 |  |
| Disk 0 |  |

- Netra 1280 disk slot lay out:

|  | DVD-ROM |
| --- | --- |
|  | Disk 1 |
|  | Disk 0 |

- Netra 20 disk slot lay out:

| D I S K 0 | D I S K 1 |  | DVD ROM |
| --- | --- | --- | --- |

- Continuous Hardware disk slot lay out:

| CD-ROM | Disk 0 |  |
| --- | --- | --- |
|  | Disk 1 |  |

Step 3   Place new disk labeled as “Release 4.5.0 EMS side B disk0, 1” in slot 0

Step 4   Power on the machine and allow the system to boot up by monitoring the boot process thru console.

- For Netra 1280 machine, please execute the following command from console:

<lom>poweron

- For other type of hardware, please use the power button to turn on the power.

Step 5   Log in as root thru console, use password “opticall”

Step 6   Rebuild network interface hardware configuration on disk

<hostname># cp –p /etc/path_to_inst /etc/path_to_inst.save

<hostname># egrep -iv “qfe|ce|eri|bge|hme|pci_pci” /etc/path_to_inst.save > /etc/path_to_inst

Step 7   Rebuild the hardware configuration

<hostname># reboot -- -r

- Wait for the system to boot up. Then log in as root.

- Verify the Ethernet port assignment in the /etc/path_to_inst file.

Step 8   Restore interfaces:

- <hostname># ifconfig <primary interface name> plumb

  - Use Interface Name recorded in Chapter 3, Task 3

- <hostname># ifconfig <primary interface name> <Interface IP> netmask <NETMASK> broadcast + up

  - Use IP and NETMASK recorded in Chapter 3, Task 3 for the interface

- Add static routes to reach Domain Name Server and Network File Server:

  - <hostname># echo “route add -net <network to reach DNS> <gateway IP>” >> /etc/rc3.d/S96StaticRoutes

  - <hostname># /etc/rc3.d/S96StaticRoutes

Example:

  - If the primary network manager interface for the machine is CE0, run “ifconfig ce0”, you should have following output:

ce0: flags=1000843<UP,BROADCAST,RUNNING,MULTICAST,IPv4> mtu 1500 index 2 inet 10.89.183.148 netmask ffffff00 broadcast 10.89.183.255 ether 0:3:ba:9d:68:85

  - If the DNS server is on network “10.89.0.0”, then add gateway 10.89.183.254 to reach that network with following command:

<hostname># echo “route add -net 10.89.0.0 10.89.183.254” >> /etc/rc3.d/S96StaticRoutes

<hostname># /etc/rc3.d/S96StaticRoutes

Step 9   Reset ssh keys:

<hostname># \rm /.ssh/known_hosts

Step 10   sftp the opticall.cfg file from Network File Server (opticall.cfg was constructed in Chapter 3, Task 2) and place it under /etc directory.

Step 11   sftp resolv.conf file from Primary EMS Side A and place it under /etc directory.

<hostname># sftp <Side A EMS IP>

sftp> get /etc/resolv.conf /etc/resolv.conf

sftp> exit

Step 12   Run script program to replace the hostname

<hostname># cd /opt/ems/upgrade

<hostname># DoTheChange -s

- The system will reboot when the script DoTheChange completes its run

Step 13   Wait for the system to boot up. Then log in as root.

Step 14   Editing /etc/default/init:

<hostname># vi /etc/default/init

- Remove lines and keep only the following lines:

#

TZ=US/Central

CMASK=022

For an example:

The original /etc/default/init file before line removal:

# @(#)init.dfl 1.5 99/05/26

#

# This file is /etc/default/init.  /etc/TIMEZONE is a symlink to this file.

# This file looks like a shell script, but it is not.  To maintain

# compatibility with old versions of /etc/TIMEZONE, some shell constructs

# (i.e., export commands) are allowed in this file, but are ignored.

#

# Lines of this file should be of the form VAR=value, where VAR is one of

# TZ, LANG, CMASK, or any of the LC_* environment variables.

#

TZ=US/Central

CMASK=022

LC_COLLATE=en_US.ISO8859-1

LC_CTYPE=en_US.ISO8859-1

LC_MESSAGES=C

LC_MONETARY=en_US.ISO8859-1

LC_NUMERIC=en_US.ISO8859-1

LC_TIME=en_US.ISO8859-1

The /etc/default/init file after line removal:

# @(#)init.dfl 1.5 99/05/26

#

# This file is /etc/default/init.  /etc/TIMEZONE is a symlink to this file.

# This file looks like a shell script, but it is not.  To maintain

# compatibility with old versions of /etc/TIMEZONE, some shell constructs

# (i.e., export commands) are allowed in this file, but are ignored.

#

# Lines of this file should be of the form VAR=value, where VAR is one of

# TZ, LANG, CMASK, or any of the LC_* environment variables.

#

TZ=US/Central

CMASK=022

Step 15   Verify interface hardware configuration match to the host configuration:

<hostname># egrep –i “qfe|ce|eri|bge|hme” /etc/path_to_inst

<hostname># ls -l /etc/hostname.*

- If the interface names match from the above two outputs, please continue on Step 16.

- If the interface names do NOT match, please match them by changing the postfix of hostname.*.

For an example:

Output from “egrep –i “qfe|ce|eri|bge|hme” /etc/path_to_inst” is:

"/pci@1f,4000/network@1,1" 0 "hme"

"/pci@1f,4000/pci@4/SUNW,qfe@0,1" 0 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@1,1" 1 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@2,1" 2 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@3,1" 3 "qfe"

Output from “ls -l /etc/hostname.*” is:

-rw-r--r--   1 root     other          14 May 16 16:03 /etc/hostname.hme0

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.hme0:1

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.eri0

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.eri0:1

After change, the output should be:

-rw-r--r--   1 root     other         14 May 16 16:03 /etc/hostname.hme0

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.hme0:1

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.qfe0

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.qfe0:1

Step 16   Correct DNS server IP address in the named daemon configuration file. Please use the name.conf file on EMS side A as reference.

<hostname># vi /etc/named.conf

- Correct the IP of each DNS server that is serving the BTS system

Step 17   Reboot the machine to pick up new TIMEZONE and interface setting:

<hostname># sync; sync; reboot

- Wait for the system to boot up. Then log in as root.

Step 18   <hostname># /opt/ems/utils/updMgr.sh –split_hub

Step 19   <hostname># svcadm disable system/cron

Step 20    CDR delimiter customization is not retained after software upgrade. If this system has been customized, either the Customer or Cisco Support Engineer must manually customize again to keep the same customization.

- <hostname># cd /opt/bdms/bin

- <hostname># vi platform.cfg

- Find the section for the command argument list for the BMG process

- Customize the CDR delimiters in the “Args=” line

- Example:

Args=-port 15260 -h localhost -u optiuser -p optiuser -fmt default_formatter -UpdIntvl 3300 -ems_local_dn blg-aSYS14EMS.cisco.com -FD semicolon -RD verticalbar

Step 21   <hostname># platform start –i oracle

Step 22   Log in as Oracle user.

<hostname># su – oracle

<hostname>$ cd /opt/oracle/admin/utl

Step 23   Set Oracle DB to simplex mode:

<hostname>$ rep_toggle –s optical2 –t set_simplex

- Answer “y” when prompt

- Answer “y” again when prompt

# Task 7: Upgrade CA/FS Side B to the new release

|  | Warning Do not proceed if you don’t have a pre-constructed opticall.cfg file for the system. The opticall.cfg file should already be constructed in Chapter 4, Task 2. |
| --- | --- |

# From CA/FS side B

Step 1   Power off the machine

Step 2   Remove disk0 from slot 0 off the machine and label it as “Release 3.5.4 CA/FS side B disk0”

- SunFire V120 disk slot lay out:

| CD-ROM | Disk 0 | Disk 1 |
| --- | --- | --- |

- SunFire V240 disk slot lay out:

| Disk 2 | Disk 3 |  |
| --- | --- | --- |
| Disk 0 | Disk 1 | DVD-ROM |

- SunFire V440 disk slot lay out:

| Disk 3 | DVD-ROM |
| --- | --- |
| Disk 2 |  |
| Disk 1 |  |
| Disk 0 |  |

- Netra 1280 disk slot lay out:

|  | DVD-ROM |
| --- | --- |
|  | Disk 1 |
|  | Disk 0 |

- Netra 20 disk slot lay out:

| D I S K 0 | D I S K 1 |  | DVD ROM |
| --- | --- | --- | --- |

- Continuous Hardware disk slot lay out:

| CD-ROM | Disk 0 | Disk 2 |
| --- | --- | --- |
|  | Disk 1 | Disk 3 |

Step 3   Place new disk labeled as “Release 4.5.0 CA/FS side B disk0, 1” in slot 0

Step 4   Power on the machine and allow the system to boot up by monitoring the boot process through console.

- For Netra 1280 machine, please execute the following command from console:

<lom>poweron

- For other type of hardware, please use the power button to turn on the power.

Step 5   Log in as root from console.

Step 6   Rebuild network interface hardware configuration on disk

<hostname># cp –p /etc/path_to_inst /etc/path_to_inst.save

<hostname># egrep -iv “qfe|ce|eri|bge|hme|pci_pci” /etc/path_to_inst.save > /etc/path_to_inst

Step 7   Rebuild the hardware configuration

<hostname># reboot -- -r

- Wait for the system to boot up. Then log in as root.

- Verify the Ethernet port assignment in the /etc/path_to_inst file.

Step 8   Restore interfaces:

- <hostname># ifconfig <primary interface name> plumb

  - Use Interface Name recorded in “Chapter 6, Task 5”

- <hostname># ifconfig <primary interface name> <Interface IP> netmask <NETMASK> broadcast + up

  - Use IP and NETMASK recorded in “Chapter 6, Task 5”

- Add static routes to reach Domain Name Server and Network File Server using “route add …” command:

  - Example: route add -net 10.89.224.1 10.89.232.254

Where: 10.89.224.1 is the destination DNS server IP
              10.89.232.254 is the gateway IP

Step 9   Reset ssh keys:

<hostname># \rm /.ssh/known_hosts

Step 10   sftp the opticall.cfg file from Network File Server (opticall.cfg was constructed in Chapter 4, Task 2) and place it under /etc directory.

Step 11   sftp resolv.conf file from Primary CA/FS Side A and place it under /etc directory.

<hostname># sftp <Side A CA/FS IP>

sftp> get /etc/resolv.conf /etc/resolv.conf

sftp> exit

Step 12   Run script program to replace the hostname

<hostname># cd /opt/ems/upgrade

<hostname># DoTheChange -s

- The system will reboot when the script DoTheChange completes its run

Step 13    Wait for the system to boot up. Then log in as root.

Step 14   Editing /etc/default/init:

<hostname># vi /etc/default/init

- Remove lines and keep only the following lines:

#

TZ=US/Central

CMASK=022

For an example:

The original /etc/default/init file before line removal:

# @(#)init.dfl 1.5 99/05/26

#

# This file is /etc/default/init.  /etc/TIMEZONE is a symlink to this file.

# This file looks like a shell script, but it is not.  To maintain

# compatibility with old versions of /etc/TIMEZONE, some shell constructs

# (i.e., export commands) are allowed in this file, but are ignored.

#

# Lines of this file should be of the form VAR=value, where VAR is one of

# TZ, LANG, CMASK, or any of the LC_* environment variables.

#

TZ=US/Central

CMASK=022

LC_COLLATE=en_US.ISO8859-1

LC_CTYPE=en_US.ISO8859-1

LC_MESSAGES=C

LC_MONETARY=en_US.ISO8859-1

LC_NUMERIC=en_US.ISO8859-1

LC_TIME=en_US.ISO8859-1

The /etc/default/init file after line removal:

# @(#)init.dfl 1.5 99/05/26

#

# This file is /etc/default/init.  /etc/TIMEZONE is a symlink to this file.

# This file looks like a shell script, but it is not.  To maintain

# compatibility with old versions of /etc/TIMEZONE, some shell constructs

# (i.e., export commands) are allowed in this file, but are ignored.

#

# Lines of this file should be of the form VAR=value, where VAR is one of

# TZ, LANG, CMASK, or any of the LC_* environment variables.

#

TZ=US/Central

CMASK=022

Step 15   Verify interface hardware configuration match to the host configuration:

<hostname># egrep –i “qfe|ce|eri|bge|hme” /etc/path_to_inst

<hostname># ls –l /etc/hostname.*

- If the interface names match from the above two outputs, please continue on Step 16.

- If the interface names do NOT match, please match them by changing the postfix of hostname.*.

For an example:

Output from “egrep –i “qfe|ce|eri|bge|hme” /etc/path_to_inst” is:

"/pci@1f,4000/network@1,1" 0 "hme"

"/pci@1f,4000/pci@4/SUNW,qfe@0,1" 0 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@1,1" 1 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@2,1" 2 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@3,1" 3 "qfe"

"/pci@1f,2000/pci@1/SUNW,qfe@0,1" 4 "qfe"

"/pci@1f,2000/pci@1/SUNW,qfe@1,1" 5 "qfe"

"/pci@1f,2000/pci@1/SUNW,qfe@2,1" 6 "qfe"

"/pci@1f,2000/pci@1/SUNW,qfe@3,1" 7 "qfe"

Output from “ls -l /etc/hostname.*” is:

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.hme0

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.eri0

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.eri1

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.eri1:1

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.eri1:2

-rw-r--r--   1 root     other         12 Jun 10 11:25 hostname.eri1:3

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.eri2

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.eri2:1

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.eri2:2

-rw-r--r--   1 root     other         12 Jun 10 11:25 hostname.eri2:3

After change, the output should be:

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.hme0

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.qfe0

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.qfe1

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.qfe1:1

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.qfe1:2

-rw-r--r--   1 root     other         12 Jun 10 11:25 hostname.qfe1:3

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.qfe2

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.qfe2:1

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.qfe2:2

-rw-r--r--   1 root     other         12 Jun 10 11:25 hostname.qfe2:3

Step 16   Correct DNS server IP address in the named daemon configuration file. Please use the name.conf file on EMS side A as reference.

<hostname># vi /etc/named.conf

- Correct the IP of each DNS server that is serving the BTS system

Step 17   Reboot the machine to pick up new TIMEZONE and interface setting:

<hostname># sync; sync; reboot

- Wait for the system to boot up. Then log in as root.

Step 18   Check for configuration errors

<hostname># cd /opt/Build

<hostname># checkCFG –u

- Correct errors generated by checkCFG

- Once the result is clean without errors, then proceed to the next step.

Step 19   Change the IPs for MGCP domain name from 4 physical IPs (2 physical IPs from primary CA/FS: 2P, and 2 physical IPs from secondary CA/FS: 2S) to two logical IPs: 2P + 2S  2L:

- <hostname># vi /etc/hosts

- Delete 2 physical IPs came from primary CA/FS.

- Delete 2 physical IPs came from secondary CA/FS.

- Add 2 new logical IPs for the MGCP domain name in release 4.5.0.

- Check to make sure there are only 2 logical IP addresses left for mgcp domain name

Step 20   <hostname># install.sh -upgrade

Step 21   Answer "y" when prompted. This process will take up to 15 minutes to complete.

- Answer "y" if prompted for reboot

- Wait for the system to boot up. Then log in as root.

- Edit /etc/hosts file to make sure there are only 2 logical IP addresses left for mgcp domain name

Step 22   Update CA/FS platform.cfg files:

<hostname># cd /opt/Build/btsauto

<hostname># update_platform.sh

Step 23   <hostname># platform start

Step 24   <hostname># mv /etc/rc3.d/_S99platform /etc/rc3.d/S99platform

# Task 8: Migrate oracle data

# From EMS side B

Step 1   Copying data.

<hostname>$ su - oracle

<hostname>$ cd /opt/oracle/admin/upd

<hostname>$ java dba.dmt.DMMgr -upgrade -auto ./config/3.5.4_to_4.5.0.cfg

Step 2   Verify the data migration is a success.

<hostname>$ echo $?

- Verify the returned value is 0.

- If not, please sftp /opt/oracle/admin/upd/DMMgr.log file off system and call for immediate technical assistance.

Step 3   <hostname>$ java dba.adm.DBUsage -sync

- Verify Number of tables “unable-to-sync” is 0

- If not, please contract Cisco support

Step 4   <hostname>$ cd /opt/oracle/opticall/create

Step 5   <hostname>$ dbinstall optical2 -load dbsize

Step 6   <hostname>$ exit

Step 7 <hostname># platform start

Step 8 <hostname># svcadm enable system/cron

Step 9 <hostname># mv /etc/rc3.d/_S99platform /etc/rc3.d/S99platform

# Task 9: Restore billing address and billing alarm

# From EMS side B

Step 1   Log in as root

Step 2   <hostname># mkdir –p /opt/.upgrade

Step 3   <hostname># cd /opt/.upgrade

Step 4   Get old billing data from EMS side A machine:

- <hostname># sftp <EMS side A hostname>

- sftp> cd /opt/.upgrade

- sftp> mget *

- sftp> exit

Step 5   <hostname># cd /opt/ems/utils

Step 6   <hostname># upgrade_billing

Step 7   <hostname># su - oracle

Step 8   <hostname>$ sqlplus optiuser/optiuser < /opt/.upgrade/upgrade_billing.sql

Step 9   <hostname>$ exit

# Task 10: To install CORBA on EMS side B, please follow Appendix I.

Chapter 7

Prepare Side A Systems for Upgrade

# Task 1: Control SS7 trunk groups out of service on release 3.5.4

# From EMS side A

Step 1   If there is NO CLI script prepared, please follow steps specified in Appendix Q and then continue on Step 3.

Step 2   If there are CLI scripts prepared, log in to the Network File Server where the pre-prepared script resides. SFTP the script to EMS side A log in as CLI user and put script under /opt/ems/ftp/deposit directory.

- Wait for the batch process to pick up the script from /opt/ems/ftp/deposit directory and provision it into the system.

- Keep running “ls” command from /opt/ems/ftp/deposit directory and till the file is gone

- <hostname># cd /opt/ems/report

- <hostname># egrep Failed:0 <filename>.html

- Verify the system returns an output. If there is no output returned, please check for errors in the report file.

Step 3    Check the status of each SS7 trunk group – the admin state should be out of service:

- Log in as CLI user to EMS side A

- CLI> status trunk-grp id=<SS7 trunk group ID>;

# Task 2: Control SS7 links out of service

In this task, you will tear down SS7 links in OMNI before switchover activity to side B.

# From CA/FS side A

Step 1   Log in as root

Step 2   Deactivate SS7 Links on CA/FS side A

1. <hostname># termhandler -node a7n1

- OMNI [date] #1:deact-slk:slk=< link on CA/FS side A >;

- Enter y to continue

- Repeat above two steps for each active link

2. OMNI [date] #2:display-slk;

- Enter “y” to continue

- Verify that the state of each link is INACTIVE

3. OMNI[date]#3:quit

# Task 3: Force side A systems to standby

This procedure will force the side A systems to standby and force the side B systems to active.

|  | Note In the commands below, "xxx", "yyy" or "zzz" is the instance for the process on your system. |
| --- | --- |

# From EMS side A

Step 1   Log in as CLI user

Step 2   CLI> control call-agent id=CAxxx; target-state=forced-standby-active;

Step 3   CLI> control feature-server id=FSPTCzzz; target-state=forced-standby-active;

Step 4   CLI> control feature-server id=FSAINyyy; target-state=forced-standby-active;

Step 5   CLI> control bdms id=BDMS01; target-state=forced-standby-active;

Step 6   CLI> control element-manager id=EM01; target-state=forced-standby-active;

Step 7   CLI session will terminate when the last CLI command completes

|  | Note If the system failed to switchover from side A to side B, please contact Cisco Support to determine whether the system should fallback. If fallback is needed, please following Appendix G. |
| --- | --- |

# Task 4: Provision 4.5.0 SS7 configuration

# From EMS side B

Note:  The script used in this task must only migrate 3.5.4 SS7 configuration from OMNI based to ITP based with no control commands. If you don’t have a pre-created CLI provisioning script for adding SS7 configuration information to the new release, you must contact Cisco Support.

Step 1   If there is NO CLI script prepared, please follow steps specified in Appendix S and Appendix T and skip over Step 2.

Step 2   If there are CLI scripts prepared, log in to the Network File Server where the pre-prepared script resides. SFTP the script to EMS side B log in as CLI user and put script under /opt/ems/ftp/deposit directory.

- Wait for the batch process to pick up the script from /opt/ems/ftp/deposit directory and provision it into the system.

- Keep running “ls” command from /opt/ems/ftp/deposit directory and till the file is gone

- <hostname># cd /opt/ems/report

- <hostname># egrep Failed:0 <filename>.html

- Verify the system returns an output. If there is no output returned, please check for errors in the report file.

# Task 5: Control SS7 trunk groups in service

# From EMS side B

Step 1   If there is NO CLI script prepared, please follow steps specified in Appendix R and then continue on Step 3.

Step 2   If there are CLI scripts prepared, log in to the Network File Server where the pre-prepared script resides. SFTP the script to EMS side B log in as CLI user and put script under /opt/ems/ftp/deposit directory.

- Wait for the batch process to pick up the script from /opt/ems/ftp/deposit directory and provision it into the system.

- Keep running “ls” command from /opt/ems/ftp/deposit directory and till the file is gone

- <hostname># cd /opt/ems/report

- <hostname># egrep Failed:0 <filename>.html

- Verify the system returns an output. If there is no output returned, please check for errors in the report file.

Step 3    Check the status of each SS7 trunk group – admin state should in service:

- Log in as CLI user to EMS side B

- CLI> status trunk-grp id=<SS7 trunk group ID>;

Step 4   Please follow steps in Appendix U to verify the new SS7 configuration is working correctly. If the system fails to make new SS7 calls, please contact Cisco Support immediately.

# Task 6: Sync Data from EMS side B to CA/FS side B

In this task, you will sync from EMS to CA/FS for several inter-platform migrated tables.

# From EMS side B

Step 1   Log in as ciscouser (password: ciscosupport)

Step 2   CLI> sync annc_tg_profile master=EMS; target=CAxxx;

Step 3   CLI> sync trunk_grp master=EMS; target=CAxxx;

Step 4   CLI> sync emergency_number_list master=EMS; target=CAxxx;

Step 5   CLI> sync pop master=EMS; target=FSPTCzzz;

Step 6   CLI> sync radius_profile master=EMS; target=FSPTCzzz;

Step 7   CLI> sync pop master=EMS; target=FSAINyyy;

Step 8   CLI> sync subscriber-profile master=EMS; target=FSPTCzzz;

Step 9   CLI> sync cpsg master=EMS; target=CAxxx;

Step 10   CLI> sync ext2subscriber master=EMS; target=CAxxx;

Step 11   CLI> sync subscriber master=EMS; target=FSPTCzzz;

Step 12   CLI> exit

# Task 7: Validate release 4.5.0 software operation

To verify the stability of the newly installed 4.5.0 Release, let CA/FS side B carry live traffic for period of time. Monitor the Cisco BTS 10200 Softswitch and the network. If there are any problems, please investigate and contact Cisco Support if necessary.

# From EMS side B

Step 1   Verify that call processing using Appendix B.

Step 2   Log in as CLI user.

Step 3   CLI> audit database type=row-count;

- Verify there is no error in the report and the database is not empty.

- If any table shows mis-match, then perform a detailed audit on these tables:

CLI> audit <table name>;

Step 4   Verify the SUP config is set up correctly

- CLI> show sup-config;

- Verify refresh rate is set to 86400.

- If not, do the following

- CLI> change sup-config type=refresh_rate; value=86400;

# From EMS side A

Step 1   <hostname># ls /opt/bms/ftp/billing

- If there are files listed, then SFTP the files to a mediation device on the network and remove the files from the /opt/bms/ftp/billing directory.

|  | Note Once the system proves stable and you decide to move ahead with the upgrade, then you must execute subsequent tasks. If fallback is needed at this stage, please follow the fallback procedure in Appendix G. |
| --- | --- |

Chapter 8

Upgrade Side A Systems

# Task 1: Stop applications and shutdown EMS side A

# From EMS side A

Step 1   Log in as root

Step 2   Record the IP address and netmask for the management interface of the system.

- For an example, if the “hme0” is used for management interface, then execute the following command:

<hostname># ifconfig hme0

- Record the IP address and netmask for the interface to be used in the next task.

IP: _____________ Netmask: ____________ Interface Name: ___________

Step 3   <hostname># mv /etc/rc3.d/S99platform /etc/rc3.d/_S99platform

Step 4   <hostname># platform stop all

Step 5   <hostname># sync; sync

Step 6   <hostname># shutdown –i5 –g0 –y

# Task 2: Stop applications and shutdown CA/FS side A

# From CA/FS side A

Step 1   Log in as root

Step 2   Record the IP address and netmask for the management interface of the system.

- For an example, if the “hme0” is used for management interface, then execute the following command:

<hostname># ifconfig hme0

- Record the IP address and netmask for the interface to be used in the next task.

IP: _____________ Netmask: ____________ Interface Name: ___________

Step 3   <hostname># mv /etc/rc3.d/S99platform /etc/rc3.d/_S99platform

Step 4   <hostname># platform stop all

Step 5    <hostname># sync; sync

Step 6   <hostname># shutdown –i5 –g0 –y

# Task 3: Upgrade EMS side A to the new release

# From EMS side A

Step 1   Power off the machine

Step 2   Remove disk0 from slot 0 off the machine and label it as “Release 3.5.4 EMS side A disk0”

- SunFire V120 disk slot lay out:

| CD-ROM | Disk 0 | Disk 1 |
| --- | --- | --- |

- SunFire V240 disk slot lay out:

| Disk 2 | Disk 3 |  |
| --- | --- | --- |
| Disk 0 | Disk 1 | DVD-ROM |

- SunFire V440 disk slot lay out:

| Disk 3 | DVD-ROM |
| --- | --- |
| Disk 2 |  |
| Disk 1 |  |
| Disk 0 |  |

- Netra 1280 disk slot lay out:

|  | DVD-ROM |
| --- | --- |
|  | Disk 1 |
|  | Disk 0 |

- Netra 20 disk slot lay out:

| D I S K 0 | D I S K 1 |  | DVD ROM |
| --- | --- | --- | --- |

- Continuous Hardware disk slot lay out:

| CD-ROM | Disk 0 |  |
| --- | --- | --- |
|  | Disk 1 |  |

Step 3   Place new disk labeled as “Release 4.5.0 EMS side A disk0, 1” in slot 0

Step 4   Power on the machine and allow the system to boot up by monitoring the boot process through console.

- For Netra 1280 machine, please execute the following command from console:

<lom>poweron

- For other type of hardware, please use the power button to turn on the power.

Step 5   Log in as root from console.

Step 6   Rebuild network interface hardware configuration on disk

<hostname># cp –p /etc/path_to_inst /etc/path_to_inst.save

<hostname># egrep -iv “qfe|ce|eri|bge|hme|pci_pci” /etc/path_to_inst.save > /etc/path_to_inst

Step 7   Rebuild the hardware configuration

<hostname># reboot -- -r

- Wait for the system to boot up. Then log in as root.

- Verify the Ethernet port assignment in the /etc/path_to_inst file.

Step 8   Restore interfaces:

- <hostname># ifconfig <primary interface name> plumb

  - Use Interface Name recorded in Chapter 3, Task 3

- <hostname># ifconfig <primary interface name> <Interface IP> netmask <NETMASK> broadcast + up

  - Use IP and NETMASK recorded in Chapter 3, Task 3 for the interface

- Add static routes to reach Domain Name Server and Network File Server:

  - <hostname># echo “route add -net <network to reach DNS> <gateway IP>” >> /etc/rc3.d/S96StaticRoutes

  - <hostname># /etc/rc3.d/S96StaticRoutes

Example:

  - If the primary network manager interface for the machine is CE0, run “ifconfig ce0”, you should have following output:

ce0: flags=1000843<UP,BROADCAST,RUNNING,MULTICAST,IPv4> mtu 1500 index 2 inet 10.89.183.148 netmask ffffff00 broadcast 10.89.183.255 ether 0:3:ba:9d:68:85

  - If the DNS server is on network “10.89.0.0”, then add gateway 10.89.183.254 to reach that network with following command:

<hostname># echo “route add -net 10.89.0.0 10.89.183.254” >> /etc/rc3.d/S96StaticRoutes

<hostname># /etc/rc3.d/S96StaticRoutes

Step 9   Reset ssh keys:

<hostname># \rm /.ssh/known_hosts

Step 10   sftp the opticall.cfg and resolv.conf from Secondary EMS side B and place it under /etc directory.

<hostname># sftp <Side B EMS IP>

sftp> get /etc/resolv.conf /etc/resolv.conf

sftp> get /etc/opticall.cfg /etc/opticall.cfg

sftp> get /etc/named.conf  /etc/named.conf

sftp> exit

Step 11   Run script program to replace the hostname

<hostname># cd /opt/ems/upgrade

<hostname># DoTheChange -p

- The system will reboot when the script DoTheChange completes its run

Step 12   Wait for the system to boot up. Then Log in as root.

Step 13   Verify interface hardware configuration match to the host configuration:

<hostname># egrep –i “qfe|ce|eri|bge|hme” /etc/path_to_inst

<hostname># ls –l /etc/hostname.*

- If the interface names match from the above two outputs, please continue on Step 15.

- If the interface names do NOT match, please match them by changing the postfix of hostname.*.

For an example:

Output from “egrep –i “qfe|ce|eri|bge|hme” /etc/path_to_inst” is:

"/pci@1f,4000/network@1,1" 0 "hme"

"/pci@1f,4000/pci@4/SUNW,qfe@0,1" 0 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@1,1" 1 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@2,1" 2 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@3,1" 3 "qfe"

Output from “ls -l /etc/hostname.*” is:

-rw-r--r--   1 root     other         14 May 16 16:03 /etc/hostname.hme0

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.hme0:1

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.eri0

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.eri0:1

After change, the output should be:

-rw-r--r--   1 root     other         14 May 16 16:03 /etc/hostname.hme0

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.hme0:1

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.qfe0

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.qfe0:1

Step 14   Reboot the machine to pick up new interface setting:

<hostname># sync; sync; reboot

- Wait for the system to boot up. Then log in as root.

Step 15   CDR delimiter customization is not retained after software upgrade. If this system has been customized, either the Customer or Cisco Support Engineer must manually customize again to keep the same customization.

- <hostname># cd /opt/bdms/bin

- <hostname># vi platform.cfg

- Find the section for the command argument list for the BMG process

- Customize the CDR delimiters in the “Args=” line

- Example:

Args=-port 15260 -h localhost -u optiuser -p optiuser -fmt default_formatter -UpdIntvl 3300 -ems_local_dn blg-aSYS14EMS.cisco.com -FD semicolon -RD verticalbar

Step 16   <hostname># platform start

# Task 4: Upgrade CA/FS Side A to the new release

# From CA/FS side A

Step 1   Power off the machine

Step 2   Remove disk0 from slot 0 off the machine and label it as “Release 3.5.4 CA/FS side A disk0”

- SunFire V120 disk slot lay out:

| CD-ROM | Disk 0 | Disk 1 |
| --- | --- | --- |

- SunFire V240 disk slot lay out:

| Disk 2 | Disk 3 |  |
| --- | --- | --- |
| Disk 0 | Disk 1 | DVD-ROM |

- SunFire V440 disk slot lay out:

| Disk 3 | DVD-ROM |
| --- | --- |
| Disk 2 |  |
| Disk 1 |  |
| Disk 0 |  |

- Netra 1280 disk slot lay out:

|  | DVD-ROM |
| --- | --- |
|  | Disk 1 |
|  | Disk 0 |

- Netra 20 disk slot lay out:

| D I S K 0 | D I S K 1 |  | DVD ROM |
| --- | --- | --- | --- |

- Continuous Hardware disk slot lay out:

| CD-ROM | Disk 0 | Disk 2 |
| --- | --- | --- |
|  | Disk 1 | Disk 3 |

Step 3   Place new disk labeled as “Release 4.5.0 CA/FS side A disk0, 1” in slot 0

Step 4   Power on the machine and allow the system to boot up by monitoring the boot process thru console.

- For Netra 1280 machine, please execute the following command from console:

<lom>poweron

- For other type of hardware, please use the power button to turn on the power.

Step 5   Log in as root.

Step 6   Rebuild network interface hardware configuration on disk

<hostname># cp –p /etc/path_to_inst /etc/path_to_inst.save

<hostname># egrep -iv “qfe|ce|eri|bge|hme|pci_pci” /etc/path_to_inst.save > /etc/path_to_inst

Step 7   Rebuild the hardware configuration

<hostname># reboot -- -r

- Wait for the system to boot up. Then log in as root.

- Verify the Ethernet port assignment in the /etc/path_to_inst file.

Step 8   Restore interfaces:

- <hostname># ifconfig <interface name> plumb

  - Use Interface Name recorded in Chapter 3, Task 3.

- <hostname># ifconfig <interface name> <Interface IP> netmask <NETMASK> broadcast + up

  - Use IP and NETMASK recorded in Chapter 3, Task 3

- Add static routes to reach Domain Name Server and Network File Server using “route add …” command:

  - Example: route add -net 10.89.0.0 10.89.232.254

Where: 10.89.0.0 is the destination DNS server network IP
              10.89.232.254 is the gateway IP

Step 9   Reset ssh keys:

<hostname># \rm /.ssh/known_hosts

Step 10   sftp required files.

<hostname># sftp <Side B CA/FS IP>

sftp> get /etc/resolv.conf /etc/resolv.conf

sftp> get /etc/opticall.cfg /etc/opticall.cfg

sftp> get /etc/named.conf  /etc/named.conf

sftp> exit

Step 11   Run script program to replace the hostname

<hostname># cd /opt/ems/upgrade

<hostname># DoTheChange -p

- The system will reboot when the script DoTheChange completes its run

Step 12    Wait for the system to boot up. Then Log in as root.

Step 13   Verify interface hardware configuration match to the host configuration:

<hostname># egrep –i “qfe|ce|eri|bge|hme” /etc/path_to_inst

<hostname># ls –l /etc/hostname.*

- If the interface names match from the above two outputs, please continue on Step 15.

- If the interface names do NOT match, please match them by changing the postfix of hostname.*.

For an example:

Output from “egrep –i “qfe|ce|eri|bge|hme” /etc/path_to_inst” is:

"/pci@1f,4000/network@1,1" 0 "hme"

"/pci@1f,4000/pci@4/SUNW,qfe@0,1" 0 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@1,1" 1 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@2,1" 2 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@3,1" 3 "qfe"

"/pci@1f,2000/pci@1/SUNW,qfe@0,1" 4 "qfe"

"/pci@1f,2000/pci@1/SUNW,qfe@1,1" 5 "qfe"

"/pci@1f,2000/pci@1/SUNW,qfe@2,1" 6 "qfe"

"/pci@1f,2000/pci@1/SUNW,qfe@3,1" 7 "qfe"

Output from “ls -l /etc/hostname.*” is:

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.hme0

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.eri0

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.eri1

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.eri1:1

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.eri1:2

-rw-r--r--   1 root     other         12 Jun 10 11:25 hostname.eri1:3

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.eri2

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.eri2:1

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.eri2:2

-rw-r--r--   1 root     other         12 Jun 10 11:25 hostname.eri2:3

After change, the output should be:

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.hme0

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.qfe0

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.qfe1

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.qfe1:1

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.qfe1:2

-rw-r--r--   1 root     other         12 Jun 10 11:25 hostname.qfe1:3

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.qfe2

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.qfe2:1

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.qfe2:2

-rw-r--r--   1 root     other         12 Jun 10 11:25 hostname.qfe2:3

Step 14   Reboot the machine to pick up new interface setting:

<hostname># sync; sync; reboot

- Wait for the system to boot up. Then log in as root.

Step 15   Install the applications

<hostname># cd /opt/Build

<hostname># install.sh –upgrade

Step 16   Answer "y" when prompted. This process will take up to 15 minutes to complete.

- Answer "y" if prompted for reboot

- Wait for the system to boot up. Then Log in as root.

Step 17   Update CA/FS platform.cfg files:

<hostname># /opt/Build/btsauto/update_platform.sh

Step 18   <hostname># platform start

Step 19   <hostname># mv /etc/rc3.d/_S99platform /etc/rc3.d/S99platform

# Task 5: Restore EMS mate communication

# From EMS side B

Step 1   Log in as root

Step 2   <hostname># /opt/ems/utils/updMgr.sh –restore_hub

Step 3   <hostname># nodestat

- Verify OMS Hub mate port status is established

- Verify HUB communication from EMS side B to CA/FS side A is established

# Task 6: Copying oracle data

# From EMS side A

Step 1   <hostname># svcadm disable system/cron

Step 2   Copying data.

<hostname>$ su – oracle

<hostname>$ cd /opt/oracle/admin/upd

<hostname>$ java dba.dmt.DMMgr –copy -auto

Step 3   Verify the data copy is a success.

<hostname>$ echo $?

- Verify the returned value is 0.

- If not, please sftp /opt/oracle/admin/upd/DMMgr.log file off system and call for immediate technical assistance.

<hostname>$ exit

Step 4   <hostname># svcadm enable system/cron

Step 5   <hostname># mv /etc/rc3.d/_S99platform /etc/rc3.d/S99platform

# Task 7: To install CORBA on EMS side A, please follow Appendix I.

Chapter 9

Finalizing Upgrade

# Task 1: Switchover activity from side B to side A

This procedure will force the active system activity from side B to side A.

# From EMS side B

|  | Note In the commands below, "xxx", "yyy" or "zzz" is the instance for the process on your system. |
| --- | --- |

Step 1   Log in to EMS side B as CLI user.

Step 2   CLI> control call-agent id=CAxxx; target-state=active-standby;

Step 3   CLI> control feature-server id=FSPTCyyy; target-state=active-standby;

Step 4   CLI> control feature-server id=FSAINzzz; target-state=active-standby;

Step 5   CLI> control bdms id=BDMS01; target-state=active-standby;

Step 6   CLI> control element-manager id=EM01; target-state=active-standby;

Step 7   CLI shell session should be terminated when last CLI commands completes.

# Task 2: Enable Oracle DB replication on EMS side B

# From EMS side B

Step 1   Restore Oracle DB to duplex mode:

<hostname># su - oracle

<hostname>$ cd /opt/oracle/admin/utl

<hostname>$ rep_toggle –s optical2 –t set_duplex

  - Answer “y” when prompt

  - Answer “y” again when prompt

Step 2   <hostname>$ exit

Step 3   Terminate applications.

<hostname># platform stop all

Step 4   Re-start applications to activate DB toggle in duplex mode.

<hostname># platform start

# Task 3: Synchronize handset provisioning data and check both mgw and cas-tg-profile tables

# From EMS side A

|  | Note In the commands below, "xxx", "yyy" or "zzz" is the instance for the process on your system. |
| --- | --- |

Step 1   Log in as ciscouser (password: ciscosupport)

Step 2   CLI>sync termination master=CAxxx; target=EMS;

- Verify the transaction is executed successfully.

Step 3   CLI>sync sc1d master=FSPTCzzz; target=EMS;

- Verify the transaction is executed successfully

Step 4   CLI>sync sc2d master=FSPTCzzz; target=EMS;

- Verify the transaction is executed successfully

Step 5   CLI>sync sle master=FSPTCzzz; target=EMS;

- Verify the transaction is executed successfully

Step 6   CLI>sync subscriber-feature-data master=FSPTCzzz; target=EMS;

- Verify the transaction is executed successfully

Step 7   CLI> audit mgw;

- If the TYPE mismatch is found between EMS Oracle DB and Call Agent, check the mismatched mgw to see if there is any trunk group associated with it. If it does, then the TYPE needs to be set to “TGW”. Otherwise, the TYPE should be set to “RGW”.

- Command syntax for making the change:

CLI> change mgw id=<mgw id>; type=<RGW|TGW>;

Step 8   CLI> audit cas-tg-profile;

- If the SIG_TYPE mismatch is found between EMS Oracle DB and Call Agent, here are steps for correction:

- Find ID of the mismatched cas-tg-profile

- Check MGCP_PKG_TYPE of trunk_grp associated with cas_tg_profile:

CLI>show trunk-grp tg_profile_id=<cas-tg-profile ID>;

- If the MGCP_PKG_TYPE (for the given trunk grp) is MS, then SIG_TYPE should be set to MF in cas-tg-profile.

- If the MGCP_PKG_TYPE (for the given trunk grp) is DT, then SIG_TYPE should be set to DTMF in cas-tg-profile.

- Default for the SIG_TYPE in cas-tg-prfoile is MF

- Command syntax for making the change:

CLI> change cas-tg-profile mgw id=<mgw id>; type=<RGW|TGW>;

Step 9   CLI>exit

# Task 4: Restore customized cron jobs

Please restore customized cron jobs by using the files saved on the network file server during system preparation stage in Chapter 4, Task 10. Please don’t copy the old crontab file over the new one. You may need to compare the back up version of the crontab file to the new crontab file to restore the previous settings. This should be done for all machines in the system.

# Task 5: Synchronize the hosts and opticall.cfg files

# From Primary CA/FS

Please ftp the /etc/hosts and /etc/opticall.cfg file to all other machines in the system.

# Task 6: Verify system status

Verify that the system is operating properly before you leave the site.

Step 1   Verify that the side A systems are in the active state. Use Appendix A for this procedure.

Step 2   Verify that call processing is working without error. Use Appendix B for this procedure.

Step 3   Verify that provisioning is operational from CLI command line, and verify database. Use Appendix C for this procedure.

Step 4   Verify that there are no outstanding major or critical alarms. Use Appendix D for this procedure.

Step 5   Use Appendix E to verify that Oracle database and replication functions are working properly.

Step 6   If you have answered NO to any of the above questions (Step 1-5) do not proceed. Instead, use the backout procedure in Appendix H. Contact Cisco Support if you need assistance.

Once the site has verified that all critical call-thru testing has successfully completed and the upgrade is complete, Appendix F should be executed to gather an up to date archive file of the system.

You have completed the Cisco BTS 10200 system upgrade process successfully.

Chapter 10

Post Upgrade Tasks

This chapter covers tasks that to be executed only after completing software upgrade successfully from release 3.5.4 to release 4.5.0.

# Task 1: Change MGCP Domain IP addresses from Domain Name Servers

From each Domain Name Server that is serving the upgraded BTS 10200, remove the 2 physical IP addresses from primary CA/FS for MGCP domain name from DNS. In other words, change IPs for the MGCP domain name in the DNS database from: 2P + 2L  2L by removing two physical IPs from primary CA/FS.

Please refresh the DNS cache and make sure the new IP addresses are taking affect.

To verify the change has taken affect, please log in to both CA/FS machines and perform “nslookup” of the MGCP domain name.

# Task 2: Correct ntp.conf File

# From EMS side B

Step 1   sftp ntp.conf file from CA/FS side B:

<hostname># sftp <CA/FS side B hostname>

sftp> get /opt/BTSxntp/etc/ntp.conf /opt/BTSxntp/etc/ntp.conf

sftp> quit

Step 2   Edit and save the ntp.conf file to change the peer information. Each machine should be “peer” to other machines in the system.

For an example:

If the local hostname is secems99, then the ntp.conf should have the following set up:

# peer all other hosts in the same BTS

peer secca99 minpoll 4 maxpoll 8

peer prica99 minpoll 4 maxpoll 8

peer priems99 minpoll 4 maxpoll 8

Step 3   Restart xntp daemon:

<hostname># /etc/rc2.d/S79xntp stop

<hostname># /etc/rc2.d/S79xntp start

# From EMS side A

Step 1   sftp ntp.conf file from CA/FS side B:

<hostname># sftp <CA/FS side B hostname>

sftp> get /opt/BTSxntp/etc/ntp.conf /opt/BTSxntp/etc/ntp.conf

sftp> quit

Step 2   Edit and save the ntp.conf file to change the peer information. Each machine should be “peer” to other machines in the system.

For an example:

If the local hostname is priems99, then the ntp.conf should have the following set up:

# peer all other hosts in the same BTS

peer secca99 minpoll 4 maxpoll 8

peer prica99 minpoll 4 maxpoll 8

peer secems99 minpoll 4 maxpoll 8

Step 3   Restart xntp daemon:

<hostname># /etc/rc2.d/S79xntp stop

<hostname># /etc/rc2.d/S79xntp start

# Task 3: Remove BTS (3.5.4) linksets

Remove BTS (3.5.4) linksets from the STP-BTS combined linkset.

# Task 4: Provisioning new features

# From EMS side A

#### Add new announcements

Step 1   The new announcements should already been prepared in Chapter 3. Please follow steps specified in Appendix J to place the new announcements to appropriate Media Gateways.

#### Associate announcements

Step 1   If there is NO CLI script prepared, please follow steps specified in Appendix K and skip over Step 2.

Step 2   If there are CLI scripts prepared, log in to the Network File Server where the pre-prepared script resides. SFTP the script to EMS side A log in as CLI user and put script under  /opt/ems/ftp/deposit directory.

- Wait for the batch process to pick up the script from /opt/ems/ftp/deposit directory and provision it into the system.

- Keep running “ls” command from /opt/ems/ftp/deposit directory and till the file is gone

- <hostname># cd /opt/ems/report

- <hostname># egrep Failed:0 <filename>.html

- Verify the system returns an output. If there is no output returned, please check for errors in the report file.

#### Create new features

Step 1   If there is NO CLI script prepared, please follow steps specified in Appendix L and skip over Step 2.

Step 2   If there are CLI scripts prepared, log in to the Network File Server where the pre-prepared script resides. SFTP the script to EMS side A log in as CLI user and put script under /opt/ems/ftp/deposit directory.

- Wait for the batch process to pick up the script from /opt/ems/ftp/deposit directory and provision it into the system.

- Keep running “ls” command from /opt/ems/ftp/deposit directory and till the file is gone

- <hostname># cd /opt/ems/report

- <hostname># egrep Failed:0 <filename>.html

- Verify the system returns an output. If there is no output returned, please check for errors in the report file.

#### Update existing features

Step 1   If there is NO CLI script prepared, please follow steps specified in Appendix M and skip over Step 2.

Step 2   If there are CLI scripts prepared, log in to the Network File Server where the pre-prepared script resides. SFTP the script to EMS side A log in as CLI user and put script under /opt/ems/ftp/deposit directory.

- Wait for the batch process to pick up the script from /opt/ems/ftp/deposit directory and provision it into the system.

- Keep running “ls” command from /opt/ems/ftp/deposit directory and till the file is gone

- <hostname># cd /opt/ems/report

- <hostname># egrep Failed:0 <filename>.html

- Verify the system returns an output. If there is no output returned, please check for errors in the report file.

# Task 5: Restore platform.cfg with logical IPs

# From EMS side A

Step 1   log in as root

Step 2   <hostname># cd /opt/Build/btsauto

Step 3   <hostname># restore_platform_cfg.exp

- Please enter the password for root when prompted

- The program will perform the task and should terminate without any error.

# Task 6: Enable disk mirroring

Please follow steps in the Appendix X to enable disk mirroring on each machine in the BTS system.

Appendix A

Check System Status

The purpose of this procedure is to verify the system is running in NORMAL mode, with the side A systems in ACTIVE state and the side B systems in STANDBY state.

|  | Note In the commands below, "xxx", "yyy" or "zzz" is the instance for the process on your system, and DomainName is your system domain name. |
| --- | --- |

# From Active EMS side A

Step 1   Log in as CLI user.

Step 2   CLI> status system;

For 3.5.x System response:

Checking Call Agent status ... Checking Feature Server status ... Checking Billing Server status ... Checking Billing MySQL status ... Checking Element Manager status ... Checking EMS MySQL status ... Checking ORACLE status ... Reply : Success: CALL AGENT STATUS IS... -> APPLICATION INSTANCE -> Call Agent [CA146] PRIMARY STATUS -> ACTIVE_NORMAL SECONDARY STATUS -> STANDBY_NORMAL FEATURE SERVER STATUS IS... -> APPLICATION INSTANCE -> Feature Server [FS235] PRIMARY STATUS -> ACTIVE_NORMAL SECONDARY STATUS -> STANDBY_NORMAL FEATURE SERVER STATUS IS... -> APPLICATION INSTANCE -> Feature Server [FS205] PRIMARY STATUS -> ACTIVE_NORMAL SECONDARY STATUS -> STANDBY_NORMAL BILLING SERVER STATUS IS... -> APPLICATION INSTANCE -> Bulk Data Management Server [BDMS1] PRIMARY STATUS -> ACTIVE_NORMAL SECONDARY STATUS -> STANDBY_NORMAL BILLING MYSQL STATUS IS... -> Daemon is running! ELEMENT MANAGER STATUS IS... -> APPLICATION INSTANCE -> Element Manager [EM1] PRIMARY STATUS -> ACTIVE_NORMAL SECONDARY STATUS -> STANDBY_NORMAL EMS MYSQL STATUS IS ... -> Daemon is running! ORACLE STATUS IS... -> Daemon is running!

For 4.5.x System response:

Checking Call Agent status ... Checking Feature Server status ... Checking Billing Server status ... Checking Billing Oracle status ... Checking Element Manager status ... Checking EMS MySQL status ... Checking ORACLE status ... CALL AGENT STATUS IS... -> APPLICATION INSTANCE -> Call Agent [CA146] PRIMARY STATUS -> ACTIVE SECONDARY STATUS -> STANDBY FEATURE SERVER STATUS IS... -> APPLICATION INSTANCE -> Feature Server [FSPTC235] PRIMARY STATUS -> ACTIVE SECONDARY STATUS -> STANDBY FEATURE SERVER STATUS IS... -> APPLICATION INSTANCE -> Feature Server [FSAIN205] PRIMARY STATUS -> ACTIVE SECONDARY STATUS -> STANDBY BILLING SERVER STATUS IS... -> APPLICATION INSTANCE -> Bulk Data Management Server [BDMS01] PRIMARY STATUS -> ACTIVE SECONDARY STATUS -> STANDBY BILLING ORACLE STATUS IS... -> Daemon is running! ELEMENT MANAGER STATUS IS... -> APPLICATION INSTANCE -> Element Manager [EM01] PRIMARY STATUS -> ACTIVE SECONDARY STATUS -> STANDBY EMS MYSQL STATUS IS ... -> Daemon is running! ORACLE STATUS IS... -> Daemon is running! Reply : Success:

Appendix B

Check Call Processing

This procedure verifies that call processing is functioning without error. The billing record verification is accomplished by making a sample phone call and verify the billing record is collected correctly.

# From EMS side A

Step 1   Log in as CLI user.

Step 2   Make a new phone call on the system. Verify that you have two-way voice communication. Then hang up both phones.

Step 3   CLI> report billing-record tail=1;

.. CALLTYPE=TOLL SIGSTARTTIME=2004-05-03 17:05:21 SIGSTOPTIME=2004-05-03 17:05:35 CALLELAPSEDTIME=00:00:00 INTERCONNECTELAPSEDTIME=00:00:00 ORIGNUMBER=4692551015 TERMNUMBER=4692551016 CHARGENUMBER=4692551015 DIALEDDIGITS=9 4692551016# 5241 ACCOUNTCODE=5241 CALLTERMINATIONCAUSE=NORMAL_CALL_CLEARING ORIGSIGNALINGTYPE=0 TERMSIGNALINGTYPE=0 ORIGTRUNKNUMBER=0 TERMTRUNKNUMBER=0 OUTGOINGTRUNKNUMBER=0 ORIGCIRCUITID=0 TERMCIRCUITID=0 ORIGQOSTIME=2004-05-03 17:05:35 ORIGQOSPACKETSSENT=0 ORIGQOSPACKETSRECD=7040877 ORIGQOSOCTETSSENT=0 ORIGQOSOCTETSRECD=1868853041 ORIGQOSPACKETSLOST=805306368 ORIGQOSJITTER=0 ORIGQOSAVGLATENCY=0 TERMQOSTIME=2004-05-03 17:05:35 TERMQOSPACKETSSENT=0 TERMQOSPACKETSRECD=7040877 TERMQOSOCTETSSENT=0 TERMQOSOCTETSRECD=1868853041 TERMQOSPACKETSLOST=805306368 TERMQOSJITTER=0 TERMQOSAVGLATENCY=0 PACKETIZATIONTIME=0 SILENCESUPPRESSION=1 ECHOCANCELLATION=0 CODECTYPE=PCMU CONNECTIONTYPE=IP OPERATORINVOLVED=0 CASUALCALL=0 INTERSTATEINDICATOR=0 OVERALLCORRELATIONID=CA14633 TIMERINDICATOR=0 RECORDTYPE=NORMAL RECORD CALLAGENTID=CA146 ORIGPOPTIMEZONE=CDT ORIGTYPE=ON NET TERMTYPE=ON NET NASERRORCODE=0 NASDLCXREASON=0 ORIGPOPID=69 TERMPOPID=69 TERMPOPTIMEZONE=CDT DIALPLANID=cdp1 CALLINGPARTYCATEGORY=Ordinary Subscriber CALLEDPARTYINDICATOR=No Indication CALLEDPARTYPORTEDIN=No CALLINGPARTYPORTEDIN=No BILLINGRATEINDICATOR=None ORIGENDPOINTADDR=c2421-227-142.ipclab.cisco.com Reply : Success: Entry 1 of 1 returned from host: priems08

Step 4   Verify that the attributes in the CDR match the call just made.

Appendix C

Check Provisioning and Database

# From EMS side A

The purpose of this procedure is to verify that provisioning is functioning without error. The following commands will add a "dummy" carrier then delete it.

Step 1   Log in as CLI user.

Step 2   CLI> add carrier id=8080;

Step 3   CLI> show carrier id=8080;

Step 4   CLI> delete carrier id=8080; 

Step 5   CLI> show carrier id=8080;

- Verify message is: Database is void of entries.

# Check transaction queue

In this task, you will verify that the OAMP transaction queue status. The queue should be empty.

Step 1   CLI> show transaction-queue;

- Verify there is no entry shown. You should get following reply back:

Reply : Success: Database is void of entries.

- If the queue is not empty, wait for the queue to empty. If the problem persists, contact Cisco Support.

Step 2   CLI>exit

# Perform database audit

In this task, you will perform a full database audit and correct any errors, if necessary.

Step 1   CLI> audit database type=full;

Step 2   Check the audit report and verify there is no discrepancy or errors. If errors are found, please try to correct them. If you are unable to correct, please contact Cisco Support.

Appendix D

Check Alarm Status

The purpose of this procedure is to verify that there are no outstanding major/critical alarms.

# From EMS side A

Step 1   Log in as CLI user.

Step 2   CLI> show alarm

- The system responds with all current alarms, which must be verified or cleared before proceeding with next step.

|  | Tip Use the following command information for reference material ONLY. |
| --- | --- |

Step 3   To monitor system alarm continuously.

CLI> subscribe alarm-report severity=all; type=all;

|  | Valid severity: MINOR, MAJOR, CRITICAL, ALL Valid types: CALLP, CONFIG, DATABASE, MAINTENANCE, OSS, SECURITY, SIGNALING, STATISTICS, BILLING, ALL, SYSTEM, AUDIT |
| --- | --- |

Step 4   System will display alarms if alarm is reported.

<------------------- START ALARM REPORT --------------------> TIMESTAMP: 20040503174759 DESCRIPTION: General MGCP Signaling Error between MGW and CA. TYPE & NUMBER: SIGNALING (79) SEVERITY: MAJOR ALARM-STATUS: OFF ORIGIN: MGA.PRIMARY.CA146 COMPONENT-ID: null ENTITY NAME: S0/DS1-0/1@64.101.150.181:5555 GENERAL CONTEXT: MGW_TGW SPECIFC CONTEXT: NA FAILURE CONTEXT: NA <------------------- END ALARM REPORT ---------------------->

Step 5   To stop monitoring system alarm.

CLI> unsubscribe alarm-report severity=all; type=all;

Step 6   CLI> exit

Appendix E

Check Oracle Database Replication and Error Correction

Perform the following steps on the Active EMS side A to check the Oracle database and replication status.

# Check Oracle DB replication status

# From EMS side A

Step 1   Log in as root.

Step 2   Log in as oracle.

<hostname># su – oracle

Step 3   Enter the command to check replication status and compare contents of tables on the side A and side B EMS databases:

<hostname>$ dbadm –C rep

Step 4   Verify that “Deferror is empty?” is “YES”.

OPTICAL1::Deftrandest is empty?           YES

OPTICAL1::dba_repcatlog is empty?       YES

OPTICAL1::Deferror is empty?              YES Make sure it is “YES”

OPTICAL1::Deftran is empty?                  YES

OPTICAL1::Has no broken job?               YES

OPTICAL1::JQ Lock is empty?                YES

OPTICAL2::Deftrandest is empty?          YES

OPTICAL2::dba_repcatlog is empty?      YES

OPTICAL2::Deferror is empty?             YES Make sure it is “YES”

OPTICAL2::Deftran is empty?                YES

OPTICAL2::Has no broken job?              YES

OPTICAL2::JQ Lock is empty?               YES

Step 5   If the “Deferror is empty?” is “NO”, please try to correct the error using steps in “Correct replication error” below. If you are unable to clear the error or if any of the individual steps fails, please contact Cisco Support.

# Correct replication error

|  | Note You must run the following steps on standby EMS side B first, then on active EMS side A. |
| --- | --- |

# From EMS Side B

Step 1   Log in as root

Step 2   <hostname># su – oracle

Step 3   <hostname>$ dbadm –C db

Step 4   For each table that is out of sync, please run the following step:

<hostname>$ dbadm -A copy -o <OWNER> -t <TABEL NAME>

- Enter “y” to continue

- Please contact Cisco Support if the above command fails.

Step 5   <hostname>$ dbadm –A truncate_deferror

- Enter “y” to continue

# From EMS Side A

Step 1   <hostname>$ dbadm –A truncate_deferror

- Enter “y” to continue

Step 2   Re-verify that “Deferror is empty?” is “YES” and none of tables is out of sync.

<hostname>$dbadm –C db

OPTICAL1::Deftrandest is empty?           YES

OPTICAL1::dba_repcatlog is empty?       YES

OPTICAL1::Deferror is empty?              YES  Make sure it is “YES”

OPTICAL1::Deftran is empty?                  YES

OPTICAL1::Has no broken job?               YES

OPTICAL1::JQ Lock is empty?                YES

OPTICAL2::Deftrandest is empty?          YES

OPTICAL2::dba_repcatlog is empty?      YES

OPTICAL2::Deferror is empty?             YES  Make sure it is “YES”

OPTICAL2::Deftran is empty?                YES

OPTICAL2::Has no broken job?              YES

OPTICAL2::JQ Lock is empty?               YES

Appendix F

Flash Archive Steps

# Task 1: Ensure side A systems are ACTIVE

In this task, you will ensure that the EMS side A applications are active.

Step 1   Log in as root to ACTIVE EMS

Step 2   Log in as CLI user

Step 3   CLI> control call-agent id=CAxxx; target-state=active-standby;

Step 4   CLI> control feature-server id=FSPTCzzz; target-state=active-standby;

Step 5   CLI> control feature-server id=FSAINyyy; target-state=active-standby;

Step 6   CLI> control bdms id=BDMS01; target-state=active-standby;

Step 7   CLI> control element-manager id=EM01; target-state=active-standby;

Step 8   CLI> status system;

- Verify applications on Side A are in ACTIVE state.

- Verify applications on Side B are in STANDBY state.

- Verify Oracle DB is in service

Step 9   CLI>exit

# Task 2: Perform a full database audit

In this task, you will go to EMS side A and perform a full database audit and correct errors, if there are any.  Contact Cisco Support if errors cannot be fixed.

# From EMS Side A

Step 1   Log in as CLI user

Step 2   CLI> audit database type=full;

Step 3   Check the audit report and verify there is no discrepancy or errors found. If errors are found, try to correct the errors. If you are unable to make the correction, contact Cisco Support.

# Task 3: Perform shared memory integrity check

In this task, you will perform shared memory integrity check to detect any potential data problems.

# From CA/FS side A

Step 1   Log in as root

Step 2   <hostname># cd /opt/OptiCall/CAxxx/bin

Step 3   <hostname># ca_tiat data

Step 4   Press “Enter” to continue

The result should be identical to the following:

All tables are OK.

For detail, see ca_tiat.out

If the result does NOT show “All tables are OK”, contact Cisco Support.

Step 5   <hostname># cd /opt/OptiCall/FSPTCzzz/bin

Step 6   <hostname># potsctx_tiat data

Step 7   Press “Enter” to continue

The result should be identical to the following:

All tables are OK.

For detail, see potsctx_tiat.out

If the result does NOT show “All tables are OK”, contact Cisco Support.

Step 8   <hostname># cd /opt/OptiCall/FSAINyyy/bin

Step 9   <hostname># ain_tiat data

Step 10   Press “Enter” to continue

The result should be identical to the following:

All tables are OK.

For detail, see ain_tiat.out

If the result does NOT show “All tables are OK”, contact Cisco Support.

# From CA/FS side B

Step 1   Log in as root

Step 2   <hostname># cd /opt/OptiCall/CAxxx/bin

Step 3   <hostname># ca_tiat data

Step 4   Press “Enter” to continue

The result should be identical to the following:

All tables are OK.

For detail, see ca_tiat.out

If the result does NOT show “All tables are OK”, contact Cisco Support.

Step 5   <hostname># cd /opt/OptiCall/FSPTCzzz/bin

Step 6   <hostname># potsctx_tiat data

Step 7   Press “Enter” to continue

The result should be identical to the following:

All tables are OK.

For detail, see potsctx_tiat.out

If the result does NOT show “All tables are OK”, contact Cisco Support.

Step 8   <hostname># cd /opt/OptiCall/FSAINyyy/bin

Step 9   <hostname># ain_tiat data

Step 10   Press “Enter” to continue

The result should be identical to the following:

All tables are OK.

For detail, see ain_tiat.out

If the result does NOT show “All tables are OK”, contact Cisco Support.

# Task 4: Perform flash archive on EMS side B

In this task, you will perform a flash archive on EMS side B to save a copy of OS and applications to a remote server. This process takes about 1 hour.

|  | Note Perform Task 4: Perform Flash Archive on EMS Side B and Task 5: Perform Flash Archive on CA/FS Side B in parallel. |
| --- | --- |

# From EMS side B

Step 1   Log in as root

Step 2   <hostname># svcadm disable system/cron

Step 3   <hostname># ps -ef | grep cron

- Verify no result is returned, which means cron daemon is no longer running.

Step 4   <hostname># cd /etc/rc3.d

Step 5   <hostname># mv S99platform _S99platform

Step 6   <hostname># platform stop all

Step 7   <hostname># nodestat

- Verify applications are out of service.

Step 8   <hostname># \rm –rf /opt/Build

Step 9   <hostname># \rm –rf /opt/8_rec

Step 10   <hostname># \rm –rf /opt/.upgrade

Step 11   Remove all directories and files that are no longer needed such as core files, patch directories.

Step 12   <hostname># mv /bin/date /bin/date.orig

Step 13   <hostname># mv /bin/.date /bin/date

Step 14   <hostname># tar –cvf - /opt/* | gzip –c > /opt/<hostname_release>.tar.gz

Where: hostname_release is the tar file name.

Example: tar –cvf - /opt/* | gzip –c > /opt/secems10_4.5.0V00.tar.gz

Step 15   <hostname># flarcreate -n <archive name> -x /opt -c /opt/<file name>

Where: archive name is the archive identification.

Example: flarcreate -n CCPU-EMS –x /opt -c /opt/secems10_4.5.0V00.archive

Step 16   SFTP the archive to an NFS server to be used later.

- <hostname># cd /opt

- <hostname># sftp <nfs server name>

- sftp> bin

- sftp> cd <target directory>

- sftp> put <archive file name>

- sftp> put <opt tar file name>

- sftp> bye

Step 17   <hostname># mv /bin/date /bin/.date

Step 18   <hostname># mv /bin/date.orig /bin/date

Step 19   <hostname># svcadm enable system/cron

Step 20   <hostname># ps -ef | grep cron

- Verify cron daemon is running.

Step 21   <hostname># cd /etc/rc3.d

Step 22   <hostname># mv _S99platform S99platform

Step 23   <hostname># platform start

Step 24   <hostname># nodestat

- Verify EM01 is in STANDBY.

- Verify BS01 is in STANDBY.

- Verify Oracle and Billing DBs are in service.

# Task 5: Perform flash archive on CA/FS side B

In this task, you will perform a flash archive on CA/FS side B to save a copy of OS and applications to a remote server. This process takes about 1 hour.

|  | Note Perform this task in parallel with Task 4: Perform Flash Archive on EMS Side B. |
| --- | --- |

# From CA/FS side B

Step 1   Log in as root

Step 2   <hostname># svcadm disable system/cron

Step 3   <hostname># ps -ef | grep cron

- Verify no result is returned, which means cron daemon is no longer running

Step 4   <hostname># cd /etc/rc3.d

Step 5   <hostname># mv S99platform _S99platform

Step 6   <hostname># platform stop all

Step 7   <hostname># nodestat

- Verify applications are out of service.

Step 8   <hostname># \rm –rf /opt/Build

Step 9   <hostname># \rm –rf /opt/8_rec

Step 10   <hostname># \rm –rf /opt/.upgrade

Step 11   Remove all directories and files that are no longer needed such as core files, patch directories.

Step 12   <hostname># mv /bin/date /bin/date.orig

Step 13   <hostname># mv /bin/.date /bin/date

Step 14   <hostname># tar –cvf - /opt/* | gzip –c > /opt/<hostname_release>.tar.gz

Where: hostname_release is the tar file name.

Example: tar –cvf - /opt/* | gzip –c > /opt/secca10_4.5.0V00.tar.gz

Step 15   <hostname># flarcreate -n <archive name> -x /opt -c /opt/<file name>

Where: archive name is the archive identification.

Example: flarcreate -n CCPU-CA –x /opt -c /opt/secca10_4.5.0V00.archive

Step 16   SFTP the archive to an NFS server to be used later.

- <hostname># cd /opt

- <hostname># sftp <nfs server name>

- sftp> bin

- sftp> cd <target directory>

- sftp> put <archive file name>

- sftp> put <opt tar file name>

- sftp> bye

Step 17   <hostname># mv /bin/date /bin/.date

Step 19   <hostname># mv /bin/date.orig /bin/date

Step 20   <hostname># svcadm enable system/cron

Step 21   <hostname># ps -ef | grep cron

- Verify cron daemon is running.

Step 22   <hostname># cd /etc/rc3.d

Step 23   <hostname># mv _S99platform S99platform

Step 24    <hostname># platform start

Step 25   <hostname># nodestat

- Verify CAxxx is in STANDBY.

- Verify FSAINyyy is in STANDBY.

- Verify FSPTCzzz is in STANDBY.

# Task 6: Switch activity from side A to side B

In this task, you will switch activity from the side A to the side B.

# From EMS side A

Step 1   Log in as CLI user

Step 2   CLI> control call-agent id=CAxxx; target-state=standby-active;

Step 3   CLI> control feature-server id=FSPTCzzz; target-state=standby-active;

Step 4   CLI> control feature-server id=FSAINyyy; target-state=standby-active;

Step 5   CLI> control bdms id=BDMS01; target-state=standby-active;

Step 6   CLI> control element-manager id=EM01; target-state=standby-active;

Step 7   CLI session will terminate when EM01 switchover is successful.

# Task 7: Perform flash archive on EMS side A

In this task, you will perform a flash archive on EMS side A to save a copy of the OS and applications to a remote server. This process takes about 1 hour.

|  | Note Perform Task 7: Perform Flash Archive on EMS Side A and Task 8: Perform Flash Archive on CA/FS Side A in parallel. |
| --- | --- |

# From EMS side A

Step 1   Log in as root

Step 2   <hostname># svcadm disable system/cron

Step 3   <hostname># ps -ef | grep cron

- Verify no result is returned, which means cron daemon is no longer running.

Step 4   <hostname># cd /etc/rc3.d

Step 5   <hostname># mv S99platform _S99platform

Step 6   <hostname># platform stop all

Step 7   <hostname># nodestat

- Verify applications are out of service.

Step 8   <hostname># \rm –rf /opt/Build

Step 9   <hostname># \rm –rf /opt/8_rec

Step 10   <hostname># \rm –rf /opt/.upgrade

Step 11   Remove all directories and files that are no longer needed such as core files, patch directories.

Step 12   <hostname># mv /bin/date /bin/date.orig

Step 13   <hostname># mv /bin/.date /bin/date

Step 14   <hostname># tar –cvf - /opt/* | gzip –c > /opt/<hostname_release>.tar.gz

Where: hostname_release is the tar file name.

Example: tar –cvf - /opt/* | gzip –c > /opt/priems10_4.5.0V00.tar.gz

Step 15   <hostname># flarcreate -n <archive name> -x /opt -c /opt/<file name>

Where: archive name is the archive identification.

Example: flarcreate -n CCPU-EMS –x /opt -c /opt/priems10_4.5.0V00.archive

Step 16   SFTP the archive to an NFS server to be used later.

- <hostname># cd /opt

- <hostname># sftp <nfs server name>

- sftp> bin

- sftp> cd <target directory>

- sftp> put <archive file name>

- sftp> put <opt tar file name>

- sftp> bye

Step 17   <hostname># mv /bin/date /bin/.date

Step 18   <hostname># mv /bin/date.orig /bin/date

Step 19   <hostname># svcadm enable system/cron

Step 20   <hostname># ps -ef | grep cron

- Verify cron daemon is running.

Step 21   <hostname># cd /etc/rc3.d

Step 22   <hostname># mv _S99platform S99platform

Step 23   <hostname># platform start

Step 24   <hostname># nodestat

- Verify EM01 is in STANDBY.

- Verify BS01 is in STANDBY.

- Verify Oracle and Billing DBs are in service.

# Task 8: Perform flash archive on CA/FS side A

In this task, you will perform flash archive on CA/FS side A to save a copy of OS and applications to a remote server. This process takes about 1 hour.

|  | Note Perform this task in parallel with Task 7: Perform Flash Archive on EMS Side A. |
| --- | --- |

# From CA/FS side A

Step 1   Log in as root

Step 2   <hostname># svcadm disable system/cron

Step 3   <hostname># ps -ef | grep cron

- Verify no result is returned, which means cron daemon is no longer running

Step 4   <hostname># cd /etc/rc3.d

Step 5   <hostname># mv S99platform _S99platform

Step 6   <hostname># platform stop all

Step 7   <hostname># nodestat

- Verify applications are out of service.

Step 8   <hostname># \rm –rf /opt/Build

Step 9   <hostname># \rm –rf /opt/8_rec

Step 10   <hostname>#\rm –rf /opt/.upgrade

Step 11   Remove all directories and files that are no longer needed such as core files, patch directories.

Step 12   <hostname># mv /bin/date /bin/date.orig

Step 13   <hostname># mv /bin/.date /bin/date

Step 14   <hostname># tar –cvf - /opt/* | gzip –c > /opt/<hostname_release>.tar.gz

Where: hostname_release is the tar file name.

Example: tar –cvf - /opt/* | gzip –c > /opt/prica10_4.5.0V00.tar.gz

Step 15   <hostname># flarcreate -n <archive name> -x /opt -c /opt/<file name>

Where: archive name is the archive identification.

Example: flarcreate -n CCPU-CA –x /opt -c /opt/prica10_4.5.0V00.archive

Step 16   SFTP the archive to an NFS server to be used later.

- <hostname># cd /opt

- <hostname># sftp <nfs server name>

- sftp> bin

- sftp> cd <target directory>

- sftp> put <archive file name>

- sftp> put <opt tar file name>

- sftp> bye

Step 17   <hostname># mv /bin/date /bin/.date

Step 18   <hostname># mv /bin/date.orig /bin/date

Step 19   <hostname># svcadm enable system/cron

Step 20   <hostname># ps -ef | grep cron

- Verify cron daemon is running.

Step 21   <hostname># cd /etc/rc3.d

Step 22   <hostname># mv _S99platform S99platform

Step 23   <hostname># platform start

Step 24   <hostname># nodestat

- Verify CAxxx is in STANDBY.

- Verify FSAINyyy is in STANDBY.

- Verify FSPTCzzz is in STANDBY.

# Task 9: Restore the system to normal mode

In this task, you will release the forced switch.

# From EMS side B

Step 1   Log in as CLI user

Step 2   CLI> control call-agent id=CAxxx; target-state=active-standby;

Step 3   CLI> control feature-server id=FSPTCyyy; target-state=active-standby;

Step 4   CLI> control feature-server id=FSAINzzz; target-state=active-standby;

Step 5   CLI> control bdms id=BDMS01; target-state=active-standby;

Step 6   CLI> control element-manager id=EM01; target-state=active-standby;

Step 7   CLI session will terminate when the EM01 switchover is successful.

# This completes the flash archive process.

Appendix G

Backout Procedure for Side B Systems

# Introduction

This procedure allows you to back out of the upgrade procedure if any verification checks (in "Verify Call/System Status" section) failed. This procedure is intended for the scenario in which the side B systems have been upgraded to the new load, while the side A systems are still at the previous load. The procedure will back out the side B systems to the previous load.

This backout procedure will:

- Restore the side A systems to active mode without making any changes to it

- Revert to the previous application load on the side B systems

- Restart the side B systems in standby mode

- Verify that the system is functioning properly with the previous load

|  | Note In addition to performing this backout procedure, you should contact Cisco Support when you are ready to retry the upgrade procedure. |
| --- | --- |

The flow for this procedure is shown in Figure F-1.

Figure F-1   Flow of Backout Procedure— Side B Only

# Task 1: Remove SCCP configuration from ITPs

Please remove all SCCP settings from ITPS by executing IOS commands. The IOS command file should already be prepared in Chapter 2, Task 3. If not, please remove the configuration manually.

# Task 2: Restore OMNI SS7 link(s) and linkset from ITP

Restore SS7 link(s) and linkset before switching over the application platforms from release 4.5.0 Primary Side A to release 3.5.4 Secondary Side B. Sample IOS command is given below:

cs7 linkset to_bts10200 7.7.7

description linkset to bts10200

link 0 Serial1/1/5:0

link 1 Serial1/1/6:0

no shut

# Task 3: Control SS7 trunk groups out of service on release 4.5.0

# From EMS side B

Step 1   If there is NO CLI script prepared, please follow steps specified in Appendix Q and then continue on Step 3.

Step 2   If there are CLI scripts prepared, log in to the Network File Server where the pre-prepared script resides. SFTP the script to EMS side B log in as CLI user and put script under /opt/ems/ftp/deposit directory.

- Wait for the batch process to pick up the script from /opt/ems/ftp/deposit directory and provision it into the system.

- Keep running “ls” command from /opt/ems/ftp/deposit directory and till the file is gone

- <hostname># cd /opt/ems/report

- <hostname># egrep Failed:0 <filename>.html

- Verify the system returns an output. If there is no output returned, please check for errors in the report file.

Step 3    Check the status of each SS7 trunk group – the admin state should be out of service:

- Log in as CLI user to EMS side B

- CLI> status trunk-grp id=<SS7 trunk group ID>;

# Task 4: Force side A CA/FS to active

This procedure will force the side A systems to forced active state, and the side B systems to forced standby state.

|  | Note In the commands below, "xxx", "yyy" or "zzz" is the instance for the process on your system. |
| --- | --- |

# From EMS side B

Step 1   Log in as CLI user.

Step 2   CLI> control call-agent id=CAxxx; target-state=active-standby;

Step 3   CLI> control feature-server id=FSPTCzzz; target-state=active-standby;

Step 4   CLI> control feature-server id=FSAINyyy; target-state=active-standby;

Step 5   CLI> exit

# Task 5: SFTP billing records to a mediation device

# From EMS side B

Step 1   Log in as root

Step 2   <hostname># platform stop

- Answer “y” if prompt for terminating the application platform

Step 3   <hostname># cd /opt/bms/ftp/billing

Step 4   <hostname># ls

Step 5   If there are files listed, then SFTP the files to a mediation device on the network and remove the files from the /opt/bms/ftp/billing directory

# Task 6: Control SS7 links in service

In this task, you will re-activate SS7 links in OMNI to fallback the system back to 3.5.4 release.

# From CA/FS side A

Step 1   Log in as root

Step 2   Activate SS7 Links on CA/FS side A.

1. <hostname># termhandler -node a7n1

- OMNI [date] #1:actv-slk:slk=< SS7 link on CA/FS side A >;

- Enter y to continue.

- Repeat above two steps for each inactive link

2. OMNI [date] #2:display-slk;

- Enter y to continue

- Verify that the state of each link is ACTIVE.

3. OMNI[date]#3:quit

# Task 7: Control SS7 trunk groups in service on release 3.5.4 side

# From EMS side A

Step 1   If there is NO CLI script prepared, please follow steps specified in Appendix R and then continue on Step 3.

Step 2   If there are CLI scripts prepared, log in to the Network File Server where the pre-prepared script resides. SFTP the script to EMS side A log in as CLI user and put script under  /opt/ems/ftp/deposit directory.

- Wait for the batch process to pick up the script from /opt/ems/ftp/deposit directory and provision it into the system.

- Keep running “ls” command from /opt/ems/ftp/deposit directory and till the file is gone

- <hostname># cd /opt/ems/report

- <hostname># egrep Failed:0 <filename>.html

- Verify the system returns an output. If there is no output returned, please check for errors in the report file.

Step 3    Check the status of each SS7 trunk group – the admin state should be in service:

- Log in as CLI user to EMS side B

- CLI> status trunk-grp id=<SS7 trunk group ID>;

# Task 8: Bounce SS7 trunks on release 3.5.4

# From EMS side A

Step 1   If there is NO CLI script prepared, please follow steps specified in Appendix Y and then continue on Step 3.

Step 2   If there are CLI scripts prepared, log in to the Network File Server where the pre-prepared script resides. SFTP the script to EMS side A log in as CLI user and put script under  /opt/ems/ftp/deposit directory.

- Wait for the batch process to pick up the script from /opt/ems/ftp/deposit directory and provision it into the system.

- Keep running “ls” command from /opt/ems/ftp/deposit directory and till the file is gone

- <hostname># cd /opt/ems/report

- <hostname># egrep Failed:0 <filename>.html

- Verify the system returns an output. If there is no output returned, please check for errors in the report file.

Step 3    Check the status of each SS7 trunk-termination – The OPER state should be in service:

- Log in as CLI user to EMS side B

- CLI> status trunk-termination id=<SS7 trunk group ID>; cic=all;

# Task 9: Sync DB usage

# From EMS side A

In this task, you will sync db-usage between two releases.

Step 1   Log in as root

Step 2   <hostname># su – oracle

Step 3   <hostname>$ java dba.adm.DBUsage –sync

- Verify Number of tables “unable-to-sync” is 0.

Step 4   <hostname>$ exit

# Task 10: Stop applications and shutdown side B systems

# From EMS side B

Step 1   Log in as root.

Step 2  <hostname># platform stop all

Step 3  <hostname># sync; sync

Step 4  <hostname># shutdown –i5 –g0 –y

# From CA/FS side B

Step 1   Log in as root.

Step 2  <hostname># platform stop all

Step 3  <hostname># sync; sync

Step 4  <hostname># shutdown –i5 –g0 –y

# Task 11: Restore side B systems to the old release

# From CA/FS side B

Step 1   Power off the machine

Step 2   Remove disk0 from slot 0 off the machine

Step 3   Place disk labeled “Release 3.5.4 CA/FS side B disk0” in slot 0

Step 4   Power on the machine

- For a Netra 1280 machine, please execute the following command from console:

<lom>poweron

- For other type of hardware, please use the power button to turn on the power.

Step 5   <hostname># platform start

Step 6   Activate SS7 Links on CA/FS side B.

1. <hostname># termhandler -node a7n1

- OMNI [date] #1:actv-slk:slk=< link on CA/FS side B >;

- Enter y to continue.

- Repeat above two steps for each inactive link

2. OMNI [date] #2:display-slk;

- Enter y to continue

- Verify that the state of each link is ACTIVE.

3. OMNI[date]#3:quit

Step 7  <hostname># mv /etc/rc3.d/_S99platform /etc/rc3.d/S99platform

# From EMS side B

Step 1   Power off the machine

Step 2   Remove disk0 from slot 0 off the machine

Step 3   Place disk labeled “Release 3.5.4 EMS side B disk0” in slot 0

Step 4   Power on the machine

- For a Netra 1280 machine, please execute the following command from console:

<lom>poweron

- For other type of hardware, please use the power button to turn on the power.

Step 5   <hostname># platform start

Step 6  <hostname># mv /etc/rc3.d/_S99platform /etc/rc3.d/S99platform

# Task 12: Restore EMS mate communication

In this task, you will restore the OMS Hub communication from EMS side A to side B.

# From EMS side A

Step 1   Log in as root

Step 2   <hostname># cd /opt/ems/utils

Step 3   <hostname># updMgr.sh –restore_hub

Step 4   <hostname># nodestat

- Verify OMS Hub mate port status is established.

- Verify HUB communication from EMS side A to CA/FS side B is established.

# Task 13: Switchover activity to EMS side B

# From Active EMS side A

Step 1   Log in as CLI user.

Step 2   CLI> control bdms id=BDMS01; target-state=forced-standby-active;

Step 3   CLI> control element-manager id=EM01; target-state=forced-standby-active;

Step 4    CLI Log in session will be terminated when switchover is completed.

# Task 14: Enable Oracle DB replication on EMS side A

# From EMS side A

Step 1   Log in as Oracle user:

<hostname># su - oracle

<hostname>$ cd /opt/oracle/admin/utl

Step2   Restore Oracle DB replication:

<hostname>$ rep_toggle –s optical1 –t set_duplex

Answer “y” when prompt

Answer “y” again when prompt

Step 3  <hostname>$ exit

Step 4   Terminate applications and all Oracle connections.

<hostname># platform stop all

Step 5   Re-start applications.

<hostname># platform start

# Task 15: Synchronize handset provisioning data

# From EMS side B

|  | Note In the commands below, "xxx", "yyy" or "zzz" is the instance for the process on your system. |
| --- | --- |

Step 1   Log in as ciscouser (password: ciscosupport)

Step 2   CLI> sync termination master=CAxxx; target=EMS;

- Verify the transaction is executed successfully.

Step 3   CLI> sync sc1d master=FSPTCzzz; target=EMS;

- Verify the transaction is executed successfully

Step 4   CLI> sync sc2d master=FSPTCzzz; target=EMS;

- Verify the transaction is executed successfully

Step 5   CLI> sync sle master=FSPTCzzz; target=EMS;

- Verify the transaction is executed successfully

Step 6   CLI> sync subscriber-feature-data master=FSPTCzzz; target=EMS;

- Verify the transaction is executed successfully

Step 7   CLI> exit

# Task 16: Switchover activity from EMS side B to EMS side A

# From EMS side B

Step 1   Log in as CLI user.

Step 2   CLI> control bdms id=BDMS01; target-state=forced-active-standby;

Step 3   CLI> control element-manager id=EM01; target-state=forced-active-standby;

Step 4    CLI Log in session will be terminated when switchover is completed.

# Task 17: Restore system to normal mode

# From EMS side A

Step 1   Log in as CLI user.

Step 2   CLI> control feature-server id=FSPTCzzz; target-state=normal;

Step 3   CLI> control feature-server id=FSAINyyy; target-state=normal;

Step 4   CLI> control call_agent id=CAxxx; target-state=normal;

Step 5   CLI> control bdms id=BDMS01; target-state=normal;

Step 6   CLI> control element-manager id=EM01; target-state=normal;

Step 7   CLI> exit

# Task 18: Verify system status

Verify that the system is operating properly before you leave the site.

Step 1   Verify that the side A systems are in the active state. Use Appendix A for this procedure.

Step 2   Verify that call processing is working without error. Use Appendix B for this procedure.

Step 3   Verify that provisioning is operational from CLI command line, and verify database. Use Appendix C for this procedure.

Step 4   Verify that there are no outstanding major or critical alarms. Use Appendix D for this procedure.

Step 5   Use Appendix E to verify that Oracle database and replication functions are working properly.

Step 6   If you answered NO to any of the above questions (Step 1 through Step 5), Contact Cisco Support for assistance.

# Task 19: Change MGCP Domain IP addresses from Domain Name Server

From each Domain Name Server that is serving the BTS 10200, restore the IP addresses for MGCP domain name from 2 physical IPs from primary CA/FS and 2 logical IPs: 2P + 2L back to 2 physical IPs from primary CA/FS and 2 physical IPs from secondary CA/FS: 2P + 2S.

You have completed the side B of Cisco BTS 10200 system fallback process successfully.

Appendix H

System Backout Procedure

# Introduction

This procedure allows you to back out of the upgrade procedure if any verification checks (in "Verify system status" section) failed. This procedure is intended for the scenario in which both the side A and side B systems have been upgraded to the new load. The procedure will back out the entire system to the previous load.

This backout procedure will:

- Revert to the previous application load on the side A systems

- Restart the side A systems and place it in active mode

- Revert to the previous application load on the side B systems

- Restart the side B systems and place it in active mode

- Verify that the system is functioning properly with the previous load

|  | Note In addition to performing this backout procedure, you should contact Cisco Support when you are ready to retry the upgrade procedure. |
| --- | --- |

# Task 1: Check MGCP Domain IP addresses from Domain Name Servers

From each Domain Name Server that is serving the upgraded BTS 10200, please make sure the IPs for the MGCP domain name in the DNS database is set to: 2P + 2L.

If not, please make the change accordingly. Then refresh the DNS cache and make sure the new IP addresses are taking affect. To verify the change has taken affect, please log in to both CA/FS machines and perform “nslookup” of the MGCP domain name.

# Task 2: Disable Oracle DB replication on EMS side B

# From Active EMS

Step 1   Log in as CLI user.

Step 2   CLI> control bdms id=BDMS01; target-state=active-standby;

Step 3   CLI> control element-manager id=EM01; target-state=active-standby;

Step 4   CLI> exit

# From EMS side B

Step 1   Log in as Oracle user:

<hostname># su – oracle

<hostname>$ cd /opt/oracle/admin/utl

Step 2   Set Oracle DB to simplex mode:

<hostname>$ rep_toggle –s optical2 –t set_simplex

Answer “y” when prompt

Answer “y” again when prompt

Step 3   Exit from Oracle Log in.

<hostname>$ exit

Step 4   Terminate applications.

<hostname># platform stop all

Step 5   Re-start applications to activate DB toggle in simplex mode.

<hostname># platform start

# Task 3: Inhibit EMS mate communication

In this task, you will isolate the OMS Hub on EMS side B from talking to CA/FS side A.

# From EMS side B

Step 1   Log in as root

Step 2   <hostname># cd /opt/ems/utils

Step 3   <hostname># updMgr.sh –split_hub

Step 4   <hostname># nodestat

- Verify there is no HUB communication from EMS side B to CA/FS side A.

# Task 4: Force side B systems to active

This procedure will force the side B systems to go active.

|  | Note In the commands below, "xxx", "yyy" or "zzz" is the instance for the process on your system. |
| --- | --- |

# From EMS side A

Step 1   Log in as CLI user.

Step 2   CLI> control call-agent id=CAxxx; target-state=standby-active;

Step 3   CLI> control feature-server id=FSPTCzzz; target-state=standby-active;

Step 4   CLI> control feature-server id=FSAINyyy; target-state=standby-active;

Step 5   CLI> control bdms id=BDMS01; target-state=standby-active;

Step 6   CLI> control element-manager id=EM01; target-state=standby-active;

Step 7   CLI session will terminate when the last CLI command completes.

# Task 5: SFTP billing records to a mediation device

# From EMS side A

Step 1   Log in as root

Step 2   <hostname># cd /opt/bms/ftp/billing

Step 3   <hostname># ls

Step 4   If there are files listed, then SFTP the files to a mediation device on the network and remove the files from the /opt/bms/ftp/billing directory

# Task 6: Stop applications

# From EMS side A

Step 1   Log in as root.

Step 2   <hostname># platform stop all

Step 3   <hostname># sync; sync

Step 4   <hostname># shutdown –i5 –g0 –y

# From CA/FS side A

Step 1   Log in as root.

Step 2   <hostname># platform stop all

Step 3   <hostname># sync; sync

Step 4   <hostname># shutdown –i5 –g0 –y

# Task 7: Restore side A systems to the old release

# From CA/FS side A

Step 1   Power off the machine

Step 2   Remove disk0 from slot 0 off the machine

Step 3   Place disk labeled “Release 3.5.4 CA/FS side A disk0” in slot 0

Step 4   Power on the machine

- For a Netra 1280 machine, please execute the following command from console:

<lom>poweron

- For other type of hardware, please use the power button to turn on the power.

Step 5   <hostname># platform start

Step 6   Activate SS7 Links on CA/FS side A.

1. <hostname># termhandler -node a7n1

- OMNI [date] #1:actv-slk:slk=< link on CA/FS side A >;

- Enter y to continue.

- Repeat above two steps for each inactive link

2. OMNI [date] #2:display-slk;

- Enter y to continue

- Verify that the state of each link is ACTIVE.

3. OMNI[date]#3:quit

#### Step 7   <hostname># mv /etc/rc3.d/_S99platform /etc/rc3.d/S99platform

# From EMS side A

Step 1   Power off the machine

Step 2   Remove disk0 from slot 0 off the machine

Step 3   Place disk labeled “Release 3.5.5 EMS side A disk0” in slot 0

Step 4   Power on the machine

- For a Netra 1280 machine, please execute the following command from console:

<lom>poweron

- For other type of hardware, please use the power button to turn on the power.

Step 5   Log in as root.

Step 6   <hostname># /opt/ems/utils/updMgr.sh –split_hub

Step 7   <hostname># /etc/rc2.d/S75cron stop

Step 8   <hostname># platform start –i oracle

Step 9   Log in as Oracle user.

<hostname># su – oracle

<hostname>$ cd /opt/oracle/admin/utl

<hostname>$ rep_toggle –s optical1 –t load_pkg

<hostname>$ rep_toggle –s optical1 –t add_toggle

Step 12   Change Oracle DB to simplex mode:

<hostname>$ rep_toggle –s optical1 –t set_simplex

- Answer “y” when prompt

- Answer “y” again when prompt

Step 13   <hostname>$ exit

Step 14   <hostname># mv /etc/rc3.d/_S99platform /etc/rc3.d/S99platform

# Task 8: Stop EMS side B billing application

# From EMS side B

Step 1   Log in as root.

Step 2   <hostname># platform stop -i BDMS01

- Answer “y” when prompt for confirmation

# Task 9: Start EMS side A applications

# From EMS side A

Step 1   <hostname># platform start

Step 2   <hostname># /etc/rc2.d/S75cron start

# Task 10: To continue fallback process, please follow Appendix G.

You have completed the Cisco BTS 10200 system fallback process successfully

Appendix I

CORBA Installation

This procedure describes how to install the Common Object Request Broker Architecture (CORBA) application on Element Management System (EMS) of the Cisco BTS 10200 Softswitch.

Note This installation process is used for both side A and side B EMS.

Caution This CORBA installation will remove existing CORBA application on EMS machines. Once you have executed this procedure, there is no backout. Do not start this procedure until you have proper authorization. If you have questions, please contact Cisco Support.

# Task 1: Open Unix Shell on EMS

Perform these steps to open a Unix shell on EMS.

Step 1   Ensure that your local PC or workstation has connectivity via TCP/IP to communicate with EMS units.

Step 2   Open a Unix shell or a XTerm window.

Note If you are unable to open a Xterm window, please contact you system administrator immediately.

# Task 2: Install OpenORB CORBA Application

### Remove Installed OpenORB Application

Step 1   Log in as root to EMS

Step 2   Enter the following command to remove the existing OpenORB package:

<hostname># pkgrm BTScis

- Respond with a “y” when prompted

<hostname># pkgrm BTSoorb

- Respond with a “y” when prompted

Step 3   Enter the following command to verify that the CORBA application is removed:

<hostname># pgrep cis3

The system will respond by displaying no data, or by displaying an error message. This verifies that the CORBA application is removed.

### Install OpenORB Packages

The CORBA application files are available for installation once the Cisco BTS 10200 Softswitch is installed.

Step 1   Log in as root to EMS

Step 2   <hostname># cd /opt/Build

Step 3   <hostname># cis-install.sh

You will get the following response, please answer appropriately:

The NameService & CIS modules listen on a specific host interface.

***WARNING*** This host name or IP address MUST resolve on the CORBA

client machine in the OSS. Otherwise, communication failures may occur.

Enter the host name or IP address [ <hostname> ]:

Step 4   If you are running EPOM or you network does not support SSLIOP (secure CORBA protocol), please answer “n”. Otherwise, answer “y” (default).

Should SSLIOP be enabled [ Y ]:

Step 5   The system will give several prompts before and during the installation process. Some prompts are repeated. Respond with a “y” when prompted.

Step 6   It will take about 5-8 minutes for the installation to complete.

Step 7   Verify CORBA Application is running On EMS:

<hostname># pgrep ins3

Note System will respond by displaying the Name Service process ID, which is a number between 2 and 32,000 assigned by the system during CORBA installation. By displaying this ID, the system confirms that the ins3 process was found and is running.

<hostname># pgrep cis3

Note The system will respond by displaying the cis3 process ID, which is a number between 2 and 32,000 assigned by the system during CORBA installation. By displaying this ID, the system confirms that the cis3 process was found and is running.

Step 8   If you do not receive both of the responses described in Step 7, or if you experience any verification problems, do not continue. Contact your system administrator. If necessary, call Cisco Support for additional technical assistance.

Appendix J

Add New Announcements

This procedure describes steps to record new announcements and place each announcement file onto proper media gateways.

# Task 1: Record Announcements

Step 1   "Your Anonymous Call Rejection service is now on. All incoming calls will be checked for number privacy before they are allowed to complete to your line". Set filename to "ann_id_540.au"

Step 2   "Your Anonymous Call Rejection service can not be successfully activated in your line. Please check with your service provider and try again.". Set filename to "ann_id_541.au"

Step 3   "Your Anonymous Call Rejection service is now off. Incoming calls will not be checked for number privacy status". Set filename to "ann_id_542.au"

Step 4   "Your Anonymous Call Rejection service can not be successfully deactivated in your line". Set filename to "ann_id_543.au"

# Task 2: Place Announcements

# From Active EMS

Step 1   Log in as CLI user.

Step 2   Show a list of announcement trunk groups.

CLI> show trunk-grp tg-type=annc;

Step 3   For each trunk group ID shown in step 2, show a list of Media Gateways being used for announcements. These media gateways are where you would place the announcement files.

CLI> show trunk tgn-id=[Trunk Group ID];

Appendix K

Associate Announcements

Associate the announcement files created in Appendix J with announcement IDs.

# Task 1: Associate announcement with route-guide

The route-guide to set is the route-guide being used for other announcements on the existing system. Do a “show route-guide” to get the list of route-guides available on the system. There should be one being used by the system.

# From Active EMS

Step 1   Log in as CLI user.

Step 2   Show a list of announcement trunk groups.

CLI> show trunk-grp tg-type=annc;

Step 3   For each trunk group ID shown in step 2, show a list of routes.

CLI> show route tgn1-id=[Trunk Group ID];

Step 4    For each route ID shown in step 3, show a list of route guides.

CLI> show route-guide policy-id=[Route ID];

- The route-guide IDs listed above now can be used for announcements in the subsequent steps. If CLI commands specified from Step 5 to Step 8 fails to execute, please choose a higher announcement ID that will work.

Step 5   CLI> add annc; id=540; type=SYSTEM; ANNOUNCEMENT_FILE=ann_id_540.au; ROUTE_GUIDE_ID=[Route Guide ID];

Step 6   CLI> add annc; id=541; type=SYSTEM; ANNOUNCEMENT_FILE=ann_id_541.au; ROUTE_GUIDE_ID=[ Route Guide ID];

Step 7   CLI> add annc; id=542; type=SYSTEM; ANNOUNCEMENT_FILE=ann_id_542.au; ROUTE_GUIDE_ID=[Route Guide ID];

Step 8   CLI> add annc; id=543; type=SYSTEM; ANNOUNCEMENT_FILE=ann_id_543.au; ROUTE_GUIDE_ID=[Route Guide ID];

# Task 2: Associate announcement with release causes

These release-causes should be provisioned as is and none of them should be changed.

# From Active EMS

Step 1   CLI> add release-cause; id=1161;  annc_id=540;

- If the above CLI command fails, execute following command:

CLI> change release-cause; id=1161; annc_id=540;

Step 2   CLI> add release-cause; id=1162; annc_id=541;

- If the above CLI command fails, execute following command:

CLI> change release-cause; id=1162; annc_id=541;

Step 3   CLI> add release-cause; id=1163; annc_id=542;

- If the above CLI command fails, execute following command:

CLI> change release-cause; id=1163; annc_id=542;

Step 4   CLI> add release-cause; id=1164; annc_id=543;

- If the above CLI command fails, execute following command:

CLI> change release-cause; id=1164; annc_id=543;

Step 5   CLI> exit

Appendix L

Created New Features

Create new features: Call Forwarding Unconditional (CFU), Call Forwarding Busy (CFB), and Call Forward No-Answer (CFNA), Call Forwarding Unconditional Interrogation (CFUI).

# From Active EMS

# Task 1: Call Forwarding Unconditional (CFU)

Step 1   Log in as CLI user.

Step 2   Add following records thru CLI to the system:

CLI> add nod-restrict-list; fname=CFU; nod=700;

CLI> add nod-restrict-list; fname=CFU; nod=900;

CLI> add nod-restrict-list; fname=CFU; nod=976;

CLI> add nod-restrict-list; fname=CFU; nod=AMBULANCE;

CLI> add nod-restrict-list; fname=CFU; nod=BUSINESS;

CLI> add nod-restrict-list; fname=CFU; nod=CUT_THRU;

CLI> add nod-restrict-list; fname=CFU; nod=DA;

CLI> add nod-restrict-list; fname=CFU; nod=DA_TOLL;

CLI> add nod-restrict-list; fname=CFU; nod=EMG;

CLI> add nod-restrict-list; fname=CFU; nod=FIRE;

CLI> add nod-restrict-list; fname=CFU; nod=INFO;

CLI> add nod-restrict-list; fname=CFU; nod=INTL_OPR;

CLI> add nod-restrict-list; fname=CFU; nod=INTL_WZ1;

CLI> add nod-restrict-list; fname=CFU; nod=NAT_OPR;

CLI> add nod-restrict-list; fname=CFU; nod=NON_EMG;

CLI> add nod-restrict-list; fname=CFU; nod=OPERATOR;

CLI> add nod-restrict-list; fname=CFU; nod=POLICE;

CLI> add nod-restrict-list; fname=CFU; nod=PREMIUM;

CLI> add nod-restrict-list; fname=CFU; nod=RELAY;

CLI> add nod-restrict-list; fname=CFU; nod=REPAIR;

CLI> add nod-restrict-list; fname=CFU; nod=SUBSCRIBER;

CLI> add nod-restrict-list; fname=CFU; nod=TIME;

CLI> add nod-restrict-list; fname=CFU; nod=TRAFFIC;

CLI> add nod-restrict-list; fname=CFU; nod=TW;

# Task 2: Call Forwarding Busy (CFB)

Step 1   Add following records thru CLI to the system:

CLI> add nod-restrict-list; fname=CFB; nod=700;

CLI> add nod-restrict-list; fname=CFB; nod=900;

CLI> add nod-restrict-list; fname=CFB; nod=976;

CLI> add nod-restrict-list; fname=CFB; nod=AMBULANCE;

CLI> add nod-restrict-list; fname=CFB; nod=BUSINESS;

CLI> add nod-restrict-list; fname=CFB; nod=CUT_THRU;

CLI> add nod-restrict-list; fname=CFB; nod=DA;

CLI> add nod-restrict-list; fname=CFB; nod=DA_TOLL;

CLI> add nod-restrict-list; fname=CFB; nod=EMG;

CLI> add nod-restrict-list; fname=CFB; nod=FIRE;

CLI> add nod-restrict-list; fname=CFB; nod=INFO;

CLI> add nod-restrict-list; fname=CFB; nod=INTL_OPR;

CLI> add nod-restrict-list; fname=CFB; nod=INTL_WZ1;

CLI> add nod-restrict-list; fname=CFB; nod=NAT_OPR;

CLI> add nod-restrict-list; fname=CFB; nod=NON_EMG;

CLI> add nod-restrict-list; fname=CFB; nod=OPERATOR;

CLI> add nod-restrict-list; fname=CFB; nod=POLICE;

CLI> add nod-restrict-list; fname=CFB; nod=PREMIUM;

CLI> add nod-restrict-list; fname=CFB; nod=RELAY;

CLI> add nod-restrict-list; fname=CFB; nod=REPAIR;

CLI> add nod-restrict-list; fname=CFB; nod=SUBSCRIBER;

CLI> add nod-restrict-list; fname=CFB; nod=TIME;

CLI> add nod-restrict-list; fname=CFB; nod=TRAFFIC;

CLI> add nod-restrict-list; fname=CFB; nod=TW;

# Task 3: Call Forward No-Answer (CFNA)

Step 1   Add following records thru CLI to the system:

CLI> add nod-restrict-list; fname=CFNA; nod=700;

CLI> add nod-restrict-list; fname=CFNA; nod=900;

CLI> add nod-restrict-list; fname=CFNA; nod=976;

CLI> add nod-restrict-list; fname=CFNA; nod=AMBULANCE;

CLI> add nod-restrict-list; fname=CFNA; nod=BUSINESS;

CLI> add nod-restrict-list; fname=CFNA; nod=CUT_THRU;

CLI> add nod-restrict-list; fname=CFNA; nod=DA;

CLI> add nod-restrict-list; fname=CFNA; nod=DA_TOLL;

CLI> add nod-restrict-list; fname=CFNA; nod=EMG;

CLI> add nod-restrict-list; fname=CFNA; nod=FIRE;

CLI> add nod-restrict-list; fname=CFNA; nod=INFO;

CLI> add nod-restrict-list; fname=CFNA; nod=INTL_OPR;

CLI> add nod-restrict-list; fname=CFNA; nod=INTL_WZ1;

CLI> add nod-restrict-list; fname=CFNA; nod=NAT_OPR;

CLI> add nod-restrict-list; fname=CFNA; nod=NON_EMG;

CLI> add nod-restrict-list; fname=CFNA; nod=OPERATOR;

CLI> add nod-restrict-list; fname=CFNA; nod=POLICE;

CLI> add nod-restrict-list; fname=CFNA; nod=PREMIUM;

CLI> add nod-restrict-list; fname=CFNA; nod=RELAY;

CLI> add nod-restrict-list; fname=CFNA; nod=REPAIR;

CLI> add nod-restrict-list; fname=CFNA; nod=SUBSCRIBER;

CLI> add nod-restrict-list; fname=CFNA; nod=TIME;

CLI> add nod-restrict-list; fname=CFNA; nod=TRAFFIC;

CLI> add nod-restrict-list; fname=CFNA; nod=TW;

# Task 4: Call Forward Unconditional Interrogation (CFUI)

Step 1   Add following records thru CLI to the system:

- CLI> add feature fname=CFUI; tdp1=COLLECTED_INFORMATION; tid1=VERTICAL_SERVICE_CODE; ttype1=R; feature_server_id=FSPTC[yyy]; description=Call Forwarding Unconditional Interrogation; grp_feature=N;

Where -- yyy is the POTS feature server instance number.

Appendix M

Updating Existing Features

you will migrate existing features to the new release.

# From EMS side A

Step 1   Log in CLI user

Step 2   CLI> show feature fname=CFUA;

- Record the value for INTL flag

- CLI> add feature FNAME=CFVABBG; FEATURE_SERVER_ID=[FSPTCzzz]; TYPE1=CC; VALUE1=N; TYPE2=FDT; VALUE2=Y; TYPE3=SDT; VALUE3=Y; TYPE4=INTL; VALUE4=[INTL value recorded]; DESCRIPTION=call forwarding activation for basic business group;

Step 3   CLI> show feature fname=CFU;

- Record the value for MCF flag and RR flag

- CLI> add feature; fname=CFVBBG; TDP1=TERMINATION_ATTEMPT_AUTHORIZED; TID1=TERMINATION_ATTEMPT_AUTHORIZED; TTYPE1=R; FNAME1=CFVABBG; FNAME2=CFUD; FNAME3=CFUI; FEATURE_SERVER_ID=[FSPTCzzz]; TYPE1=MCF; VALUE1=[MCF value recorded]; TYPE2=RR; VALUE2=[RR value recorded]; DESCRIPTION=call forwarding for basic business group; GRP_FEATURE=N;

Step 4   CLI> show service fname1=CFU;

- Record the service IDs

- For each service ID found, change existing feature CFU to the new feature CFVBBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME1=CFVBBG;

Step 5   CLI>show service fname2=CFU;

- Record the service IDs

- For each service ID found, change existing feature CFU to the new feature CFVBBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME2=CFVBBG;

Step 6   CLI>show service fname3=CFU;

- Record the service IDs

- For each service ID found, change existing feature CFU to the new feature CFVBBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME3=CFVBBG;

Step 7   CLI>show service fname4=CFU;

- Record the service IDs

- For each service ID found, change existing feature CFU to the new feature CFVBBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME4=CFVBBG;

Step 8  CLI>show service fname5=CFU;

- Record the service IDs

- For each service ID found, change existing feature CFU to the new feature CFVBBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME5=CFVBBG;

Step 9  CLI>show service fname6=CFU;

- Record the service IDs

- For each service ID found, change existing feature CFU to the new feature CFVBBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME6=CFVBBG;

Step 10 CLI>show service fname7=CFU;

- Record the service IDs

- For each service ID found, change existing feature CFU to the new feature CFVBBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME7=CFVBBG;

Step 11  CLI>show service fname8=CFU;

- Record the service IDs

- For each service ID found, change existing feature CFU to the new feature CFVBBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME8=CFVBBG;

Step 12  CLI>show service fname9=CFU;

- Record the service IDs

- For each service ID found, change existing feature CFU to the new feature CFVBBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME9=CFVBBG;

Step 13  CLI>show service fname10=CFU;

- Record the service IDs

- For each service ID found, change existing feature CFU to the new feature CFVBBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME10=CFVBBG;

Step 14  CLI>show service fname1=CFUA;

- Record the service IDs

- For each service ID found, change existing feature CFU to the new feature CFVABBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME1=CFVABBG;

Step 15  CLI>show service fname2=CFUA;

- Record the service IDs

- For each service ID found, change existing feature CFU to the new feature CFVABBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME2=CFVABBG;

Step 16 CLI>show service fname3=CFUA;

- Record the service IDs

- For each service ID found, change existing feature CFU to the new feature CFVABBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME3=CFVABBG;

Step 17  CLI>show service fname4=CFUA;

- Record the service IDs

- For each service ID found, change existing feature CFU to the new feature CFVABBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME4=CFVABBG;

Step 18 CLI>show service fname5=CFUA;

- Record the service IDs

- For each service ID found, change existing feature CFU to the new feature CFVABBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME5=CFVABBG;

Step 19 CLI>show service fname6=CFUA;

- Record the service IDs

- For each service ID found, change existing feature CFU to the new feature CFVABBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME6=CFVABBG;

Step 20 CLI>show service fname7=CFUA;

- Record the service IDs

- For each service ID found, change existing feature CFU to the new feature CFVABBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME7=CFVABBG;

Step 21 CLI>show service fname8=CFUA;

- Record the service IDs

- For each service ID found, change existing feature CFU to the new feature CFVABBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME8=CFVABBG;

Step 22 CLI>show service fname9=CFUA;

- Record the service IDs

- For each service ID found, change existing feature CFU to the new feature CFVABBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME9=CFVABBG;

Step 23 CLI>show service fname10=CFUA;

- Record the service IDs

- For each service ID found, change existing feature CFU to the new feature CFVABBG to each service record using the sample CLI command below:

- CLI>change service; id=[service id]; FNAME10=CFVABBG;

Step 24 CLI>show cdp fname=CFUA;

- Record all cdp IDs and respective DIGIT_STRING for the cdp record shown

- For each cdp ID found, change the record using the sample CLI command below:

CLI>change cdp id=[cdp id]; DIGIT_STRING=[DIGIT_STRING value recorded]; FNAME=CFVABBG

Step 25   CLI>exit

Appendix N

Setting up MTP and M3UA Configuration on the Signaling Gateway

This section describes the steps a user must complete to setup the MTP and M3UA configuration on an ITP signaling gateway. This must be completed a week before upgrading from release 3.5.4 to release 4.5.0.

# Requirements and Prerequisites

- One or two Cisco ITP signaling gateways

# Preparation

You must perform the following list of tasks in preparation for setting up the MTP and M3UA configuration:

- Obtain the SS7 configuration from the Omni stack.

- Obtain the IP addresses of the ITP(s).

- Obtain the point code of the ITP(s).

- Decide whether to use A-link or D-link configuration. Refer to Appendix V for details about these configurations.

- Obtain the IP addresses of the primary and secondary Cisco BTS 10200 call agent boxes.

- Physical link connection(s) should be made between the STP and the ITP. A linkset (and all associated links) should be added to the STP. A route should be added to the STP towards the Cisco BTS 10200 via the ITP. Note that details concerning the method to configure the STP are specific to the STP manufacturer.

Note: - All commands to be executed on the ITP are marked in Italics. - As with all IOS commands, a question mark ‘?’ can be typed at any part of the command sequence to get a listing of the possible user inputs. - For details related to the ITP please refer to ITP Configuration Guide http://www.cisco.com/en/US/products/sw/iosswrel/ps5012/products_feature_guide_book09186a00801752aa.html.

# Task 1: Define MTP Variant

In this task, you will define an MTP variant to be used by the ITP.

Step 1   Log in as root to ITP

Step 2   cs7 variant ANSI

- The format is as follows: cs7 variant <MTP variant>

# Task 2: Define Point Code

In this task, you will define the point code of the ITP. In the A-link setup, ITPs will have the same point code as the Cisco BTS 10200. In the D-link setup, each ITP will have its own point code.

Step 1   cs7 point-code 7.7.7

- The format is as follows:  cs7 point-code <local point code>

# Task 3: Define Ethernet Configuration

Step 1   interface Ethernet0/0

ip address 10.89.232.9 255.255.255.0

ip directed-broadcast

no ip mroute-cache

full-duplex

!

interface Ethernet0/1

ip address 10.89.233.41 255.255.255.0

ip directed-broadcast

no ip mroute-cache

full-duplex

!

- The format is as follows:
>interface Ethernet <interface# / interface#> {see back of ITP for labeling 0/0 or 0/1}
>ip address <local ip address> <net mask>

# Task 4: Define SGMP (When SG Mated Pair is used)

In this task, you will define the SGMP which provides redundancy for the SG mated pair.

Step 1  cs7 sgmp 9101

local-ip 10.89.232.9

local-ip 10.89.233.41

!

- The format is as follows:

cs7sgmp  <Sigtran Adaption Layer> <local port #>

local-ip <local ip address 1>

local-ip <local ip address 2>

Step 2  cs7 mated-sg ITP2 9101

remote-ip 10.89.232.10

remote-ip 10.89.233.42

- The format is as follows:

cs7mated-sg  <name of mated signaling gateway> <remote port #>

remote-ip <remote  ip address 1>

remote-ip <remote ip address 2>

# Task 5: Define M3UA Port Number

In this task, you will define the local port number used by the M3UA layer.

Step 1 cs7 m3ua 2905

local-ip 10.89.232.9

local-ip 10.89.233.41

!

- The format is as follows:

cs7 <Sigtran Adaption Layer> <port #>

local-ip <local ip address 1>

local-ip <local ip address 2>

# Task 6: Define Application Server Process (ASP)

In this task, you will define the ASPs. Two ASPs are defined for each Cisco BTS 10200 call agent, one for the primary and one for secondary.

Step 1   cs7 asp caAspPri 11146 2905 m3ua

remote-ip 10.89.225.209

remote-ip 10.89.226.209

!

- This command defines the ASP for the primary call agent. The format is as follows:
cs7 asp <aspName> <remote IP port> <local IP port> <Sigtran Adaption layer>
remote-ip <remote IP address 1>
remote-ip <remote IP address 2>

Step 2   cs7 asp caAspSec 11146 2905 m3ua

remote-ip 10.89.225.210

remote-ip 10.89.226.210

!

- This command defines the ASP for the secondary call agent

# Task 7: Define SS7 Links

In this task, you will define SS7 links. The syntax of the commands is different for a 2651 and 7505. The equivalent Omni commands are given below.

CRTE-SLK: LSET=LNK0, LSET=LSET0, SLC=0, SPEED=56K, PORT=0, CHANNEL=1;

CRTE-SLK: LSET=LNK1, LSET=LSET1, SLC=0, SPEED=56K, PORT=48, CHANNEL=1;

For 2651:

Step 1   interface Serial0/0:0

no ip address

encapsulation mtp2

bandwidth 56

no ip route-cache

no ip mroute-cache

- The format is as follows:

Interface Serial Slot/Port

encapsulation <layer 2 protocol>

bandwidth <bandwith counted by Kb>

{note that all ip related parameters are disabled by adding the ‘no’ prefix.}

Step 2   interface Serial0/1:0

no ip address

encapsulation mtp2

bandwidth 56

no ip route-cache

no ip mroute-cache

!

For 7507:

Step 1  interface Serial0/0/0

no ip address

encapsulation mtp2

bandwidth 56

no ip route-cache

no ip mroute-cache

- The format is as follows:

Interface Serial Slot/Bay/Port

encapsulation <layer 2 protocol>

{note that all ip related parameters are disabled by adding the ‘no’ prefix.}

Step 2   interface Serial0/0/1

no ip address

encapsulation mtp2

bandwidth 56

no ip route-cache

no ip mroute-cache

!

# Task 8: Define Linkset

In this task you will define linksets. The equivalent Omni commands are given below.

CRTE-LSET:LSET=LSET0,PC=20-20-20,TYPE=A;

CRTE-LSET:LSET=LSET1,PC=50-50-50,TYPE=A;

20-20-20 and 50-50-50 are the STP point codes.

Step 1  cs7 linkset lset1 20.20.20

link 0 Serial0/0

!

Step 2  cs7 linkset lset2 50.50.50

link 0 Serial0/1

- The format is as follows:

cs7linkset <lset name> <adjacent point code>

link <slc number> <serial interface>

# Task 9: Define SS7 Route Sets

In this task you will define SS7 route sets to the DPCs.

Step 1  Configure adjacent routes.

cs7 route-table system

update route 20.20.20 255.255.255 linkset lset1 priority 1

update route 50.50.50 255.255.255 linkset lset2 priority 1

update route 20.20.20 255.255.255 linkset lset2 priority 2

update route 50.50.50 255.255.255 linkset lset1 priority 2

- The format is as follows:
update route <PSTN DPC> <point code mask value> <linkset to APC> <route priority>

- These commands define adjacent routes. Note that the direct routes to the adjacent point codes are given a priority of 1 while the indirect routes are given a priority of 2.

- The equivalent Omni commands are given below:

CRTE-RSET:RSET=RSET0,PC=20-20-20,RTES=CLSET0;

ALW-RSET:RSET=RSET0;

CRTE-RSET:RSET=RSET1,PC=50-50-50,RTES=CLSET0;

ALW-RSET:RSET=RSET1;

Step 2  Configure routes for DPCs behind the STP (APC)

cs7 route-table system

update route 1.1.2 255.255.255 linkset lset1 priority 1

update route 1.1.2 255.255.255 linkset lset2 priority 1

update route 1.1.3 255.255.255 linkset lset1 priority 1

update route 1.1.3 255.255.255 linkset lset2 priority 1

!

- The format is as follows:
update route <PSTN DPC> <point code mask value> <linkset to APC> <route priority>

- These commands defines two routes of equal priority to each DPC with point codes 1.1.2, and 1.1.3 through linksets lset1 and lset2. The equivalent Omni commands are given below.

CRTE-RSET:RSET=RSET2,PC=1-1-2,RTES=CLSET0;

ALW-RSET:RSET=RSET2;

CRTE-RSET:RSET=RSET3,PC=1-1-3,RTES=CLSET0;

ALW-RSET:RSET=RSET3;

Step 3  Check the status of the links to STP (APC) and DPC

show cs7 route

- This command displays the status of the links to STPs and DPCs. All links in the system routing table should have the status of “avail”.

Step4  Save the routes

cs7 save route-table flash:routedata.txt

- This command saves the routes configured in steps 1 and 2 in the file named routedata.txt. The saved information can be used to configure another ITP.

# Task 10: Define Routing Key

In this task, you will create a routing key used by M3UA layer.

Step 1   cs7 as btsAs m3ua

routing-key 100 7.7.7

asp caAspPrimary

asp caAspSecondary

!

- The format is as follows:
cs7 as <AS Name> <Sigtran Adaption layer>
routing-key <routing context value> <Cisco BTS 10200 point code>
asp <ASP name for primary Cisco BTS 10200>
asp <ASP name for secondary Cisco BTS 10200>

- 100 is the routing context. This value must match the value defined later (Appendix S, Task 5, Step 1) on in the Cisco BTS 10200 call agent.

- Additional routing key values can be added to the routing-key command to further filter the routing from the ITP to the Cisco BTS 10200. For details concerning adding additional keys please see ITP Configuration Guide http://www.cisco.com/en/US/products/sw/iosswrel/ps5012/products_feature_guide_book09186a00801752aa.html

Appendix O

Setting up SCCP configuration on the Signaling Gateway

This section describes the tasks you must perform to setup the SCCP configuration on the ITP signaling gateway. This must be completed a week before upgrading from release 3.5.4 to release 4.5.0.

Note: All commands to be executed on the ITP are marked in Italics.

# Task 1: Define SUA Port Number

In this task, you will define the local port number used by the SUA layer.

Step 1  Log in as root to ITP

Step 2   cs7 sua 14001

local-ip 10.89.232.9

local-ip 10.82.233.41

!

# Task 2: Define ASP

In this task, you will define the ASPs. Two ASPs are defined for each Cisco BTS 10200 AIN and POTS feature servers, one for the primary and one for secondary.

Step 1   cs7 asp fsAinAspPri 12205 14001 m3ua

remote-ip 10.89.225.209

remote-ip 10.89.226.209

!

- This command defines the ASP for the primary AIN feature server

- 14001 is the local port

- 12205 is the remote port

- 10.89.225.209 and 10.89.226.209 are the IP addresses of the primary Cisco BTS 10200

Step 2  cs7 asp fsAinAspSec 12205 14001 m3ua

remote-ip 10.89.225.210

remote-ip 10.89.226.210

!

- This command defines the ASP for the secondary AIN feature server.

- 14001 is the local port

- 12205 is the remote port

- 10.89.225.210 and 10.89.226.210 are the IP addresses of the secondary Cisco BTS 10200

Step 3 cs7 asp fsPotsAspPri 12235 14001 m3ua

remote-ip 10.89.225.209

remote-ip 10.89.226.209

!

- This command defines the ASP for the secondary POTS feature server.

- 14001 is the local port

- 12235 is the remote port

- 10.89.225.209 and 10.89.226.209 are the IP addresses of the primary Cisco BTS 10200

Step 4  cs7 asp fsPotsAspSec 12235 14001 m3ua

remote-ip 10.89.225.210

remote-ip 10.89.226.210

!

- This command defines the ASP for the secondary POTS feature server.

- 14001 is the local port

- 12235 is the remote port

- 10.89.225.210 and 10.89.226.210 are the IP addresses of the secondary Cisco BTS 10200

# Task 3: Define Routing Keys for Various Services

In this task, you will define routing keys for CNAM, LNP, IN1 toll free, AIN toll free, and AC and AR services. All the routing context values associated with each routing key must match the routing context values defined later on in the Cisco BTS 10200 AIN and POTS feature servers.

Step 1   cs7 as cnamAs sua

routing-key 103 2-42-10

asp fsPtcAspPri

asp fsPtcAspSec

!

- This command defines the routing key used for CNAM service

- 103 is the routing context value

Step 2  cs7 as lnpAs sua

routing-key 101 2-42-10

asp fsAinAspPri

asp fsAinAspSec

!

- This command defines the routing key used for LNP service

- 101 is the routing context value

Step 3 cs7 as in1TfAs sua

routing-key 102 2-42-10

asp fsAinAspPri

asp fsAinAspSec

!

- This command defines the routing key used for IN1 toll free service

###### 102 is the routing context value

Step 4 cs7 as in1TfAs sua

routing-key 104 2-42-10

asp fsAinAspPri

asp fsAinAspSec

!

- This command defines the routing key used for AIN toll free service

###### 104 is the routing context value

Step 5 cs7 as acarAs sua

routing-key 105 2-42-10

asp fsPtcAspPri

asp fsPtcAspSec

!

- This command defines the routing key used for AC and AR services

###### 105 is the routing context value

# Task 4: Define GTT Configuration

In this task, you will define the GTT configuration for AC and AR service. The equivalent Omni commands are given below. All the commands have a subsystem number of 251.

CREATE-GT:TT=251,NP=ISDN-TEL,DIG="972",PC=238-14-250,SSN=251,RI=DEF;

CREATE-GT:TT=251,NP=ISDN-TEL,DIG="469",PC=238-14-251,SSN=251,RI=DEF;

CREATE-GT:TT=251,NP=ISDN-TEL,DIG="214",PC=238-14-252,SSN=251,RI=DEF;

CREATE-REMSSN: PC=238-14-250,SSN=251;

CREATE-REMSSN: PC=238-14-251,SSN=251;

CREATE-REMSSN: PC=238-14-252,SSN=251;

Step 1  conf t

cs7 gtt selector acar_sel tt 251

gta 972 pcssn 238.14.250 pcssn ssn 251

gta 469 pcssn 238.14.251 pcssn ssn 251

gta 214 pcssn 238.14.252 pcssn ssn 251

!

Step 2  Save the GTT configuration

cs7 save gtt-table flash:gttdata.txt

- This command saves the GTT configuration in the file named gttdata.txt. This information can be used to load the GTT configuration during every reload.

Step 3  Load GTT configuration from a file

cs7gtt load gtt-table flash:gttdata.txt

- This command loads the GTT configuration from the gttdata.txt file during every reload.

# Task 5: Saving the Configuration

In this task, you will save all the ITP configuration changes made so far.

Step 1 wr

Appendix P

Preparing for the SS7 Upgrade

This section describes the procedure for preparing for the upgrade. The tasks in this section must be performed right before the starting the upgrade procedure.

# Requirements and Prerequisites

- Primary and secondary Cisco BTS 10200 EMS and call agent boxes must be up and running with primary being the active and secondary being the standby.

# Preparation

You must perform the following list of tasks in preparation for the upgrade:

- Obtain the point code of the Cisco BTS 10200.

# Task 1: Define OPC and NET-AP in SS7-CIC Table

In this task, you will define the OPC and NET-AP values in the SS7-CIC table.

# From CA/FS side A

Step 1   Log in as root

Step 2   Find out the OPCs for the system.

1. <hostname># termhandler -node a7n1

2. OMNI [date] #1:display-ospc;

- Enter y to continue

- Record the OPC

# From Active EMS

Step 1   Log in as CLI user

Step 2   CLI> change ss7-cic opc=<OPC for the Cisco BTS 10200 system>; net-ap=0;

Step 3   CLI> show ss7-cic

- Verify each record with proper OPC and NET-AP value

Step 4   CLI> exit

Appendix Q

Controlling SS7 Trunk Groups Out of Service

This section describes the steps for preparing a script to control all SS7 trunk groups defined in a Cisco BTS 10200 system out of service. The trunk list must be prioritized in an order that the critical ones are controlled out of service last to reduce the SS7 outage time.

This task will accomplish:

- Identify all the SS7 trunk groups

- Prepare a CLI provisioning script to control all SS7 trunk groups out of service

# Task 1: Control SS7 Trunk Groups Out of Service

In this task, you will put all SS7 trunk groups out of service (oos).

# From Active EMS

Step 1   Log in as CLI user

Step 2   Identify SS7 trunk groups.

CLI> show trunk-grp tg_type=SS7;

Step 3   Following commands must be repeated for all SS7 trunk groups to control each SS7 trunk out of service:

CLI> control trunk-grp id=<Trunk Group ID>; mode=forced; target-state=oos;

Appendix R

Controlling SS7 Trunk Groups in service

This section describes the steps for preparing a script to control all SS7 trunk groups defined in a Cisco BTS 10200 system in service. The trunk list must be prioritized in an order that the critical ones are controlled in service first. This way it will reduce the SS7 outage time.

This task will accomplish:

- Identify all the SS7 trunk groups

- Prepare a CLI provisioning script to control all SS7 trunk groups in service

# Task 1: Control SS7 Trunk Groups in Service

In this task, you will put all SS7 trunk groups in service (ins).

# From Active EMS

Step 1   Log in as CLI user

Step 2   Identify all the SS7 trunk groups.

CLI> show trunk-grp tg_type=SS7;

Step 3   Following commands must be repeated for all SS7 trunk groups to control each SS7 trunk into service:

CLI> control trunk-grp tgn-id=<Trunk Group ID>; mode=forced; target-state=ins;

Appendix S

Provisioning Release 4.5.0 Specific Configuration on the Cisco BTS 10200 Call Agent

This chapter describes the procedure for setting up ISUP configuration on the Cisco BTS 10200 call agent.

# Requirements and Prerequisites

- Secondary Cisco BTS 10200 EMS and call agent boxes must be up and running as active on release 4.5.0.

- Primary Cisco BTS 10200 EMS and call agent boxes must be up and running as standby on release 3.5.4.

# Preparation

You must perform the following list of tasks in preparation for setting up ISUP configuration:

- Obtain the IP addresses of the ITP(s).

# Task 1: Define Signaling Gateway Components

In this task, you will define signaling gateway components, which include signaling gateway, signaling gateway process, and signaling gateway group. These components are common to call agent and feature servers and need not be defined multiple times for each platform.

# From Active EMS

Step 1   Log in as CLI user

###### Step 2   Add signaling gateway to the Cisco BTS 10200 system.

###### CLI> add sg id=sg-1; description=Signaling Gateway 1;

CLI> add sg id=sg-2; description=Signaling Gateway 2;

Step 3   Add signaling gateway process to the Cisco BTS 10200 system.

CLI> add sgp id=sgp-1; sg-id=sg-1; description=Signaling Gateway Process ITP 1;

CLI> add sgp id=sgp-2; sg-id=sg-2; description=Signaling Gateway Process ITP 2;

Step 4   Add signaling gateway group to the Cisco BTS 10200 system.

CLI> add sg-grp id=sg-grp-1; sg1-id=sg-1; sg2-id=sg-2; description=Signaling Gateway Group consisting of both sg-1 and sg-2 for redundancy;

# Task 2: Define SCTP Association

In this task, you will define SCTP association profile and SCTP association to each of the signaling gateway processes.

Step 1  CLI> add sctp-assoc-profile id=sctp-prof;

Step 2  CLI> add sctp-assoc id=ca-sctp-assoc1; sgp-id=sgp-1; sctp-assoc-profile-id=sctp-prof; platform-id=CAxxx; remote-tsap-addr1=<ITP 1 First IP Address>; remote-port=2905; dscp=AF11; ip-tos-precedence=ROUTINE;  remote-tsap-addr2=<ITP 1 Second IP Address>;

CLI> add sctp-assoc id=ca-sctp-assoc2; sgp-id=sgp-2; sctp-assoc-profile-id=sctp-prof; platform-id=CAxxx; remote-tsap-addr1=<ITP 2 First IP Address>; remote-port=2905; dscp=AF11; ip-tos-precedence=ROUTINE; remote-tsap-addr2=<ITP 2 Second IP Address>;

- Remote TSAP addresses 1 and 2 are the ITP’s redundant IP addresses.

# Task 3: Define ISUP Variant

In this task, you will define the ISUP variant. In release 3.5.4 only ANSI variant was supported.

Step 1   CLI> add user-part-variant id=ANSISS7_GR317;

# Task 4: Define OPC

In this task, you will define the OPC of Cisco BTS 10200. The equivalent Omni command is given below.

CRTE-OSPC:PC=7-7-7, NI=NAT0;

Step 1   CLI> add opc id=opc; point-code=7-7-7;

- Point code 7-7-7 is an example.

# Task 5: Define Routing Key

In this task, you will define a routing key used by the Cisco BTS 10200 call agent.

Step 1   CLI> add routing-key id=rk1; opc-id=opc; sg-grp-id=sg-grp-1; si=ISUP; rc=100; platform-id=CAxxx;

- The routing context defined by the value of the parameter “rc” must be same as the value defined in the ITP (Appendix N, Task 10, Step 1).

Step 2   Bring the SCTP associations in service.

CLI> control sctp-assoc id=ca-sctp-assoc1; target-state=ins; mode=forced;

CLI> control sctp-assoc id=ca-sctp-assoc2; target-state=ins; mode=forced;

# Task 6: Define DPC

In this task, you will define DPC. The equivalent Omni commands are given below.

CRTE-RSET:RSET=RSET2, PC=1-1-2, RTES=CLSET0;

ALW-RSET:RSET=RSET2;

CRTE-RSET:RSET=RSET3, PC=1-1-3, RTES=CLSET0;

ALW-RSET:RSET=RSET3;

Step 1  CLI> add dpc id=dpc1; point-code=1-1-2; description=DPC 1;

CLI> add dpc id=dpc2; point-code=1-1-3; description=DPC 2;

- Point code 1-1-2 and 1-1-3 are examples.

# Task 7: Define Call Control Routes

In this task, you will define call control routes to the DPCs defined in the previous task.

Step 1   CLI> add call-ctrl-route id=route-dpc1; dpc-id=dpc1; routing-key-id=rk1; si=ISUP; user-part-variant-id=ANSISS7_GR317;

CLI> add call-ctrl-route id=route-dpc2; dpc-id=dpc2; routing-key-id=rk1; si=ISUP; user-part-variant-id=ANSISS7_GR317;

Appendix T

Provisioning Release 4.5.0 Specific Configuration on the Cisco BTS 10200 Feature Server

This chapter describes the procedure for setting up SCCP configuration on the BTS feature server.

# Requirements and Prerequisites

- Secondary EMS and feature server boxes must be up and running as active on release 4.5.0.

- Primary EMS and feature server boxes must be up and running as standby on release 3.5.4.

# Task 1: Define SCTP Association

In this task, you will define the one SCTP association to each of the signaling gateway processes for AIN and POTS feature servers. The SCTP association profile is already defined as part of the call agent configuration.

From Active EMS

Step 1   Log in as CLI user

Step 2   CLI> add sctp-assoc id=ain-sctp-assoc1; sgp-id=sgp-1; sctp-assoc-profile-id=sctp-prof; platform-id=FSAINyyy; remote-tsap-addr1=<IP Address>; remote-port=14001; dscp=AF11; ip-tos-precedence=ROUTINE; remote-tsap-addr2=<IP Address>;

CLI> add sctp-assoc id=ain-sctp-assoc2; sgp-id=sgp-2; sctp-assoc-profile-id=sctp-prof; platform-id=FSAINyyy; remote-tsap-addr1=<IP Address>; remote-port=14001; dscp=AF11; ip-tos-precedence=ROUTINE; remote-tsap-addr2=<IP Address>;

- These commands add SCTP associations for the AIN feature server.

Step 3   CLI> add sctp-assoc id=pots-sctp-assoc1; sgp-id=sgp-1; sctp-assoc-profile-id=sctp_prof; platform-id=FSPTCzzz; remote-tsap-addr1=<IP Address>; remote-port=14001; dscp=AF11; ip-tos-precedence=ROUTINE;  remote-tsap-addr2=<IP Address>;

CLI> add sctp-assoc id=pots-sctp-assoc2; sgp-id=sgp-2; sctp-assoc-profile-id=sctp_prof; platform-id=FSPTCzzz; remote-tsap-addr1=<IP Address>; remote-port=14001; dscp=AF11; ip-tos-precedence=ROUTINE; remote-tsap-addr2=<IP Address>;

- These commands add SCTP associations for the POTS feature server.

Step 4   CLI> control sctp-assoc id=ain-sctp-assoc1; target-state=ins; mode=forced;

CLI> control sctp-assoc id=ain-sctp-assoc2; target-state=ins; mode=forced;

CLI> control sctp-assoc id=pots-sctp-assoc1; target-state=ins; mode=forced;

CLI> control sctp-assoc id=pots-sctp-assoc2; target-state=ins; mode=forced;

- These commands bring the SCTP associations in service.

# Task 2: Add SCCP Network

In this task, you will define a SCCP network.

Step 1   CLI> add sccp-nw id=1; net-ind=NATIONAL; sub-svc=NATIONAL; hop-count=15;

# Task 3: Add OPC in POP table

In this task, you will change the old pop table to include the opc-id. If opc-id is not added in the pop table for Rel4.x, the related features (AC, AR, Toll-Free, CNAM, LNP) will not work.

Step 1  CLI> change pop id=1; opc-id=opc;

# Task 4: Define CNAM Service

In this task, you will define a CNAM service. The equivalent Omni commands are given below. All of them have a subsystem number of 232.

CREATE-GT:TT=5,NP=ISDN-TEL,DIG="201",PC=238-3-0,SSN=232,RI=DEF;

CREATE-GT:TT=5,NP=ISDN-TEL,DIG="202",PC=238-3-0,SSN=232,RI=DEF;

CREATE-GT:TT=5,NP=ISDN-TEL,DIG="203",PC=238-3-0,SSN=232,RI=DEF;

CREATE-GT:TT=5,NP=ISDN-TEL,DIG="205",PC=238-3-0,SSN=232,RI=DEF;

Step 1   CLI> add dpc id=stp1; point-code=238-3-0; description=STP for CNAM service;

Step 2   CLI> add subsystem-grp id=ss_cnam; tcap-version=ANS92;platform-id=FSPTCzzz; description=CNAM Subsystem;

Step 3   CLI> add subsystem id=ss_cnam; opc_id=opc; local-ssn=232; remote-ssn=232; sccp-nw-id=1; sccp-version=ANS92; application-version=IN1;

Step 4   CLI> add routing-key id=rk_cnam; opc-id=opc; subsystem-grp-id=ss_cnam; sg-grp-id=sg-grp-1; si=SCCP; rc=103; platform-id=FSPTCzzz;

- The routing context defined by the value of the parameter “rc” must be same as the value defined in the ITP (Appendix O, Task 3, Step 1).

Step 5   CLI> add sccp-route opc-id=opc; dpc-id=stp1; subsystem-grp-id=ss_cnam; rk-id=rk_cnam;

Step 6   CLI> add slhr-profile id=slhr_cnam; description=Service Logic Host Routing Table for IN1 CNAM service;

Step 7   CLI> add slhr id=slhr_cnam; opc-id=opc; dpc-id=stp1; subsystem-grp-id=ss_cnam; gtt-req=Y; tt=5; gtt_addr_type=CDPN;

Step 8   CLI> add ca-config type=DEFAULT-LIDB-SLHR-ID; datatype=string; value=slhr_cnam;

Step 9   CLI> control subsystem-grp id=ss_cnam; mode=forced; target-state=uis;

- This command puts the subsystem in service.

# Task 5: Define LNP Service

In this task, you will define a LNP service. The equivalent Omni commands are given below. All of them have a subsystem number of 247.

CREATE-GT:TT=11,NP=ISDN-TEL,DIG="334",PC=238-153-0,SSN=247,RI=DEF;

CREATE-GT:TT=11,NP=ISDN-TEL,DIG="678",PC=238-153-0,SSN=247,RI=DEF;

CREATE-GT:TT=11,NP=ISDN-TEL,DIG="404",PC=238-153-0,SSN=247,RI=DEF;

CREATE-GT:TT=11,NP=ISDN-TEL,DIG="706",PC=238-153-0,SSN=247,RI=DEF;

CREATE-GT:TT=11,NP=ISDN-TEL,DIG="770",PC=238-153-0,SSN=247,RI=DEF;

Step 1   CLI> add dpc id=stp2; point-code=238-153-0; description=STP for LNP service;

Step 2   CLI> add subsystem-grp id=ss_lnp; tcap-version=ANS92; platform-id=FSAINyyy; description=LNP Subsystem;

Step 3   CLI> add subsystem id=ss_lnp; opc_id=opc; local-ssn=247; remote-ssn=247; sccp-nw-id=1; sccp-version=ANS92; application-version=AIN01;

Step 4   CLI> add routing-key id=rk_lnp; opc-id=opc; subsystem-grp-id=ss_lnp; sg-grp-id=sg-grp; si=SCCP; rc=101; platform-id=FSAINyyy;

- The routing context defined by the value of the parameter “rc” must be same as the value defined in the ITP (Appendix O, Task 3, Step 2).

Step 5   CLI> add sccp-route opc-id=opc; dpc-id=stp2; subsystem-grp-id=ss_lnp; rk-id=rk_lnp;

Step 6   CLI> add slhr-profile id=slhr_lnp; description=Service Logic Host Routing Table for AIN LNP service;

Step 7   CLI> add slhr id=slhr_lnp; opc-id=opc; dpc-id=stp2; subsystem-grp-id=ss_lnp; gtt-req=Y; tt=11;

Step 8   CLI> add ca-config type=DEFAULT-LNP-SLHR-ID; datatype=string; value=slhr_lnp;

Step 9   CLI> control subsystem-grp id=ss_lnp; mode=forced; target-state=uis;

- This command puts the subsystem in service.

# Task 6: Define IN1 Toll Free Service

In this task, you will define IN1 toll free service. The equivalent Omni commands are given below. All of them have a subsystem number of 254.

CREATE-GT:TT=254,NP=ISDN-TEL,DIG="800",PC=238-14-254,SSN=254,RI=DEF;

CREATE-GT:TT=254,NP=ISDN-TEL,DIG="866",PC=238-14-254,SSN=254,RI=DEF;

CREATE-GT:TT=254,NP=ISDN-TEL,DIG="888",PC=238-14-254,SSN=254,RI=DEF;

CREATE-GT:TT=254,NP=ISDN-TEL,DIG="877",PC=238-14-254,SSN=254,RI=DEF;

Step 1   CLI> add dpc id=stp3; point-code=238-14-254; description=STP for Toll Free service;

Step 2   CLI> add subsystem-grp id=ss_tf; tcap-version=ANS92; platform-id=FSAINyyy; description=Toll Free Subsystem;

Step 3   CLI> add subsystem id=ss_tf; opc_id=opc; local-ssn=254; remote-ssn=254; sccp-nw-id=1; sccp-version=ANS92; application-version=IN1;

Step 4   CLI> add routing-key id=rk_tf; opc-id=opc; subsystem-grp-id=ss_tf; sg-grp-id=sg-grp; si=SCCP; rc=102; platform-id=FSAINyyy;

- The routing context defined by the value of the parameter “rc” must be same as the value defined in the ITP (Appendix O, Task 3, Step 3).

Step 5   CLI> add sccp-route opc-id=opc; dpc-id=stp3; subsystem-grp-id=ss_tf; rk-id=rk_tf;

Step 6   CLI> add slhr-profile id=slhr_tf; description=Service Logic Host Routing Table for IN1 Toll Free service;

Step 7   CLI> add slhr id=slhr_tf; opc-id=opc; dpc-id=stp3; subsystem-grp-id=ss_tf; gtt-req=Y; tt=254;

Step 8   CLI> add ca-config type=DEFAULT-TOLL-FREE-SLHR-ID; datatype=string; value=slhr_tf;

Step 9   CLI> control subsystem-grp id=ss_tf; mode=forced; target-state=uis;

- This command puts the subsystem in service.

# Task 7: Define AIN Toll Free Service

In this task, you will define an AIN toll free service. The equivalent Omni commands are given below. All of them have a subsystem number of 248.

CREATE-GT:TT=8,NP=ISDN-TEL,DIG="800",PC=238-14-253,SSN=248,RI=DEF;

CREATE-GT:TT=8,NP=ISDN-TEL,DIG="866",PC=238-14-253,SSN=248,RI=DEF;

CREATE-GT:TT=8,NP=ISDN-TEL,DIG="888",PC=238-14-253,SSN=248,RI=DEF;

CREATE-GT:TT=8,NP=ISDN-TEL,DIG="877",PC=238-14-253,SSN=248,RI=DEF;

Step 1   CLI> add dpc id=stp4; point-code=238-14-253; description=STP for Toll Free service;

Step 2   CLI> add subsystem-grp id=ss_aintf; tcap-version=ANS92; platform-id=FSAINyyy; description=Toll Free Subsystem;

Step 3   CLI> add subsystem id=ss_aintf; opc_id=opc; local-ssn=248; remote-ssn=248; sccp-nw-id=1;  sccp-version=ANS92; application-version=IN1;

Step 4   CLI> add routing-key id=rk_aintf; opc-id=opc; subsystem-grp-id=ss_ aintf; sg-grp-id=sg-grp; si=SCCP; rc=104; platform-id=FSAINyyy;

- The routing context defined by the value of the parameter “rc” must be same as the value defined in the ITP (Appendix O, Task 3, Step 4).

Step 5   CLI> add sccp-route opc-id=opc; dpc-id=stp4; subsystem-grp-id=ss_aintf; rk-id=rk_aintf;

Step 6   CLI> add slhr-profile id=slhr_aintf; description=Service Logic Host Routing Table for IN1 Toll Free service;

Step 7   CLI> add slhr id=slhr_aintf; opc-id=opc; dpc-id=stp4; subsystem-grp-id=ss_aintf; gtt-req=Y; tt=8;

Step 8   CLI> add ca-config type=DEFAULT-TOLL-FREE-SLHR-ID; datatype=string; value=slhr_aintf;

Step 9   CLI> control subsystem-grp id=ss_aintf; mode=forced; target-state=uis;

- This command puts the subsystem in service.

# Task 8: Define AC and AR Services

In this task, you will define AC and AR services. The equivalent Omni commands are defined below. All of them have a subsystem number of 251.

CREATE-GT:TT=251,NP=ISDN-TEL,DIG="972",PC=238-14-250,SSN=251,RI=DEF;

CREATE-GT:TT=251,NP=ISDN-TEL,DIG="469",PC=238-14-251,SSN=251,RI=DEF;

CREATE-GT:TT=251,NP=ISDN-TEL,DIG="214",PC=238-14-252,SSN=251,RI=DEF;

……

CREATE-GT:TT=251,NP=ISDN-TEL,DIG="919",PC=238-10-201,SSN=251,RI=DEF;

……

CREATE-REMSSN: PC=238-14-250,SSN=251;

CREATE-REMSSN: PC=238-14-251,SSN=251;

CREATE-REMSSN: PC=238-14-252,SSN=251;

……

CREATE-REMSSN: PC=238-10-201,SSN=251;

……

CREATE-GT:TT=251,NP=ISDN-TEL,DIG="512",PC=2-42-10,SSN=251,RI=DEF;

Step 1   CLI> add dpc id=itp1; point-code=7-7-7; description=ITP for AC and AR services;

Step 2   CLI> add subsystem-grp id=ss_acar; tcap-version=ANS92; platform-id=FSPTCzzz; description=AC AR Subsystem;

Step 3   CLI> add subsystem id=ss_acar; opc_id=opc; local-ssn=251; remote-ssn=251; sccp-nw-id=1;  sccp-version=ANS92; application-version=IN1;

Step 4   CLI> add routing-key id=rk_acar; opc-id=opc; subsystem-grp-id=ss_acar; sg-grp-id=sg-grp; si=SCCP; rc=105; platform-id=FSPTCzzz;

- The routing context defined by the value of the parameter “rc” must be same as the value defined in the ITP (Appendix O, Task 3, Step 5).

Step 5   CLI> add sccp-route opc-id=opc; dpc-id=itp1; subsystem-grp-id=ss_acar; rk-id=rk_acar;

Step 6   CLI> add slhr-profile id=slhr_acar; description=Service Logic Host Routing Table for IN1 Toll Free service;

Step 7   CLI> add slhr id=slhr_acar; opc-id=opc; dpc-id=itp1; subsystem-grp-id=ss_acar; gtt-req=Y; tt=251;

Step 8   CLI> add ca-config type=ACAR -SLHR-ID; datatype=string; value=slhr_acar;

Step 9   CLI> add dpc id=sw_mask1; point-code=238-14-0; description=ITP for AC and AR services;

Step 10   CLI> add dpc id=sw1; point-code=238-10-201; description=ITP for AC and AR services;

Step 11   CLI> add sccp-route opc-id=opc; dpc-id=sw_mask; subsystem-grp-id=ss_acar; rk-id=rk_acar;

Step 12   CLI> add sccp-route opc-id=opc; dpc-id=sw1; subsystem-grp-id=ss_acar; rk-id=rk_acar;

Step 13   CLI> control subsystem-grp id=ss_acar; mode=forced; target-state=uis;

- This command puts the subsystem in service.

Appendix U

Testing the SS7 Upgrade

This chapter describes the procedure for testing to check if the SS7 part of the upgrade from release 3.5.4 to release 4.5.0 was successful.

# Requirements and Prerequisites

- Secondary EMS and feature server boxes must be up and running as active on release 4.5.0.

- Primary EMS and feature server boxes must be up and running as standby on release 3.5.4.

- Release 4.5.0 specific configuration should be provisioned on the Cisco BTS 10200 call agent and feature servers.

# Task 1: Check the Status of SCTP Associations

In this task, you will check the status of SCTP associations.

# From Active EMS

Step 1   Log in as CLI user

Step 2  CLI> status sctp-assoc id=ca-sctp-assoc1;

CLI> status sctp-assoc id=ca-sctp-assoc2;

CLI> status sctp-assoc id=ain-sctp-assoc1;

CLI> status sctp-assoc id=ain-sctp-assoc2;

CLI> status sctp-assoc id=pots-sctp-assoc1;

CLI> status sctp-assoc id=pots-sctp-assoc2;

- The admin state of all SCTP associations should be ADMIN_INS and operational state should be “SCTP-ASSOC in service”.

# Task 2: Check the Status of the Signaling Gateway Processes

In this task, you will check the status of signaling gateway processes.

Step 1  CLI> status sgp id=sgp-1;

CLI> status sgp id=sgp-2;

- The operational state returned by the call agent and feature server platforms should be “SGP in service”.

# Task 3: Check the Status of DPCs

In this task, you will check the status of DPCs associated with the call agent and AIN feature server.

Step 1  CLI> status dpc id=dpc1; platform-id=CAxxx;

CLI> status dpc id=dpc2; platform-id=CAxxx;

CLI> status dpc id=stp1; platform-id=FSAINyyy;

CLI> status dpc id=stp2; platform-id=FSAINyyy;

CLI> status dpc id=stp3; platform-id=FSAINyyy;

CLI> status dpc id=stp4; platform-id=FSAINyyy;

CLI> status dpc id=itp1; platform-id=FSAINyyy;

- The operational state of all the DPCs should be “DPC available”.

# Task 4: Check the Status of Trunk Terminations

In this task, you will check the status of all the SS7 trunk terminations.

Step 1   CLI> status tt tgn-id=1; cic=all;

- All trunks should have an admin state of ADMIN_INS (3rd column) and operation state of NON_FAULTY (7th column).

- This command should be repeated for all SS7 trunk groups.

# Task 5: Check the Status of Subsystems

In this task, you will check the status of various subsystems.

Step 1  CLI> status subsystem id=ss_cnam; opc-id=opc;

CLI> status subsystem id=ss_lnp; opc-id=opc;

CLI> status subsystem id=ss_tf; opc-id=opc;

CLI> status subsystem id=ss_aintf; opc-id=opc;

CLI> status subsystem id=ss_acar; opc-id=opc;

- The operational state of all subsystems should be “Subsystem allowed”.

# Task 6: Make a SS7 Call

In this task, you will make a subscriber to SS7 call to verify the SS7 upgrade.

Step 1   From a subscriber line, dial a number, which is handled by one of the DPCs and verify if the call completes successfully.

Appendix V

# D-Link Configuration

The following diagram illustrates a ‘D’ link configuration when an ITP is utilized

Figure 1:  Cisco BTS 10200 release 4.5.0 D link configuration

The D link configuration has advantages and disadvantages compared to the A link configuration. The advantages are:

- The Signaling Gateways can be geographically separated (even after distributed MTP3 is implemented, the ‘A’ link configuration will not allow two SGPs to be geographically separated  - in separate cities for example).

- Multiple small (2651) SGs can currently be utilized for hardware redundancy.

The disadvantage of the ‘D’ link configuration is that each SG has to have its own point code.

Appendix W

# Preparing Disks for Upgrade

This software upgrade will need 8 disks: 2 for each machine. Each set of 2 disks must have the same model number in order for disk mirroring to work.

The NIDS information required for disk preparation must be different from that used on the system to be upgraded.

# Task 1: Locate and label the disks

# Label disks for EMS Side A

Locate two new disk drives identical to the machine to be upgraded and label the first disk drive as “Release 4.5.0 EMS side A disk0, 1” and the second disk drive as “Release 4.5.0 EMS side A disk0, 2”.  Please follow the steps below to prepare the two disk drives. The second disk drive will be used as the backup in case the first disk drive goes bad.

# Label disks for EMS Side B

Locate two new disk drives identical to the machine to be upgraded and label the first disk drive as “Release 4.5.0 EMS side B disk0, 1” and the second disk drive as “Release 4.5.0 EMS side B disk0, 2”.  Please follow the steps below to prepare the two disk drives. The second disk drive will be used as the backup in case the first disk drive goes bad.

# Label disks for CA/FS Side A

Locate two new disk drives identical to the machine to be upgraded and label the first disk drive as “Release 4.5.0 CA/FS side A disk0, 1” and the second disk drive as “Release 4.5.0 CA/FS side A disk0, 2”.  Please follow the steps below to prepare the two disk drives. The second disk drive will be used as the backup in case the first disk drive goes bad.

# Label disks for CA/FS Side B

Locate two new disk drives identical to the machine to be upgraded and label the first disk drive as “Release 4.5.0 CA/FS side B disk0, 1” and the second disk drive as “Release 4.5.0 CA/FS side B disk0, 2”.  Please follow the steps below to prepare the two disk drives. The second disk drive will be used as the backup in case the first disk drive goes bad.

# Task 2: Disk slot lay out

- Netra 240 disk slot lay out:

| Disk 0 | Disk 1 |
| --- | --- |
| DVD-ROM |  |

- SunFire V240 disk slot lay out:

| Disk 2 | Disk 3 |  |
| --- | --- | --- |
| Disk 0 | Disk 1 | DVD-ROM |

- SunFire V440 disk slot lay out:

| Disk 3 | DVD-ROM |
| --- | --- |
| Disk 2 |  |
| Disk 1 |  |
| Disk 0 |  |

- Netra 1280 disk slot lay out:

|  | DVD-ROM |
| --- | --- |
|  | Disk 1 |
|  | Disk 0 |

- Netra 20 disk slot lay out:

| D I S K 0 | D I S K 1 |  | DVD ROM |
| --- | --- | --- | --- |

- Continuous Hardware disk slot lay out:

| CD-ROM | Disk 0 | Disk 2 |
| --- | --- | --- |
|  | Disk 1 | Disk 3 |

# Task 3: Construct opticall.cfg

Step 1   Get a copy of Cisco BTS 10200 Software Release 4.5.0 Network Information Data Sheets (NIDS)

Step 2   Fill in NIDS information for the BTS system used for disk preparation. The NIDS information must be different from that used on the system to be upgraded.

Step 3   Get a copy of Cisco BTS 10200 Software Release 4.5.0 opticall.cfg

Step 4   Fill in parameters in opticall.cfg from NIDS and placed it on a server which will be accessible from the BTS system used for disk preparation.

# Task 4: Disk preparation

Repeat the steps in Task 4 to prepare the second disk.

Note:  Please perform disk preparation for each machine in parallel.

# For both EMS side A and B

Step 1   Locate a system with the identical hardware as the machine to be upgraded

Step 2   Put disk to disk slot 0

Step 3   Jumpstart the machine with Solaris 10 OS

Step 4   Configured the machine with 4/2 network configuration

Step 5   Stage Cisco BTS 10200 Software Release 4.5.0 to the /opt/Build directory.

Step 6   ftp opticall.cfg from the server (where the file was placed in Task 3) and place it under /etc directory.

<hostname># /opt/Build/checkCFG

  - To verify the information entered are free of errors.

Note: The EMS side A and side B installation must be started in parallel.

Step 7   Install Cisco BTS 10200 Software Release 4.5.0 application software.

<hostname># /opt/Build/install.sh

  - Answer “y” when prompt. This installation process could take up to 1hour and 30 minutes.

  - Answer "y” when prompt for “reboot”

  - Wait for the system to boot up. Then Log in as root

Step 8   <hostname># mv /etc/rc3.d/S99platform /etc/rc3.d/_S99platform

Step 9   Remove all network interface configuration information from the machine:

<hostname>#  \rm /etc/hostname.*

Step 10   Shutdown the system

<hostname># platform stop all

<hostname># sync;sync

<hostname># shutdown –i5 –g0 -y

Step 11   Power off the machine and remove the disks to be used for upgrade.

# For both CA/FS side A and B

Step 1   Locate a system with the identical hardware as the machine to be upgraded

Step 2   Put disk to disk slot 0

Step 3   Jumpstart the machine with Solaris 10 OS

Step 4   Configured the machine with 4/2 network configuration

Step 5   Stage Cisco BTS 10200 Software Release 4.5.0 to the /opt/Build directory.

Step 6   ftp opticall.cfg from the server (where the file was placed in Task 3) and place it under /etc directory.

Step 7   Install BTSbase, BTSinst, BTSossh and BTShard packages:

- <hostname># cd /opt/Build

- <hostname># pkgadd –d . BTSbase

- Answer “y” when prompt

- <hostname># pkgadd –d . BTSinst

- Answer “y” when prompt

- <hostname># pkgadd –d . BTSossh

- Answer “y” when prompt

- <hostname># pkgadd –d . BTShard

- Answer “y” when prompt

- <hostname># cd /etc/rc3.d

- <hostname># mv S99platform _S99platform

- <hostname># shutdown –i6 –g0 -y

- Wait for system to boot up and login as root.

Step 8   Remove all network interface configuration information from the machine:

<hostname>#  \rm /etc/hostname.*

Step 9   Update version information:

<hostname># cd /opt/ems/utils

<hostname># echo “900-04.04.00.V00” > Version

Step 10   Shutdown the system

<hostname>#  shutdown –i5 –g0 -y

Step 11   Power off the machine and remove the disks to be used for upgrade.

Appendix X

# Disk Mirroring after Upgrade

It takes about two to two and half hours to complete the disk mirroring process on each machine. Please schedule the task accordingly. Cisco recommends a separate maintenance window to complete the upgrade process.

The disk mirroring process assumes the Applications on Primary side A are in Active state and Secondary Side B are in standby state.

# Task 1: Configuring the Secondary EMS Side B

Step 1   Login as root.

Step 2	 <hostname># /opt/setup/setup_mirror_ems

Step 3    <hostname># reboot -- -r

Step 4    Wait for the system to boot, then log in to EMS Side B as root.

Step 5    <hostname># /opt/setup/sync_mirror

Note Wait for the disks to synchronize before continuing. The synchronization can be verified by running the Resync_status command from the /opt/utils directory. The display will show the resyncing in progress and reports resync completion.

Step 6    Exit EMS Side B

# Task 2: Configuring the Secondary CA/FS Side B

Step 1   Login as root.

Step 2	 <hostname># /opt/setup/setup_mirror_ca

Step 3    <hostname># reboot -- -r

Step 4    Wait for the system to boot, then log in to CA/FS Side B as root.

Step 5    <hostname># /opt/setup/sync_mirror

Note Wait for the disks to synchronize before continuing. The synchronization can be verified by running the Resync_status command from the /opt/utils directory. The display will show the resyncing in progress and reports resync completion.

Step 6	  Exit CA/FS Side B

# Task 3: Switchover Applications to the Secondary Side

# From Active EMS Side A

Step 1   Log in as CLI user

Step 2   CLI> control call-agent id=CAxxx; target-state=standby-active;

Step 3   CLI> control feature-server id=FSPTCyyy; target-state=standby-active;

Step 4   CLI> control feature-server id=FSAINzzz; target-state=standby-active;

Step 5   CLI> control bdms id=BDMS01; target-state=standby-active;

Step 6   CLI> control element-manager id=EM01; target-state=standby-active;

Step 7   CLI> exit

# Task 4: Configuring the Primary EMS Side A

Step 1   Login as root.

Step 2	 <hostname># /opt/setup/setup_mirror_ems

Step 3    <hostname># reboot -- -r

Step 4    Wait for the system to boot, then log in to EMS Side A as root.

Step 5    <hostname># /opt/setup/sync_mirror

Note Wait for the disks to synchronize before continuing. The synchronization can be verified by running the Resync_status command from the /opt/utils directory. The display will show the resyncing in progress and reports resync completion.

Step 6    Exit EMS Side A

# Task 5: Configuring the Primary CA/FS Side A

Step 1   Login as root.

Step 2	 <hostname># /opt/setup/setup_mirror_ca

Step 3    <hostname># reboot -- -r

Step 4    Wait for the system to boot, then log in to CA/FS Side A as root.

Step 5    <hostname># /opt/setup/sync_mirror

Note Wait for the disks to synchronize before continuing. The synchronization can be verified by running the Resync_status command from the /opt/utils directory. The display will show the resyncing in progress and reports resync completion.

Step 6   Exit CA/FS Side A

# Task 6: Restore the System to Normal Mode

# From Active EMS Side A

Step 1   Log in as CLI user

Step 2   CLI> control call-agent id=CAxxx; target-state=active-standby;

Step 3   CLI> control feature-server id=FSPTCyyy; target-state=active-standby;

Step 4   CLI> control feature-server id=FSAINzzz; target-state=active-standby;

Step 5   CLI> control bdms id=BDMS01; target-state=active-standby;

Step 6   CLI> control element-manager id=EM01; target-state=active-standby;

Step 7   CLI> exit

This completes the disk mirroring process

Appendix Y

Bounce SS7 Trunks

This section describes the steps for preparing a script to bounce SS7 trunks. The trunk list must be prioritized in an order that the critical ones are controlled out of service last and controlled in service first. This way it will reduce the SS7 outage time.

This task will accomplish:

- Identify all the SS7 trunks

- Prepare a CLI provisioning script to bounce SS7 trunks

# Task 1: Bounce SS7 Trunks

You’ll identify SS7 trunks, control each trunk out of service, unequip, equip, in service.

# From Active EMS

Step 1   Log in as CLI user

Step 2   Identify all the SS7 trunk groups.

CLI> show trunk-grp tg_type=SS7;

Step 3   Following commands must be repeated for all SS7 trunk groups:

CLI> control trunk-termination tgn-id=<Trunk Group ID>; cic=all; mode=forced; target-state=oos;

CLI> unequip trunk-termination tgn-id=<Trunk Group ID>; cic=all;

CLI> equip trunk-termination tgn-id=<Trunk Group ID>; cic=all;

CLI> control trunk-termination tgn-id=<Trunk Group ID>; cic=all; mode=forced; target-state=ins;