Document Number 	EDCS-572658

Revision		5.0

Cisco BTS 10200 Softswitch Software Upgrade for Release

4.4.0 to 4.5.1 MR1

March 21, 2007

Corporate Headquarters

Cisco Systems, Inc.

170 West Tasman Drive

San Jose, CA 95134-1706

USA

http://www.cisco.com

Tel: 	408 526-4000

800 553-NETS (6387)

Fax:	408 526-4100

THE SPECIFICATIONS AND INFORMATION REGARDING THE PRODUCTS IN THIS MANUAL ARE SUBJECT TO CHANGE WITHOUT NOTICE. ALL STATEMENTS, INFORMATION, AND RECOMMENDATIONS IN THIS MANUAL ARE BELIEVED TO BE ACCURATE BUT ARE PRESENTED WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED. USERS MUST TAKE FULL RESPONSIBILITY FOR THEIR APPLICATION OF ANY PRODUCTS.

THE SOFTWARE LICENSE AND LIMITED WARRANTY FOR THE ACCOMPANYING PRODUCT ARE SET FORTH IN THE INFORMATION PACKET THAT SHIPPED WITH THE PRODUCT AND ARE INCORPORATED HEREIN BY THIS REFERENCE. IF YOU ARE UNABLE TO LOCATE THE SOFTWARE LICENSE OR LIMITED WARRANTY, CONTACT YOUR CISCO REPRESENTATIVE FOR A COPY.

The Cisco implementation of TCP header compression is an adaptation of a program developed by the University of California, Berkeley (UCB) as part of UCB’s public domain version of the UNIX operating system. All rights reserved. Copyright © 1981, Regents of the University of California.

NOTWITHSTANDING ANY OTHER WARRANTY HEREIN, ALL DOCUMENT FILES AND SOFTWARE OF THESE SUPPLIERS ARE PROVIDED “AS IS” WITH ALL FAULTS. CISCO AND THE ABOVE-NAMED SUPPLIERS DISCLAIM ALL WARRANTIES, EXPRESSED OR IMPLIED, INCLUDING, WITHOUT LIMITATION, THOSE OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT OR ARISING FROM A COURSE OF DEALING, USAGE, OR TRADE PRACTICE.

IN NO EVENT SHALL CISCO OR ITS SUPPLIERS BE LIABLE FOR ANY INDIRECT, SPECIAL, CONSEQUENTIAL, OR INCIDENTAL DAMAGES, INCLUDING, WITHOUT LIMITATION, LOST PROFITS OR LOSS OR DAMAGE TO DATA ARISING OUT OF THE USE OR INABILITY TO USE THIS MANUAL, EVEN IF CISCO OR ITS SUPPLIERS HAVE BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES.

CCIP, CCSP, the Cisco Arrow logo, the Cisco Powered Network mark, the Cisco Systems Verified logo, Cisco Unity, Follow Me Browsing, FormShare, iQ Breakthrough, iQ FastTrack, the iQ Logo, iQ Net Readiness Scorecard, Networking Academy, ScriptShare, SMARTnet, TransPath, and Voice LAN are trademarks of Cisco Systems, Inc.; Changing the Way We Work, Live, Play, and Learn, The Fastest Way to Increase Your Internet Quotient, and iQuick Study are service marks of Cisco Systems, Inc.; and Aironet, ASIST, BPX, Catalyst, CCDA, CCDP, CCIE, CCNA, CCNP, Cisco, the Cisco Certified Internetwork Expert logo, Cisco IOS, the Cisco IOS logo, Cisco Press, Cisco Systems, Cisco Systems Capital, the Cisco Systems logo, Empowering the Internet Generation, Enterprise/Solver, EtherChannel, EtherSwitch, Fast Step, GigaStack, Internet Quotient, IOS, IP/TV, iQ Expertise, LightStream, MGX, MICA, the Networkers logo, Network Registrar, Packet, PIX, Post-Routing, Pre-Routing, RateMUX, Registrar, SlideCast, StrataView Plus, Stratm, SwitchProbe, TeleRouter, and VCO are registered trademarks of Cisco Systems, Inc. and/or its affiliates in the U.S. and certain other countries.

All other trademarks mentioned in this document or Web site are the property of their respective owners. The use of the word partner does not imply a partnership relationship between Cisco and any other company. (0301R)

Cisco BTS 10200 Softswitch Software Upgrade

Copyright © 2005, Cisco Systems, Inc.

All rights reserved.

| Revision History |  |  |
| --- | --- | --- |
| Date | Version | Description |
| 01/29/2007 | 1.0 | - Initial Version |
| 03/09/2007 | 2.0 | - Added IP00 Patch Procedure |
| 03/21/2007 | 5.0 | - Added IP01 Patch Procedure |

Table of Contents

Table of Contents	4

Table of Contents	4

Preface	10

Obtaining Documentation	10

World Wide Web	10

Documentation CD-ROM	10

Ordering Documentation	10

Documentation Feedback	11

Obtaining Technical Assistance	11

Cisco.com	11

Technical Assistance Center	12

Cisco TAC Web Site	12

Cisco TAC Escalation Center	13

Chapter 1	14

Upgrade Requirements	14

Introduction	14

Assumptions	15

Requirements	15

Important notes about this procedure	16

Chapter 2	18

Preparation	18

Referenced documents	18

Prerequisites	19

. Chapter 3	20

Chapter 3	21

Complete one week before the scheduled upgrade	21

Task 1: Add new domain names to DNS	21

Task 2: Pre-construct opticall.cfg for the system to be upgraded to 4.5.1 MR1 release	22

Task 3: Verify value of CA CONTROL PORT for IVR devices connected to BTS. 	23

Note : The value of the CA_CONTROL_PORT for IVR devices connected to BTS should not be 0.	23

From Active EMS	23

Task 4: Verify Default routes	27

27

Note : Verify that there are a total of 4 default routes on CA (including primary and secondary) and ensure that they are on the correct (signaling) VLANS.	27

Chapter 4	28

Complete the night before the scheduled upgrade	28

Task 1: Check mlhg_terminal table	28

From Active EMS	28

Task 2: Check SLE table	28

From Active EMS	29

Task 3: Check Feature table	29

From Active EMS	29

Task 4: Check Feature table and Service-trigger table	29

From Active EMS	29

Task 5: Save customized cron jobs	31

From Each BTS machine	31

Task 6 : Perform full database audit	31

Note : Depending on the size of the database the audit could take multiple hours to complete. This task should be completed the night before the upgrade.	31

Chapter 5	32

Prepare System for Upgrade	32

Task 1: Verify System Status	32

Task 2: Perform database row count audit (sanity check)	32

Task 3: Alarms.	33

Verify that there are no outstanding major or critical alarms. Use Appendix D for this procedure.	33

Task 4: Oracle Database and Replication.	33

Use Appendix E to verify that Oracle database and replication functions are working properly.	33

Task 5: Backup user account	33

From EMS Side A	33

Task 6: Verify and record Virtual IP (VIP) information	34

From EMS Side A	34

Task 7: Verify and record VSM Macro information	34

From EMS Side A	35

Chapter 6	36

Upgrade Side B Systems	36

Task 1: Block provisioning path	36

Task 2: Disable Oracle DB replication	36

From Active EMS	36

From EMS side A	36

Task 3: Force side A systems to active	37

From Active EMS Side B	37

Task 4: Inhibit EMS mate communication	38

From EMS side A	38

Task 5: Stop applications and shutdown EMS side B	38

From EMS side B	38

Task 6: Stop applications and shutdown CA/FS side B	39

From CA/FS side B	39

Task 7: Upgrade EMS side B to the new release	39

From EMS side B	39

Task 8: Upgrade CA/FS Side B to the new release	45

From CA/FS side B	45

Task 9: Migrate oracle data	50

From EMS side B	50

Chapter 7	52

Prepare Side A Systems for Upgrade	52

Task 1: Force side A systems to standby	52

From EMS side A	52

Task 2: Check and Correct Feature Server and Call Agent TSAP address	53

From EMS side B	53

Task 3: Sync Data from EMS side B to CA/FS side B	55

From EMS side B	55

Task 4:  Block Provisioning Path	55

From EMS side B	55

Task 5: Validate release 4.5.1 MR1 software operation	56

From EMS side B	56

Task 6: Ftp billing records off the system	57

From EMS side A	57

Chapter 8	58

Upgrade Side A Systems	58

Task 1: Stop applications and shutdown EMS side A	58

From EMS side A	58

Task 2: Stop applications and shutdown CA/FS side A	58

From CA/FS side A	58

Task 3: Upgrade EMS side A to the new release	59

From EMS side A	59

Task 4: Upgrade CA/FS Side A to the new release	63

From CA/FS side A	63

Task 5: Restore Hub communication	67

From EMS side B	67

Task 6: Copying oracle data	68

From EMS side A	68

Task 7: Verify Hub communication	69

From EMS side B	69

Chapter 9	70

Finalizing Upgrade	70

Task 1: Switchover activity from side B to side A	70

From EMS side B	70

Task 2: Enable Oracle DB replication on EMS side B	70

From EMS side B	70

Task 3 : Enable Provisioning Path	71

From EMS side B	71

Task 4: Check and correct sctp-assoc table	71

From EMS side A	72

Task 5: Synchronize handset provisioning data	72

From EMS side A	72

Task 6: SFTP hosts and opticall.cfg file	73

Task 7: Restore customized cron jobs	73

From EMS side A	73

From EMS side B	74

From CA side B	74

Task 8: Reconfigure VSM Macro information	75

Task 9: To install CORBA on EMS side A and B, please follow Appendix I.	76

Task 10: Verify system status	76

Appendix A	77

Check System Status	77

From Active EMS side A	77

Appendix B	80

Check Call Processing	80

From EMS side A	80

Appendix C	82

Check Provisioning and Database	82

From EMS side A	82

Check transaction queue	82

Perform database audit	83

Appendix D	84

Check Alarm Status	84

From EMS side A	84

Appendix E	86

Check Oracle Database Replication and Error Correction	86

Check Oracle DB replication status	86

From EMS side A	86

Correct replication error	87

From EMS Side B	87

From EMS Side A	88

Appendix F	89

Backout Procedure for Side B Systems	89

Introduction	89

Task 1: Force side A CA/FS to active	90

From EMS side B	91

Task 2: Force side A EMS to active 	91

From EMS side B	91

Task 3: Sync TSAP-ADDR  	91

From EMS side A	91

Task 4: SFTP billing records to a mediation device	91

From EMS side B	92

Task 5: Sync DB usage	92

From EMS side A	92

Task 6: Stop applications and shutdown side B systems	92

From EMS side B	92

From CA/FS side B	93

Task 7: Restore side B systems to the old release	93

From CA/FS side B	93

From EMS side B	94

Task 8: Restore EMS mate communication	95

From EMS side A	95

Task 9: Switchover activity to EMS side B	95

From Active EMS side A	95

Task 10: Enable Oracle DB replication on EMS side A	95

From EMS side A	96

Task 11: Synchronize handset provisioning data	96

From EMS side B	96

Task 12: Switchover activity from EMS side B to EMS side A	97

From EMS side B	97

Task 13: Restore system to normal mode	97

From EMS side A	97

Task 14: Verify system status	98

Appendix G	99

Software Upgrade Disaster Recovery Procedure	99

Assumptions	99

Requirements	100

Important notes about this procedure	100

System Disaster Recovery Procedure	101

Task 1: Shutdown each machine	101

Task 2: Restore CA/FS side B to the old release	103

Task 3: Bring up applications on CA/FS side B	104

Task 4: Restore EMS side B to the old release	104

Task 5: Verify system health	104

Task 6: Restore CA/FS side A to the old release	106

Task 7: Restore EMS side A to the old release	107

Task 8:Enable Bulk Provisioning from EMS side A & B	109

Task 9: Verify system status	109

Appendix H	112

Preparing Disks for Upgrade	112

Task 1: Locate CD-ROM Discs	112

Task 2: Locate and label the Disks	112

Label disks for EMS Side A	112

Label Disks for EMS Side B	113

Label Disks for CA/FS Side A	113

Label Disks for CA/FS Side B	113

Task 3: Disk slot lay out	113

Task 4: Construct opticall.cfg	114

Task 5: Disk preparation	114

For both EMS side A and B	114

For both CA/FS side A and B	118

Appendix I	121

CORBA Installation	121

Task 1: Open Unix Shell on EMS	121

Task 2: Install OpenORB CORBA Application	121

Remove Installed OpenORB Application	121

Install OpenORB Packages	122

Appendix J	124

Block Provisioning Path	124

From EMS side A and B	124

Appendix K	126

Files handled by DoTheChange script	126

Appendix L	128

Disable and Enable Radius Server	128

Task 1: Disable Radius Server	128

From Each Machine	128

Task 2: Enable Radius Server	128

From Each Machine	128

Appendix M	129

Install IP01 Patch on EMS	129

Appendix N	130

Install IP01 Patch on CA	130

Backout Procedure for IP01 Patch.	131

Preface

# Obtaining Documentation

These sections explain how to obtain documentation from Cisco Systems.

### World Wide Web

You can access the most current Cisco documentation on the World Wide Web at this URL: http://www.cisco.com/

Translated documentation is available at this URL: http://www.cisco.com/public/countries_languages.shtml

### Documentation CD-ROM

Cisco documentation and additional literature are available in a Cisco Documentation CD-ROM package, which is shipped with your product. The Documentation CD-ROM is updated monthly and may be more current than printed documentation. The CD-ROM package is available as a single unit or through an annual subscription.

### Ordering Documentation

You can order Cisco documentation in these ways:

Registered Cisco.com users (Cisco direct customers) can order Cisco product documentation from the Networking Products MarketPlace: http://www.cisco.com/cgi-bin/order/order_root.pl

Registered Cisco.com users can order the Documentation CD-ROM through the online Subscription Store: http://www.cisco.com/go/subscription

Nonregistered Cisco.com users can order documentation through a local account representative by calling Cisco Systems Corporate Headquarters (California, U.S.A.) at 408 526-7208 or, elsewhere in North America, by calling 800 553-NETS (6387).

### Documentation Feedback

You can submit comments electronically on Cisco.com. In the Cisco Documentation home page, click the Fax or Email option in the “Leave Feedback” section at the bottom of the page.

You can e-mail your comments to mailto:bug-doc@cisco.com.

You can submit your comments by mail by using the response card behind the front cover of your document or by writing to the following address:

Cisco Systems, INC.

Attn: Document Resource Connection

170 West Tasman Drive

San Jose, CA 95134-9883

# Obtaining Technical Assistance

Cisco provides Cisco.com as a starting point for all technical assistance. Customers and partners can obtain online documentation, troubleshooting tips, and sample configurations from online tools by using the Cisco Technical Assistance Center (TAC) Web Site. Cisco.com registered users have complete access to the technical support resources on the Cisco TAC Web Site: http://www.cisco.com/tac

### Cisco.com

Cisco.com is the foundation of a suite of interactive, networked services that provides immediate, open access to Cisco information, networking solutions, services, programs, and resources at any time, from anywhere in the world.

Cisco.com is a highly integrated Internet application and a powerful, easy-to-use tool that provides a broad range of features and services to help you with these tasks:

Streamline business processes and improve productivity

Resolve technical issues with online support

Order Cisco learning materials and merchandise

Register for online skill assessment, training, and certification programs

If you want to obtain customized information and service, you can self-register on Cisco.com. To access Cisco.com, go to this URL: http://www.cisco.com/

### Technical Assistance Center

The Cisco Technical Assistance Center (TAC) is available to all customers who need technical assistance with a Cisco product, technology, or solution. Two levels of support are available: the Cisco TAC Web Site and the Cisco TAC Escalation Center.

Cisco TAC inquiries are categorized according to the urgency of the issue:

Priority level 4 (P4)—You need information or assistance concerning Cisco product capabilities, product installation, or basic product configuration.

Priority level 3 (P3)—Your network performance is degraded. Network functionality is noticeably impaired, but most business operations continue.

Priority level 2 (P2)—Your production network is severely degraded, affecting significant aspects of business operations. No workaround is available.

Priority level 1 (P1)—Your production network is down, and a critical impact to business operations will occur if service is not restored quickly. No workaround is available.

The Cisco TAC resource that you choose is based on the priority of the problem and the conditions of service contracts, when applicable.

### Cisco TAC Web Site

You can use the Cisco TAC Web Site to resolve P3 and P4 issues yourself, saving both cost and time. The site provides around-the-clock access to online tools, knowledge bases, and software. To access the Cisco TAC Web Site, go to this URL: http://www.cisco.com/tac

All customers, partners, and resellers who have a valid Cisco service contract have complete access to the technical support resources on the Cisco TAC Web Site. The Cisco TAC Web Site requires a Cisco.com Log in ID and password. If you have a valid service contract but do not have a Log in ID or password, go to this URL to register: http://www.cisco.com/register/

If you are a Cisco.com registered user, and you cannot resolve your technical issues by using the Cisco TAC Web Site, you can open a case online by using the TAC Case Open tool at this URL: http://www.cisco.com/tac/caseopen

If you have Internet access, we recommend that you open P3 and P4 cases through the Cisco TAC Web Site: http://www.cisco.com/tac

### Cisco TAC Escalation Center

The Cisco TAC Escalation Center addresses priority level 1 or priority level 2 issues. These classifications are assigned when severe network degradation significantly impacts business operations. When you contact the TAC Escalation Center with a P1 or P2 problem, a Cisco TAC engineer automatically opens a case.

To obtain a directory of toll-free Cisco TAC telephone numbers for your country, go to this URL: http://www.cisco.com/warp/public/687/Directory/DirTAC.shtml

Before calling, please check with your network operations center to determine the level of Cisco support services to which your company is entitled: for example, SMARTnet, SMARTnet Onsite, or Network Supported Accounts (NSA). When you call the center, please have available your service agreement number and your product serial number.

Chapter 1

Upgrade Requirements

# Introduction

Application software loads are designated as Release 900-aa.bb.cc.Vxx, where

- aa=major release number, for example, 01

- bb=minor release number, for example, 03

- cc=maintenance release, for example, 00

- Vxx=Version number, for example V04

This procedure can be used on an in-service system, but the steps must be followed as shown in this document in order to avoid traffic interruptions.

|  | Caution Performing the steps in this procedure will bring down and restart individual platforms in a specific sequence. Do not perform the steps out of sequence, as it could affect traffic. If you have questions, contact Cisco Support. |
| --- | --- |

This procedure should be performed during a maintenance window.

Note   In this document, the following designations are used:

- EMS -- Element Management System

- CA/FS -- Call Agent / Feature Server

- Primary -- Also referred to as "Side A"

- Secondary -- Also referred to as "Side B"

# Assumptions

The following assumptions are made.

- The installer has a basic understanding of UNIX and Oracle commands.

- The installer has the appropriate user name(s) and password(s) to log on to each EMS/CA/FS platform as root user, and as Command Line Interface (CLI) user on the EMS.

|  | Note Contact Cisco Support before you start if you have any questions. |
| --- | --- |

# Requirements

Verify that opticall.cfg has the correct information for each of the following machines.

- Side A EMS

- Side B EMS

- Side A CA/FS

- Side B CA/FS

Please follow the steps below:

- Make sure the opticall.cfg file have the exact same content on all four machines

- Verify the information is correct by running: /opt/ems/utils/checkCFG

Determine the oracle and root passwords for the systems you are upgrading. If you do not know these passwords, ask your system administrator.

Refer to local documentation to determine if CORBA installation is required on this system. If unsure, ask your system administrator.

# Important notes about this procedure

Throughout this procedure, each command is shown with the appropriate system prompt, followed by the command to be entered in bold. The prompt is generally one of the following:

- Host system prompt (<hostname>#)

- Oracle prompt (<hostname>$)

- SQL prompt (SQL>)

- CLI prompt (CLI>)

- SFTP prompt (sftp>)

Note the following conventions used throughout the steps in this procedure:

- Enter commands as shown, as they are case sensitive (except for CLI commands).

- Default BTS user (for example btsuser, btsadmin, ciscouser) attributes are reset to factory defaults of the release to which the system is being upgraded.

It is recommended that you read through the entire procedure before performing any steps.

It will take approximately 6-7 hours to complete the entire upgrade process for a system with about 80K subscribers. Please plan accordingly to minimize any negative service impacts.

Please be aware that when mid-upgrade point is reached and that is when side Bs are upgraded to the new release and became active, you should spend no more than one hour run the necessary mid-upgrade checks and complete the side A upgrade ASAP to avoid any negative affects.

This procedure must be run thru its entirety in one maintenance window.

CDR delimiter customization is not retained after software upgrade. The customer or Cisco engineer must manually customize again to keep the same customization.

There will be no CLI provisioning allowed during entire upgrade process.

Chapter 2

Preparation

This chapter describes the tasks a user must complete at least two weeks before the scheduled upgrade.

Each customer must purchase 8 disk drives with matching disk size to the existing system that is to be upgraded.

Cisco highly recommends two sets of 8 mirrored disks (16 disks in total) should be prepared for each system. The second set of disks will serve as a backup in case there is disk failure in the first set. Then the second set can be rotated to upgrade other systems.

# Referenced documents

Please go to Cisco CCO Web site below to access BTS documentations: http://www.cisco.com/univercd/cc/td/doc/product/voice/bts10200/index.htm

- Site Preparation and Network Communications Requirements: http://www.cisco.com/univercd/cc/td/doc/product/voice/bts10200/bts4_5/install/site45.htm

- Cisco BTS 10200 Network Site Survey (Release 4.5): http://www.cisco.com/univercd/cc/td/doc/product/voice/bts10200/bts4_5/install/bts45nss.doc

- Cisco BTS 10200 NIDS Generator (Release 4.5): http://www.cisco.com/univercd/cc/td/doc/product/voice/bts10200/bts4_5/install/nids_45.xls

- Cisco BTS 10200 CD Jumpstart Procedure for Solaris 10: http://www.cisco.com/univercd/cc/td/doc/product/voice/bts10200/bts4_5/install/sol10cdj.doc

- Cabling, VLAN, and IRDP Setup Procedure for 4-2 Configuration (Release 4.4.x): http://www.cisco.com/univercd/cc/td/doc/product/voice/bts10200/bts4_4/install/r44cbl.doc

# Prerequisites

- Create or allocate a BTS system based on exactly the same Sun hardware as the BTS to be upgraded with a fully functioning BTS network including DNS, NTP, and IRDP.

- Each BTS will require 8 extra disks with matching disk size to swap with the existing system during the upgrade.  The disks taken out can then be recycled.

- Four disk drives jumpstarted with Solaris 10 with the other four as mirror disks. Disks must be prepared in a hardware platform that matches the target system.  Please refer to Appendix H disk preparation details.

- Two disk drives for EMS side A as a mirrored pair. The first disk is the primary disk and second disk is a mirrored disk. Disks should have:

- Jumpstarted with Solaris 10 OS.

- Staged with BTS 10200 Software Release 4.5.1

- Installed EMS application software and databases

- Two disk drives for EMS side B as a mirrored pair. The first disk is the primary disk and second disk is a mirrored disk. Disks should have:

- Jumpstarted with Solaris 10 OS

- Staged with BTS 10200 Software Release 4.5.1

- Installed EMS application software and databases

- Two disk drives for CA/FS side A as a mirrored pair. The first disk is the primary disk and second disk is a mirrored disk. Disks should have:

- Jumpstarted with Solaris 10 OS

- Staged with BTS 10200 Software Release 4.5.1 MR1

- Two disk drives for CA/FS side B as a mirrored pair. The first disk is the primary disk and second disk is a mirrored disk. Disks should have:

- Jumpstarted with Solaris 10 OS

  - Staged with BTS 10200 Software Release 4.5.1 MR1

- Locate CD-ROM Disc labeled as “BTS 10200 Application”

- Locate CD-ROM Disc labeled as “BTS 10200 Database”

- Locate CD-ROM Disc labeled as “BTS 10200 Oracle Engine”

- Locate CD-ROM Disc labeled as “BTS_04_05_01_V24_IP01”

- There is secure shell (ssh) access to the Cisco BTS 10200 system.

- There is console access to the Cisco BTS 10200 system.

- Verify the target system to be upgraded has the latest 4.4.0 release deployed and the most recent patches applied if any. Please contact Cisco support if you are not sure what patch level the system is on.

- A Network File Server (NFS) with at least 20 GB disk space is accessible from the Cisco BTS 10200 system to store system archives, backups, and configuration files.

- H323_ENABLED Parameter:

  - If the parameter H323_ENABLED is set to “y” in opticall.cfg on 4.5.1, make sure that the following parameter returns TWO LOGICAL IP addresses.

DNS_FOR_CA146_H323_COM=h3a-SYS08CA146.ipclab.cisco.com

  - In case of fallback from 4.5.1 to 4.4.x make sure that the following parameter returns ONE LOGICAL IP address on 4.4.x

DNS_FOR_CA146_H323_COM=h3a-SYS08CA146.ipclab.cisco.com

.

Chapter 3

# Complete one week before the scheduled upgrade

This chapter describes the tasks a user must complete one week before the scheduled upgrade.

# Task 1: Add new domain names to DNS

This task must be performed on Domain Name Servers that are serving the Cisco BTS 10200 system.

The following three new 4.5.1 domain names are used to replace three 4.4.0 domain names used by existing 4.4.0 processes. Three 4.4.0 domain names return 4 Physical IP addresses.  By changing the 4 physical IP with 2 floating logical IP, BTS will be seen as one single entity.

Step 1   Log in to Domain Name Servers for Cisco BTS 10200

Step 2   Add domain names for the following opticall.cfg parameters to Domain Name Server database where xxx – is the application instance number specific to the site.

- DNS_FOR_CAxxx_SIM_COM

Note This is a qualified DNS name used by SIM process in Call Agent for LOCAL communication. Each name resolves to two logical IP addresses in the same subnet as the third virtual interface of management network(s). Each instance must have a unique DNS name and two uniquely associated LOGICAL IP addresses. They must NOT be the same as the domain names of POTS and ASM. (i.e. DNS_FOR_FSPTCxxx_POTS_COM and DNS_FOR_FSAINxxx_ASM_COM).

- DNS_FOR_FSAINxxx_ASM_COM

Note This is a qualified DNS name used by ASM process in AIN Feature Server for LOCAL communication. Each name should return two logical IP addresses of AIN Feature Server which must be in the same subnet as the third virtual interface of management network(s). Each instance must have a unique DNS name and two uniquely associated LOGICAL IP addresses. They must NOT be the same as the domain names of SIM and POTS. (i.e. DNS_FOR_CAxxx_SIM_COM and DNS_FOR_FSPTCxxx_POTS_COM).

- DNS_FOR_FSPTCxxx_POTS_COM

Note This is fully qualified DNS name used by POTS process in Feature Server FSPTC for LOCAL communication. Each name should return two logical IP addresses of a POTS/CENTRIX Feature Server which match the subnet of the third virtual interface of management network(s). Each instance must have a unique DNS name and two uniquely associated LOGICAL IP addresses. They must NOT be the same as the domain names of SIM and ASM (i.e. DNS_FOR_CAxxx_SIM_COM and DNS_FOR_FSAINxxx_ASM_COM).

# Task 2: Pre-construct opticall.cfg for the system to be upgraded to 4.5.1 MR1 release

Step 1   Get a copy of the completed Network Information Data Sheets (NIDS)

Step 2   Use following CCO link to the Cisco BTS 10200 NIDS Generator for Release 4.5 to generate the opticall.cfg file.

http://www.cisco.com/univercd/cc/td/doc/product/voice/bts10200/bts4_5/install/nids_45.xls

Note : Opticall.cfg generated from the 4.5.1 NIDS must be compared with the existing 4.4.0 opticall.cfg in the existing system.

Step 3   Place the file on the Network File Server (NFS)

Note   New parameters added to the 4.5.1 release:

Where xxx – is the application instance number specific to the site.

- NSCD_ENABLED

- CAxxx_LAF_PARAMETER

- FSPTCxxx_LAF_PARAMETER

- FSAINxxx_LAF_PARAMETER

- EMS_LAF_PARAMETER

- BDMS_LAF_PARAMETER

- DNS_FOR_CAxxx_SIM_COM

- DNS_FOR_FSAINxxx_ASM_COM

- DNS_FOR_FSPTCxxx_POTS_COM

- SHARED_MEMORY_BACKUP_START_TIME_CA

- SHARED_MEMORY_BACKUP_START_TIME_FSAIN

- SHARED_MEMORY_BACKUP_START_TIME_FSPTC

- SHARED_MEMORY_BACKUP_START_TIME_EMS

- SHARED_MEMORY_BACKUP_START_TIME_BDMS

- BILLING_FILENAME_TYPE

For further information and descriptions of the above parameters, please refer to opticall.cfg.

# Task 3: Verify value of CA CONTROL PORT for IVR devices connected to BTS.

# Note : The value of the CA_CONTROL_PORT for IVR devices connected to BTS should not be 0.

Note : IVR services may be interrupted during this task.

# From Active EMS

Step 1   Login as “btsuser” User.

Step 2   Find the ANNC trunk-grp. The one with the main subscriber should be an IVR trunk-grp

CLI > show trunk_grp tg-type=ANNC;

ID=80031

CALL_AGENT_ID=CA146

TG_TYPE=ANNC

NUM_OF_TRUNKS=30

TG_PROFILE_ID=ivr-ipunity

STATUS=INS

DIRECTION=BOTH

SEL_POLICY=ASC

GLARE=SLAVE

ALT_ROUTE_ON_CONG=N

SIGNAL_PORTED_NUMBER=N

MAIN_SUB_ID=806-888-2000

DEL_DIGITS=0

TRAFFIC_TYPE=LOCAL

ANI_BASED_ROUTING=N

MGCP_PKG_TYPE=ANNC_CABLE_LABS

ANI_SCREENING=N

SEND_RDN_AS_CPN=N

STATUS_MONITORING=N

SEND_EARLY_BKWD_MSG=N

EARLY_BKWD_MSG_TMR=5

SCRIPT_SUPP=N

VOICE_LAYER1_USERINFO=AUTO

VOICE_INFO_TRANSFER_CAP=AUTO

PERFORM_LNP_QUERY=N

Step 3:  Ensure that subscriber CATEGORY is set to IVR.

CLI>show sub id=806-888-2000

ID=806-888-2000

CATEGORY=IVR

NAME=tb06 806-888-2000

STATUS=ACTIVE

DN1=8068882000

PRIVACY=NONE

RING_TYPE_DN1=1

TGN_ID=80031

PIC1=NONE

PIC2=NONE

PIC3=NONE

GRP=N

USAGE_SENS=N

SUB_PROFILE_ID=tb06-ivr-1

TERM_TYPE=ROUTE

POLICY_ID=80031

IMMEDIATE_RELEASE=N

TERMINATING_IMMEDIATE_REL=N

SEND_BDN_AS_CPN=N

SEND_BDN_FOR_EMG=N

SEND_BDN_AS_CPN=N

SEND_BDN_FOR_EMG=N

PORTED_IN=N

BILLING_TYPE=NONE

VMWI=Y

SDT_MWI=Y

Step 4: Use the trunk-grp id to find the trunks that have the IVR gateway ID

CLI>show trunk tgn-id=80031;

ID=1

TGN_ID=80031

TERM_ID=ivr/1

MGW_ID=ipunity-227-103

ID=2

TGN_ID=80031

TERM_ID=ivr/2

MGW_ID=ipunity-227-103

ID=3

TGN_ID=80031

TERM_ID=ivr/3

MGW_ID=ipunity-227-103

ID=4

TGN_ID=80031

TERM_ID=ivr/4

MGW_ID=ipunity-227-103

ID=5

TGN_ID=80031

TERM_ID=ivr/5

MGW_ID=ipunity-227-103

ID=6

TGN_ID=80031

TERM_ID=ivr/6

MGW_ID=ipunity-227-103

Step 5: Show IVR Gateway to verify CALL_AGENT_CONTROL PORT value

CLI > show mgw id = ipunity-227-103

ID=ipunity-227-13

TSAP_ADDR=ms-ipunity.ipclab.cisco.com

CALL_AGENT_ID=CA146

MGW_PROFILE_ID=ivr-ipunity

STATUS=INS

CALL_AGENT_CONTROL_PORT=0

TYPE=TGW

Reply : Success: Entry 1 of 1 returned.

Note: If the CALL_AGENT_CONTROL_PORT is set to 0 continue to step 6. If the value of the CALL_AGENT_CONTROL_PORT set to a value other than 0 like 2427 or 2428, do not execute further steps in this task.

Step 6: Control the IVR gateway to OOS state.

Note: Make sure that no calls exist on these trunks associated to the gateway.

CLI > status tt tgn-id=<IVR trunk-grp id>; cic=all.

80031  1   ADMIN_INS     TERM_ACTIVE_IDLE      ACTV   IDLE   NON_FAULTY

80031  2   ADMIN_INS     TERM_ACTIVE_IDLE      ACTV   IDLE   NON_FAULTY

80031  3   ADMIN_INS     TERM_ACTIVE_IDLE      ACTV   IDLE   NON_FAULTY

In the example output above note that the state of the endpoint/CIC is IDLE.

CLI>control mgw id=ipunity-227-103;mode=forced;target_state=oos;

MGW ID -> ipunity-227-103

INITIAL STATE -> ADMIN_INS

REQUEST STATE -> ADMIN_OOS

RESULT STATE -> ADMIN_OOS

FAIL REASON -> ADM found no failure

REASON -> ADM executed successfully

RESULT -> ADM configure result in success

Step 7: Change the CALL_AGENT_CONTROL_PORT for the IVR gateway

CLI>change mgw id=ipunity-227-103;CALL_AGENT_CONTROL_PORT =2427

Step 8: Control the IVR gateway to the INS state.

CLI>control mgw id=ipunity-227-103;mode=forced;target_state=ins

MGW ID -> ipunity-227-103

INITIAL STATE -> ADMIN_OOS

REQUEST STATE -> ADMIN_INS

RESULT STATE -> ADMIN_INS

FAIL REASON -> ADM found no failure

REASON -> ADM executed successfully

RESULT -> ADM configure result in success

Step 9: Control the trunk-termination for the IVR gateway to the INS state.

CLI>control tt tgn_id=80031;mode=forced;cic=all; target-state=ins

REQUEST STATE -> ADMIN_INS

RESULT STATE -> ADMIN_INS

FAIL REASON -> ADM found no failure

REASON -> ADM executed successfully

RESULT -> ADM configure result in success

TGN ID -> 80031

CIC START -> 1

CIC END -> 30

Step 10: Make sure the IVR trunk-terminations are in the INS state.

CLI>status tt tgn-id=80031;cic=all

80031  1   ADMIN_INS     TERM_ACTIVE_IDLE      ACTV   IDLE   NON_FAULTY

80031  2   ADMIN_INS     TERM_ACTIVE_IDLE      ACTV   IDLE   NON_FAULTY

80031  3   ADMIN_INS     TERM_ACTIVE_IDLE      ACTV   IDLE   NON_FAULTY

80031  4   ADMIN_INS     TERM_ACTIVE_IDLE      ACTV   IDLE   NON_FAULTY

80031  5   ADMIN_INS     TERM_ACTIVE_IDLE      ACTV   IDLE   NON_FAULTY

80031  6   ADMIN_INS     TERM_ACTIVE_IDLE      ACTV   IDLE   NON_FAULTY

80031  7   ADMIN_INS     TERM_ACTIVE_IDLE      ACTV   IDLE   NON_FAULTY

80031  8   ADMIN_INS     TERM_ACTIVE_IDLE      ACTV   IDLE   NON_FAULTY

80031  9   ADMIN_INS     TERM_ACTIVE_IDLE      ACTV   IDLE   NON_FAULTY

CLI > Exit

# Task 4: Verify Default routes

# Note : Verify that there are a total of 4 default routes on CA (including primary and secondary) and ensure that they are on the correct (signaling) VLANS.

Issue the following command for CA (primary and secondary).

<hostname># netstat –rn  |grep default

Example output.

default              10.89.225.254        UG        1   1003

default              10.89.226.254        UG        1    836

Caution: Call Processing application will not start, if there are other than 4 default routes or are not on the correct (signaling) VLANS.

Chapter 4

# Complete the night before the scheduled upgrade

This chapter describes the tasks a user must complete the night before the scheduled upgrade.

If root access is disabled, to enable, please use Appendix L Task 1 to enable the root access.

CLI provisioning activity should be suspended before running the following pre-upgrade DB integrity checks. If any of these commands fail, please contact Cisco support.

# Task 1: Check mlhg_terminal table

# From Active EMS

Step 1   <hostname># su – oracle

Step 2    <hostname>$ sqlplus optiuser/optiuser

Step 3    sql> select term_id, mlhg_id, mgw_id from mlhg_terminal group by term_id,mlhg_id,mgw_id having count(*) > 1;

Please check:

- Check for duplicated records with TERM_ID, MLHG_ID, MGW_ID

- If there is any record shown from the above query, remove the duplicated records from CLI. Failure to do so will result in an upgrade failure.

# Task 2: Check SLE table

# From Active EMS

Step 1    sql> select fname from sle where fname not in

('SCA','SCR','SCF','DRCW','NSA');

Please check:

- If the above query returns any record, you either have to delete the sle record or upgrade the sle record with a valid fname from CLI. Failure to do so will result in an upgrade failure.

# Task 3: Check Feature table

# From Active EMS

Step 1   sql> select tid1, tid2, tid3 from feature where tid1 not in (select tid from trigger_id) or tid2 not in (select tid from trigger_id) or tid3 not in (select tid from trigger_id);

Please check:

- If the above query returns any record, you either have to change or remove each feature record returned. Failure to do so will result in an upgrade failure.

# Task 4: Check Feature table and Service-trigger table

# From Active EMS

Step 1   From Oracle DB:

sql> select tid from service_trigger where tid not in (select tid from trigger_id);

Please check:

- If the above query returns any record, you either have to change or remove each service-trigger record returned. Failure to do so will result in an upgrade failure.

Step 2   Exit from Oracle:

sql> quit;

<hostname>$ exit

Step 3   Log in as “btsuser” user

Step 4   CLI> show feature tid1=<TRIGGER_ID>

Step 5   CLI> show feature tid2=<TRIGGER_ID>

Step 6   CLI> show feature tid3=<TRIGGER_ID>

Step 7   CLI> show service-trigger tid=<TRIGGER_ID>

Where <TRIGGER_ID> is any one of the following obsolete triggers:

ORIGINATION_ATTEMPT
O_ABANDON
O_ANSWER
O_CALLED_PARTY_BUSY
O_DISCONNECT
O_EXCEPTION
O_NOT_REACHABLE
O_NO_ANSWER
O_REANSWER
O_SUSPEND
ROUTE_SELECTED_FAILURE
T_ABANDON_DP
T_DISCONNECT
T_EXCEPTION
T_NOT_REACHABLE
T_REANSWER
T_SUSPEND

Please check:

- If the above show commands return any record, you either have to change or remove of each TRIGGER_ID record returned. Failure to do so will result in an upgrade failure.

Step 8   CLI> exit

# Task 5: Save customized cron jobs

This upgrade process requires disk replacement. Because of this, all customized cron jobs in the system will be lost. Please save the cron jobs to your network file servers to be restored once the entire system is upgraded to the 4.5.1 MR1 release.

# From Each BTS machine

Step 1   Log in as root user

Step 2   <hostname># cd /var/spool/cron/crontabs

- Ftp each cron job to a Network File Server that the BTS system can access. .

# Task 6 : Perform full database audit

This task verifies that provisioning is operational from CLI and a full database audit is performed and errors if any are corrected. Please refer to Appendix C to perform full data base Audit.

# Note : Depending on the size of the database the audit could take multiple hours to complete. This task should be completed the night before the upgrade.

Chapter 5

Prepare System for Upgrade

This chapter describes the steps a user must complete the morning or the night before the scheduled upgrade.

If root access is disabled, to enable, please use Appendix L Task 1 to enable the root access.

# Task 1: Verify System Status

Step 1   Verify that the side A systems are in the active state. Use Appendix A for this procedure.

Step 2   Verify that call processing is working without error. Use Appendix B for this procedure.

# Task 2: Perform database row count audit (sanity check)

Step 1   In this task, you will perform database row count audit (sanity check) and correct any errors, if necessary.

Note: The database row count audit is a quick verification (sanity check) right before the upgrade that the number of rows match for tables in EMS and CA nodes.

Step 2  Login as “ciscouser”

Step 3   CLI> audit database type=row_count;

Step 4   Check the audit report and verify there is no discrepancy or error. If errors are found, please try to correct them. If you are unable to correct, please contact Cisco Support.

Please follow the sample command provided below to correct the mismatches:

For the 4 handset provisioning tables (SLE, SC1D, SC2D, SUBSCRIBER-FEATURE-DATA), please use:

CLI> sync <table name> master=FSPTCyyy; target=EMS;

For all other tables, please use:

CLI> sync <table name> master=EMS; target=<CAxxx/FSPTCyyy/FSAINzzz>;

# Task 3: Alarms.

# Verify that there are no outstanding major or critical alarms. Use Appendix D for this procedure.

# Task 4: Oracle Database and Replication.

# Use Appendix E to verify that Oracle database and replication functions are working properly.

|  | Caution Do not continue until the above verifications have been made. Call Cisco Support if you need assistance. |
| --- | --- |

# Task 5: Backup user account

The user accounts saved in this task is to be restored to side B EMS once it is upgraded to 4.5.1 release.

# From EMS Side A

Step 1   Log in as root

Step 2   Save the /opt/ems/users directory:

<hostname># mkdir –p /opt/.upgrade

<hostname># tar -cvf /opt/.upgrade/users.tar /opt/ems/users

# Task 6: Verify and record Virtual IP (VIP) information

Verify if virtual IP is configured on the EMS machine. If VIP is configured, record the VIP, otherwise go to task 7. VIP will need to be re-configured after the upgrade procedure is complete.

# From EMS Side A

Step 1   Log in as root user

Step 2  <hostname> su – btsadmin

Step 3   btsadmin> show ems

IP_ALIAS=10.89.224.177
INTERFACE=eri0
NTP_SERVER=10.89.224.44,

Step 4  Record the IP_ALIAS (VIP) and INTERFACE.

IP_ALIAS:

INTERFACE:

Step 5  btsadmin> exit

# Task 7: Verify and record VSM Macro information

Verify if VSM Macros are configured on the EMS machine. If VSM is configured, record the VSM, otherwise go to chapter 6.  VSM will need to be re-configured after the upgrade procedure is complete.

# From EMS Side A

Step 1   Log in as root user

Step 2  <hostname> su – btsadmin

Step 3   btsadmin> show macro id=VSM%

ID=VSMSubFeature

PARAMETERS=subscriber.id,subscriber.dn1,subscriber_service_profile.service-id,service.fname1,service.fname2,service.fname3,service.fname4,service.fname5,service.fname6,service.fname7,service.fname8,service.fname9,service.fname10

AND_RULES=subscriber.id=subscriber_service_profile.sub-id,subscriber_service_profile.service-id=service.id

Step 4  Record the VSM Macro information

Step 5  btsadmin> exit

Chapter 6

Upgrade Side B Systems

# Task 1: Block provisioning path

It is critical to block unwanted provisioning traffics during upgrade. Please execute steps in Appendix J.

# Task 2: Disable Oracle DB replication

# From Active EMS

Step 1   Log in to Active EMS as “btsuser” user.

Step 2   CLI> control bdms id=BDMS01; target-state=forced-standby-active;

Step 3   CLI> control element-manager id=EM01; target-state=forced-standby-active;

Step 4    CLI session will terminate when application platform switchover is complete.

# From EMS side A

Step 1   Log in as Oracle user:

<hostname># su – oracle

<hostname>$ cd /opt/oracle/admin/utl

Step 2   Set Oracle DB to simplex mode:

<hostname>$ rep_toggle –s optical1 –t set_simplex

- Answer “y” when prompt

- Answer “y” again when prompt

Step 3   <hostname>$ exit

Step 4   Restart applications to release Oracle connections:

<hostname># platform stop all

<hostname># platform start

# Task 3: Force side A systems to active

This procedure will force the side A systems to remain active.

|  | Note In the commands below, "xxx", "yyy" or "zzz" is the instance for the process on your system. |
| --- | --- |

# From Active EMS Side B

Step 1   Log in to Active EMS as “btsuser” user.

Step 2   CLI> control call-agent id=CAxxx; target-state=forced-active-standby;

Step 3   CLI> control feature-server id=FSPTCyyy; target-state=forced-active-standby;

Step 4   CLI> control feature-server id=FSAINzzz; target-state=forced-active-standby;

Step 5   CLI> control bdms id=BDMS01; target-state=forced-active-standby;

Step 6   CLI> control element-manager id=EM01; target-state=forced-active-standby;

Note : CLI session will terminate when application platform switchover is complete.

# Task 4: Inhibit EMS mate communication

In this task, you will isolate the OMS Hub on EMS side A from talking to EMS side B.

# From EMS side A

Step 1   Log in as root

Step 2   <hostname># /opt/ems/utils/updMgr.sh –split_hub

Step 3   <hostname># nodestat

- Verify there is no HUB communication from EMS side A to CA/FS side B

- Verify OMS Hub mate port status: No communication between EMS

# Task 5: Stop applications and shutdown EMS side B

# From EMS side B

Step 1   Log in as root

Step 2   Record the IP address and netmask for the management interface of the system.

- For an example, if the “hme0” is used for management interface, then execute the following command:

<hostname># ifconfig hme0

- Record the IP address and netmask for the interface to be used in the next task.

IP: _____________ Netmask: ____________ Interface Name: ___________

Step 3   <hostname># mv /etc/rc3.d/S99platform /etc/rc3.d/_S99platform

Step 4   <hostname># platform stop all

Step 5   <hostname># sync; sync

Step 6   <hostname># shutdown –i5 –g0 –y

# Task 6: Stop applications and shutdown CA/FS side B

# From CA/FS side B

Step 1   Log in as root

Step 2   Record the IP address and netmask for the management interface of the system.

- For an example, if the “hme0” is used for management interface, then execute the following command:

<hostname># ifconfig hme0

- Record the IP address and netmask for the interface to be used in the next task.

IP: _____________ Netmask: ____________ Interface Name: ___________

Step 3   <hostname># mv /etc/rc3.d/S99platform /etc/rc3.d/_S99platform

Step 4   <hostname># platform stop all

Step 5    <hostname># sync; sync

Step 6   <hostname># shutdown –i5 –g0 –y

# Task 7: Upgrade EMS side B to the new release

# From EMS side B

Step 1   Power off the machine

Step 2   Remove disk0 from slot 0 off the machine and label it as “Release 4.4.0 EMS side B disk0”.  Also remove disk1 from slot 1 off the machine and label it as “Release 4.4.0 EMS side B disk1”.

Replace 2 disks with two new disks mirrored.

- SunFire V440 disk slot lay out:

| Disk 3 | DVD-ROM |
| --- | --- |
| Disk 2 |  |
| Disk 1 |  |
| Disk 0 |  |

- Sunfire 1280 disk slot lay out:

|  | DVD-ROM |
| --- | --- |
|  | Disk 1 |
|  | Disk 0 |

Step 3   Place new disk labeled as “Release 4.5.1 MR1 EMS side B disk0” in slot 0. Also place new disk labeled as “Release 4.5.1 MR1 EMS side B disk1” in slot 1.

Step 4   Power on the machine and allow the system to boot up by monitoring the boot process thru console

- For a Sunfire 1280 machine, please execute the following command from console:

<lom>poweron

- For other type of hardware, please use the power button to turn on the power.

Step 5   Log in as root thru console

Step 6   Remove network interface hardware configuration

<hostname># cp -fp /etc/path_to_inst   /etc/path_to_inst.save

<hostname># \rm –f /etc/path_to_inst

<hostname># \rm –f /etc/path_to_inst.old

Step 7   Rebuild the hardware configuration

<hostname># reboot -- -r

- Wait for the system to boot up. Then log in as root.

Step 8   Restore management interface (recorded in Task 5):

- <hostname># ifconfig <primary interface name> plumb

  - Use Interface Name recorded in “Chapter 5, Task 4”

- <hostname># ifconfig <primary interface name> <Interface IP> netmask <NETMASK> broadcast + up

  - Use IP and NETMASK recorded in “Chapter 5, Task 4”

- Execute the following command to match the mode and speed on the bts system and CAT switch interfaces.

  - <hostname> # /etc/rc2.d/S68autoneg

- If the system has  IRDP enabled, please go to step 9, otherwise add static routes to reach Domain Name Server and Network File Server using “route add …” command:

  - Example: route add -net 10.89.224.1 10.89.232.254

Where: 10.89.224.1 is the destination DNS server IP
              10.89.232.254 is the gateway IP

Step 9   Reset ssh keys:

<hostname># \rm /.ssh/known_hosts

Step 10   sftp the opticall.cfg file from Network File Server (opticall.cfg was constructed in Chapter 3, Task 2) and place it under /etc directory.

Step 11   sftp resolv.conf file from Primary EMS Side A and place it under /etc directory.

<hostname># sftp <Side A EMS IP>

sftp> lcd /etc

sftp> cd /etc

sftp> get resolv.conf

sftp> exit

Step 12   Run script program to replace the hostname

<hostname># cd /opt/ems/upgrade

<hostname># DoTheChange -s

- The system will prompt you for the root password of the mate, please enter it now.

- The system will reboot when the script DoTheChange completes execution.

- Please see Appendix K for the files handled by the script.

Step 13    Wait for the system to boot up. Then log in as root. At this point, ignore the warning message ‘could not find locale directory’ as the next steps would resolve it.

Step 14   Editing /etc/default/init with the proper time zone for the system:

<hostname># vi /etc/default/init

- Remove lines and keep only the following lines:

#

TZ=<Timezone>

CMASK=022

For an example:

The original /etc/default/init file before line removal:

# @(#)init.dfl 1.5 99/05/26

#

# This file is /etc/default/init.  /etc/TIMEZONE is a symlink to this file.

# This file looks like a shell script, but it is not.  To maintain

# compatibility with old versions of /etc/TIMEZONE, some shell constructs

# (i.e., export commands) are allowed in this file, but are ignored.

#

# Lines of this file should be of the form VAR=value, where VAR is one of

# TZ, LANG, CMASK, or any of the LC_* environment variables.

#

TZ=US/Central

CMASK=022

LC_COLLATE=en_US.ISO8859-1

LC_CTYPE=en_US.ISO8859-1

LC_MESSAGES=C

LC_MONETARY=en_US.ISO8859-1

LC_NUMERIC=en_US.ISO8859-1

LC_TIME=en_US.ISO8859-1

The /etc/default/init file after line removal:

# @(#)init.dfl 1.5 99/05/26

#

# This file is /etc/default/init.  /etc/TIMEZONE is a symlink to this file.

# This file looks like a shell script, but it is not.  To maintain

# compatibility with old versions of /etc/TIMEZONE, some shell constructs

# (i.e., export commands) are allowed in this file, but are ignored.

#

# Lines of this file should be of the form VAR=value, where VAR is one of

# TZ, LANG, CMASK, or any of the LC_* environment variables.

#

TZ=US/Central

CMASK=022

Step 15   Verify interface hardware configuration match to the host configuration. If not, you’ll need to change the interface pre-fix.

<hostname># egrep –i “qfe|ce|eri|bge|hme” /etc/path_to_inst

<hostname># ls –l /etc/hostname.*

- If the interface names match from the above two outputs, please continue on to Step 16.

- If the interface names do NOT match, please match them by changing the postfix of hostname.*.

For an example:

Output from “egrep –i “qfe|ce|eri|bge|hme” /etc/path_to_inst” is:

"/pci@1f,4000/network@1,1" 0 "hme"

"/pci@1f,4000/pci@4/SUNW,qfe@0,1" 0 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@1,1" 1 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@2,1" 2 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@3,1" 3 "qfe"

Output from “ls -l /etc/hostname.*” is:

-rw-r--r--   1 root     other          14 May 16 16:03 /etc/hostname.hme0

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.hme0:1

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.eri0

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.eri0:1

After change, the output should be:

-rw-r--r--   1 root     other          14 May 16 16:03 /etc/hostname.hme0

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.hme0:1

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.qfe0

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.qfe0:1

Step 16   Reboot the machine to pick up new TIMEZONE and interface setting:

<hostname># sync; sync; reboot

- Wait for the system to boot up. Then log in as root.

Step 17   Install 4.5.1 V24 IP01 Patch. Refer to Appendix M, Install IP01 Patch on EMS.

Caution: 4.5.1 V24 IP01 is a mandatory patch and has to be installed at this step. Failure to install the IP01 patch at this step will impact the traffic and result in call failure.

Step 18   <hostname># /opt/ems/utils/updMgr.sh –split_hub

Step 19   <hostname># svcadm disable system/cron

Step 20    CDR delimiter customization is not retained after software upgrade. If this system has been customized, either the Customer or Cisco Support Engineer must manually customize again to keep the same customization.

- <hostname># cd /opt/bdms/bin

- <hostname># vi platform.cfg

- Find the section for the command argument list for the BMG process

- Customize the CDR delimiters in the “Args=” line

- Example:

Args=-port 15260 -h localhost -u optiuser -p optiuser -fmt default_formatter -UpdIntvl 3300 -ems_local_dn blg-aSYS14EMS.cisco.com -FD semicolon -RD linefeed

Step 21   <hostname># platform start –i oracle

Step 22   Log in as Oracle user.

<hostname># su – oracle

<hostname>$ cd /opt/oracle/admin/utl

Step 23   Set Oracle DB to simplex mode:

<hostname>$ rep_toggle –s optical2 –t set_simplex

- Answer “y” when prompt

- Answer “y” again when prompt

Step 24: <hostname> exit

# Task 8: Upgrade CA/FS Side B to the new release

|  | Warning Do not proceed if you don’t have a pre-constructed opticall.cfg file for the system. The opticall.cfg file should already be constructed in Chapter 3, Task 2. |
| --- | --- |

# From CA/FS side B

Step 1   Power off the machine

Step 2   Remove disk0 from slot 0 off the machine and label it as “Release 4.4.0 CA/FS side B disk0”. Also remove disk1 from slot 1 off the machine and label it as “Release 4.4.0 CA/FS side B disk1”.

Replace 2 disks with two new disks mirrored.

- SunFire V440 disk slot lay out:

| Disk 3 | DVD-ROM |
| --- | --- |
| Disk 2 |  |
| Disk 1 |  |
| Disk 0 |  |

- Sunfire 1280 disk slot lay out:

|  | DVD-ROM |
| --- | --- |
|  | Disk 1 |
|  | Disk 0 |

Step 3   Place new disk labeled as “Release 4.5.1 MR1 CA/FS side B disk0” in slot 0. Also place new disk labeled as “Release 4.5.1 MR1 CA/FS side B disk1” in slot 1.

Step 4   Power on the machine and allow the system to boot up by monitoring the boot process thru console

- For a Sunfire 1280 machine, please execute the following command from console:

<lom>poweron

- For other type of hardware, please use the power button to turn on the power.

Step 5   Log in as root thru console

Step 6   Remove network interface hardware configuration

<hostname># cp -fp /etc/path_to_inst   /etc/path_to_inst.save

<hostname># \rm –f /etc/path_to_inst

<hostname># \rm –f /etc/path_to_inst.old

Step 7   Rebuild the hardware configuration

<hostname># reboot -- -r

- Wait for the system to boot up. Then log in as root.

Step 8   Restore management interface (recorded in Task 6):

- <hostname># ifconfig <primary interface name> plumb

  - Use Interface Name recorded in “Chapter 5, Task 5”

- <hostname># ifconfig <primary interface name> <Interface IP> netmask <NETMASK> broadcast + up

  - Use IP and NETMASK recorded in “Chapter 5, Task 5”

- Execute the following command to match the mode and speed on the bts system and CAT switch interfaces.

  - <hostname> # /etc/rc2.d/S68autoneg

- If the system has  IRDP enabled, please go to step 9, otherwise add static routes to reach Domain Name Server and Network File Server using “route add …” command:

  - Example: route add -net 10.89.224.1 10.89.232.254

Where: 10.89.224.1 is the destination DNS server IP
              10.89.232.254 is the gateway IP

Step 9   Reset ssh keys:

<hostname># \rm /.ssh/known_hosts

Step 10   sftp the opticall.cfg file from Network File Server (opticall.cfg was constructed in Chapter 3, Task 2) and place it under /etc directory.

Step 11   sftp resolv.conf file from Primary CA/FS Side A and place it under /etc directory.

<hostname># sftp <Side A CA/FS IP>

sftp> lcd /etc

sftp> cd /etc

sftp> get resolv.conf

sftp> exit

Step 12   Run script program to replace the hostname

<hostname># cd /opt/ems/upgrade

<hostname># DoTheChange -s

- The system will prompt you for the root password of the mate, please enter it now.

- The system will reboot when the script DoTheChange completes its run

- Please see Appendix K for the files handled by the script.

Step 13    Wait for the system to boot up. Then log in as root. At this point, ignore the warning message ‘could not find locale directory’ as the next steps would resolve it.

Step 14   Editing /etc/default/init with the proper time zone for the system:

<hostname># vi /etc/default/init

- Remove lines and keep only the following lines:

#

TZ=<Timezone>

CMASK=022

For an example:

The original /etc/default/init file before line removal:

# @(#)init.dfl 1.5 99/05/26

#

# This file is /etc/default/init.  /etc/TIMEZONE is a symlink to this file.

# This file looks like a shell script, but it is not.  To maintain

# compatibility with old versions of /etc/TIMEZONE, some shell constructs

# (i.e., export commands) are allowed in this file, but are ignored.

#

# Lines of this file should be of the form VAR=value, where VAR is one of

# TZ, LANG, CMASK, or any of the LC_* environment variables.

#

TZ=US/Central

CMASK=022

LC_COLLATE=en_US.ISO8859-1

LC_CTYPE=en_US.ISO8859-1

LC_MESSAGES=C

LC_MONETARY=en_US.ISO8859-1

LC_NUMERIC=en_US.ISO8859-1

LC_TIME=en_US.ISO8859-1

The /etc/default/init file after line removal:

# @(#)init.dfl 1.5 99/05/26

#

# This file is /etc/default/init.  /etc/TIMEZONE is a symlink to this file.

# This file looks like a shell script, but it is not.  To maintain

# compatibility with old versions of /etc/TIMEZONE, some shell constructs

# (i.e., export commands) are allowed in this file, but are ignored.

#

# Lines of this file should be of the form VAR=value, where VAR is one of

# TZ, LANG, CMASK, or any of the LC_* environment variables.

#

TZ=US/Central

CMASK=022

Step 15   Verify interface hardware configuration match to the host configuration. If not, you’ll need to change the interface pre-fix.

<hostname># egrep –i “qfe|ce|eri|bge|hme” /etc/path_to_inst

<hostname># ls –l /etc/hostname.*

- If the interface names match from the above two outputs, please continue on Step 16.

- If the interface names do NOT match, please match them by changing the postfix of hostname.*.

For an example:

Output from “egrep –i “qfe|ce|eri|bge|hme” /etc/path_to_inst” is:

"/pci@1f,4000/network@1,1" 0 "hme"

"/pci@1f,4000/pci@4/SUNW,qfe@0,1" 0 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@1,1" 1 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@2,1" 2 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@3,1" 3 "qfe"

"/pci@1f,2000/pci@1/SUNW,qfe@0,1" 4 "qfe"

"/pci@1f,2000/pci@1/SUNW,qfe@1,1" 5 "qfe"

"/pci@1f,2000/pci@1/SUNW,qfe@2,1" 6 "qfe"

"/pci@1f,2000/pci@1/SUNW,qfe@3,1" 7 "qfe"

Output from “ls -l /etc/hostname.*” is:

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.hme0

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.eri0

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.eri1

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.eri1:1

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.eri1:2

-rw-r--r--   1 root     other         12 Jun 10 11:25 hostname.eri1:3

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.eri2

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.eri2:1

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.eri2:2

-rw-r--r--   1 root     other         12 Jun 10 11:25 hostname.eri2:3

After change, the output should be:

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.hme0

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.qfe0

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.qfe1

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.qfe1:1

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.qfe1:2

-rw-r--r--   1 root     other         12 Jun 10 11:25 hostname.qfe1:3

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.qfe2

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.qfe2:1

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.qfe2:2

-rw-r--r--   1 root     other         12 Jun 10 11:25 hostname.qfe2:3

Step 16   Reboot the machine to pick up new TIMEZONE and interface setting:

<hostname># sync; sync; reboot

- Wait for the system to boot up. Then log in as root.

Step 17   Check for configuration errors

<hostname># cd /opt/Build

<hostname># checkCFG –u

- Correct errors generated by checkCFG

- Once the result is clean without errors, then proceed to the next step.

Step 18   <hostname># install.sh -upgrade

Step 19   Answer "y" when prompted. This process will take up to 15 minutes to complete.

- Some of the OS patch installation requires immediate system reboot.  If system is rebooted because of the OS patch installation, you must re-run “install.sh -upgrade” after system boots up.  Fail to do so will result in installation failure.

- When installation is completed, answer "y" if prompted for reboot.

- Wait for the system to boot up. Then log in as root..

Step 20  Install 4.5.1 V24 IP01 Patch. Refer to Appendix N, Install IP01 Patch on CA.

Caution: 4.5.1 V24 IP01 is a mandatory patch and has to be installed at this step. Failure to install the IP01 patch at this step will impact the traffic and result in call failure.

Step 21   <hostname># platform start

Step 22   <hostname># mv /etc/rc3.d/_S99platform /etc/rc3.d/S99platform

# Task 9: Migrate oracle data

The subscriber data migration breaks into two parts:

- Oracle DB -- data migration rules are defined in the configuration file /opt/oracle/admin/upd/config/4.4.0_to_4.5.1.cfg. Java process DMMgr reads in rules and then reads data from mate EMS Oracle DB thru DB connection established by the Java process.

- Shared Memory DB for Call Agent/Feature Server -- data is replicated thru RDM and utilizes DBM checkpoint library to move data from an old release to a new release.

# From EMS side B

Step 1   Copying data.

<hostname>$ su - oracle

<hostname>$ cd /opt/oracle/admin/upd

<hostname>$ java dba.dmt.DMMgr -upgrade -auto ./config/4.4.0_to_4.5.1.cfg

Step 2   Verify the data migration is a success.

<hostname>$ echo $?

- Verify the returned value is 0.

- If not, please sftp /opt/oracle/admin/upd/DMMgr.log file off system and call for immediate technical assistance.

Step 3   <hostname>$ java dba.adm.DBUsage -sync

- Verify Number of tables “unable-to-sync” is 0

- If not, please contract Cisco support

Step 4   <hostname>$ cd /opt/oracle/opticall/create

Step 5   <hostname>$ dbinstall optical2 -load dbsize

Step 6   <hostname>$ exit

Step 7    Enable daemon manager:

<hostname># /opt/ems/bin/daemon_mgr.sh add

<hostname># /opt/ems/bin/daemon_mgr.sh start

Step 8 <hostname># platform start

Step 9 <hostname># svcadm enable system/cron

Step 10 <hostname># mv /etc/rc3.d/_S99platform /etc/rc3.d/S99platform

Chapter 7

Prepare Side A Systems for Upgrade

# Task 1: Force side A systems to standby

This procedure will force the side A systems to standby and force the side B systems to active.

|  | Note In the commands below, "xxx", "yyy" or "zzz" is the instance for the process on your system. |
| --- | --- |

# From EMS side A

Step 1   Log in as “btsuser” user

Step 2   CLI> control call-agent id=CAxxx; target-state=forced-standby-active;

Step 3   CLI> control feature-server id=FSPTCyyy; target-state=forced-standby-active;

Step 4   CLI> control feature-server id=FSAINzzz; target-state=forced-standby-active;

Step 5   CLI> control bdms id=BDMS01; target-state=forced-standby-active;

Step 6   CLI> control element-manager id=EM01; target-state=forced-standby-active;

Step 7   Depending on the CLI user type, the CLI session may terminate upon successful completion of last CLI command.

|  | Note If the system fails to switchover from side A to side B, please contact Cisco Support to determine whether the system should fallback. If fallback is needed, please following Appendix F. |
| --- | --- |

# Task 2: Check and Correct Feature Server and Call Agent TSAP address

# From EMS side B

Step 1   Log in to EMS side B as “btsuser” user.

Step 2  Check TSAP_ADDR token for  AIN , POTS feature server and CA and make sure that they match with the ones populated in opticall.cfg.

Example 1

- In Opticall.cfg the following entry is populated as follows

DNS_FOR_FSAIN205_ASM_COM=asm-SYS76AIN205.ipclab.cisco.com

- The corresponding FSAIN feature server TSAP_ADDR token should be as indicated below.

Btsuser > show feature_server

ID=FSAIN205

TSAP_ADDR=asm-SYS76AIN205.ipclab.cisco.com:11205

TYPE=AIN

EXTERNAL_FEATURE_SERVER=N

Example 2

- In Opticall.cfg the following entry is populated as follows

DNS_FOR_FSPTC235_POTS_COM=pots-SYS76PTC235.ipclab.cisco.com

- The corresponding POTS feature server TSAP_ADDR token should be as indicated below.

Btsuser > show_feature_server

ID=POTS

TSAP_ADDR= pots-SYS76PTC235.ipclab.cisco.com:11235

TYPE=POTS

EXTERNAL_FEATURE_SERVER=N

Example 3

- In Opticall.cfg the following entry is populated as follows

DNS_FOR_CA146_SIM_COM=sim-SYS76CA146.ipclab.cisco.com

- The corresponding Call Agent TSAP_ADDR token should be as indicated below.

Btsuser > show_call agent

ID=CA146

TSAP_ADDR= sim-SYS76CA146.ipclab.cisco.com:9146

MGW_MONITORING_ENABLED=Y

NOTE : Make sure that TSAP_ADDR token is provisioned correctly for Call Agent,  AIN and POTS feature servers

If not, please use the change command to correct the TSAP_ADDR token:

CLI> change feature-server id=<FS ID>; TSAP_ADDR=<dn:port>;

CLI > change call agent id = <Call Agent ID> ;TSAP ADDR =<dn:port>;

Step 3  Check TSAP port number for both AIN and POTS feature servers:

CLI> show feature-server;

Sample output:

ID=FSAIN205

TSAP_ADDR=asm-SYS36AIN205.ipclab.cisco.com:11205

TYPE=AIN

EXTERNAL_FEATURE_SERVER=N

ID=FSPTC235

TSAP_ADDR=pots-SYS36PTC235.ipclab.cisco.com:11235

TYPE=POTS

EXTERNAL_FEATURE_SERVER=N

Make sure the port number provisioned is:

  - AIN Feature Server: 11205

  - POTS Feature Server: 11235

If not, please use the change command:

CLI> change feature-server id=<FS ID>; TSAP_ADDR=<dn:port>;

Step 4   CLI> exit

# Task 3: Sync Data from EMS side B to CA/FS side B

In this task, you will sync from EMS to CA/FS for several inter-platform migrated tables.

# From EMS side B

Step 1   Log in as ciscouser (password: ciscosupport)

Step 2   CLI> sync emergency_number_list master=EMS; target=CAxxx;

Step 3   CLI> sync radius-profile master=EMS; target=FSPTCyyy;

Step 4   CLI> sync lnp-profile master=EMS; target=CAxxx;

Step 5   CLI> sync lnp-profile master=EMS; target=FSAINzzz;

Step 6   CLI> sync subsystem_grp master=EMS; target=FSPTCyyy;

Step 7   CLI> sync subsystem_grp master=EMS; target=FSAINzzz;

Step 8   CLI> sync pop master=EMS; target=CAxxx;

Step 9   CLI> sync pop master=EMS; target=FSPTCyyy;

Step 10   CLI> sync pop master=EMS; target=FSAINzzz;

Step 11   CLI> exit

Task 4:  Block Provisioning Path

# From EMS side B

Make sure the provisioning path is blocked.

Step 1   Login as root

Step 2    Disable bulk provisioning:

<hostname># cd /opt/ems/bin

<hostname># echo "update PlatformPrograms set enable='N' where id='DLP';" | mysql_cli.sh

<hostname># pkill smg

# Task 5: Validate release 4.5.1 MR1 software operation

To verify the stability of the newly installed 4.5.1 MR1 Release, let CA/FS side B carry live traffic for period of time. Monitor the Cisco BTS 10200 Softswitch and the network. If there are any problems, please investigate and contact Cisco Support if necessary.

This activity should NOT last more than an hour.  The tasks should include:

- Basic call on-net to on-net, and on-net to off-net calls

- Pre-existed feature calls

- Verify billing CDR for calls made

# From EMS side B

Step 1   Verify that call processing using Appendix B.

Step 2   Log in as “ciscouser” user.

Step 3   CLI> audit database type=row-count;

- Please ignore mismatches for the following north bound traffic tables:

  - SLE

  - SC1D

  - SC2D

  - SUBSCRIBER-FEATURE-DATA

- Please check the report to any mismatched tables.

- If any table shows mis-match, sync the table from EMS to CA/FS, then perform a detailed audit on each mismatched table:

CLI> sync <table name> master=EMS; target=<CA/FSPTC|FSAIN>;

CLI> audit <table name>;

Step 4   Verify the SUP config is set up correctly

- CLI> show sup-config;

- Verify refresh rate is set to 86400.

- If not, do the following

- CLI> change sup-config type=refresh_rate; value=86400;

- CLI> exit

# Task 6: Ftp billing records off the system

# From EMS side A

Step 1   Log in as root

Step 2   <hostname># ls /opt/bms/ftp/billing

- If there are files listed, then SFTP the files to a mediation device on the network and remove the files from the /opt/bms/ftp/billing directory.

|  | Note Once the system proves stable and you decide to move ahead with the upgrade, then you must execute subsequent tasks. If fallback is needed at this stage, please follow the fallback procedure in Appendix F. |
| --- | --- |

Chapter 8

Upgrade Side A Systems

# Task 1: Stop applications and shutdown EMS side A

# From EMS side A

Step 1   Log in as root

Step 2   Record the IP address and netmask for the management interface of the system.

- For an example, if the “hme0” is used for management interface, then execute the following command:

<hostname># ifconfig hme0

- Record the IP address and netmask for the interface to be used in the next task.

IP: _____________ Netmask: ____________ Interface Name: ___________

Step 3   <hostname># mv /etc/rc3.d/S99platform /etc/rc3.d/_S99platform

Step 4   <hostname># platform stop all

Step 5   <hostname># sync; sync

Step 6   <hostname># shutdown –i5 –g0 –y

# Task 2: Stop applications and shutdown CA/FS side A

# From CA/FS side A

Step 1   Log in as root

Step 2   Record the IP address and netmask for the management interface of the system.

- For an example, if the “hme0” is used for management interface, then execute the following command:

<hostname># ifconfig hme0

- Record the IP address and netmask for the interface to be used in the next task.

IP: _____________ Netmask: ____________ Interface Name: ___________

Step 3   <hostname># mv /etc/rc3.d/S99platform /etc/rc3.d/_S99platform

Step 4   <hostname># platform stop all

Step 5    <hostname># sync; sync

Step 6   <hostname># shutdown –i5 –g0 –y

# Task 3: Upgrade EMS side A to the new release

# From EMS side A

Step 1   Power off the machine

Step 2   Remove disk0 from slot 0 off the machine and label it as “Release 4.4.0 EMS side A disk0”.  Also remove disk1 from slot 1 off the machine and label it as “Release 4.4.0 EMS side A disk1”.

Replace 2 disks with two new disks mirrored.

- SunFire V440 disk slot lay out:

| Disk 3 | DVD-ROM |
| --- | --- |
| Disk 2 |  |
| Disk 1 |  |
| Disk 0 |  |

- Sunfire 1280 disk slot lay out:

|  | DVD-ROM |
| --- | --- |
|  | Disk 1 |
|  | Disk 0 |

Step 3   Place new disk labeled as “Release 4.5.1 MR1 EMS side A disk0” in slot 0. Also place new disk labeled as “Release 4.5.1 MR1 EMS side A disk1” in slot 1.

Step 4   Power on the machine and allow the system to boot up by monitoring the boot process thru console

- For a Sunfire 1280 machine, please execute the following command from console:

<lom>poweron

- For other type of hardware, please use the power button to turn on the power.

Step 5   Log in as root thru console

Step 6   Remove network interface hardware configuration

<hostname># cp -fp /etc/path_to_inst   /etc/path_to_inst.save

<hostname># \rm –f /etc/path_to_inst

<hostname># \rm –f /etc/path_to_inst.old

Step 7   Rebuild the hardware configuration

<hostname># reboot -- -r

- Wait for the system to boot up. Then log in as root.

Step 8   Restore management interface (recorded in Task 1):

- <hostname># ifconfig <primary interface name> plumb

  - Use Interface Name recorded in “Chapter 7, Task 1”

- <hostname># ifconfig <primary interface name> <Interface IP> netmask <NETMASK> broadcast + up

  - Use IP and NETMASK recorded in “Chapter 7, Task 1”

- Execute the following command to match the mode and speed on the bts system and CAT switch interfaces.

  - <hostname> # /etc/rc2.d/S68autoneg

- If the system has  IRDP enabled, please go to step 9, otherwise add static routes to reach Domain Name Server and Network File Server using “route add …” command:

  - Example: route add -net 10.89.224.1 10.89.232.254

Where: 10.89.224.1 is the destination DNS server IP
              10.89.232.254 is the gateway IP

Step 9   Reset ssh keys:

<hostname># \rm /.ssh/known_hosts

Step 10   sftp the opticall.cfg and resolv.conf from Secondary EMS side B and place it under /etc directory.

<hostname># sftp <Side B EMS IP>

sftp> lcd /etc

sftp> cd /etc

sftp> get resolv.conf

sftp> get opticall.cfg

sftp> exit

Step 11   Run script program to replace the hostname

<hostname># cd /opt/ems/upgrade

<hostname># DoTheChange -p

- The system will prompt you for the root password of the mate, please enter it now.

- The system will reboot when the script DoTheChange completes its run

- Please see Appendix K for the files handled by the script.

Step 12   Wait for the system to boot up. Then Log in as root. At this point, ignore the warning message ‘could not find locale directory’ as the next steps would resolve it.

Step 13   Verify interface hardware configuration to match the host configuration. If not, you’ll need to change the interface pre-fix.

<hostname># egrep –i “qfe|ce|eri|bge|hme” /etc/path_to_inst

<hostname># ls –l /etc/hostname.*

- If the interface names match from the above two outputs, please continue on Step 15.

- If the interface names do NOT match, please match them by changing the postfix of hostname.*.

For an example:

Output from “egrep –i “qfe|ce|eri|bge|hme” /etc/path_to_inst” is:

"/pci@1f,4000/network@1,1" 0 "hme"

"/pci@1f,4000/pci@4/SUNW,qfe@0,1" 0 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@1,1" 1 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@2,1" 2 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@3,1" 3 "qfe"

Output from “ls -l /etc/hostname.*” is:

-rw-r--r--   1 root     other         14 May 16 16:03 /etc/hostname.hme0

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.hme0:1

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.eri0

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.eri0:1

After change, the output should be:

-rw-r--r--   1 root     other         14 May 16 16:03 /etc/hostname.hme0

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.hme0:1

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.qfe0

-rw-r--r--   1 root     other         14 May 16 16:04 /etc/hostname.qfe0:1

Step 14   Reboot the machine to pick up new interface setting:

<hostname># sync; sync; reboot

- Wait for the system to boot up. Then log in as root.

Step 15   Install 4.5.1 V24 IP01 Patch. Refer to Appendix M, Install IP01 Patch on EMS.

Caution: Caution: 4.5.1V24 IP01 is a mandatory patch and has to be installed at this step.  Failure to install the IP01 patch at this step will impact the traffic, and result in call failures.

Step 15   CDR delimiter customization is not retained after software upgrade. If this system has been customized, either the Customer or Cisco Support Engineer must manually customize again to keep the same customization.

- <hostname># cd /opt/bdms/bin

- <hostname># vi platform.cfg

- Find the section for the command argument list for the BMG process

- Customize the CDR delimiters in the “Args=” line

- Example:

Args=-port 15260 -h localhost -u optiuser -p optiuser -fmt default_formatter -UpdIntvl 3300 -ems_local_dn blg-aSYS14EMS.cisco.com -FD semicolon -RD linefeed

# Task 4: Upgrade CA/FS Side A to the new release

# From CA/FS side A

Step 1   Power off the machine

Step 2   Remove disk0 from slot 0 off the machine and label it as “Release 4.4.0 CA/FS side A disk0”. Also remove disk1 from slot 1 off the machine and label it as “Release 4.4.0 CA/FS side A disk1”.

Replace 2 disks with two new disks mirrored.

- SunFire V440 disk slot lay out:

| Disk 3 | DVD-ROM |
| --- | --- |
| Disk 2 |  |
| Disk 1 |  |
| Disk 0 |  |

- Sunfire 1280 disk slot lay out:

|  | DVD-ROM |
| --- | --- |
|  | Disk 1 |
|  | Disk 0 |

Step 3   Place new disk labeled as “Release 4.5.1 MR1 CA/FS side A disk0” in slot 0. Also place new disk labeled as “Release 4.5.1 MR1 CA/FS side A disk1” in slot 1.

Step 4   Power on the machine and allow the system to boot up by monitoring the boot process thru console

- For a Sunfire 1280 machine, please execute the following command from console:

<lom>poweron

- For other type of hardware, please use the power button to turn on the power.

Step 5   Log in as root thru console

Step 6   Remove network interface hardware configuration

<hostname># cp -fp /etc/path_to_inst   /etc/path_to_inst.save

<hostname># \rm –f /etc/path_to_inst

<hostname># \rm –f /etc/path_to_inst.old

Step 7   Rebuild the hardware configuration

<hostname># reboot -- -r

- Wait for the system to boot up. Then log in as root.

Step 8   Restore management interface (recorded in Task 2):

- <hostname># ifconfig <interface name> plumb

  - Use Interface Name recorded in “Chapter 7, Task 2”

- <hostname># ifconfig <interface name> <Interface IP> netmask <NETMASK> broadcast + up

  - Use IP and NETMASK recorded in “Chapter 7, Task 2”

- Execute the following command to match the mode and speed on the bts system and CAT switch interfaces.

  - <hostname> # /etc/rc2.d/S68autoneg

- If the system has  IRDP enabled, please go to step 9, otherwise add static routes to reach Domain Name Server and Network File Server using “route add …” command:

  - Example: route add -net 10.89.224.1 10.89.232.254

Where: 10.89.224.1 is the destination DNS server IP
              10.89.232.254 is the gateway IP

Step 9   Reset ssh keys:

<hostname># \rm /.ssh/known_hosts

Step 10   sftp the opticall.cfg and resolv.conf from Secondary CA/FS side B and place it under /etc directory.

<hostname># sftp <Side B CA/FS IP>

sftp> lcd /etc

sftp> cd /etc

sftp> get resolv.conf

sftp> get opticall.cfg

sftp> exit

Step 11   Run script program to replace the hostname

<hostname># cd /opt/ems/upgrade

<hostname># DoTheChange -p

- The system will prompt you for the root password of the mate, please enter it now.

- The system will reboot when the script DoTheChange completes its run

- Please see Appendix K for the files handled by the script.

Step 12    Wait for the system to boot up. Then Log in as root. At this point, ignore the warning message ‘could not find locale directory’ as the next steps would resolve it.

Step 13   Verify interface hardware configuration match to the host configuration. If not, you’ll need to change the interface pre-fix.

<hostname># egrep –i “qfe|ce|eri|bge|hme” /etc/path_to_inst

<hostname># ls –l /etc/hostname.*

- If the interface names match from the above two outputs, please continue on Step 15.

- If the interface names do NOT match, please match them by changing the postfix of hostname.*.

For an example:

Output from “egrep –i “qfe|ce|eri|bge|hme” /etc/path_to_inst” is:

"/pci@1f,4000/network@1,1" 0 "hme"

"/pci@1f,4000/pci@4/SUNW,qfe@0,1" 0 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@1,1" 1 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@2,1" 2 "qfe"

"/pci@1f,4000/pci@4/SUNW,qfe@3,1" 3 "qfe"

"/pci@1f,2000/pci@1/SUNW,qfe@0,1" 4 "qfe"

"/pci@1f,2000/pci@1/SUNW,qfe@1,1" 5 "qfe"

"/pci@1f,2000/pci@1/SUNW,qfe@2,1" 6 "qfe"

"/pci@1f,2000/pci@1/SUNW,qfe@3,1" 7 "qfe"

Output from “ls -l /etc/hostname.*” is:

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.hme0

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.eri0

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.eri1

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.eri1:1

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.eri1:2

-rw-r--r--   1 root     other         12 Jun 10 11:25 hostname.eri1:3

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.eri2

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.eri2:1

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.eri2:2

-rw-r--r--   1 root     other         12 Jun 10 11:25 hostname.eri2:3

After change, the output should be:

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.hme0

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.qfe0

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.qfe1

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.qfe1:1

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.qfe1:2

-rw-r--r--   1 root     other         12 Jun 10 11:25 hostname.qfe1:3

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.qfe2

-rw-r--r--   1 root     other         13 Jun 10 11:25 hostname.qfe2:1

-rw-r--r--   1 root     other         14 Jun 10 11:25 hostname.qfe2:2

-rw-r--r--   1 root     other         12 Jun 10 11:25 hostname.qfe2:3

Step 14   Reboot the machine to pick up new interface setting:

<hostname># sync; sync; reboot

- Wait for the system to boot up. Then log in as root.

Step 15   Install the applications

<hostname># cd /opt/Build

<hostname># install.sh –upgrade

Step 16   Answer "y" when prompted. This process will take up to 15 minutes to complete.

- Some of the OS patch installation requires immediate system reboot.  If system is rebooted because of the OS patch installation, you must re-run “install.sh -upgrade” after system boots up.  Fail to do so will result in installation failure.

- When installation is completed, answer "y" if prompted for reboot.

- Wait for the system to boot up. Then Log in as root.

Step 17   Install 4.5.1 V24 IP01 Patch. Refer to Appendix N , Install IP01 Patch on CA.

Caution: 4.5.1V24 IP01 patch is a mandatory patch and has to be installed at this step.  Failure to install the IP01 patch at this step will impact the traffic and result in call failures.

Step 17   <hostname># platform start

Step 18   <hostname># mv /etc/rc3.d/_S99platform /etc/rc3.d/S99platform

# Task 5: Restore Hub communication

# From EMS side B

Step 1   Log in as root

Step 2   <hostname># /opt/ems/utils/updMgr.sh –restore_hub

Step 3   Verify hub communication on EMS Side B

<hostname># nodestat

# Task 6: Copying oracle data

# From EMS side A

Step 1   Log in as root

Step 2   <hostname># svcadm disable system/cron

Step 3   <hostname># platform start –i oracle

Step 4   Copying data.

<hostname>$ su – oracle

<hostname>$ cd /opt/oracle/admin/upd

<hostname>$ java dba.dmt.DMMgr –copy -auto

Step 5   Verify the data copy is a success.

<hostname>$ echo $?

- Verify the returned value is 0.

- If not, please sftp /opt/oracle/admin/upd/DMMgr.log file off system and call for immediate technical assistance.

<hostname>$ exit

Step 6   Enable daemon manager:

<hostname># /opt/ems/bin/daemon_mgr.sh add

<hostname># /opt/ems/bin/daemon_mgr.sh start

Step 7   <hostname># platform start

Step 8   <hostname># svcadm enable system/cron

Step 9   <hostname># mv /etc/rc3.d/_S99platform /etc/rc3.d/S99platform

# Task 7: Verify Hub communication

# From EMS side B

Step 1   Log in as root

Step 2   <hostname># nodestat

- Verify OMS Hub mate port status is established

- Verify HUB communication from EMS side B to CA/FS side A is established

Chapter 9

Finalizing Upgrade

# Task 1: Switchover activity from side B to side A

This procedure will force the active system activity from side B to side A.

# From EMS side B

|  | Note In the commands below, "xxx", "yyy" or "zzz" is the instance for the process on your system. |
| --- | --- |

Step 1   Log in to EMS side B as “btsuser” user.

Step 2   CLI> control call-agent id=CAxxx; target-state=active-standby;

Step 3   CLI> control feature-server id=FSPTCyyy; target-state=active-standby;

Step 4   CLI> control feature-server id=FSAINzzz; target-state=active-standby;

Step 5   CLI> control bdms id=BDMS01; target-state=active-standby;

Step 6   CLI> control element-manager id=EM01; target-state=active-standby;

Step 7   CLI shell session should be terminated when last CLI commands completes.

# Task 2: Enable Oracle DB replication on EMS side B

# From EMS side B

Step 1   Restore Oracle DB to duplex mode:

<hostname># su - oracle

<hostname>$ cd /opt/oracle/admin/utl

<hostname>$ rep_toggle –s optical2 –t set_duplex

- Answer “y” when prompt

- Answer “y” again when prompt

Step 2   <hostname>$ exit

Step 3   Restart applications to connect to Oracle DB in duplex mode.

<hostname># platform stop all

<hostname># platform start

Task 3 : Enable Provisioning Path

# From EMS side B

Make sure the provisioning path is enabled.

Step 1   Login as root

Step 2    Enable bulk provisioning:

<hostname># cd /opt/ems/bin

<hostname># echo "update PlatformPrograms set enable='Y' where id='DLP';" | mysql_cli.sh

<hostname># pkill smg

# Task 4: Check and correct sctp-assoc table

# From EMS side A

Step 1   Log in to EMS side A as “btsuser” user.

Step 2   Change the value for field max-init-rto to 1000 in the table sctp-assoc:

CLI> audit sctp-assoc;

- If the audit produces mismatched result for field max-init-rto. Please use the following steps to correct each mismatched record:

CLI> control sctp-assoc id=<Assoc ID>; mode=forced; target_state=oos;

CLI> change sctp-assoc id=<Assoc ID>; max-init-rto=1000;

CLI> control sctp-assoc id=<Assoc ID>; mode=forced; target_state=ins;

# Task 5: Synchronize handset provisioning data

# From EMS side A

|  | Note In the commands below, "xxx", "yyy" or "zzz" is the instance for the process on your system. |
| --- | --- |

Step 1   Log in as ciscouser (password: ciscosupport)

Step 2   CLI>sync termination master=CAxxx; target=EMS;

- Verify the transaction is executed successfully.

Step 3   CLI>sync sc1d master=FSPTCyyy; target=EMS;

- Verify the transaction is executed successfully

Step 4   CLI>sync sc2d master=FSPTCyyy; target=EMS;

- Verify the transaction is executed successfully

Step 5   CLI>sync sle master=FSPTCyyy; target=EMS;

- Verify the transaction is executed successfully

Step 6   CLI>sync subscriber-feature-data master=FSPTCyyy; target=EMS;

- Verify the transaction is executed successfully

Step 7   CLI>exit

# Task 6: SFTP hosts and opticall.cfg file

Please take /etc/hosts and /etc/opticall.cfg on CA/FS side A as the master copy and SFTP each file to all other BTS nodes.

# Task 7: Restore customized cron jobs

Please restore customized cron jobs by using the files saved on the network file server during system preparation stage in Chapter 4, Task 5. Please don’t copy the old crontab file over the new one. You may need to compare the back up version of the crontab file to the new crontab file to restore the previous settings. This should be done for all machines in the system.

# From EMS side A

Step 1   Log in as root.

Step 2   Update any customized root cron jobs using “crontab –e root” command.

Step 3   Update any customized oracle cron jobs using “crontab –e oracle” command.

To enable EMS database hot backup, make sure the backup_crosscheck process is executed as follows:

# su - oracle -c "dbadm -E backup_crosscheck"

Step 4   <hostname># svcadm disable system/cron

Step 5   <hostname># svcadm enable system/cron

# From EMS side B

Step 1   Log in as root.

Step 2   Update any customized root cron jobs using “crontab –e root” command.

Step 3   Update any customized oracle cron jobs using “crontab –e oracle” command.

To enable EMS database hot backup, make sure the backup_crosscheck process is executed as follows:

# su - oracle -c "dbadm -E backup_crosscheck"

Step 4   <hostname># svcadm disable system/cron

Step 5   <hostname># svcadm enable system/cron

From CA side A

Step 1   Log in as root.

Step 2   Update any customized root cron jobs using “crontab –e root” command.

Step 3   Update any customized oracle cron jobs using “crontab –e oracle” command.

Step 4   <hostname># svcadm disable system/cron

Step 5   <hostname># svcadm enable system/cron

# From CA side B

Step 1   Log in as root.

Step 2   Update any customized root cron jobs using “crontab –e root” command.

Step 3   Update any customized oracle cron jobs using “crontab –e oracle” command.

Step 4   <hostname># svcadm disable system/cron

Step 5   <hostname># svcadm enable system/cron

# Task 8: Reconfigure VSM Macro information

Step 1   Log in as root to EMS

Note : If  VSM was configured and recorded in the pre-upgrade step in Chapter 5 task 7 then, reconfigure the VSM on the Active EMS, otherwise, go to task 9.

Step 2   Reconfigure VSM (Note that VSM needs to be configured on Active EMS (Side A)

<hostname> su - btsadmin

<btsadmin> add macro ID=VSMSubFeature;PARAMETERS=subscriber.id,subscriber.dn1,subscriber_service_profile.service-id,service.fname1,service.fname2,service.fname3,service.fname4,service.fname5,service.fname6,service.fname7,service.fname8,service.fname9,service.fname10;AND_RULES=subscriber.id=subscriber_service_profile.sub-id,subscriber_service_profile.service-id=service.id

Macro_id = Macro value recorded in chapter 5 , task 7

- Verify that VSM is configured

<btsadmin> show macro id= VSM%

ID=VSMSubFeature

PARAMETERS=subscriber.id,subscriber.dn1,subscriber_service_profile.service-id,service.fname1,service.fname2,service.fname3,service.fname4,service.fname5,service.fname6,service.fname7,service.fname8,service.fname9,service.fname10

AND_RULES=subscriber.id=subscriber_service_profile.sub-id,subscriber_service_profile.service-id=service.id

<btsadmin> quit

# Task 9: To install CORBA on EMS side A and B, please follow Appendix I.

# Task 10: Verify system status

Verify that the system is operating properly before you leave the site.

Step 1   Verify that the side A systems are in the active state. Use Appendix A for this procedure.

Step 2   Verify that call processing is working without error. Use Appendix B for this procedure.

Step 3   Verify that provisioning is operational from CLI command line, and verify database. Use Appendix C for this procedure.

Step 4   Verify that there are no outstanding major or critical alarms. Use Appendix D for this procedure.

Step 5   Use Appendix E to verify that Oracle database and replication functions are working properly.

Step 6   From each machine, please run “/opt/utils/Resync_status” to check for disk mirroring status.

Step 7   If you have answered NO to any of the above questions (Step 1-5) do not proceed. Instead, use the software upgrade disaster recovery procedure in Appendix G. Contact Cisco Support if you need assistance.

If radius server is disabled at beginning of the upgrade, please use Appendix L Task 2 to enable the radius server.

You have completed the Cisco BTS 10200 system upgrade process successfully.

Appendix A

Check System Status

The purpose of this procedure is to verify the system is running in NORMAL mode, with the side A systems in ACTIVE state and the side B systems in STANDBY state.

|  | Note In the commands below, "xxx", "yyy" or "zzz" is the instance for the process on your system, and DomainName is your system domain name. |
| --- | --- |

# From Active EMS side A

Step 1   Log in as “btsuser” user.

Step 2   CLI> status system;

Step 3   CLI> exit;

Release 4.4.0 sample output:

Checking Call Agent status ... Checking Feature Server status ... Checking Billing Server status ... Checking Billing Oracle status ... Checking Element Manager status ... Checking EMS MySQL status ... Checking ORACLE status ... CALL AGENT STATUS IS... -> APPLICATION INSTANCE -> Call Agent [CA146] PRIMARY STATUS -> ACTIVE_NORMAL SECONDARY STATUS -> STANDBY_NORMAL FEATURE SERVER STATUS IS... -> APPLICATION INSTANCE -> Feature Server [FSPTC235] PRIMARY STATUS -> ACTIVE_NORMAL SECONDARY STATUS -> STANDBY_NORMAL FEATURE SERVER STATUS IS... -> APPLICATION INSTANCE -> Feature Server [FSAIN205] PRIMARY STATUS -> ACTIVE_NORMAL SECONDARY STATUS -> STANDBY_NORMAL BILLING SERVER STATUS IS... -> APPLICATION INSTANCE -> Bulk Data Management Server [BDMS01] PRIMARY STATUS -> ACTIVE_NORMAL SECONDARY STATUS -> STANDBY_NORMAL BILLING ORACLE STATUS IS... -> Daemon is running! ELEMENT MANAGER STATUS IS... -> APPLICATION INSTANCE -> Element Manager [EM01] PRIMARY STATUS -> ACTIVE_NORMAL SECONDARY STATUS -> STANDBY_NORMAL EMS MYSQL STATUS IS ... -> Daemon is running! ORACLE STATUS IS... -> Daemon is running! Reply : Success:

Release 4.5.1 MR1 sample output:

Checking Call Agent status ... Checking Feature Server status ... Checking Billing Server status ... Checking Billing Oracle status ... Checking Element Manager status ... Checking EMS MySQL status ... Checking ORACLE status ... CALL AGENT STATUS IS... -> APPLICATION INSTANCE -> Call Agent [CA146] PRIMARY STATUS -> ACTIVE SECONDARY STATUS -> STANDBY FEATURE SERVER STATUS IS... -> APPLICATION INSTANCE -> Feature Server [FSPTC235] PRIMARY STATUS -> ACTIVE SECONDARY STATUS -> STANDBY FEATURE SERVER STATUS IS... -> APPLICATION INSTANCE -> Feature Server [FSAIN205] PRIMARY STATUS -> ACTIVE SECONDARY STATUS -> STANDBY BILLING SERVER STATUS IS... -> APPLICATION INSTANCE -> Bulk Data Management Server [BDMS01] PRIMARY STATUS -> ACTIVE SECONDARY STATUS -> STANDBY BILLING ORACLE STATUS IS... -> Daemon is running! ELEMENT MANAGER STATUS IS... -> APPLICATION INSTANCE -> Element Manager [EM01] PRIMARY STATUS -> ACTIVE SECONDARY STATUS -> STANDBY EMS MYSQL STATUS IS ... -> Daemon is running! ORACLE STATUS IS... -> Daemon is running! Reply : Success:

Appendix B

Check Call Processing

This procedure verifies that call processing is functioning without error. The billing record verification is accomplished by making a sample phone call and verify the billing record is collected correctly.

# From EMS side A

Step 1   Log in as “btsuser” user.

Step 2   Make a new phone call on the system. Verify that you have two-way voice communication. Then hang up both phones.

Step 3   CLI> report billing-record tail=1;

.. CALLTYPE=TOLL SIGSTARTTIME=2004-05-03 17:05:21 SIGSTOPTIME=2004-05-03 17:05:35 CALLELAPSEDTIME=00:00:00 INTERCONNECTELAPSEDTIME=00:00:00 ORIGNUMBER=4692551015 TERMNUMBER=4692551016 CHARGENUMBER=4692551015 DIALEDDIGITS=9 4692551016# 5241 ACCOUNTCODE=5241 CALLTERMINATIONCAUSE=NORMAL_CALL_CLEARING ORIGSIGNALINGTYPE=0 TERMSIGNALINGTYPE=0 ORIGTRUNKNUMBER=0 TERMTRUNKNUMBER=0 OUTGOINGTRUNKNUMBER=0 ORIGCIRCUITID=0 TERMCIRCUITID=0 ORIGQOSTIME=2004-05-03 17:05:35 ORIGQOSPACKETSSENT=0 ORIGQOSPACKETSRECD=7040877 ORIGQOSOCTETSSENT=0 ORIGQOSOCTETSRECD=1868853041 ORIGQOSPACKETSLOST=805306368 ORIGQOSJITTER=0 ORIGQOSAVGLATENCY=0 TERMQOSTIME=2004-05-03 17:05:35 TERMQOSPACKETSSENT=0 TERMQOSPACKETSRECD=7040877 TERMQOSOCTETSSENT=0 TERMQOSOCTETSRECD=1868853041 TERMQOSPACKETSLOST=805306368 TERMQOSJITTER=0 TERMQOSAVGLATENCY=0 PACKETIZATIONTIME=0 SILENCESUPPRESSION=1 ECHOCANCELLATION=0 CODECTYPE=PCMU CONNECTIONTYPE=IP OPERATORINVOLVED=0 CASUALCALL=0 INTERSTATEINDICATOR=0 OVERALLCORRELATIONID=CA14633 TIMERINDICATOR=0 RECORDTYPE=NORMAL RECORD CALLAGENTID=CA146 ORIGPOPTIMEZONE=CDT ORIGTYPE=ON NET TERMTYPE=ON NET NASERRORCODE=0 NASDLCXREASON=0 ORIGPOPID=69 TERMPOPID=69 TERMPOPTIMEZONE=CDT DIALPLANID=cdp1 CALLINGPARTYCATEGORY=Ordinary Subscriber CALLEDPARTYINDICATOR=No Indication CALLEDPARTYPORTEDIN=No CALLINGPARTYPORTEDIN=No BILLINGRATEINDICATOR=None ORIGENDPOINTADDR=c2421-227-142.ipclab.cisco.com Reply : Success: Entry 1 of 1 returned from host: priems08

Step 4   Verify that the attributes in the CDR match the call just made.

Step 5	  CLI> exit;

Appendix C

Check Provisioning and Database

# From EMS side A

The purpose of this procedure is to verify that provisioning is functioning without error. The following commands will add a "dummy" carrier then delete it.

Step 1   Log in as “btsuser” user.

Step 2   CLI> add carrier id=8080;

Step 3   CLI> show carrier id=8080;

Step 4   CLI> delete carrier id=8080; 

Step 5   CLI> show carrier id=8080;

- Verify message is: Database is void of entries.

# Check transaction queue

In this task, you will verify that the OAMP transaction queue status. The queue should be empty.

Step 1   CLI> show transaction-queue;

- Verify there is no entry shown. You should get following reply back:

Reply : Success: Database is void of entries.

- If the queue is not empty, wait for the queue to empty. If the problem persists, contact Cisco Support.

Step 2 	CLI> exit

# Perform database audit

In this task, you will perform a full database audit and correct any errors, if necessary.

Step 1  Login as “ciscouser”

Step 2   CLI> audit database type=full;

Step 3   Check the audit report and verify there is no discrepancy or error. If errors are found, please try to correct them. If you are unable to correct, please contact Cisco Support.

Please follow the sample command provided below to correct the mismatches:

For the 4 handset provisioning tables (SLE, SC1D, SC2D, SUBSCRIBER-FEATURE-DATA), please use:

CLI> sync <table name> master=FSPTCyyy; target=EMS;

For all other tables, please use:

CLI> sync <table name> master=EMS; target=<CAxxx/FSPTCyyy/FSAINzzz>;

Step 4   CLI> exit

Appendix D

Check Alarm Status

The purpose of this procedure is to verify that there are no outstanding major/critical alarms.

# From EMS side A

Step 1   Log in as “btsuser” user.

Step 2   CLI> show alarm

- The system responds with all current alarms, which must be verified or cleared before proceeding with next step.

|  | Tip Use the following command information for reference material ONLY. |
| --- | --- |

Step 3   To monitor system alarm continuously.

CLI> subscribe alarm-report severity=all; type=all;

|  | Valid severity: MINOR, MAJOR, CRITICAL, ALL Valid types: CALLP, CONFIG, DATABASE, MAINTENANCE, OSS, SECURITY, SIGNALING, STATISTICS, BILLING, ALL, SYSTEM, AUDIT |
| --- | --- |

Step 4   System will display alarms if alarm is reported.

<------------------- START ALARM REPORT --------------------> TIMESTAMP: 20040503174759 DESCRIPTION: General MGCP Signaling Error between MGW and CA. TYPE & NUMBER: SIGNALING (79) SEVERITY: MAJOR ALARM-STATUS: OFF ORIGIN: MGA.PRIMARY.CA146 COMPONENT-ID: null ENTITY NAME: S0/DS1-0/1@64.101.150.181:5555 GENERAL CONTEXT: MGW_TGW SPECIFC CONTEXT: NA FAILURE CONTEXT: NA <------------------- END ALARM REPORT ---------------------->

Step 5   To stop monitoring system alarm.

CLI> unsubscribe alarm-report severity=all; type=all;

Step 6   CLI> exit

Appendix E

Check Oracle Database Replication and Error Correction

Perform the following steps on the Active EMS side A to check the Oracle database and replication status.

# Check Oracle DB replication status

# From EMS side A

Step 1   Log in as root.

Step 2   Log in as oracle.

<hostname># su – oracle

Step 3   Enter the command to check replication status and compare contents of tables on the side A and side B EMS databases:

<hostname>$ dbadm –C rep

Step 4   Verify that “Deferror is empty?” is “YES”.

OPTICAL1::Deftrandest is empty?           YES

OPTICAL1::dba_repcatlog is empty?       YES

OPTICAL1::Deferror is empty?              YES Make sure it is “YES”

OPTICAL1::Deftran is empty?                  YES

OPTICAL1::Has no broken job?               YES

OPTICAL1::JQ Lock is empty?                YES

OPTICAL2::Deftrandest is empty?          YES

OPTICAL2::dba_repcatlog is empty?      YES

OPTICAL2::Deferror is empty?             YES Make sure it is “YES”

OPTICAL2::Deftran is empty?                YES

OPTICAL2::Has no broken job?              YES

OPTICAL2::JQ Lock is empty?               YES

Step 5   If the “Deferror is empty?” is “NO”, please try to correct the error using steps in “Correct replication error” below. If you are unable to clear the error or if any of the individual steps fails, please contact Cisco Support.

Step 6 <hostname> $ exit

# Correct replication error

|  | Note You must run the following steps on standby EMS side B first, then on active EMS side A. |
| --- | --- |

# From EMS Side B

Step 1   Log in as root

Step 2   <hostname># su – oracle

Step 3   <hostname>$ dbadm –C db

- The above command will generate a list of out of sync tables.

- Example Output:

  - <hostname> $ dbadm –C db Checking table => OAMP.SCHEDULED COMMAND…..Different

- In the above output,  OWNER = OAMP and TABEL NAME = SCHEDULED COMMAND

Step 4   For each table that is out of sync, please run the following step:

<hostname>$ dbadm -A copy -o <OWNER> -t <TABEL NAME>

- Enter “y” to continue

- Please contact Cisco Support if the above command fails.

Step 5   <hostname>$ dbadm –A truncate_deferror

- Enter “y” to continue

Step 6 <hostname> $ exit

# From EMS Side A

Step 1   Login in as root.

Step 2   <hostname> # su – oracle

Step 3   <hostname>$ dbadm –A truncate_deferror

- Enter “y” to continue

Step 4   Re-verify that “Deferror is empty?” is “YES” and none of tables is out of sync.

<hostname>$dbadm –C db

OPTICAL1::Deftrandest is empty?           YES

OPTICAL1::dba_repcatlog is empty?       YES

OPTICAL1::Deferror is empty?              YES  Make sure it is “YES”

OPTICAL1::Deftran is empty?                  YES

OPTICAL1::Has no broken job?               YES

OPTICAL1::JQ Lock is empty?                YES

OPTICAL2::Deftrandest is empty?          YES

OPTICAL2::dba_repcatlog is empty?      YES

OPTICAL2::Deferror is empty?             YES  Make sure it is “YES”

OPTICAL2::Deftran is empty?                YES

OPTICAL2::Has no broken job?              YES

OPTICAL2::JQ Lock is empty?               YES

Step 5   <hostname> # exit

Appendix F

Backout Procedure for Side B Systems

Warning   After entire system is upgraded to release 4.5.1, if the system is provisioned with new CLI data fallback is not recommended. If fallback is absolutely required, please contact Cisco support.

# Introduction

This procedure allows you to back out of the upgrade procedure if any verification checks (in "Verify Call/System Status" section) failed. This procedure is intended for the scenario in which the side B systems have been upgraded to the new load, while the side A systems are still at the previous load. The procedure will back out the side B systems to the previous load.

This backout procedure will:

- Restore the side A systems to active mode without making any changes to it

- Revert to the previous application load on the side B systems

- Restart the side B systems in standby mode

- Verify that the system is functioning properly with the previous load

|  | Note In addition to performing this backout procedure, you should contact Cisco Support when you are ready to retry the upgrade procedure. |
| --- | --- |

The flow for this procedure is shown in Figure F-1.

Figure F-1   Flow of Backout Procedure— Side B Only

# Task 1: Force side A CA/FS to active

This procedure will force the side A systems to forced active state, and the side B systems to forced standby state.

|  | Note In the commands below, "xxx", "yyy" or "zzz" is the instance for the process on your system. |
| --- | --- |

# From EMS side B

Step 1   Log in as “btsuser” user.

Step 2   CLI> control call-agent id=CAxxx; target-state=active-standby;

Step 3   CLI> control feature-server id=FSPTCyyy; target-state=active-standby;

Step 4   CLI> control feature-server id=FSAINzzz; target-state=active-standby;

Step 5   CLI> exit

# Task 2: Force side A EMS to active

# From EMS side B

Step 1   Log in as root

Step 2   <hostname># platform stop all

# Task 3: Sync TSAP-ADDR

# From EMS side A

Step 1   Log in as “ciscouser”

CLI> sync call_agent master=EMS; target=CAxxx;

CLI> sync feature_server master=EMS; target=FSAINxxx;

CLI> sync feature_server master=EMS; target=FSPTCxxx;

# Task 4: SFTP billing records to a mediation device

# From EMS side B

Step 1   Log in as root

Step 2   <hostname># cd /opt/bms/ftp/billing

Step 3   <hostname># ls

Step 4   If there are files listed, then SFTP the files to a mediation device on the network and remove the files from the /opt/bms/ftp/billing directory

# Task 5: Sync DB usage

# From EMS side A

In this task, you will sync db-usage between two releases.

Step 1   Log in as root

Step 2   <hostname># su – oracle

Step 3   <hostname>$ java dba.adm.DBUsage –sync

- Verify Number of tables “unable-to-sync” is 0.

Step 4   <hostname>$ exit

# Task 6: Stop applications and shutdown side B systems

# From EMS side B

Step 1   Log in as root.

Step 2   <hostname># sync; sync

Step 3   <hostname># shutdown –i5 –g0 –y

# From CA/FS side B

Step 1   Log in as root.

Step 2   <hostname># platform stop all

Step 3   <hostname># sync; sync

Step 4   <hostname># shutdown –i5 –g0 –y

# Task 7: Restore side B systems to the old release

# From CA/FS side B

Step 1   Power off the machine

Step 2   Remove disk0 from slot 0 off the machine. Also remove disk1 from slot 1 off the machine.

- SunFire V440 disk slot lay out:

| Disk 3 | DVD-ROM |
| --- | --- |
| Disk 2 |  |
| Disk 1 |  |
| Disk 0 |  |

- Sunfire 1280 disk slot lay out:

|  | DVD-ROM |
| --- | --- |
|  | Disk 1 |
|  | Disk 0 |

Step 3   Place disk labeled “Release 4.4.0 CA/FS side B disk0” in slot 0. Also place disk labeled “Release 4.4.0 CA/FS side B disk1” in slot 1.

Step 4   Power on the machine

- For a Sunfire 1280 machine, please execute the following command from console:

<lom>poweron

- For other type of hardware, please use the power button to turn on the power.

Step 5   Log in as root.

Step 6   <hostname># platform start

Step 7   <hostname># mv /etc/rc3.d/_S99platform /etc/rc3.d/S99platform

# From EMS side B

Step 1   Power off the machine

Step 2   Remove disk0 from slot 0 off the machine. Also remove disk1 from slot 1 off the machine.

- SunFire V440 disk slot lay out:

| Disk 3 | DVD-ROM |
| --- | --- |
| Disk 2 |  |
| Disk 1 |  |
| Disk 0 |  |

- Sunfire 1280 disk slot lay out:

|  | DVD-ROM |
| --- | --- |
|  | Disk 1 |
|  | Disk 0 |

Step 3   Place disk labeled “Release 4.4.0 EMS side B disk0” in slot 0. Also place disk labeled “Release 4.4.0 EMS side B disk1” in slot 1.

Step 4   Power on the machine

- For a Sunfire 1280 machine, please execute the following command from console:

<lom>poweron

- For other type of hardware, please use the power button to turn on the power.

Step 5   Log in as root.

Step 6   <hostname># platform start

Step 7  <hostname># mv /etc/rc3.d/_S99platform /etc/rc3.d/S99platform

# Task 8: Restore EMS mate communication

In this task, you will restore the OMS Hub communication from EMS side A to side B.

# From EMS side A

Step 1   Log in as root

Step 2   <hostname># /opt/ems/utils/updMgr.sh –restore_hub

Step 3   <hostname># nodestat

- Verify OMS Hub mate port status is established.

- Verify HUB communication from EMS side A to CA/FS side B is established.

# Task 9: Switchover activity to EMS side B

# From Active EMS side A

Step 1   Log in as “btsuser” user.

Step 2   CLI> control bdms id=BDMS01; target-state=forced-standby-active;

Step 3   CLI> control element-manager id=EM01; target-state=forced-standby-active;

Step 4    CLI Log in session will be terminated when switchover is completed.

# Task 10: Enable Oracle DB replication on EMS side A

# From EMS side A

Step 1   Log in as Oracle user:

<hostname># su - oracle

<hostname>$ cd /opt/oracle/admin/utl

Step2   Restore Oracle DB replication:

<hostname>$ rep_toggle –s optical1 –t set_duplex

Answer “y” when prompt

Answer “y” again when prompt

Step 3   <hostname>$ exit

Step 4   Restart applications to connect to Oracle DB in duplex mode:

<hostname># platform stop all

<hostname># platform start

# Task 11: Synchronize handset provisioning data

# From EMS side B

|  | Note In the commands below, "xxx", "yyy" or "zzz" is the instance for the process on your system. |
| --- | --- |

Step 1   Log in as ciscouser (password: ciscosupport)

Step 2   CLI> sync termination master=CAxxx; target=EMS;

- Verify the transaction is executed successfully.

Step 3   CLI> sync sc1d master=FSPTCyyy; target=EMS;

- Verify the transaction is executed successfully

Step 4   CLI> sync sc2d master=FSPTCyyy; target=EMS;

- Verify the transaction is executed successfully

Step 5   CLI> sync sle master=FSPTCyyy; target=EMS;

- Verify the transaction is executed successfully

Step 6   CLI> sync subscriber-feature-data master=FSPTCyyy; target=EMS;

- Verify the transaction is executed successfully

Step 7   CLI> exit

# Task 12: Switchover activity from EMS side B to EMS side A

# From EMS side B

Step 1   Log in as “btsuser” user.

Step 2   CLI> control bdms id=BDMS01; target-state=forced-active-standby;

Step 3   CLI> control element-manager id=EM01; target-state=forced-active-standby;

Step 4    CLI Log in session will be terminated when switchover is completed.

# Task 13: Restore system to normal mode

# From EMS side A

Step 1   Log in as “btsuser” user.

Step 2   CLI> control feature-server id=FSPTCyyy; target-state=normal;

Step 3   CLI> control feature-server id=FSAINzzz; target-state=normal;

Step 4   CLI> control call_agent id=CAxxx; target-state=normal;

Step 5   CLI> control bdms id=BDMS01; target-state=normal;

Step 6   CLI> control element-manager id=EM01; target-state=normal;

Step 7   CLI> exit

# Task 14: Verify system status

Verify that the system is operating properly before you leave the site.

Step 1   Verify that the side A systems are in the active state. Use Appendix A for this procedure.

Step 2   Verify that call processing is working without error. Use Appendix B for this procedure.

Step 3   Verify that provisioning is operational from CLI command line, and verify database. Use Appendix C for this procedure.

Step 4   Verify that there are no outstanding major or critical alarms. Use Appendix D for this procedure.

Step 5   Use Appendix E to verify that Oracle database and replication functions are working properly.

Step 6   From each machine, please run “/opt/utils/Resync_status” to check for disk mirroring status.

Step 7   If you answered NO to any of the above questions (Step 1 through Step 5), Contact Cisco Support for assistance.

You have completed the side B of Cisco BTS 10200 system fallback process successfully.

Appendix G

Software Upgrade Disaster Recovery Procedure

Disaster Recovery Requirements

This procedure is recommended only when full system upgrade to release 4.5.1 MR1 has been completed and the system is experiencing unrecoverable problems for which the only solution is to take a full system service outage and restore to the previously working release as quickly as possible.

# Assumptions

The following assumptions are made.

- The installer has a basic understanding of UNIX and Oracle commands.

- The installer has the appropriate user name(s) and password(s) to log on to each EMS/CA/FS platform as root user, and as Command Line Interface (CLI) user on the EMS.

- Total live traffic outage for a period of approximately 30 minutes is acceptable

- All analysis of the outage is leading to the BTS as the main contributor in the current situation

|  | Note: Contact Cisco Support before you start if you have any questions. |
| --- | --- |

# Requirements

Locate release 4.4.X disks with following label:

- Side A EMS	--  “Release 4.4.X EMS side A”

- Side B EMS	--  “Release 4.4.X EMS side B”

- Side A CA/FS	--  “Release 4.4.X CA/FS side A”

- Side B CA/FS	--  “Release 4.4.X CA/FS side B”

# Important notes about this procedure

Throughout this procedure, each command is shown with the appropriate system prompt, followed by the command to be entered in bold. The prompt is generally one of the following:

- Host system prompt (<hostname>#)

- Oracle prompt (<hostname>$)

- SQL prompt (SQL>)

- CLI prompt (CLI>)

- SFTP prompt (sftp>)

- Ok prompt (ok>)

- Throughout the steps in this procedure, enter commands as shown, as they are case sensitive (except for CLI commands).

- It is recommended that you read through the entire procedure before performing any steps.

- The system will incur about 30 minutes of total live traffic outage before side B systems can be fully functional. Please plan accordingly to minimize any negative service impact.

- There will be no new provisioning allowed during entire disaster recovery process.

- Newly provisioned data including handset and CLI data, after the point that side B system disks were swapped out are lost.

System Disaster Recovery Procedure

Warning   Executing this backout procedure will result in loss of all provisioning data which includes north bound handset data from the point where the side B system disks were swapped out. Loss of billing CDR is also expected.

Introduction

This backout procedure allows you to restore the BTS system back to release 4.4.X. This procedure is intended for the disaster scenario in which the entire system has been upgraded to the release 4.5.1 MR1 load and calls cannot be made or maintained.

This backout procedure will:

- Power off each BTS machine

- Restore the side-B systems with release 4.4.X disks

- Bring up applications to active mode without making any changes to it

- Verify that the system is functioning properly with the previous load

- Restore the side-A systems with release 4.4.X disks

- Restart the side-A systems in standby mode

- Control the system to a normal operational state  -- side A active, side B standby

Note:   In addition to performing this backout procedure, you should contact Cisco Support when you are ready to retry the upgrade procedure.

# Task 1: Shutdown each machine

From CA/FS side B

Step 1   Login to the CA/FS Side B machine through console server and shutdown the machine.

<hostname># eeprom diag-level=init (For SUN 1280’s only)

<hostname># sync;sync;shutdown –i5 –g0 –y

- Notice the normal console prompt is now changed to lom>

Step 2   Remove disk0 from slot 0 off the machine with label “Release 4.5.1 MR1 CA/FS side B disk0”. Also remove disk1 from slot 1 off the machine with label “Release 4.5.1 MR1 CA/FS side B disk1”.

- Sunfire 1280 disk slot lay out:

|  | DVD-ROM |
| --- | --- |
|  | Disk 1 |
|  | Disk 0 |

Step 3   Place disk labeled with “Release 4.4.X CA/FS side B disk0” in slot 0.  Also place disk labeled with “Release 4.4.X CA/FS side B disk1” in slot 1.

From EMS side B

Step 1   Login to the EMS Side B machine through console server and shutdown the machine.

<hostname># eeprom diag-level=init (For SUN 1280’s only)

<hostname># sync;sync;shutdown –i5 –g0 –y

- Notice the normal console prompt is now changed to lom>

Step 2   Remove disk0 from slot 0 off the machine with label “Release 4.5.1 MR1 EMS side B disk0”. Also remove disk1 from slot 1 off the machine with label “Release 4.5.1 MR1 EMS side B disk1”.

- Sunfire 1280 disk slot lay out:

|  | DVD-ROM |
| --- | --- |
|  | Disk 1 |
|  | Disk 0 |

Step 3   Place disk labeled with “Release 4.4.X EMS side B disk0” in slot 0.  Also place disk labeled with “Release 4.4.X EMS side B disk1” in slot 1.

From CA/FS side A

Step 1   Login to the CA/FS Side A machine through console server and shutdown all applications

<hostname># platform stop all

<hostname># cd /opt/ems/bin

<hostname># daemon_mgr.sh stop

<hostname># daemon_mgr.sh remove

From EMS side A

Step 1   Login to the EMS Side A machine through console server and shutdown all applications

<hostname># platform stop all

<hostname># cd /opt/ems/bin

<hostname># daemon_mgr.sh stop

<hostname># daemon_mgr.sh remove

# Task 2: Restore CA/FS side B to the old release

From CA/FS side B

Step 1   Power on the machine

<lom>poweron

Step 2   Monitor the system boot progress thru console, log back in when system completes boot process

Note:   It is recommended to perform Task 3 and Task 4 in parallel.

# Task 3: Bring up applications on CA/FS side B

From CA/FS side B

Step 1   Login as root.

Step 2   <hostname># platform start

Step 3   <hostname># mv /etc/rc3.d/_S99platform /etc/rc3.d/S99platform

# Task 4: Restore EMS side B to the old release

From EMS side B

Step 1   Power on the machine

<lom>poweron

Step 2   Monitor the system boot progress thru console, log back in when system completes boot process

Step 3   Login as root.

Step 4   <hostname># platform start

Step 5   <hostname># mv /etc/rc3.d/_S99platform /etc/rc3.d/S99platform

# Task 5: Verify system health

Verify that the system is functioning properly with the previous load

Step 1   Verify that call processing is working by making on-net and off net calls

Step 2   Verify that Billing CDRs are being generated with correct billing details for the calls just made

- Log in to EMS side B as CLI user

- CLI> report billing-record tail=<Choose # CDRs to be displayed>;

Step 3   Verify CLI data provision is functioning properly from EMS side B

- CLI> add carrier id=8080;

- CLI> show carrier id=8080;

- CLI> delete carrier id=8080;

- CLI> show carrier id=8080;

- CLI> show transaction-queue;

  - You should expect to see following:

Reply : Success: Database is void of entries.

- CLI> exit;

Step 4   Verify Oracle DB replication on EMS side B is enabled:

<hostname># su - oracle

<hostname>$ cd /opt/oracle/admin/utl

<hostname>$ rep_toggle -s optical2 -t show_mode

  - You should expect to see following:

The optical2 database is set to DUPLEX now.

<hostname># exit

Step 5   If you answered NO to any of the above questions (Step 1 through Step 4), Contact Cisco Support for assistance.

Note:   It is recommended to perform Task 6 and Task 7 in parallel.

# Task 6: Restore CA/FS side A to the old release

From CA/FS side A

Step 1   Login to the CA/FS Side A machine through console server and shutdown the machine.

<hostname># eeprom diag-level=init (For SUN 1280’s only)

<hostname># sync;sync;shutdown –i5 –g0 –y

- Notice the normal console prompt is now changed to lom>

Step 2   Remove disk0 from slot 0 off the machine with label “Release 4.5.1 MR1 CA/FS side A disk0”. Also remove disk1 from slot 1 off the machine with label “Release 4.5.1 MR1 CA/FS side A disk1”.

- Sunfire 1280 disk slot lay out:

|  | DVD-ROM |
| --- | --- |
|  | Disk 1 |
|  | Disk 0 |

Step 3   Place disk labeled with “Release 4.4.X CA/FS side A disk0” in slot 0.  Also place disk labeled with “Release 4.4.X CA/FS side A disk1” in slot 1.

Step 4   Power on the machine

<lom>poweron

Step 5   Monitor the system boot progress thru console, log back in when system completes boot process

Step 6   <hostname># /opt/ems/utils/install.sh -clearshm

- Answer “y” when prompt

Step 7   Login as root.

Step 8   <hostname># platform start

Step 9   <hostname># mv /etc/rc3.d/_S99platform /etc/rc3.d/S99platform

# Task 7: Restore EMS side A to the old release

From EMS side A

Step 1   Login to the EMS Side A machine through console server and shutdown the machine.

<hostname># eeprom diag-level=init (For SUN 1280’s only)

<hostname># sync;sync;shutdown –i5 –g0 –y

- Notice the normal console prompt is now changed to lom>

Step 2   Remove disk0 from slot 0 off the machine with label “Release 4.5.1 MR1 EMS side A disk0”. Also remove disk1 from slot 1 off the machine with label “Release 4.5.1 MR1 EMS side A disk1”.

- Sunfire 1280 disk slot lay out:

|  | DVD-ROM |
| --- | --- |
|  | Disk 1 |
|  | Disk 0 |

Step 3   Place disk labeled with “Release 4.4.X EMS side A disk0” in slot 0.  Also place disk labeled with “Release 4.4.X EMS side A disk1” in slot 1.

Step 4   Power on the machine:

<lom>poweron

Step 5   Monitor the system boot progress thru console, log back in when system completes boot process

Step 6   <hostname># /opt/ems/utils/updMgr.sh -restore_hub

Step 7   Login as root.

Step 8   <hostname># platform start -i oracle

Step 9   Log in as Oracle user:

<hostname># su – oracle

Step 10   Copy data from EMS side B:

<hostname>$ cd /opt/oracle/admin/upd

<hostname>$ java dba.upd.UPDMgr -loadconfig

<hostname>$ java dba.upd.UPDMgr -skip reset copy

<hostname>$ java dba.upd.UPDMgr -copy all

<hostname>$ grep "FAIL=" UPDMgr.log

- Verify the FAIL count 0 is reported

<hostname>$ grep constraint UPDMgr.log | grep –i warning

- Verify there is no constraint warning reported

Note: if FAIL count is not 0 and/or there is constraint warning, please contact Cisco support.

Step 11   Set EMS side A replication to duplex:

<hostname>$ cd /opt/oracle/admin/utl

<hostname>$ rep_toggle -s optical1 -t set_duplex

- Answer “y” when prompt

- Answer “y” again when prompt

Step 12   <hostname>$ java dba.adm.DBUsage –sync

- Verify Number of tables “unable-to-sync” is 0.

Step 13   <hostname>$ exit

Step 14   Make sure all oracle DB connections are terminated:

<hostname># platform stop -i oracle

Step 15   Bring up all BTS applications:

<hostname># platform start

Step 16   <hostname># mv /etc/rc3.d/_S99platform /etc/rc3.d/S99platform

# Task 8:Enable Bulk Provisioning from EMS side A & B

Step 1    Login as root on EMS

Step 2	  Enable bulk provisioning.

<hostname># cd /opt/ems/bin

<hostname># echo "update PlatformPrograms set enable='Y' where id='DLP';" | mysql_cli.sh

<hostname># pkill smg

# Task 9: Verify system status

Verify that the system is operating properly before you leave the site.

Step 1   Perform full database audit between EMS and CA/FS:

- Log in to EMS side B as CLI user

- CLI> audit database;

  - Check audit report.  If there are any mismatches found, please contact Cisco support immediately.

- CLI> exit;

Step 2   Performance Oracle DB audit between EMSs:

<hostname># su - oracle

<hostname>$ dbadm -C db

  - You should expect to see following:

optical2:secems36:/opt/orahome$ dbadm -C db

OPTICAL2::Deftrandest is empty?         YES

OPTICAL2::dba_repcatlog is empty?     YES

OPTICAL2::Deferror is empty?              YES

OPTICAL2::Deftran is empty?               YES

OPTICAL2::Has no broken job?            YES

OPTICAL2::JQ Lock is empty?             YES

OPTICAL1::Deftrandest is empty?         YES

OPTICAL1::dba_repcatlog is empty?     YES

OPTICAL1::Deferror is empty?            YES

OPTICAL1::Deftran is empty?              YES

OPTICAL1::Has no broken job?            YES

OPTICAL1::JQ Lock is empty?             YES

You are connecting to OPTICAL2, remote DB is OPTICAL1

…

Number of tables to be checked: 195

Number of tables checked OK:    195

Number of tables out-of-sync:   0

  - If “Number of tables out-of-sync” is not 0, please contact Cisco support immediately.

<hostname>$ exit

Step 3   Check for critical and major alarms

- Log in to EMS side B as CLI user

- CLI> show alarm severity=MAJOR;

- CLI> show alarm severity=CRITICAL;

- CLI> exit

  - Check report.  Please determine each major and critical condition, contact Cisco support if necessary.

Step 4   If you encounter issues in any of the above steps (Step 1 through Step 3), Contact Cisco Support for assistance.

You have completed the disaster recovery process successfully.

Appendix H

# Preparing Disks for Upgrade

This software upgrade will need 8 disks: 2 for each machine. Each set of 2 disks must have the same model number in order for disk mirroring to work.

The NIDS information required for disk preparation must be different from that used on the system to be upgraded.

The disk preparation/staging should be done on a separate platform (from the system to be upgraded), but the hardware should be identical.

# Task 1: Locate CD-ROM Discs

Please locate following 3 CD-ROM Discs:

- Locate CD-ROM Disc labeled as “BTS 10200 Application”

- Locate CD-ROM Disc labeled as “BTS 10200 Database”

- Locate CD-ROM Disc labeled as “BTS 10200 Oracle Engine”

# Task 2: Locate and label the Disks

# Label disks for EMS Side A

Locate two new disk drives identical to the machine to be upgraded and label the first disk drive as “Release 4.5.1 MR1 EMS side A disk0” and the second disk drive as “Release 4.5.1 MR1 EMS side A disk1”.  Please follow the steps below to prepare the two disk drives. The second disk drive will be used as the backup in case the first disk drive goes bad.

# Label Disks for EMS Side B

Locate two new disk drives identical to the machine to be upgraded and label the first disk drive as “Release 4.5.1 MR1 EMS side B disk0” and the second disk drive as “Release 4.5.1 MR1 EMS side B disk1”.  Please follow the steps below to prepare the two disk drives. The second disk drive will be used as the backup in case the first disk drive goes bad.

# Label Disks for CA/FS Side A

Locate two new disk drives identical to the machine to be upgraded and label the first disk drive as “Release 4.5.1 MR1 CA/FS side A disk0” and the second disk drive as “Release 4.5.1 MR1 CA/FS side A disk1”.  Please follow the steps below to prepare the two disk drives. The second disk drive will be used as the backup in case the first disk drive goes bad.

# Label Disks for CA/FS Side B

Locate two new disk drives identical to the machine to be upgraded and label the first disk drive as “Release 4.5.1 MR1 CA/FS side B disk0” and the second disk drive as “Release 4.5.1 MR1 CA/FS side B disk1”.  Please follow the steps below to prepare the two disk drives. The second disk drive will be used as the backup in case the first disk drive goes bad.

# Task 3: Disk slot lay out

- SunFire V440 disk slot lay out:

| Disk 3 | DVD-ROM |
| --- | --- |
| Disk 2 |  |
| Disk 1 |  |
| Disk 0 |  |

- Sunfire 1280 disk slot lay out:

|  | DVD-ROM |
| --- | --- |
|  | Disk 1 |
|  | Disk 0 |

# Task 4: Construct opticall.cfg

Step 1   Get a copy of Cisco BTS 10200 Software Release 4.5.1 Network Information Data Sheets (NIDS). Please follow the link below for NIDS and opticall.cfg information: http://lbj/push_targets1/ucdit/cc/td/doc/product/voice/bts10200/bts4_5/install/nids_45.xls

Step 2   Fill in NIDS information for the BTS system used for disk preparation. The NIDS information must be different from that used on the system to be upgraded.

Step 3   Get a copy of Cisco BTS 10200 Software Release 4.5.1 opticall.cfg

Step 4   Fill in parameters in opticall.cfg from NIDS and placed it on a server which will be accessible from the BTS system used for disk preparation.

# Task 5: Disk preparation

Repeat the steps in Task 5 to prepare the second disk.

Note:  Please perform disk preparation for each machine in parallel.

# For both EMS side A and B

Step 1   Locate a system with the identical hardware as the machine to be upgraded

Step 2   Place disk0 to slot 0 and disk 1 to slot 1

Step 3   Jumpstart the machine with Solaris 10 OS by following the jumpstart procedure: http://lbj/push_targets1/ucdit/cc/td/doc/product/voice/bts10200/bts4_5/install/sol10cdj.doc

Step 4   Configured the machine with 4/2 network configuration

Step 5   Stage Cisco BTS 10200 Software Release 4.5.1 to the /opt/Build directory.

From EMS Side B:

- Log in as root

- Put Disc labeled as BTS 10200 Application Disc in the CD-ROM drive

- Remove old files and mount CD-ROM drive

<hostname># cd /

<hostname># \rm -rf /opt/Build

<hostname># mkdir -p /cdrom

<hostname># mount -o ro -F hsfs /dev/dsk/c0t0d0s0 /cdrom

- Copy file from the CD-ROM to the /opt directory.

<hostname># cp –f /cdrom/K9-opticall.tar.gz  /opt

- Verify that the check sum values match with the values located in the “checksum.txt” file located on Application Disc

<hostname># cat /cdrom/checksum.txt

<hostname># cksum /opt/K9-opticall.tar.gz

<hostname># umount /cdrom

- Manually eject the CD-ROM and take out Disc from drive

- Put Disc labeled as BTS 10200 Database Disc in the CD-ROM drive

- Mount and copy file

<hostname># mount -o ro -F hsfs /dev/dsk/c0t0d0s0 /cdrom

<hostname># cp –f /cdrom/K9-btsdb.tar.gz  /opt

<hostname># cp –f /cdrom/K9-extora.tar.gz  /opt

- Verify that the check sum values match with the values located in the “checksum.txt” file located on Oracle CD-ROM.

<hostname># cat /cdrom/checksum.txt

<hostname># cksum /opt/K9-btsdb.tar.gz

<hostname># cksum /opt/K9-extora.tar.gz

<hostname># umount /cdrom

- Manually eject the CD-ROM and take out Disc from drive

- If you have Customer Built Oracle DB engine, please use the Disc provided by the customer. Otherwise, put Disc labeled as BTS 10200 Oracle Engine Disc in the CD-ROM drive

- Mount and copy file

<hostname># mount -o ro -F hsfs /dev/dsk/c0t0d0s0 /cdrom

<hostname># cp –f /cdrom/K9-oraengine.tar.gz  /opt

- Verify that the check sum values match with the values located in the “checksum.txt” file located on Oracle CD-ROM.

<hostname># cat /cdrom/checksum.txt

<hostname># cksum /opt/K9-oraengine.tar.gz

<hostname># umount /cdrom

- Manually eject the CD-ROM and take out Disc from drive

- Ftp files to EMS side A

<hostname># cd /opt

<hostname># sftp <EMS Side A hostname>

sftp> cd /opt

sftp> put K9-opticall.tar.gz

sftp> put K9-btsdb.tar.gz

sftp> put K9-extora.tar.gz

sftp> put K9-oraengine.tar.gz

sftp> exit

- Extract tar files.

<hostname># gzip -cd K9-opticall.tar.gz | tar -xvf -

<hostname># gzip -cd K9-btsdb.tar.gz | tar -xvf -

<hostname># gzip -cd K9-extora.tar.gz | tar -xvf -

<hostname># gzip -cd K9-oraengine.tar.gz | tar -xvf -

|  | Note: Each file will take up 5-10 minutes to extract. |
| --- | --- |

From EMS Side A:

- Log in as root

- Remove old files and extract new files

<hostname># cd /opt

<hostname># \rm -rf Build

<hostname># gzip -cd K9-opticall.tar.gz | tar -xvf -

<hostname># gzip -cd K9-btsdb.tar.gz | tar -xvf -

<hostname># gzip -cd K9-extora.tar.gz | tar -xvf -

<hostname># gzip -cd K9-oraengine.tar.gz | tar -xvf -

|  | Note: Each file will take up 5-10 minutes to extract. |
| --- | --- |

Step 6   ftp opticall.cfg from the server (where the file was placed in Task 4 above) and place it under /etc directory.

<hostname># /opt/Build/checkCFG

- Verify the information in /etc/opticall.cfg is free of errors.

Note: The EMS side A and side B installation must be started in parallel.

From both EMS:

Step 7   Install Cisco BTS 10200 Software Release 4.5.1 application software.

<hostname># /opt/Build/install.sh

  - Answer “y” when prompt. This installation process could take up to 1hour and 30 minutes.

  - Answer "y” when prompt for “reboot”

  - Wait for the system to boot up. Then Log in as root

Step 8   <hostname># mv /etc/rc3.d/S99platform /etc/rc3.d/_S99platform

Step 9   Shutdown the applications

<hostname># platform stop all

Step 10   Mirror the disks.

<hostname># /opt/setup/setup_mirror_ems

<hostname># sync;sync;reboot -- -r

  - Wait for the system to boot then log in to EMS Side B as root.

<hostname># /opt/setup/sync_mirror

Note: It takes 2 to 2.5 hours to complete the disk mirroring process on each machine. Please run “/opt/utils/Resync_status” to check for the disk mirroring status. The display will show the resyncing in progress and reports resync completion.

Step 11   Remove all network interface configuration information from the machine:

<hostname>#  \rm /etc/hostname.*

Step 12   Shutdown and power off the machine then remove the disks to be used for upgrade.

<hostname># sync;sync;shutdown –i5 –g0 -y

# For both CA/FS side A and B

Step 1   Locate a system with the identical hardware as the machine to be upgraded

Step 2   Place disk0 to slot 0 and disk 1 to slot 1

Step 3   Jumpstart the machine with Solaris 10 OS by following the jumpstart procedure: http://lbj/push_targets1/ucdit/cc/td/doc/product/voice/bts10200/bts4_5/install/sol10cdj.doc

Step 4   Configured the machine with 4/2 network configuration

Step 5   Stage Cisco BTS 10200 Software Release 4.5.1 to the /opt/Build directory.

From CA/FS Side B:

- Log in as root

- Put Disc labeled as BTS 10200 Application Disc in the CD-ROM drive

- Remove old files and mount CD-ROM drive

<hostname># cd /

<hostname># \rm -rf /opt/Build

<hostname># mkdir -p /cdrom

<hostname># mount -o ro -F hsfs /dev/dsk/c0t0d0s0 /cdrom

- Copy file from the CD-ROM to the /opt directory.

<hostname># cp –f /cdrom/K9-opticall.tar.gz  /opt

- Verify that the check sum values match with the values located in the “checksum.txt” file located on Application Disc

<hostname># cat /cdrom/checksum.txt

<hostname># cksum /opt/K9-opticall.tar.gz

<hostname># umount /cdrom

- Manually eject the CD-ROM and take out Disc from drive

- Ftp files to CA/FS side A

<hostname># cd /opt

<hostname># sftp <CA/FS Side A hostname>

sftp> cd /opt

sftp> put K9-opticall.tar.gz

sftp> exit

- Extract tar files.

<hostname># gzip -cd K9-opticall.tar.gz | tar -xvf –

|  | Note: The file will take up 5-10 minutes to extract. |
| --- | --- |

From CA/FS Side A:

- Log in as root

- Remove old files and extract new files

<hostname># cd /opt

<hostname># \rm -rf Build

<hostname># gzip -cd K9-opticall.tar.gz | tar -xvf -

|  | Note: The file will take up 5-10 minutes to extract. |
| --- | --- |

From both CA/FS:

Step 6   ftp opticall.cfg from the server (where the file was placed in Task 4 above) and place it under /etc directory.

Step 7   Install BTSbase, BTSinst, BTSossh and BTShard packages:

- <hostname># cd /opt/Build

- <hostname># pkgadd –d . BTSbase

- Answer “y” when prompt

- <hostname># pkgadd –d . BTSinst

- Answer “y” when prompt

- <hostname># pkgadd –d . BTSossh

- Answer “y” when prompt

- <hostname># pkgadd –d . BTShard

- Answer “y” when prompt

- <hostname># cd /etc/rc3.d

- <hostname># mv S99platform _S99platform

- <hostname># sync;sync;reboot

- Wait for system to boot up and login as root.

Step 8   Update version information:

<hostname># echo “900-04.04.00.V00” > /opt/ems/utils/Version

Step 9   Mirror the disks.

<hostname># /opt/setup/setup_mirror_ca

<hostname># sync;sync;reboot -- -r

  - Wait for the system to boot then log in to EMS Side B as root.

<hostname># /opt/setup/sync_mirror

Note: It takes 2 to 2.5 hours to complete the disk mirroring process on each machine. Please run “/opt/utils/Resync_status” to check for the disk mirroring status. The display will show the resyncing in progress and reports resync completion.

Step 10   Remove all network interface configuration information from the machine:

<hostname>#  \rm /etc/hostname.*

Step 11   Shutdown and power off the machine then remove the disks to be used for upgrade.

<hostname># sync;sync;shutdown –i5 –g0 -y

Appendix I

CORBA Installation

This procedure describes how to install the Common Object Request Broker Architecture (CORBA) application on Element Management System (EMS) of the Cisco BTS 10200 Softswitch.

Note This installation process is used for both side A and side B EMS.

Caution This CORBA installation will remove existing CORBA application on EMS machines. Once you have executed this procedure, there is no backout. Do not start this procedure until you have proper authorization. If you have questions, please contact Cisco Support.

# Task 1: Open Unix Shell on EMS

Perform these steps to open a Unix shell on EMS.

Step 1   Ensure that your local PC or workstation has connectivity via TCP/IP to communicate with EMS units.

Step 2   Open a Unix shell or a XTerm window.

Note If you are unable to open a Xterm window, please contact you system administrator immediately.

# Task 2: Install OpenORB CORBA Application

### Remove Installed OpenORB Application

Step 1   Log in as root to EMS

Step 2   Enter the following command to remove the existing OpenORB package:

<hostname># pkgrm BTScis

- Respond with a “y” when prompted

<hostname># pkgrm BTSoorb

- Respond with a “y” when prompted

Step 3   Enter the following command to verify that the CORBA application is removed:

<hostname># pgrep cis3

The system will respond by displaying no data, or by displaying an error message. This verifies that the CORBA application is removed.

### Install OpenORB Packages

The CORBA application files are available for installation once the Cisco BTS 10200 Softswitch is installed.

Step 1   Log in as root to EMS

Note : If  VIP was configured and recorded in the pre-upgrade step in Chapter 5 task 6 then, reconfigure the VIP on the Active EMS, otherwise, go to Step 3.

Step 2   Reconfigure VIP (Note that VIP needs to be configured on Active EMS (Side A)

<hostname> su - btsadmin

<btsadmin> change ems interface=<INTERFACE>;ip_alias=<VIP>

INTERFACE = Interface value recorded in chapter 5 , task 6

VIP =  ip-alias value recorded in chapter 5, task 6

- Verify that VIP is configured

<btsadmin> show ems

IP_ALIAS=10.89.224.177
INTERFACE=eri0
NTP_SERVER=10.89.224.

<btsadmin> quit

Step 3   <hostname># cd /opt/Build

Step 4   <hostname># cis-install.sh

- The system will give several prompts before and during the installation process. Some prompts are repeated. Respond with a “y” when prompted.

Step 5   It will take about 5-8 minutes for the installation to complete.

Step 6   Verify CORBA Application is running On EMS:

<hostname># pgrep ins3

Note System will respond by displaying the Name Service process ID, which is a number between 2 and 32,000 assigned by the system during CORBA installation. By displaying this ID, the system confirms that the ins3 process was found and is running.

<hostname># pgrep cis3

Note The system will respond by displaying the cis3 process ID, which is a number between 2 and 32,000 assigned by the system during CORBA installation. By displaying this ID, the system confirms that the cis3 process was found and is running.

Step 7   If you do not receive both of the responses described in Step 5, or if you experience any verification problems, do not continue. Contact your system administrator. If necessary, call Cisco Support for additional technical assistance.

Appendix J

Block Provisioning Path

# From EMS side A and B

Make sure the provisioning path is blocked.

Step 1   Terminate all existing CLI sessions:

<hostname># pkill cli.sh

Step 2    Disable bulk provisioning:

<hostname># cd /opt/ems/bin

<hostname># echo "update PlatformPrograms set enable='N' where id='DLP';" | mysql_cli.sh

<hostname># pkill smg

Step 3    Check to see if CORBA is installed:

<hostname># grep –w cis3 /etc/inittab

- If no results returned, please skip over the rest of the steps

Step 4    Disable CORBA:

<hostname># pkgrm BTScis

<hostname># pkgrm BTSoorb

<hostname># ps –ef |grep –i D_CIS |grep –v grep|awk ‘{print $2;}’

The output of the above command may display the process ID of one or more orphaned processes that need to be killed. If the PID is displayed, kill each of the process individually using the command below.

<hostname># kill -9 PID

<hostname># ps –ef |grep –i D_INS |grep –v grep|awk ‘{print $2;}’

The output of the above command may display the process ID of one or more orphaned processes that need to be killed. If the PID is displayed, kill each of the process individually using the command below

<hostname># kill -9 PID

Appendix K

Files handled by DoTheChange script

The following file list is FTPed from mate as the first execution step after replacing disks with new release:

- resolv.conf

- netmasks

- TIMEZONE

- Defaultdomain

- hostname.*

- passwd

- shadow

- group

- users.tar

- ntp.conf

- S96StaticRoutes

The following file list is modified with either new hostname or to preserve existing user accounts in the system:

- /etc/opticall.cfg

- /etc/nodetype

- /etc/hostname.*

- /etc/nodename

- /etc/hosts

- /etc/resolv.conf

- /etc/passwd

- /etc/shadow

- /etc/group

- /etc/TIMEZONE

- /etc/defaultdomain

- /etc/rc3.d/S96StaticRoutes

- /etc/inet/netmasks

- /etc/inet/ipsecinit.conf

- /etc/snmp/conf/snmpd.conf

- /opt/BTSxntp/etc/ntp.conf

- /opt/BTSossh/etc/sshd_config

- /opt/SMCapache/conf/httpd.conf

- /opt/SMCapache/conf/ssl.crt/*.crt

- /opt/SMCapache/conf/ssl.crt/*.key

- /opt/BTSoorb/config/OpenORB.xml

- /opt/BTSoorb/config/OpenORB-INS.xml

- /opt/ems/etc/ems.props

- /opt/oracle/admin/etc/listener.ora

- /opt/oracle/admin/etc/tnsnames.ora

- /opt/oracle/admin/etc/protocol.ora

- /opt/BTSordba/etc/dba.properties

- /opt/BTSoramg/etc/ora.properties

- /opt/ems/bin/platform.cfg

- /opt/bdms/bin/platform.cfg

Appendix L

Disable and Enable Radius Server

# Task 1: Disable Radius Server

# From Each Machine

Step 1   Log in as root. If root access is disabled, log in as an admin user.

Step 2   <hostname># vi /etc/pam.conf

- Search for the line end with: /usr/lib/security/pam_radius_auth.so.1

- Put the sign “#” at beginning of the line

- Save the file

# Task 2: Enable Radius Server

# From Each Machine

Step 1   Log in as root

Step 2   <hostname># vi /etc/pam.conf

- Search for the line end with: /usr/lib/security/pam_radius_auth.so.1

- Remove the sign “#” at beginning of the line

- Save the file

Appendix M

Install IP01 Patch on EMS

Step 1  Locate CD-ROM Disc labeled as " BTS_04_05_01_V24_IP01”

- Log in as root

- Put Disc labeled as " BTS_04_05_01_V24_IP01” in the CD-ROM drive

- Mount CD-ROM drive

<hostname># cd /

<hostname># mkdir -p /cdrom

<hostname># mount -o ro -F hsfs /dev/dsk/c0t0d0s0 /cdrom

- Copy file from the CD-ROM to the /opt directory.

<hostname># cp –f /cdrom/ BTS_04_05_01_V24_IP01.tar /opt

<hostname># umount /cdrom

- Manually eject the CD-ROM and take out Disc from drive

Step 2	Untar the file on EMS platform.

<hostname># cd /opt

<hostname># tar -xvf /opt/BTS_04_05_01_V24_IP01.tar

Step 3	Update PatchLevel

<hostname># cp /opt/BTS_04_05_01_V24_IP01/PatchLevel /opt/ems/utils

<hostname># date >> /opt/ems/utils/PatchLevel

Appendix N

Install IP01 Patch on CA

Step 1  Locate CD-ROM Disc labeled as "BTS_04_05_01_V24_IP01"

- Log in as root

- Put Disc labeled as " BTS_04_05_01_V24_IP01” in the CD-ROM drive

- Mount CD-ROM drive

<hostname># cd /

<hostname># mkdir -p /cdrom

<hostname># mount -o ro -F hsfs /dev/dsk/c0t0d0s0 /cdrom

- Copy file from the CD-ROM to the /opt directory.

<hostname># cp –f /cdrom/ BTS_04_05_01_V24_IP01.tar /opt

<hostname># umount /cdrom

- Manually eject the CD-ROM and take out Disc from drive

Step 2	Untar the file on CA platform.

<hostname># cd /opt

<hostname># tar -xvf /opt/BTS_04_05_01_V24_IP01.tar

Step 3	Verify the checksum for each file is valid

<hostname># cd /opt/BTS_04_05_01_V24_IP01

<hostname># cksum libSIS.so.BTS_04_05_01_V24_IP01

<hostname># cksum sga.BTS_04_05_01_V24_IP01

Output must match the checksums shown in BTS_04_05_01_V24_IP01.cksum

Step 4	Ensure the file has execute permission

<hostname># chmod +x libSIS.so.BTS_04_05_01_V24_IP01

<hostname># chmod +x sga.BTS_04_05_01_V24_IP01

Step 5	Ensure the file has the correct owner and group

<hostname># chown oamp:staff libSIS.so.BTS_04_05_01_V24_IP01

Step 6	Backup the original files and put the patches in place.

<hostname>cd /opt/BTSlib/lib

<hostname>cp -p libSIS.so libSIS.so.IP01

<hostname>cp -p /opt/BTS_04_05_01_V24_IP010/libSIS.so.BTS_04_05_01_V24_IP01 libSIS.so

<hostname>cd /opt/Opticall/ca/bin

<hostname> cp -p  /opt/BTS_04_05_01_V24_IP01/sga.BTS_04_05_01_V24_IP01  sga.BTS_04_05_01_V24_IP01

<hostname> cd /opt/OptiCall/CAxxx/bin

<hostname> rm sga.CAxxx

<hostname> ln –s /opt/OptiCall/ca/bin/sga.BTS_04_05_01_V24_IP01 sga.CAxxx

Step 7 Update PatchLevel on CA

<hostname># cp /opt/BTS_04_05_01_V24_IP01/PatchLevel /opt/ems/utils

<hostname># date >> /opt/ems/utils/PatchLevel

Backout Procedure for IP01 Patch.

In the steps below CAxxx must be replaced with the appropriate CA instance number for the system being patched. If you are unsure of the instance number run the "nodestat" command to find the instance number.

This section of the backout procedure assumes the CAxxx application is active on the primary side before you begin the patch backout.

Step 1 Stop the CA application on the Secondary CA

<hostname>#platform stop all

Step 2 Reinstall the saved files (before patching).

<hostname>#cd /opt/BTSlib/lib

<hostname>#mv libSIS.so.IP01 libSIS.so

<hostname>#cd /opt/OptiCall/CAxxx/bin

<hostname>#rm sga.CAxxx

<hostname>#ln –s /opt/OptiCall/ca/bin/sga sga.CAxxx

Step 3 On the secondary CA

<hostname>#mv /opt/ems/utils/PatchLevel.IP01 /opt/ems/utils/PatchLevel

Step 4 Start the secondary CA. Wait 10 minutes for the platform to stabilize and force it to active

<hostname>#platform start all

CAUTION: Wait 10 minutes before executing Step 5

Step 5 Login to the CLI as “btsuser”

<hostname># su – btsuser

CLI> control call-agent id=CAxxx; target-state=standby_active

CLI> control feature-server id=FSPTCyyy; target-state=standby_active

CLI> control feature-server id=FSAINzzz; target-state=standby_active

Step 6. Perform test calls.  If all goes well repeat steps 2, 3, 4 and 5 on the primary CA , and return the system to normal operating mode with the following commands...

CLI> control call-agent id=CAxxx; target-state=active_standby

CLI> control feature-server id=FSPTCyyy; target-state=active_standby

CLI> control feature-server id=FSAINzzz; target-state=active_standby

Step 7. On the primary EMS

<hostname>#cd /opt/ems/utils

<hostname>#mv PatchLevel.IP01 PatchLevel

Step 8. On the secondary EMS

<hostname>#cd /opt/ems/utils

<hostname>#mv PatchLevel.IP01 PatchLevel