Document Number 	EDCS-597586		Revision		27.0

Cisco BTS 10200 Softswitch Software Upgrade for Release

5.x to 6.x (where x is 0 – 99)

May 06, 2009

Corporate Headquarters

Cisco Systems, Inc.

170 West Tasman Drive

San Jose, CA 95134-1706

USA

http://www.cisco.com

THE SPECIFICATIONS AND INFORMATION REGARDING THE PRODUCTS IN THIS MANUAL ARE SUBJECT TO CHANGE WITHOUT NOTICE. ALL STATEMENTS, INFORMATION, AND RECOMMENDATIONS IN THIS MANUAL ARE BELIEVED TO BE ACCURATE BUT ARE PRESENTED WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED. USERS MUST TAKE FULL RESPONSIBILITY FOR THEIR APPLICATION OF ANY PRODUCTS.

THE SOFTWARE LICENSE AND LIMITED WARRANTY FOR THE ACCOMPANYING PRODUCT ARE SET FORTH IN THE INFORMATION PACKET THAT SHIPPED WITH THE PRODUCT AND ARE INCORPORATED HEREIN BY THIS REFERENCE. IF YOU ARE UNABLE TO LOCATE THE SOFTWARE LICENSE OR LIMITED WARRANTY, CONTACT YOUR CISCO REPRESENTATIVE FOR A COPY.

The Cisco implementation of TCP header compression is an adaptation of a program developed by the University of California, Berkeley (UCB) as part of UCB’s public domain version of the UNIX operating system. All rights reserved. Copyright © 1981, Regents of the University of California.

NOTWITHSTANDING ANY OTHER WARRANTY HEREIN, ALL DOCUMENT FILES AND SOFTWARE OF THESE SUPPLIERS ARE PROVIDED “AS IS” WITH ALL FAULTS. CISCO AND THE ABOVE-NAMED SUPPLIERS DISCLAIM ALL WARRANTIES, EXPRESSED OR IMPLIED, INCLUDING, WITHOUT LIMITATION, THOSE OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT OR ARISING FROM A COURSE OF DEALING, USAGE, OR TRADE PRACTICE.

IN NO EVENT SHALL CISCO OR ITS SUPPLIERS BE LIABLE FOR ANY INDIRECT, SPECIAL, CONSEQUENTIAL, OR INCIDENTAL DAMAGES, INCLUDING, WITHOUT LIMITATION, LOST PROFITS OR LOSS OR DAMAGE TO DATA ARISING OUT OF THE USE OR INABILITY TO USE THIS MANUAL, EVEN IF CISCO OR ITS SUPPLIERS HAVE BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES.

CCDE, CCSI, CCENT, Cisco Eos, Cisco HealthPresence, the Cisco logo, Cisco Lumin, Cisco Nexus, Cisco Nurse Connect, Cisco Stackpower, Cisco StadiumVision, Cisco TelePresence, Cisco WebEx, DCE, and Welcome to the Human Network are trademarks; Changing the Way We Work, Live, Play, and Learn and Cisco Store are service marks; and Access Registrar, Aironet, AsyncOS, Bringing the Meeting To You, Catalyst, CCDA, CCDP, CCIE, CCIP, CCNA, CCNP, CCSP, CCVP, Cisco, the Cisco Certified Internetwork Expert logo, Cisco IOS, Cisco Press, Cisco Systems, Cisco Systems Capital, the Cisco Systems logo, Cisco Unity, Collaboration Without Limitation, EtherFast, EtherSwitch, Event Center, Fast Step, Follow Me Browsing, FormShare, GigaDrive, HomeLink, Internet Quotient, IOS, iPhone, iQuick Study, IronPort, the IronPort logo, LightStream, Linksys, MediaTone, MeetingPlace, MeetingPlace Chime Sound, MGX, Networkers, Networking Academy, Network Registrar, PCNow, PIX, PowerPanels, ProConnect, ScriptShare, SenderBase, SMARTnet, Spectrum Expert, StackWise, The Fastest Way to Increase Your Internet Quotient, TransPath, WebEx, and the WebEx logo are registered trademarks of Cisco Systems, Inc. and/or its affiliates in the United States and certain other countries.

All other trademarks mentioned in this document or website are the property of their respective owners. The use of the word partner does not imply a partnership relationship between Cisco and any other company. (0903R)

Cisco BTS 10200 Softswitch Software Upgrade

Copyright © 2009, Cisco Systems, Inc.

All rights reserved.

| Date | Version | Description |
| --- | --- | --- |
| 04/19/2007 | 1.0 | - Initial Version |
| 09/17/2007 | 2.0 | - Updated with Automated scripts |
| 10/16/2007 | 6.0 | - Updated Chapter 5 with latest upgrade prompts |
| 10/22/2007 | 7.0 | - Added Task #3 in Chapter 3 to check office code index table |
| 11/22/2007 | 8.0 | - Added task #7 in Chapter 6 to check dn2subscriber table |
| 12/04/2007 | 9.0 | Updated Task#3 in Chapter#3 per William’s comments |
| 12/13/2007 | 10.0 | Added task#9 in Chapter#3 to check CA-CONFIG for ACCT & AUTH Code Prompt Resolved CSCsl47643, CSCsk55228, CSCsk55272, Added Task#5 in chapter#3 check dial plan Updated Appendix I per Juann’s comments on Audit oracle database steps |
| 12/21/2007 | 11.0 | Updated task#10 in Chapter#3 to resolve CSCsl87367 Resolved CSCsl87550 Also added Task#6 in Chapter#3 to resolve CSCsl90966 |
| 01/18/2008 | 12.0 | Removed Task# 7 from Chapter#3 (Verify and record Virtual IP (VIP) information), This task has been automated. Removed Task#6 in Chapter#3 (check MLHG-TERMINAL) Updated Appendix E to resolve CSCsm09452 Removed Appendix M and Changed Appendix N to Appendix M Added step#3 to srep#10 on Appendix J to resolve CSCsl89665 Added steps on Appendix A & B to resolve CSCsm12370 Added Appendix N for verifying disk mirror Added steps on Appendix E |
| 02/08/08 | 13.0 | Updated TASK#1 in Chapter#3. Added Note on Appendix J to make it clear for creating backup disk script. Added Task#9 in Chapter#3 Added Appendix O to verify index id for table aor2sub, user-auth and siaregcontact. Updated Task#8 in Chapter#5 for entering new password |
| 02/12/08 | 15.0 | Updated Task#8 in Chapter#5 |
| 02/26/08 | 17.0 | Added step#3 on Appendix E to resolve CSCsm73181 |
| 03/31/08 | 18.0 | Added step#2 in Chapter 5 Task 8 |
| 04/01/08 | 19.0 | Updated Task#4 in Chapter#3. Removed Task#6 (Enable DB statistics collection) in Chapter#6. Updated Appendix F. Updated Task#3 in Chapter#6 for CORBA installation on regards to 6.0MR1 release. |
| 06/23/08 | 20.0 | Added step#25 in Appendix A Updated Appendix A to resolve CSCsq60617 Updated Appendix I to resolve CSCsq18734 Updated Appendix A and Appendix B per Matthew’s comments. |
| 06/23/08 | 21.0 | Updated Appendix B step#6 |
| 07/30/08 | 22.0 | Removed TASK 1 from Chapter 6 per Chris Koath’s verification ( Resolved CSCsr36886) |
| 08/06/08 | 23.0 | Added step 1 on Task 2 Appendix J, per Matthew’s comment. |
| 11/19/08 | 24.0 | Added MidPoint Patch Install /Uninstall Procedure for 5.x->6.0.1V02PXX upgrade. Appendix P and Appendix Q added for install procedure and Appendix R and S added for uninstall procedure. |
| 04/17/09 | 25.0 | Added Patch procedure in Appendix T for upgrading directly from 5.0.3 to 6.0.1V02PXX |
| 04/17/09 | 26.0 | Done some correction given by reviewer team. |
| 05/06/09 | 27.0 | Added Steps#1-3 in Task 1 to Unblock the CLI session and Steps#4-6 in Task 2 to Block the CLI Session in Appendix L to resolve CSCsy89109. Added appendix U to resolve CSCsy52409. |

Table of Contents

Table of Contents	6

Chapter 1	10

Meeting upgrade requirements	10

12

Completing the Upgrade Requirements Checklist	12

Understanding Conventions	13

Chapter 2	14

14

Preparation	14

Task 1: Requirements and Prerequisites	14

Task 2: Stage the load on the system	14

From EMS Side A	14

Task 3: Delete Checkpoint files from Secems System	15

Task 4: CDR delimiter customization	15

Task 5: Check for HW errors	16

Task 6: Change SPARE2-SUPP	16

From Active EMS	16

Chapter 3	17

17

Complete the following tasks 24-48 hours before the scheduled upgrade	17

Task 1: Check DIAL PLAN PROFILE Table	17

From Active EMS	17

Task 2: Check AUTH CODE Table	18

From Active EMS	18

Task 3: Check OFFICE CODE INDEX Table	19

From Active EMS	19

Task 4: Check MGW PROFILE Table	20

From Active EMS	20

Task 5: Check DIAL PLAN Table	22

From Active EMS	22

Task 6: Verify and record VSM Macro information	23

From EMS Side A	23

Task 7: Record subscriber license record count	24

From EMS Side A	24

Task 8: Check Ca-Config for ACCT and AUTH Code	24

From Active EMS	25

Task 9: Check SIP Subscriber for Commercial Memory Configuration	26

From Active EMS	26

Chapter 4	27

27

Complete the following tasks the night before the scheduled upgrade	27

Task 1: Perform full database audit	27

Chapter 5	28

28

Upgrade the System	28

Task 1: Verify system in normal operating status	29

From Active EMS	29

Task 2: Alarms	30

Refer to Appendix H to verify that there are no outstanding major or critical alarms. 	30

Task 3: Audit Oracle Database and Replication	30

30

Refer to Appendix I to verify Oracle database and replication functionality.	30

Task 4: Creation of Backup Disks	30

Task 5: Verify Task 1, 2 & 3	30

Task 6: Start Upgrade Process by Starting the Upgrade Control Program	31

On all 4 BTS nodes	31

From EMS side B	31

Task 7: Validate New Release operation	35

Task 8: Upgrade Side A	35

Chapter 6	39

Finalizing Upgrade	39

Task 1: CDR delimiter customization	39

Task 2: To install CORBA on EMS, follow Appendix C.	40

Task 3: Reconfigure VSM Macro information	40

Task 4: Restore subscriber license record count	41

From EMS Side A	41

Task 5: Check DN2SUBSCRIBER Table	42

From Active EMS	42

43

Task 6: Audit Oracle Database and Replication	43

43

Refer to Appendix I to verify Oracle database and replication functionality.	43

Task 7: Initiate disk mirroring by using Appendix E	43

Appendix A	44

Backout Procedure for Side B Systems	44

Appendix B	51

Full System Backout Procedure	51

Appendix C	58

CORBA Installation	58

Task 1: Install OpenORB CORBA Application	58

Remove Installed OpenORB Application	58

Task 2 Install OpenORB Packages	59

Appendix D	62

Staging the 6.0.x load to the system	62

From EMS Side B	62

From EMS Side A	65

From CA/FS Side A	66

From CA/FS Side B	67

Appendix E	68

Full System Successful Upgrade Procedure	68

Appendix F	70

Emergency Fallback Procedure Using the Backup Disks	70

Appendix G	73

Check database	73

Perform database audit	73

Appendix H	75

Check Alarm Status	75

From EMS side A	75

Appendix I	77

Audit Oracle Database and Replication	77

Check Oracle DB replication status	77

From STANDBY EMS	77

Correct replication error for Scenario #1	80

From EMS Side B	80

From EMS Side A	80

Correct replication error for Scenario #2	81

From EMS Side A	81

Appendix J	83

Creation Of Backup Disks	83

83

Task 1: Creating a Bootable Backup Disk	83

86

Task 2: Perform Switchover to prepare Side A CA and EMS Bootable Backup Disk	86

87

Task 3: Repeat task 1 on the Side A EMS and CA Nodes	87

Appendix K	88

88

Caveats and solutions	88

Appendix L	91

91

Sync Data from Active EMS to Active CA/FS	91

91

Task 1: Sync Data from Active EMS to Active CA/FS	91

From Active EMS	91

92

Task 2: Execute DB Audit (Row Count)	92

Appendix M	94

Opticall.cfg parameters	94

Appendix N	97

Verifying the Disk mirror	97

Appendix O	99

Verifying index id for aor2sub, user-auth and SiaRegContact Tables	99

From Active CA	99

Appendix P	101

Midpoint Patch Install Procedure for CA	101

From CA	101

Appendix Q	103

Midpoint Patch Install Procedure for EMS	103

From EMS	103

Appendix R	105

Midpoint Patch Uninstall Procedure for CA	105

From CA	105

Appendix S	106

Midpoint Patch Uninstall Procedure for EMS	106

From EMS	106

Appendix T	107

Patch Procedure for 5.0.3 to 6.0.1V02PXX before starting Upgrade Control Program.	107

Appendix U	109

Procedure to verify the entries in ACG table	109

From Active CA	109

Chapter 1

Meeting upgrade requirements

- This procedure MUST be executed during a maintenance window.

- Executions of steps in this procedure shut down and restart individual platforms in a certain sequence. The steps should not be executed out of sequence, doing so could result in traffic loss.

- Provisioning is not allowed during the entire upgrade process.  All provisioning sessions (CLI, external) MUST be closed before starting the upgrade until the upgrade process is complete.

Upgrade process overview.

Meeting

Upgrade

Requirements

Preparing

1 Week

Before

Upgrade

Preparing

24-48 Hours

Before

Upgrade

Preparing

the Night

Before

Upgrade

Upgrading

Finalizing

the

Upgrade

# Completing the Upgrade Requirements Checklist

Before upgrading, ensure the following requirements are met:

| Upgrade Requirements Checklist |  |
| --- | --- |
|  | You have a basic understanding of UNIX and ORACLE commands. |
|  | Make sure that that console access is available |
|  | You have user names and passwords to log into each EMS/CA/FS platform as root user. |
|  | You have user names and passwords to log into the EMS as a CLI user. |
|  | You have the ORACLE passwords from your system administrator. |
|  | You have a completed NETWORK INFORMATION DATA SHEET (NIDS). |
|  | Confirm that all domain names in /etc/opticall.cfg are in the DNS server |
|  | You have the correct BTS software version on a readable CD-ROM. |
|  | Verify opticall.cfg has the correct information for all four nodes (Side A EMS, Side B EMS, Side A CA/FS, Side B CA/FS |
|  | You know whether or not to install CORBA. Refer to local documentation or ask your system administrator. |
|  | Ensure that all non used/not required tar files and not required large data files on the systems are removed from the system before the upgrade. |
|  | Verify that the CD ROM drive is in working order by using the mount command and a valid CD ROM. |
|  | Confirm host names for the target system |
|  | Document the location of archive(s) |

# Understanding Conventions

Application software loads are named Release 900-aa.bb.cc.Vxx, where

- aa=major release number.

- bb=minor release number.

- cc=maintenance release.

- Vxx=Version number.

Platform naming conventions

- EMS = Element Management System;

- CA/FS = Call Agent/Feature Server

- Primary is also referred to as Side A

- Secondary is also referred to as Side B

Commands appear with the prompt, followed by the command in bold. The prompt is usually one of the following:

- Host system prompt (<hostname>#)

- Oracle prompt (<hostname>$)

- SQL prompt (SQL>)

- CLI prompt (CLI>)

- SFTP prompt (sftp>)

Chapter 2

Preparation

This chapter describes the tasks a user must complete in the week prior to the upgrade.

# Task 1: Requirements and Prerequisites

- For 6.0.x load

  - One CD-ROM disc labeled as Release 6.0.x Vxx BTS 10200 Application Disk

    - Where x is 00 -99

  - One CD-ROM disc labeled as Release 6.0.x Vxx BTS 10200 Database Disk

    - Where x is 00 -99

  - One CD-ROM disc labeled as Release 6.0.x Vxx BTS 10200 Oracle Disk

    - Where x is 00 -99

# Task 2: Stage the load on the system

# From EMS Side A

Step 1   Log in as root user.

Step 2   If /opt/Build contains the currently running load, save it, in case fallback is needed.  Use the following commands to save /opt/Build.

<hostname># cat /opt/Build/Version

- Assume the above command returns the following output

900-05.00.01.V20

- Use “05.00.01.V20” as part of the new directory name

<hostname># mv /opt/Build /opt/Build.05.00.01.V20

Step 3   Repeat Step 1 and Step 2 for EMS Side B.

Step 4   Repeat Step 1 and Step 2 for CA/FS Side A.

Step 5   Repeat Step 1 and Step 2 for CA/FS side B.

Step 6   Refer to Appendix D for staging the Rel 6.0.x load on the system

# Task 3: Delete Checkpoint files from Secems System

Step 1 Log in as root.

Step 2 Delete the checkpoint files.

- <hostname> # \rm –f /opt/.upgrade/checkpoint.*

# Task 4: CDR delimiter customization

CDR delimiter customization is not retained after software upgrade. If the system has been customized, then the operator must manually recustomize the system after the upgrade.

The following steps must be executed on both EMS side A and side B

Step 1 <hostname># cd /opt/bdms/bin

Step 2 <hostname># vi platform.cfg

Step 3 Locate the section for the command argument list for the BMG process

Args=-port 15260 -h localhost -u optiuser -p optiuser -fmt default_formatter -UpdIntvl 3300 -ems_local_dn blg-aSYS14EMS.cisco.com -FD semicolon -RD linefeed

Step 4 Record the customized values. These values will be used for CDR customization in the post upgrade steps.

# Task 5: Check for HW errors

On all four systems, check /var/adm/messages file for any hardware related errors conditions. Rectify the error conditions before proceeding with the upgrade.

# Task 6: Change SPARE2-SUPP

# From Active EMS

Step 1 Login to CLI as “btsuser”.

<hostname> su – btsuser

Step 2 Issue the following CLI command.

CLI>  show mgw-profile SPARE2_SUPP=n;display=id

Make a note of each mgw-profile listed in the output.

Step 3 Issue the following CLI command for each mgw-profile listed in step 2.

CLI> change mgw-profile id=xxxx; SPARE2-SUPP=Y

Chapter 3

Complete the following tasks 24-48 hours before the scheduled upgrade

This chapter describes the tasks a user must complete 24-48 hours before the scheduled upgrade.

# Task 1: Check DIAL PLAN PROFILE Table

# From Active EMS

Step 1   Log in the active EMS as “root” user

Step 2   <hostname># su – oracle

Step 3    <hostname>$ sqlplus optiuser/optiuser

Step 4	SQL> select ID,NAT_DIAL_PLAN_ID from DIAL_PLAN_PROFILE a

where not exists (select 'x' from DIAL_PLAN_PROFILE b

where a.NAT_DIAL_PLAN_ID = b.id) and a.NAT_DIAL_PLAN_ID is not null;

- If the above query returns a result. For example,

ID                       NAT_DIAL_PLAN_ID
----------------       --------------------------------
NTE                     tb76-ivr-2

You may notice from above example that NTE is assigned with incorrect NAT_DIAL_PLAN_ID, since tb76-ivr-2 is not a valid ID in the table DIAL_PLAN_PROFILE.

Please use CLI commands to change NAT_DIAL_PLAN_ID for the DIAL_PLAN_PROFILE ID=NTE. The new NAT_DIAL_PLAN_ID needs to be valid ID in the table DIAL_PLAN_PROFILE, and then run the sql command again to verify all errors are fixed.

Failure to do so will result in an upgrade failure.

# Task 2: Check AUTH CODE Table

# From Active EMS

Step 1   Log in the active EMS as “root” user

Step 2   <hostname># su – oracle

Step 3    <hostname>$ sqlplus optiuser/optiuser

Step 4   SQL> select AUTH_CODE_GRP_ID,ID from auth_code where length(id) < 3;

- If the above query returns a result. For example,

AUTH_CODE_GRP_ID                 ID
--------------------------------                 -----------------------
DEFAULT_ACGROUP                  12

You may notice from above example that the ID has only 2 characters (12), the number of characters for ID should be in the range of 3 to 23.

Please use CLI commands to change these records and then run the sql command again to verify all errors are fixed.

Failure to do so will result in an upgrade failure.

# Task 3: Check OFFICE CODE INDEX Table

# From Active EMS

Step 1   Log in the active EMS as “root” user

Step 2   <hostname># su – oracle

Step 3    <hostname>$ sqlplus optiuser/optiuser

Step 4   SQL> select b.office_code_index, b.dn from dn2subscriber b
              where not exists (select 'x' from exchange_code a
              where a.office_code_index = b.office_code_index )
              and (dn like '%x' or dn like '%xx' or dn like '%xxx' or dn like 'xxxx');

- If the above query returns a result. For example,

OFFICE_CODE_INDEX           	DN
    -----------------------------------    ------------
                35                            	 3xxx
                66                            	 xxxx
              123                            	 4x

You may notice from above example that these OFFICE_CODE_INDEX used in the table DN2SUBSCRIBER do not exist in the table EXCHANGE_CODE.

Please use CLI commands to fix these office_code_index for the table DN2SUBSCRIBER and then run the sql command again to verify all errors are fixed.

Failure to do so will result in an upgrade failure.

For Example:

1) Show dn2subscriber office-code-index=35;dn=3xxx;

OFFICE_CODE_INDEX=35

DN=3xxx

STATUS=ASSIGNED

RING_TYPE=1

LNP_TRIGGER=N

NP_RESERVED=N

SUB_ID=818-888-2001

LAST_CHANGED=2008-07-28 16:17:41

ADMIN_DN=N

PORTED_IN=N

Reply : Success: Entry 1 of 1 returned.

2) change dn2subscriber office-code-index=35;status=vacant;sub-id=null;

3) delete dn2subscriber office-code-index=35;dn=3xxx;

4) add dn2subscriber office-code-index= < valid office code index from table exchange code>;dn=3xxx;

Note: valid office code index needs to be determining from exchange code table (show exchange code).

# Task 4: Check MGW PROFILE Table

# From Active EMS

Note: Following steps are only valid if the upgrade process is prior to 5.0.2.

Please do not execute following steps on any upgrade which is from 5.0.2 and above.

Step 1   Log in the active EMS as “root” user

Step 2   <hostname># su – oracle

Step 3    <hostname>$ sqlplus optiuser/optiuser

Step 4   SQL> col origfield for a9;

Step 5   SQL> col sessname for a8;

Step 6   SQL> col email for a5;

Step 7   SQL> col phone for a5;

Step 8   SQL> col uri for a3;

Step 9   SQL> col supp for a4;

Step 10 SQL> col info for a4;

Step 11 SQL> col time for a4;

Step 12 SQL> col attrib for a6;

Step 13 SQL> col bandwidth for a8;

Step 14 SQL> select id, sdp_origfield_supp origfield,sdp_sessname_supp sessname,sdp_email_supp email,sdp_phone_supp phone,sdp_uri_supp uri,sdp_bandwidth_supp bandwidth, sdp_info_supp info, sdp_time_supp time, sdp_attrib_supp attrib from mgw_profile

Step 15 Press enter to get a new line for below remaining sql statements to be entered

where sdp_origfield_supp = 'N' or sdp_sessname_supp = 'N' or sdp_email_supp = 'N' or sdp_phone_supp = 'N' or sdp_uri_supp = 'N' or sdp_bandwidth_supp = 'N' or sdp_info_supp = 'N' or sdp_time_supp = 'N' or sdp_attrib_supp = 'N';

Note: Please execute the above sql command in two lines.

- If the above query returns a result. For example,

ID                ORIGFIELD 	    SESSNAME   EMAIL    PHONE URI BANDWIDTH INFO TIME ATTRIB
---------------  ----------------  -----------------  ---------  ----------- ----- -------------------  -------  ------  ----------
test1           	 Y                   N            	  Y         	     Y         Y          Y                  Y         Y          Y

abcd         	  Y                  Y             	  Y               N         Y          Y                  Y         Y         Y

efgh           	  N                   Y           	   Y              Y         Y          Y                  N         Y        Y

Please use CLI commands to update value “N” to “Y” for each ID, and then run the sql command again to verify all errors are fixed.

Failure to do so will result in an upgrade failure.

# Task 5: Check DIAL PLAN Table

# From Active EMS

Step 1   Log in the active EMS as “root” user

Step 2   <hostname># su – oracle

Step 3    <hostname>$ sqlplus optiuser/optiuser

Step 4   SQL> select id,digit_string,noa from dial_plan where digit_string like '%-%';

- If the above query returns a result. For example,

ID               DIGIT_STRING   NOA
    ---------------- --------------     ----------------
    tb67             667-904         NATIONAL
    tb67             667-905         NATIONAL
    tb67             667-906         NATIONAL
    tb67             667-907         NATIONAL
    tb67             667-908         NATIONAL

Please use CLI command “show dial-plan” to display and preserve attributes for these dial plan IDs. Then, use CLI command to delete these dial plan IDs, and re-enter it with same preserved attributes except no “-“ in the digit string. Failure to do so will result in an upgrade failure.

For Example:

1) show dial-plan; id=tb67; digit-string=667-904;

ID=tb67

DIGIT_STRING=667-904

DEST_ID=30085

SPLIT_NPA=NONE

DEL_DIGITS=0

MIN_DIGITS=10

MAX_DIGITS=10

NOA=NATIONAL

2) delete dial-plan; id=tb67; digit-string=667-904;

3) add dial-plan; id=tb67; digit-string=667904;dest-id=30085;spilt-npa=none;del-digits=0;min-digits=10;max-digits=10;noa=national;

Step 5 Exit from Oracle:

SQL> quit;

<hostname>$ exit

# Task 6: Verify and record VSM Macro information

Verify if VSM Macros are configured on the EMS machine. If VSM is configured, record the VSM information. VSM will need to be re-configured after the upgrade procedure is complete.

# From EMS Side A

Step 1   btsadmin> show macro id=VSM%

ID=VSMSubFeature

PARAMETERS=subscriber.id,subscriber.dn1,subscriber_service_profile.service-id,service.fname1,service.fname2,service.fname3,service.fname4,service.fname5,service.fname6,service.fname7,service.fname8,service.fname9,service.fname10

AND_RULES=subscriber.id=subscriber_service_profile.sub-id,subscriber_service_profile.service-id=service.id

Step 2 Record the VSM Macro information

# Task 7: Record subscriber license record count

Record the subscriber license record count.

# From EMS Side A

Step 1   btsadmin> show db_usage table_name=subscriber;

For example:

TABLE_NAME=SUBSCRIBER

MAX_RECORD_COUNT=150000

LICENSED_RECORD_COUNT=150000

CURRENT_RECORD_COUNT=0

MINOR_THRESHOLD=80

MAJOR_THRESHOLD=85

CRITICAL_THRESHOLD=90

ALERT_LEVEL=NORMAL

SEND_ALERT=ON

Reply : Success: Entry 1 of 1 returned.

Step 2 Exit

# Task 8: Check Ca-Config for ACCT and AUTH Code

# From Active EMS

Step 1 Login to CLI as “btsuser”.

<hostname> su – btsuser

Step 2 Issue the following CLI command.

CLI> show ca-config type=ACCT-CODE-PROMPT-TONE;

Step 3 If it’s not provisioned, then execute step#4.

Step 4 Issue the following CLI command.

CLI> add ca-config type=ACCT-CODE-PROMPT-TONE; value=cf;

Step 5 Issue the following CLI command.

CLI> show ca-config type=AUTH-CODE-PROMPT-TONE;

Step 6 If it’s not provisioned, then execute step#7.

Step 7 Issue the following CLI command.

CLI> add ca-config type=AUTH-CODE-PROMPT-TONE; value=cf;

CLI> exit

# Task 9: Check SIP Subscriber for Commercial Memory Configuration

# From Active EMS

Note: Following steps are only valid if the system has been configured as Commercial Memory.

Step 1 Issue following command on active EMS to verify system is configured as commercial memory.

<hostname> vi /etc/opticall.cfg

Step 2 If MEM_CFG_SELECTION is set to Commercial in opticall.cfg file, then execute following steps, otherwise task 9 is complete.

Step 3 Login to CLI as “btsuser”.

<hostname> su – btsuser

Step 4 Issue the following CLI command.

CLI> show aor2sub sub_id=%;

Step 5 If the above command return with results, then refer to Appendix O.

Example Output:

AOR_ID=208-262-3614@sia-sys08ca146.ipclab.cisco.com

SUB_ID=208-262-3614

STATUS=INS

RING_TYPE=R1

IN_IRS=N

Reply : Success: at 2008-02-06 13:15:47 by btsadmin

Entries 1-4 of 4 returned.

Chapter 4

# Complete the following tasks the night before the scheduled upgrade

This chapter describes the tasks a user must complete the night before the scheduled upgrade.

# Task 1: Perform full database audit

All provisioning activity MUST be suspended before executing the following pre-upgrade DB integrity checks.

In this task a full database audit is performed and errors if any are corrected. Refer to Appendix G to perform full data base Audit.

|  | Caution: It is recommended that a full-data base audit be executed 24 hours prior to performing the upgrade. Execution of full database audit within this time period will provide the ability to bypass a full database audit during the upgrade. In deployments with large databases the full database audit can take several hours which may cause the upgrade to extend beyond the maintenance window. |
| --- | --- |

Chapter 5

Upgrade the System

1. Caution: Suspend all CLI provisioning activity during the entire upgrade process.  Close all the CLI provisioning sessions.

2Caution: Refer to Appendix K for known caveats and corresponding solutions

3Note: In the event of the following conditions, use Appendix A to fallback side B systems to the old release.

- Failure to bring up the side B systems to standby state with the new release

- Failure to switch over from Side A systems to side B systems

4.  Note: In the event of the following conditions, use Appendix B to fallback the entire system to the old release.

- Failure to bring up the side A systems to standby state with the new release

- Failure to switch over from Side B systems to side A systems

5.  Note: If the upgrade of the entire systems is successful but it is still required to rollback the entire system to the old release then use Appendix B to fallback the entire system.

6.  Note: If the upgrade of the entire system needs to abandon due to call processing failure or the upgrade performance is so degraded that it is not possible to continue operations with the upgrade release, to restore service as quickly as possible to the old release then use Appendix F.

# Task 1: Verify system in normal operating status

Make sure the Primary systems are in ACTIVE state, and Secondary systems are in STANDBY state.

# From Active EMS

Step 1   <hostname> btsstat

- Verify the Primary systems are in ACTIVE state and the Secondary systems are in STANDBY state. If not, please use the control command to bring the system to the desired state.

# Task 2: Alarms

# Refer to Appendix H to verify that there are no outstanding major or critical alarms.

# Task 3: Audit Oracle Database and Replication

# Refer to Appendix I to verify Oracle database and replication functionality.

|  | Caution Do NOT continue until all data base mismatches and errors have been completely rectified. |
| --- | --- |

# Task 4: Creation of Backup Disks

Refer to Appendix J for creation of backup disks. It will take 30-45 minutes to complete the task.

Caution: Appendix J must be executed before starting the upgrade process. Creation of backup disks procedure (Appendix J) will split the mirror between the disk set and create two identical and bootable drives on each of the platforms for fallback purpose.

# Task 5: Verify Task 1, 2 & 3

Repeat Task 1, 2 & 3 again to verify that system is in normal operating state.

Note: The upgrade script must be executed from the Console port

Note: If the upgrade script exits as a result of any errors or otherwise, the operator can continue the upgrade process by restarting the upgrade script after rectifying the error that caused the script execution failure. The script will restart at the last recorded successful checkpoint.

# Task 6: Start Upgrade Process by Starting the Upgrade Control Program

# On all 4 BTS nodes

Step 1   Log in as root user.

Step 2   Execute the following commands on all 4 BTS nodes and remove the install.lock file if it is present. .

<hostname># ls /tmp/install.lock

- If the lock file is present, please do the following command to remove it.

<hostname># \rm -f /tmp/install.lock

# From EMS side B

Step 1   Log in as root user.

Step 2   Log all upgrade activities and output to a file

<hostname># script /opt/.upgrade/upgrade.log

- If you get an error from the above command, “/opt/.upgrade” may not exist yet.

  - Please do the following command to create this directory.

<hostname># mkdir –p /opt/.upgrade

  - Run the “script /opt/.upgrade/upgrade.log”again.

Caution: If upgrading from 5.0.3 to 6.0.1V02P03(or above patch level) then Refer to Appendix T before proceeding with next step.

Step 3   Execute the BTS software upgrade script.

- <hostname># /opt/Build/bts_upgrade.exp -stopBeforeStartApps

Step 4   If this BTS system does not use the default root password, you will be prompted for the root password.  The root password must be identical on all the 4 BTS nodes.  Enter the root password when you get following message:

root@[Side A EMS hostname]'s password:

Step 5	The upgrade procedure prompts the user to populate the values of certain parameters in opticall.cfg file. Be prepared to populate the values when prompted.

Caution: The parameter values that the user provides will be written into /etc/opticall.cfg and sent to all 4 BTS nodes. Ensure that you enter the correct values when prompted to do so. Refer to Appendix M for further details on the following parameters.

- Please provide a value for CA146_LAF_PARAMETER:

- Please provide a value for FSPTC235_LAF_PARAMETER:

- Please provide a value for FSAIN205_LAF_PARAMETER:

- Please provide a value for BILLING_FILENAME_TYPE:

- Please provide a value for BILLING_FD_TYPE:

- Please provide a value for BILLING_RD_TYPE:

- Please provide a value for  DNS_FOR_EM01_DIA_COM:

- Please provide a value for EMS_DIA_ORIGIN_HOST:

Step 6   Answer “n” to the following prompt.

- Would you like to perform a full DB audit again (y/n)?  n

Note: If you pressed “y” for the above prompts and found some DB mismatches. Refer to appendix L to sync the data, else continue with the next step. After executing the tasks in Appendix L, restart the upgrade script. The script will restart at the last recorded successful checkpoint.

Step 7   Caution: It is not recommended to continue the upgrade with outstanding major/critical alarms. Refer to appendix H to mitigate outstanding alarms.

- Question: Do you want to continue (y/n)? y

Step 8    Caution: It is not recommended to continue the upgrade with outstanding major/critical alarms. Refer to appendix H to mitigate outstanding alarms.

- Question: Are you sure you want to continue (y/n)?  y

Step 9 Answer “y” to the following prompts.

- <hostname># About to stop platforms on secemsxx and seccaxx, Continue? (y/n) y

Step 10    Caution: When the following prompt comes :

<hostname># About to start platform on secondary side, Continue? (y/n)

- Login into secondary CA and EMS and refer to Appendix P and Appendix Q for applying mid upgrade patches on secondary CA and EMS respectively.

- After executing the tasks in Appendix P and Appendix Q execute step 11.

Note: If the mid point upgrade patch fails on either of secondary CA or secondary EMS then please refer Appendix R and Appendix S for uninstalling mid upgrade patch on secondary CA and EMS respectively.

Caution: The entry in the ACG table must be verified before continuing to the next step. Please follow the appendix U to verify the ACG table.

Step 11   Answer “y” to the following prompts.

- <hostname># About to start platform on secondary side, Continue? (y/n) y

- <hostname># About to change platform to standby-active.  Continue? (y/n) y

Note: If the upgrade script exits due to DB mismatch errors during mid upgrade row count audit, then refer to Appendix L to sync data from EMS side B to CA/FS side B.  After executing the tasks in Appendix L, restart the upgrade script. The script will restart at the last recorded successful checkpoint.

- The following NOTE will be displayed once the Side B EMS and Side B CA/FS have been upgraded to the new release. After the following NOTE is displayed proceed to Task 7,

***********************************************************************

NOTE: The mid-upgrade point has been reached successfully. Now is the time to verify functionality by making calls, if desired, before proceeding with the upgrade of side A of the BTS.

***********************************************************************

# Task 7: Validate New Release operation

Step 1   Once the side B systems are upgraded and are in ACTIVE state, validate the new release software operation.  If the validation is successful, continue to next step, otherwise refer to Appendix A, Backout Procedure for Side B Systems.

- Verify existing calls are still active

- Verify new calls can be placed

- Verify billing records generated for the new calls just made are correct

  - Log in as CLI user

  - CLI> report billing-record tail=1;

  - Verify that the attributes in the CDR match the call just made.

# Task 8: Upgrade Side A

Note: These prompts are displayed on EMS Side B.

Step 1   Answer “y” to the following prompt.

- <hostname># About to stop platforms on priemsxx and pricaaxx. Continue? (y/n) y

Note: Following steps (step#2 & #3) are only valid for upgrade prior to 6.0.1 (MR1). If you are in the upgrade process of 6.0.1 (MR1) or above then please skip step#2 & #3 and continue from step#4.

Step 2   Answer “n” to the following prompt

- <hostname># About to start platform on primary side, Continue? (y/n) n

************************************************************

***********************Exiting******************************

************************************************************

Warning: After answer ‘n’ to the above prompt, the upgrade script will exit. And we need to login to primary EMS to apply the following SQL commands.

- ssh login as root to primary EMS

- <hostname>#  su  –  oracle

- <hostname>$  dba

- Execute following SQL commands

SQL> ALTER DATABASE TEMPFILE'/data1/oradata/optical2/db2/deftemp.dbf' DROP;
SQL> ALTER DATABASE TEMPFILE'/data1/oradata/optical2/db2/temp01.dbf' DROP;
SQL> exit;

- <hostname>$  exit

Note: The following exit command will get you back in secondary EMS login

- <hostname>#  exit

Warning: After the above SQL commands are done, re-start the upgrade script on secondary EMS.

- <hostname># /opt/Build/bts_upgrade.exp -stopBeforeStartApps

Step 3   Answer “y” to the following prompt.

- <hostname>#  Staged load version is same as current running version.  Continue (y/n)? y

Step 4    Caution: When the following prompt comes :

<hostname># About to start platform on primary side, Continue? (y/n)

- Login into primary CA and EMS and refer to Appendix P and Appendix Q for applying mid upgrade patches on primary CA and EMS respectively.

- After executing the tasks in Appendix P and Appendix Q execute step 5.

Note: If the mid point upgrade patch fails on either of primary CA or primary EMS then please refer Appendix R and Appendix S for uninstalling mid upgrade patch on Primary CA and EMS respectively.

Step 5   Answer “y” to the following prompts.

- <hostname># About to start platform on primary side, Continue? (y/n) y

- <hostname># About to change platform to active-standby.  Continue? (y/n) y

*** CHECKPOINT syncHandsetData ***

Handset table sync may take long time.  Would you like to do it now?

Please enter “Y” if you would like to run handset table sync, otherwise enter “N”.

Step 6   Please enter new passwords to the following prompts. Following password changes are mandatory.

Note: The password must be longer than or equal to 6 characters and less than or equal to 8 characters.

User account - root - is using default password

Enter new Password:

Enter new Password again:

Password has been changed successfully.

User account - btsadmin - is using default password

Enter new Password:

Enter new Password again:

Password has been changed successfully.

User account - btsuser - is using default password

Enter new Password:

Enter new Password again:

Password has been changed successfully.

User account - btsoper - is using default password

Enter new Password:

Enter new Password again:

Password has been changed successfully.

==================================================

===============Upgrade is complete==================

==================================================

Chapter 6

Finalizing Upgrade

# Task 1: CDR delimiter customization

CDR delimiter customization is not retained after software upgrade. The system must be manually recustomized the system after the upgrade.

The following steps must be executed on both EMS side A and side B

Step 1 <hostname># cd /opt/bdms/bin

Step 2 <hostname># vi platform.cfg

Step 3 Locate the section for the command argument list for the BMG process

Note:These values were recorded in pre-upgrade steps in Chapter 2 Task 4.

Args=-port 15260 -h localhost -u optiuser -p optiuser -fmt default_formatter -UpdIntvl 3300 -ems_local_dn blg-aSYS14EMS.cisco.com -FD semicolon -RD linefeed

Step 4 Modify the customized values. These values were recorded in Chapter 2 Task 6. Customize the CDR delimiters in the “Args=” line according to customer specific requirement. For Example:

Args=-port 15260 -h localhost -u optiuser -p optiuser -fmt default_formatter -UpdIntvl 3300 -ems_local_dn blg-aSYS14EMS.cisco.com -FD semicolon -RD linefeed

# Task 2: To install CORBA on EMS, follow Appendix C.

Note: Please skip Task# 2, if you have upgraded to 6.0.1 (MR1) or above releases.

# Task 3: Reconfigure VSM Macro information

Step 1   Log in as root to EMS

Note: If VSM was configured and recorded in the pre-upgrade step in Chapter 3 task 6 then, reconfigure the VSM on the Active EMS, otherwise, skip this task.

Note: VSM must be configured on the Active EMS (Side A)

Step 2 Reconfigure VSM

<hostname> su - btsadmin

<btsadmin> add macro ID=VSMSubFeature;PARAMETERS=subscriber.id,subscriber.dn1,subscriber_service_profile.service-id,service.fname1,service.fname2,service.fname3,service.fname4,service.fname5,service.fname6,service.fname7,service.fname8,service.fname9,service.fname10;AND_RULES=subscriber.id=subscriber_service_profile.sub-id,subscriber_service_profile.service-id=service.id

Macro_id = Macro value recorded in chapter 3 , task 6

- Verify that VSM is configured

<btsadmin> show macro id= VSM%

ID=VSMSubFeature

PARAMETERS=subscriber.id,subscriber.dn1,subscriber_service_profile.service-id,service.fname1,service.fname2,service.fname3,service.fname4,service.fname5,service.fname6,service.fname7,service.fname8,service.fname9,service.fname10

AND_RULES=subscriber.id=subscriber_service_profile.sub-id,subscriber_service_profile.service-id=service.id

<btsadmin> quit

# Task 4: Restore subscriber license record count

Restore the subscriber license record count recorded earlier in pre-upgrade steps.

# From EMS Side A

Step 1 login as ciscouser

<hostname># su - ciscouser

Step 2 CLI> change db-license table-name=SUBSCRIBER; licensed-record-count=XXXXXX

Where XXXXXX is the number that was recorded in the pre-upgrade steps.

Step 3 CLI> show db_usage table_name=subscriber;

For example:

TABLE_NAME=SUBSCRIBER

MAX_RECORD_COUNT=150000

LICENSED_RECORD_COUNT=150000

CURRENT_RECORD_COUNT=0

MINOR_THRESHOLD=80

MAJOR_THRESHOLD=85

CRITICAL_THRESHOLD=90

ALERT_LEVEL=NORMAL

SEND_ALERT=ON

Reply : Success: Entry 1 of 1 returned.

# Task 5: Check DN2SUBSCRIBER Table

# From Active EMS

Step 1   Log in the active EMS as “root” user

Step 2   <hostname># su – oracle

Step 3    <hostname>$ sqlplus optiuser/optiuser

Step 4    SQL> select office_code_index,dn from dn2subscriber where dn like '_x' or dn like '_xx' or dn like '_xxx' or dn like 'xxxx';

- If the above query returns a result. For example,

OFFICE_CODE_INDEX         DN
 ---------------------------------      ---------
                2                                 2x
                2                                 xxxx
                2                                 4xx
                2                                 1xxx

Please use CLI command to delete these records from dn2subscriber table.

Step 5 Exit from Oracle:

SQL> quit;

<hostname>$ exit

# Task 6: Audit Oracle Database and Replication

# Refer to Appendix I to verify Oracle database and replication functionality.

# Task 7: Initiate disk mirroring by using Appendix E

Refer to Appendix E for initiating disk mirroring. It will take about 2.5 hours for each side to complete the mirroring process.

Warning: It is strongly recommended to wait for next maintenance window for initiating disk mirroring process. After disk mirroring is completed by using Appendix E, the system will no longer have the ability to fallback to the previous release. Make sure the entire software upgrade process is completed successfully and the system does not experience any call processing issue before executing Appendix E.

The entire software upgrade process is now complete.

Note: Please remember to close the upgrade.log file after the upgrade process completed.

Appendix A

Backout Procedure for Side B Systems

Caution: After the side B systems are upgraded to release 6.0, and if the system is provisioned with new CLI data, fallback is not recommended.

This procedure allows you to back out of the upgrade procedure if any verification checks (in "Verify system status" section) failed. This procedure is intended for the scenario in which the side B system has been upgraded to the new load and in active state, or side B system failed to upgrade to the new release, while the side A system is still at the previous load and in standby state. The procedure will back out the side B system to the previous load.

This backout procedure will:

- Restore the side A system to active mode without making any changes to it

- Revert to the previous application load on the side B system

- Restart the side B system in standby mode

- Verify that the system is functioning properly with the previous load

This procedure is used to restore the previous version of the release on Side B using a fallback release on disk 1.

The system must be in split mode so that the Side B EMS and CA can be reverted back to the previous release using the fallback release on disk 1.

Step 1 Verify that oracle is in simplex mode and Hub is in split state on EMS Side A

<hostname># nodestat

- Verify ORACLE DB REPLICATION should be IN SIMPLEX SERVICE

- Verify OMSHub mate port status: No communication between EMS

- Verify OMSHub slave port status: should not contain Side B CA IP address

Note: If the above verification is not correct then follow following bullets, otherwise go to step 2

- On the EMS Side A place oracle in the simplex mode and split the Hub.

  - <hostname> su – oracle

  - <hostname>$ cd /opt/oracle/opticall/create

  - <hostname>$ ./dbinstall optical1 disable replication

  - <hostname> $ exit

  - <hostname> /opt/ems/utils/updMgr.sh -split_hub

Step 2 Verify that the Side A EMS and CA are ACTIVE and Side B EMS and CA are in OOS-FAULTY or STANDBY state. If side A EMS and CA are in STANDBY state, the following “platform stop all” command will switchover.

<hostname> btsstat

Step 3 Stop Side B EMS and CA platforms.  Issue the following command on Side B EMS and CA.

<hostname> platform stop all

Note: At this point, Side B system is getting prepared to boot from fallback release on disk 1.

Step 4 To boot from disk1 (bts10200_FALLBACK release), do the following commands

<hostname># eeprom boot-device=”disk1 disk0”

<hostname># shutdown –y –g0 –i6

Step 5 After logging in as root, execute following commands to verify system booted on disk1 (bts10200_FALLBACK release) and that the platform on the Secondary side is not started.

<hostname> nodestat

<hostname># lustatus   (Verification for Boot Environment)

Example Output

<hostname># lustatus

Boot Environment             Is       	Active 	Active    	Can    	Copy

Name                       	   Complete 	Now    On Reboot 	Delete 	Status

--------------------------     -------- 	------ 	---------		 ------ 	----------

d2                         	     yes      	no   	 no       	yes     	-

bts10200_FALLBACK   yes      	yes     	yes       	no   	 -

Step 6 On the EMS and CA Side B

<hostname> platform start all

Step 7 Verify that the Side A EMS and CA are ACTIVE and Side B EMS and CA are in STANDBY state.

<hostname> btsstat

Step 8 Restore hub on the Side A EMS.

<hostname>  /opt/ems/utils/updMgr.sh -restore_hub

Step 9 On Side A EMS set mode to Duplex

<hostname>  su - oracle

<hostname> $ cd /opt/oracle/opticall/create

<hostname> $ ./dbinstall optical1 enable replication

<hostname> $ exit

Step 10 Verify HUB and EMS communication restored on Side B EMS.

<hostname> nodestat

- Verify  HUB communication is restored.

- Verify OMS Hub mate port status: communication between EMS nodes is restored

Step 11 Verify call processing is working normally with new call completion.

Step 12 Perform an EMS database audit on Side A EMS and verify that there are no mismatch between side A EMS and Side B EMS.

<hostname> su - oracle

<hostname> dbadm -C db

<hostname> exit;

Note: If there are any mismatch errors found, please refer to Appendix I on correcting replication error section.

Step 13 Perform an EMS/CA database audit and verify that there are no mismatches.

<hostname> su - btsadmin

CLI>audit database type=full;

CLI> exit

Note: At this point Side B is running on disk 1. Please refer to Appendix K if you need to access disk 0 for traces/logs, otherwise continue on step 15.

Step 14   Log in as root user on Side B EMS and CA nodes.

Step 15   Execute the Fallback script from Side B EMS and CA nodes.

Note: fallback_proc.exp script will first prepare the EMS & CA nodes for disk mirroring process and then initiate disk mirroring process from disk 1 to disk 0. It will take about 2.5 hours to complete the process.

<hostname># cd /opt/Build

<hostname># ./fallback_proc.exp

Note: If the system fails to reboot during the fallback script execution, then it needs to be run manually from the prompt as “reboot -- -r”.

Step 16 System will reboot with below note.

Note: At this point the system will be rebooted...

Restart the fallback procedure once it comes up.

Step 17 After logging in as root on EMS and CA nodes, execute the Fallback script again from Side B EMS and CA nodes.

<hostname># cd /opt/Build

<hostname># ./fallback_proc.exp

Step 18 The script will display following notes, please verify and answer “y” to the following prompts.

Checkpoint 'syncMirror1' found.  Resuming aborted backup disk procedure

from this point and continuing.

- Do you want to continue (y/n)? y

hostname# display _boot _env _state

Printing boot environment status...

ERROR: No boot environments are configured on this system

ERROR: cannot determine list of all boot environment names

If status is okay, press y to continue or n to abort...

- Please enter your choice... Do you want to continue? [y,n,?,q] y

Step 19 The Fallback script will display following note.

=================================================

====  Disk mirroring preparation is completed  ====

====  Disk resync is now running at background ====

==== Resync will take about 2.5 hour to finish ====

=========== Mon Jan 14 11:14:00 CST 2008 ============

==================================================

Step 20 Verify that disk mirroring process is in progress on Side B EMS and CA nodes by using following command.

<hostname># metastat |grep %

Example Output

<hostname># metastat | grep %

Resync in progress: 0 % done

Resync in progress: 4 % done

Resync in progress: 6 % done

Resync in progress: 47 % done

Step 21 Once the fallback script completed successfully, verify that phone calls are processed correctly.

Step 22 Execute below command to boot the system on disk 0.

<hostname># shutdown –y –g0 –i6

Note: Refer to Appendix N “Verifying the disk mirror” to verify if the mirror process was completed properly.

Note: The following commands must be executed on Primary EMS to clean up the flag. Fail to do so will disable Oracle DB heart beat process when platform is re-started.

Step 23 Login as root to primary EMS and execute following commands.

<hostname># cd /opt/ems/etc

<hostname># cp ems.props ems.props.$$

<hostname># grep –v upgradeInProgress ems.props.$$ > ems.props

<hostname># /bin/rm ems.props.$$

<hostname># btsstat (Ensure Secondary EMS is in Standby state)

<hostname># platform stop all (Primary EMS only)

<hostname># platform start all (Primary EMS only)

Fallback of side B systems is now complete

Appendix B

Full System Backout Procedure

CAUTION: This procedure is recommended only when full system upgrade to release 6.x has been completed and the system is experiencing unrecoverable problems for which the only solution is to take a full system service outage and restore the systems to the previous release as quickly as possible.

This procedure is used to restore the previous version of the release using a fallback release on disk 1.

The system must be in split mode so that the Side B EMS and CA can be reverted back to the previous release using the fallback release on disk 1.

Step 1 On the EMS Side A place oracle in the simplex mode and split the Hub.

<hostname> su – oracle

<hostname>$ cd /opt/oracle/opticall/create

<hostname>$ ./dbinstall optical1 disable replication

<hostname>$ exit

<hostname> /opt/ems/utils/updMgr.sh -split_hub

Step 2 Verify that the Side A EMS and CA are ACTIVE and Side B EMS and CA are in STANDBY state.

<hostname> btsstat

Step 3 Stop Side B EMS and CA platforms.  Issue the following command on Side B EMS and CA.

<hostname> platform stop all

Note: At this point, Side B system is getting prepared to boot from fallback release on disk 1.

Step 4 To boot from disk1 (bts10200_FALLBACK release) on Side B EMS & CA, do the following command.

<hostname># eeprom boot-device=”disk1 disk0”

<hostname># shutdown –y –g0 –i6

Step 5 After logging in as root, execute following commands to verify Side B system booted on disk 1 (bts10200_FALLBACK release) and that the platform on Secondary side is not started.

<hostname> nodestat

<hostname># lustatus   (Verification for Boot Environment)

Example Output

<hostname># lustatus

Boot Environment             Is       	Active 	Active    	Can    	Copy

Name                       	   Complete 	Now    On Reboot 	Delete 	Status

--------------------------     -------- 	------ 	---------		 ------ 	----------

d2                         	     yes      	no   	 no       	yes     	-

bts10200_FALLBACK   yes      	yes     	yes       	no   	 -

Step 6 Log into the Side B EMS as root

<hostname> /opt/ems/utils/updMgr.sh -split_hub

<hostname> platform start -i oracle

<hostname> su – oracle

<hostname>$ cd /opt/oracle/opticall/create

<hostname>$ ./dbinstall optical2 disable replication

<hostname>$ exit

The next steps will cause FULL system outage

Step 7 Stop Side A EMS and CA nodes.

Note: Wait for Side A EMS and CA nodes to stop completely before executing Step 8 below.

<hostname> platform stop all

Step 8 Start Side B EMS and CA nodes.

<hostname> platform start all

Step 9 Verify that Side B EMS and CA are ACTIVE on the “fallback release” and calls are being processed.

<hostname> btsstat

Note: At this point, Side A system is getting prepared to boot from fallback release on disk 1.

Step 10 To boot from disk1 (bts10200_FALLBACK release) on Side A EMS and CA, do the following command.

<hostname># eeprom boot-device=”disk1 disk0”

<hostname># shutdown –y –g0 –i6

Step 11 After logging in as root, execute following commands to verify Side A system booted on disk 1 (bts10200_FALLBACK release) and that the platform on Primary side is not started.

<hostname> nodestat

<hostname># lustatus   (Verification for Boot Environment)

Example Output

<hostname># lustatus

Boot Environment             Is       	Active 	Active    	Can    	Copy

Name                       	   Complete 	Now    On Reboot 	Delete 	Status

--------------------------     -------- 	------ 	---------		 ------ 	----------

d2                         	     yes      	no   	 no       	yes     	-

bts10200_FALLBACK   yes      	yes     	yes       	no   	 -

Step 12 Issue the platform start command to start up the Side A EMS and CA nodes.

<hostname> platform start all

Step 13 Verify that Side A EMS and CA platforms are in standby state.

<hostname>  btsstat

Step 14 Restore hub on Side B EMS.

<hostname>  /opt/ems/utils/updMgr.sh -restore_hub

Step 15 On Side B EMS set mode to Duplex

<hostname>  su - oracle

<hostname> $ cd /opt/oracle/opticall/create

<hostname> $ ./dbinstall optical2 enable replication

<hostname> $ exit

Step 16 Verify that the Side A EMS and CA are in active state.

<hostname> nodestat

* Verify  HUB communication is restored.
* Verify OMS Hub mate port status: communication between EMS nodes is restored

Step 17 Verify call processing is working normally with new call completion.

Step 18 Perform an EMS database audit on Side A EMS and verify that there are no mismatch between side A EMS and Side B EMS.

<hostname> su - oracle

<hostname> dbadm -C db

<hostname> exit;

Step 19 Perform an EMS/CA database audit and verify that there are no mismatches.

<hostname> su - btsadmin

CLI>audit database type=full;

CLI> exit

The backup version is now fully restored and running on non-mirrored disk.

Note: At this point, Side A and Side B are running on disk 1 (bts10200_FALLBACK release). Also both systems Side A and Side B are running on non-mirrored disk. To get back to state prior to upgrade on Side A and Side B, execute fallback script on Side A and Side B as follows.

Step 20   Log in as root user on Side A and B EMS and CA nodes.

Step 21   Execute the Fallback script from Side A (EMS & CA) first and then after about 30 minutes start the same script from Side B (EMS & CA) nodes.

Note: fallback_proc.exp script will first prepare the EMS & CA nodes for disk mirroring process and then initiate disk mirroring process from disk 1 to disk 0. It will take about 2.5 hours to complete the process.

<hostname># cd /opt/Build

<hostname># ./fallback_proc.exp

Note: If the system fails to reboot during the fallback script execution, then it needs to be run manually from the prompt as “reboot -- -r”.

Step 22 System will reboot with below note.

Note: At this point the system will be rebooted...

Restart the fallback procedure once it comes up.

Step 23 After logging in as root on EMS and CA nodes, execute the Fallback script again from EMS and CA nodes.

<hostname># cd /opt/Build

<hostname># ./fallback_proc.exp

Step 24 The script will display following notes, please verify and answer “y” to the following prompts.

Checkpoint 'syncMirror1' found.  Resuming aborted backup disk procedure

from this point and continuing.

- Do you want to continue (y/n)? y

hostname# display _boot _env _state

Printing boot environment status...

ERROR: No boot environments are configured on this system

ERROR: cannot determine list of all boot environment names

If status is okay, press y to continue or n to abort...

- Please enter your choice... Do you want to continue? [y,n,?,q] y

Step 25 The Fallback script will display following note.

==================================================

====  Disk mirroring preparation is completed  ====

====  Disk resync is now running at background ====

==== Resync will take about 2.5 hour to finish ====

=========== Mon Jan 14 11:14:00 CST 2008 ============

===================================================

Step 26 Verify that disk mirroring process is in progress on EMS and CA nodes by using following command.

<hostname># metastat |grep %

Example Output

<hostname># metastat | grep %

Resync in progress: 0 % done

Resync in progress: 4 % done

Resync in progress: 6 % done

Resync in progress: 47 % done

Step 27 Once the fallback script completed successfully, verify that phone calls are processed correctly.

Note: Refer to Appendix N “Verifying the disk mirror” to verify if the mirror process was completed properly.

This completes the entire system fallback

Appendix C

CORBA Installation

This procedure describes how to install the OpenORB Common Object Request Broker Architecture (CORBA) application on Element Management System (EMS) of the Cisco BTS 10200 Softswitch.

NOTE: During the upgrade this installation process has to be executed on both EMS side A and EMS side B.

Caution  This CORBA installation will remove existing CORBA application on EMS machines. Once you have executed this procedure, there is no backout. Do not start this procedure until you have proper authorization.

# Task 1: Install OpenORB CORBA Application

### Remove Installed OpenORB Application

Step 1   Log in as root to EMS.

Step 2   Remove the OpenORB CORBA packages if they are installed, other wise go to next step.

<hostname># pkginfo | grep BTScis

- If the output of the above command indicates that BTScis package is installed, then follow the next step to remove the BTScis package.

<hostname># pkgrm BTScis

  - Answer “y” when prompted

<hostname># pkginfo | grep BTSoorb

- If the output of the above command indicates that BTSoorb package is installed, then follow the next step to remove the BTSoorb package.

<hostname># pkgrm BTSoorb

  - Answer “y” when prompted

Step 3   Enter the following command to verify that the CORBA application is removed:

<hostname># pgrep cis3

The system will respond by displaying no data, or by displaying an error message. This verifies that the CORBA application is removed.

# Task 2 Install OpenORB Packages

The CORBA application files are available for installation once the Cisco BTS 10200 Softswitch is installed.

Step 1   Log in as root to EMS

Note : If  VIP was configured and recorded in the pre-upgrade step in Chapter 3 task 13 then, reconfigure the VIP on the Active EMS, otherwise, go to Step 4.

Note that VIP needs to be configured on Active EMS (Side A)

Step 2   Reconfigure VIP

<hostname> su - btsadmin

<btsadmin> change ems interface=<INTERFACE>;ip_alias=<VIP>; netmask=<NETMASK ADDRESS> broadcast =<BROADCAST ADDRESS>

INTERFACE = Interface value recorded in chapter 3, task 13

VIP =  ip-alias value recorded in chapter 3, task 13

Step 3   Verify that VIP is configured

<btsadmin> show ems

IP_ALIAS=10.89.224.177
INTERFACE=eri0
NTP_SERVER=10.89.224.

<btsadmin> quit

Step 4   <hostname># cd /opt/Build

Step 5   <hostname># cis-install.sh

- Answer “y” when prompted.

It will take about 5-8 minutes for the installation to complete.

Step 6   Verify CORBA Application is running On EMS:

<hostname># init q

<hostname># pgrep ins3

Note : System will respond by displaying the Name Service process ID, which is a number between 2 and 32,000 assigned by the system during CORBA installation. By displaying this ID, the system confirms that the ins3 process was found and is running.

<hostname># pgrep cis3

Note : The system will respond by displaying the cis3 process ID, which is a number between 2 and 32,000 assigned by the system during CORBA installation. By displaying this ID, the system confirms that the cis3 process was found and is running.

Step 7   If you do not receive both of the responses described in Step 6, or if you experience any verification problems, do not continue. Contact your system administrator. If necessary, call Cisco TAC for additional technical assistance.

Appendix D

Staging the 6.0.x load to the system

This Appendix describes how to stage the 6.0.x load to the system using CD-ROM.

Note: Ensure that you have the correct CD-ROM for the release you want to fall back to.

# From EMS Side B

Step 1   Log in as root.

Step 2   Put BTS 10200 Application Disk CD-ROM in the CD-ROM drive.

Step 3   Remove old files.

<hostname># cd /

<hostname># \rm –rf /opt/Build

Step 4   Create /cdrom directory and mount the directory.

<hostname># mkdir -p /cdrom

- A system with Continuous Computing hardware, please run:

<hostname># mount -o ro -F hsfs /dev/dsk/c0t6d0s0 /cdrom

- Other hardware platform, please run:

<hostname># mount -o ro -F hsfs /dev/dsk/c0t0d0s0 /cdrom

Step 5   Use the following commands to copy file from the CD-ROM to the /opt directory.

<hostname># cp –f /cdrom/K9-opticall.tar.gz /opt

Step 6   Verify that the check sum value match with the values located in the “checksum.txt” file located on Application CD-ROM.

<hostname># cat /cdrom/checksum.txt

<hostname># cksum /opt/K9-opticall.tar.gz

- Record the checksum value for later use.

Step 7   Unmount the CD-ROM.

<hostname># umount /cdrom

Step 8   Manually eject the CD-ROM and take out BTS 10200 Application Disk CD-ROM from CD-ROM drive.

Step 9   Put BTS 10200 Database Disk CD-ROM in the CD-ROM drive of EMS Side B.

Step 10   Mount the /cdrom directory.

- A system with Continuous Computing hardware, please run:

<hostname># mount -o ro -F hsfs /dev/dsk/c0t6d0s0 /cdrom

- Other hardware platform, please run:

<hostname># mount -o ro -F hsfs /dev/dsk/c0t0d0s0 /cdrom

Step 11   Use the following commands to copy file from the CD-ROM to the /opt directory.

<hostname># cp –f /cdrom/K9-btsdb.tar.gz /opt

<hostname># cp –f /cdrom/K9-extora.tar.gz /opt

Step 12   Verify that the check sum values match with the values located in the “checksum.txt” file located on BTS 10200 Database Disk CD-ROM.

<hostname># cat /cdrom/checksum.txt

<hostname># cksum /opt/K9-btsdb.tar.gz

<hostname># cksum /opt/K9-extora.tar.gz

- Record the checksum values for later use.

Step 13   Unmount the CD-ROM.

<hostname># umount /cdrom

Step 14   Manually eject the CD-ROM and take out BTS 10200 Database Disk CD-ROM from CD-ROM drive.

Step 15   Put BTS 10200 Oracle Engine Disk CD-ROM in the CD-ROM drive of EMS Side B.

Step 16   Mount the /cdrom directory.

- A system with Continuous Computing hardware, please run:

<hostname># mount -o ro -F hsfs /dev/dsk/c0t6d0s0 /cdrom

- Other hardware platform, please run:

<hostname># mount -o ro -F hsfs /dev/dsk/c0t0d0s0 /cdrom

Step 17   Use the following commands to copy file from the CD-ROM to the /opt directory.

<hostname># cp –f /cdrom/K9-oraengine.tar.gz /opt

Step 18   Verify that the check sum values match with the values located in the “checksum.txt” file located on Oracle Engine CD-ROM.

<hostname># cat /cdrom/checksum.txt

<hostname># cksum /opt/K9-oraengine.tar.gz

- Record the checksum value for later use.

Step 19   Unmount the CD-ROM.

<hostname># umount /cdrom

Step 20   Manually eject the CD-ROM and take out BTS 10200 Oracle Engine Disk CD-ROM from CD-ROM drive.

Step 21   Extract tar files.

<hostname># cd /opt

<hostname># gzip -cd K9-opticall.tar.gz | tar -xvf -

<hostname># gzip -cd K9-btsdb.tar.gz | tar -xvf -

<hostname># gzip -cd K9-oraengine.tar.gz | tar -xvf -

<hostname># gzip –cd K9-extora.tar.gz | tar -xvf -

|  | Note : It may take up to 30 minutes to extract the files. |
| --- | --- |

# From EMS Side A

Step 1   <hostname># cd /opt

Step 2   <hostname># \rm –rf /opt/Build

Step 3   <hostname># sftp <hostname of EMS Side B>

Step 4   sftp> cd /opt

Step 5   sftp> get K9-opticall.tar.gz

Step 6   sftp> get K9-btsdb.tar.gz

Step 7   sftp> get K9-oraengine.tar.gz

Step 8   sftp> get K9-extora.tar.gz

Step 9   sftp> exit

Step 10  Compare and verify the checksum values of the following files with the values that were recorded in earlier tasks.

<hostname># cksum /opt/K9-opticall.tar.gz

<hostname># cksum /opt/K9-btsdb.tar.gz

<hostname># cksum /opt/K9-oraengine.tar.gz

<hostname># cksum /opt/K9-extora.tar.gz

Step 11   <hostname># gzip -cd K9-opticall.tar.gz | tar -xvf -

Step 12   <hostname># gzip -cd K9-btsdb.tar.gz | tar -xvf -

Step 13   <hostname># gzip -cd K9-oraengine.tar.gz | tar -xvf -

Step 14   <hostname># gzip –cd K9-extora.tar.gz  | tar –xvf -

|  | Note: It may take up to 30 minutes to extract the files |
| --- | --- |

# From CA/FS Side A

Step 1   <hostname># cd /opt

Step 2   <hostname># \rm –rf /opt/Build

Step 3   <hostname># sftp <hostname of EMS Side B>

Step 4   sftp> cd /opt

Step 5   sftp> get K9-opticall.tar.gz

Step 6   sftp> exit

Step 7  Compare and verify the checksum values of the following file with the value that was recorded in earlier tasks.

<hostname># cksum /opt/K9-opticall.tar.gz

Step 8   <hostname># gzip -cd K9-opticall.tar.gz | tar -xvf -

|  | Note : It may take up to 10 minutes to extract the files |
| --- | --- |

# From CA/FS Side B

Step 1   <hostname># cd /opt

Step 2   <hostname># \rm –rf /opt/Build

Step 3   <hostname># sftp <hostname of EMS side B>

Step 4   sftp> cd /opt

Step 5   sftp> get K9-opticall.tar.gz

Step 6   sftp> exit

Step 7  Compare and verify the checksum values of the following file with the value that was recorded in earlier tasks.

<hostname># cksum /opt/K9-opticall.tar.gz

Step 8   <hostname># gzip -cd K9-opticall.tar.gz | tar -xvf -

|  | Note : It may take up to 10 minutes to extract the files |
| --- | --- |

Appendix E

Full System Successful Upgrade Procedure

Note: This procedure is recommended only when full system upgrade has been completed successfully and the system is not experiencing any issues.

This procedure is used to initiate the disk mirroring from disk 0 to disk 1, once Side A and Side B have been successfully upgraded. It will take about 2.5 hours on each side to complete the disk mirroring process.

The system must be in split mode and both Side A and Side B (EMS and CA) have been upgraded successfully on disk 0, with disk 1 remains as fallback release. Both Side A and Side B (EMS and CA) disk 1 can be mirrored to disk0, so that both disks will have the upgrade release.

Step 1   Log in as root user on Side A and B EMS and CA nodes.

Step 2 Execute following command on all four nodes to verify disk status.

<hostname># lustatus   (Verification for Boot Environment)

Example Output

<hostname># lustatus

Boot Environment             Is       	Active 	Active    	Can    	Copy

Name                       	   Complete 	Now    On Reboot 	Delete 	Status

--------------------------     -------- 	------ 	---------		 ------ 	----------

d2                         	     yes      	yes   	 yes       	no     	-

bts10200_FALLBACK  yes      	no     	no        	yes   	 -

Step 3   Execute the Sync mirror script from Side A and B EMS and CA nodes.

<hostname># cd /opt/Build

<hostname># ./bts_sync_disk.sh

Step 4 The Sync mirror script will display following note.

=============== =============== =================
> ====  Disk mirroring preparation is completed  ====
> ====  Disk sync is now running at background ====
> ==== Disk syncing  will take about 2.5 hour to finish ====

=========== Mon Jan 14 11:14:00 CST 2008 ============

=================================================

Step 5 Verify that disk mirroring process is in progress on all four nodes by using following command.

<hostname># metastat |grep %

Example Output

<hostname># metastat | grep %

Resync in progress: 0 % done

Resync in progress: 4 % done

Resync in progress: 6 % done

Resync in progress: 47 % done

Step 6 Once the Sync mirror script completed successfully, verify that phone calls are processed correctly.

Note: Refer to Appendix N “Verifying the disk mirror” to verify if the mirror process was completed properly.

Appendix F

Emergency Fallback Procedure Using the Backup Disks

This procedure should be used to restore service as quickly as possible in the event that there is a need to abandon the upgrade version due to call processing failure.

This procedure will be used when there is either no successful call processing, or the upgrade performance is so degraded that it is not possible to continue operations with the upgrade release.

Step 1   Log in as root user on Side A and B EMS and CA nodes.

Step 2   Execute the Fallback script from Side A and B EMS and CA nodes.

<hostname># cd /opt/Build

<hostname># ./fallback_proc.exp “emergency fallback”

Note: If the system fails to reboot during the fallback script execution, then it needs to be run manually from the prompt as “reboot -- -r”.

Step 3 System will reboot with below note.

Note: At this point the system will be rebooted...

Restart the fallback procedure once it comes up.

Step 4 After logging in as root on EMS and CA nodes, execute the Fallback script again from Side A and B EMS and CA nodes.

<hostname># cd /opt/Build

<hostname># ./fallback_proc.exp “emergency fallback”

Step 5 The script will display following notes, please verify and answer “y” to the following prompts.

Checkpoint 'changeBootDevice1' found.  Resuming aborted backup disk procedure from this point and continuing.

- Do you want to continue (y/n)? y

hostname# display _boot _env _state

Printing boot environment status...

Boot Environment             Is       	Active 	Active    	Can    	Copy

Name                       	   Complete 	Now    On Reboot 	Delete 	Status

--------------------------     -------- 	------ 	---------		 ------ 	----------

d2                         	     yes      	no   	 no	      	yes     	-

bts10200_FALLBACK  yes      	yes    	yes        	no   	 -

If status is okay, press y to continue or n to abort..

- Please enter your choice... Do you want to continue? [y,n,?,q] y

Step 6 System will reboot with below note.

Note: At this point the system will be rebooted...

Restart the disk backup procedure once it comes up.

Step 7 After logging in as root on EMS and CA nodes, execute the Fallback script again from Side A and B EMS and CA nodes.

<hostname># cd /opt/Build

<hostname># ./fallback_proc.exp “emergency fallback”

Step 8 The script will display following notes, please verify and answer “y” to the following prompts.

Checkpoint 'syncMirror1' found.  Resuming aborted backup disk procedure

from this point and continuing.

- Do you want to continue (y/n)? y

hostname# display _boot _env _state

Printing boot environment status...

ERROR: No boot environments are configured on this system

ERROR: cannot determine list of all boot environment names

If status is okay, press y to continue or n to abort...

- Please enter your choice... Do you want to continue? [y,n,?,q] y

Step 9 The Fallback script will display following note.

=================================================

====  Disk mirroring preparation is completed  ====

====  Disk resync is now running at background ====

==== Resync will take about 2.5 hour to finish ====

=========== Mon Jan 14 11:14:00 CST 2008 ============

==================================================

Step 10 Verify that disk mirroring process is in progress on Side B EMS and CA nodes by using following command.

<hostname># metastat |grep %

Example Output

<hostname># metastat | grep %

Resync in progress: 0 % done

Resync in progress: 4 % done

Resync in progress: 6 % done

Resync in progress: 47 % done

Step 11 Once the fallback script completed successfully, verify that phone calls are processed correctly.

Step 12 Execute below command to boot the system on disk 0.

<hostname># shutdown –y –g0 –i6

Note: Refer to Appendix N “Verifying the disk mirror” to verify if the mirror process was completed properly.

Emergency Fallback of side A and B systems is now completed

Appendix G

Check database

This procedure describes how to perform database audit and correct database mismatch as a result of the DB audit.

# Perform database audit

In this task, you will perform a full database audit and correct any errors, if necessary.  The results of the audit can be found on the active EMS via the following Web location. For example ….https://priems108/report/

Step 1 Login as “ciscouser”

<hostname># su - ciscouser

Step 2 CLI> audit database type=full;

Step 3  Check the audit report and verify there is no discrepancy or error. If errors are found, please try to correct them. If you are unable to correct, please contact Cisco Support.

Please follow the sample command provided below to correct the mismatches:

CLI> sync <table name> master=EMS; target=<CAxxx/FSPTCyyy/FSAINzzz>;

CLI> audit <table name>

Step 4 CLI> exit

Use the following command to clear data base mismatches for the following tables.

- SLE

- SC1D

- SC2D

- SUBSCRIBER-FEATURE-DATA

Step 1 CLI> sync <table name> master=FSPTC; target=<EMS>;

Step 2 CLI> audit <table name>

Step 3 CLI> exit

Appendix H

Check Alarm Status

The purpose of this procedure is to verify that there are no outstanding major/critical alarms.

# From EMS side A

Step 1   Log in as “btsuser” user.

Step 2   CLI> show alarm

- The system responds with all current alarms, which must be verified or cleared before proceeding with next step.

|  | Tip Use the following command information for reference material ONLY. |
| --- | --- |

Step 3   To monitor system alarm continuously.

CLI> subscribe alarm-report severity=all; type=all;

|  | Valid severity: MINOR, MAJOR, CRITICAL, ALL Valid types: CALLP, CONFIG, DATABASE, MAINTENANCE, OSS, SECURITY, SIGNALING, STATISTICS, BILLING, ALL, SYSTEM, AUDIT |
| --- | --- |

Step 4   System will display alarms if alarm is reported.

<------------------- START ALARM REPORT --------------------> TIMESTAMP: 20040503174759 DESCRIPTION: General MGCP Signaling Error between MGW and CA. TYPE & NUMBER: SIGNALING (79) SEVERITY: MAJOR ALARM-STATUS: OFF ORIGIN: MGA.PRIMARY.CA146 COMPONENT-ID: null ENTITY NAME: S0/DS1-0/1@64.101.150.181:5555 GENERAL CONTEXT: MGW_TGW SPECIFC CONTEXT: NA FAILURE CONTEXT: NA <------------------- END ALARM REPORT ---------------------->

Step 5   To stop monitoring system alarm.

CLI> unsubscribe alarm-report severity=all; type=all;

Step 6   CLI> exit

Appendix I

Audit Oracle Database and Replication

Perform the following steps on the Standby EMS side to check the Oracle database and replication status.

# Check Oracle DB replication status

# From STANDBY EMS

Step 1   Log in as root.

Step 2   Log in as oracle.

<hostname># su – oracle

Step 3   Enter the command to compare contents of tables on the side A and side B EMS databases:

Note: This may take 5-20 minutes time, depending on the size of the database.

<hostname>$ dbadm –C db

Step 4 Please check following two possible return results:

A) If all tables are in sync, output will be as follows:

Number of tables to be checked:  218

Number of tables checked OK:    218

Number of tables out-of-sync:      0

Step 5 If the tables are in sync as above, then Continue on Step 7 and skip Step 6.

B) If tables are out of sync, output will be as follows:

Number of tables to be checked: 157
Number of tables checked OK:    154
Number of tables out-of-sync:   3

Below is a list of out-of-sync tables:

OAMP.SECURITYLEVELS   				=> 1/0

OPTICALL.SUBSCRIBER_FEATURE_DATA		=> 1/2
OPTICALL.MGW                   				=> 2/2

Step 6 If the tables are out of sync as above, then Continue on Step C to sync the tables.

- For each table that is out of sync, please run the following step:

Note: Execute below “dbadm –A copy” command from the EMS side that has *BAD* data.

<hostname>$ dbadm -A copy -o <OWNER> -t <TABEL NAME>

Example:    dbadm –A copy –o opticall –t subscriber_feature_data

- Enter “y” to continue

- Please contact Cisco Support if the above command fails.

Step 7   Enter the command to check replication status:

<hostname>$ dbadm –C rep

Scenario #1 Verify that “Deferror is empty?” is “YES”.

OPTICAL2::Deftrandest is empty?           YES

OPTICAL2::dba_repcatlog is empty?       YES

OPTICAL2::Deferror is empty?              YES Make sure it is “YES”

OPTICAL2::Deftran is empty?                  YES

OPTICAL2::Has no broken job?               YES

OPTICAL2::JQ Lock is empty?                YES

OPTICAL1::Deftrandest is empty?          YES

OPTICAL1::dba_repcatlog is empty?      YES

OPTICAL1::Deferror is empty?             YES Make sure it is “YES”

OPTICAL1::Deftran is empty?                YES

OPTICAL1::Has no broken job?              YES

OPTICAL1::JQ Lock is empty?               YES

If the “Deferror is empty?” is “NO”, please try to correct the error using steps in “Correct replication error for scenario #1” below. If you are unable to clear the error or if any of the individual steps fails, please contact Cisco Support. If the “Deferror is empty?” is “YES”, then proceed to step 8.

Scenario #2 Verify that “Has no broken job?” is “YES”.

OPTICAL1::Deftrandest is empty?           YES Make sure it is “YES”

OPTICAL1::dba_repcatlog is empty?          YES

OPTICAL1::Deferror is empty?                  YES

OPTICAL1::Deftran is empty?                  YES Make sure it is “YES”

OPTICAL1::Has no broken job?               YES Make sure it is “YES”

OPTICAL1::JQ Lock is empty?                   YES

OPTICAL2::Deftrandest is empty?            YES

OPTICAL2::dba_repcatlog is empty?        YES

OPTICAL2::Deferror is empty?                 YES

OPTICAL2::Deftran is empty?                   YES

OPTICAL2::Has no broken job?                 YES

OPTICAL2::JQ Lock is empty?                  YES

If the “Has no broken job?” is “NO”, please try to correct the error using steps in “Correct replication error for scenario #2” below. If you are unable to clear the error or if any of the individual steps fails, please contact Cisco Support. If the “Has no broken job?” is “YES”, then proceed to step 8.

Step 8 <hostname> $ exit

# Correct replication error for Scenario #1

|  | Note You must run the following steps on standby EMS side B first, then on active EMS side A. |
| --- | --- |

# From EMS Side B

Step 1   Log in as root

Step 2   <hostname># su – oracle

Step 5   <hostname>$ dbadm –A truncate_deferror

- Enter “y” to continue

Step 6 <hostname> $ exit

# From EMS Side A

Step 1   Login in as root.

Step 2   <hostname> # su – oracle

Step 3   <hostname>$ dbadm –A truncate_deferror

- Enter “y” to continue

Step 4   Re-verify that “Deferror is empty?” is “YES” and none of tables is out of sync.

<hostname>$dbadm –C rep

OPTICAL1::Deftrandest is empty?           YES

OPTICAL1::dba_repcatlog is empty?       YES

OPTICAL1::Deferror is empty?              YES  Make sure it is “YES”

OPTICAL1::Deftran is empty?                  YES

OPTICAL1::Has no broken job?               YES

OPTICAL1::JQ Lock is empty?                YES

OPTICAL2::Deftrandest is empty?          YES

OPTICAL2::dba_repcatlog is empty?      YES

OPTICAL2::Deferror is empty?             YES  Make sure it is “YES”

OPTICAL2::Deftran is empty?                YES

OPTICAL2::Has no broken job?              YES

OPTICAL2::JQ Lock is empty?               YES

Step 5   <hostname> # exit

# Correct replication error for Scenario #2

|  | Note Scenario #2 indicates the replication PUSH job on the optical1 database (side-A) is broken. When PUSH job is broken, all outstanding replicated data is held in the replication queue (Deftrandest). In this case, the broken PUSH job needs to be enabled manually, so all the unpushed replicated transactions are propagated. Follow the steps below on the side with broken PUSH job to enable the PUSH job. In this case, side A has broken job. |
| --- | --- |

# From EMS Side A

Step 1   Log in as root

Step 2   <hostname># su – oracle

Step 3   <hostname>$ dbadm –A enable_push_job -Q

Note: This may take a while, until all the unpushed transactions are drained.

Step 4   Re-verify that “Has no broken job?” is “YES” and none of tables is out of sync.

<hostname>$dbadm –C rep

OPTICAL1::Deftrandest is empty?           YES

OPTICAL1::dba_repcatlog is empty?       YES

OPTICAL1::Deferror is empty?                YES

OPTICAL1::Deftran is empty?                  YES

OPTICAL1::Has no broken job?               YES  Make sure it is “YES”

OPTICAL1::JQ Lock is empty?                YES

OPTICAL2::Deftrandest is empty?          YES

OPTICAL2::dba_repcatlog is empty?      YES

OPTICAL2::Deferror is empty?             YES

OPTICAL2::Deftran is empty?                YES

OPTICAL2::Has no broken job?              YES

OPTICAL2::JQ Lock is empty?               YES

Step 5   <hostname> # exit

Appendix J

Creation Of Backup Disks

The following script and instructions split the mirror between the disk set and create two identical and bootable drives on each of the platforms.

Caution: Before continuing with the following procedure, Refer to Appendix N “Verifying the disk mirror” to verify that the disks are mirrored properly.

If it’s not mirrored properly then the below backup script (bts_backup_disk) will first initiate mirroring process and it will take 2.5 hours to complete before creating backup disks.

# Task 1: Creating a Bootable Backup Disk

The following script can be executed in parallel on both the CA and EMS nodes.

Note: This script has to be executed on Side B EMS and CA nodes while side A is active and processing calls.  Subsequently, it has to be executed on Side A EMS and CA nodes.

Step 1   Log in as root user on EMS and CA nodes.

Step 2   Execute the Creation of backup disks script from EMS and CA nodes.

<hostname># cd /opt/Build

<hostname># ./bts_backup_disk.exp

Step 3 The script will display following notes, please verify and answer “y” to the following prompts.

This utility will assist in creating a bootable backup disk of the

currently running BTS system.

- Do you want to continue (y/n)? y

Note: At this point the backup script is in the process of creating Alternate Boot Environments for  Fallback purpose, it will take about 15-30 minutes to complete and will display below prompt. Please be patients on the display “Copying” before you get below prompt.

hostname# display _boot _env _state

Printing boot environment status...

Boot Environment             Is       	Active 	Active    	Can    	Copy

Name                       	   Complete 	Now    On Reboot 	Delete 	Status

--------------------------     -------- 	------ 	---------		 ------ 	----------

d2                         	     yes      	yes   	 yes       	no     	-

bts10200_FALLBACK  yes      	no     	no        	yes   	 -

If status is okay, press y to continue or n to abort..

- Please enter your choice... Do you want to continue? [y,n,?,q] y

Step 4 System will reboot with below note.

Note: At this point the system will be rebooted...

Restart the disk backup procedure once it comes up.

Step 5 After logging in as root on EMS and CA nodes, execute the Creation of backup disks script from EMS and CA nodes again.

<hostname># cd /opt/Build

<hostname># ./bts_backup_disk.exp

Step 6 The script will display following notes, please verify and answer “y” to the following prompts.

This utility will assist in creating a bootable backup disk of the

currently running BTS system.

- Do you want to continue (y/n)? y

Checkpoint 'setBootDisk1' found.  Resuming aborted backup disk procedure

from this point and continuing.

- Do you want to continue (y/n)? y

hostname# display _boot _env _state

Printing boot environment status...

Boot Environment             Is       	Active 	Active    	Can    	Copy

Name                       	   Complete 	Now    On Reboot 	Delete 	Status

--------------------------     -------- 	------ 	---------		 ------ 	----------

d2                         	     yes      	no   	 no	      	yes     	-

bts10200_FALLBACK  yes      	yes    	yes        	no   	 -

If status is okay, press y to continue or n to abort..

- Please enter your choice... Do you want to continue? [y,n,?,q] y

Step 7 System will reboot with below note.

Note: At this point the system will be rebooted...

Restart the disk backup procedure once it comes up.

Step 8 After logging in as root on EMS and CA nodes, execute the Creation of backup disks script from EMS and CA node again.

<hostname># cd /opt/Build

<hostname># ./bts_backup_disk.exp

Step 9 The script will display following notes, please verify and answer “y” to the following prompts.

This utility will assist in creating a bootable backup disk of the

currently running BTS system.

- Do you want to continue (y/n)? y

Checkpoint 'setBootDisk0' found.  Resuming aborted backup disk procedure

from this point and continuing.

- Do you want to continue (y/n)? y

hostname# display _boot _env _state

Printing boot environment status...

Boot Environment             Is       	Active 	Active    	Can    	Copy

Name                       	   Complete 	Now    On Reboot 	Delete 	Status

--------------------------     -------- 	------ 	---------		 ------ 	----------

d2                         	     yes      	yes   	 yes       	no     	-

bts10200_FALLBACK  yes      	no     	no        	yes   	 -

If status is okay, press y to continue or n to abort..

- Please enter your choice... Do you want to continue? [y,n,?,q] y

Step 10 The following message will be displayed to complete the Creation of backup disks script.

=====================================================

=============== Backup disk created =================

=========== Thu Jan 10 14:29:51 CST 2008 ============

=====================================================

# Task 2: Perform Switchover to prepare Side A CA and EMS Bootable Backup Disk

Step 1 Perform the following command on Side A EMS.

<hostname># echo upgradeInProgress=yes >> /opt/ems/etc/ems.props

Step 2 Control all the platforms to standby-active. Login into the EMS side A and execute the following commands

<hostname>#   su - btsadmin

CLI> control call-agent id=CAxxx; target-state=STANDBY_ACTIVE;

CLI> control feature-server id=FSPTCyyy; target-state= STANDBY_ACTIVE;

CLI> control feature-server id=FSAINzzz; target-state= STANDBY_ACTIVE;

CLI> control bdms id=BDMSxx; target-state= STANDBY_ACTIVE;

CLI> control element_manager id=EMyy; target-state= STANDBY_ACTIVE;

CLI>Exit

Note:  It is possible that the mirror process for Side A nodes was previously started and not completed.  If this is the case, the Creation of Backup Disk script will not work and the disks will be left in an indeterminate state.

# Task 3: Repeat task 1 on the Side A EMS and CA Nodes

Note: At this point both Side A and Side B are running in a split mirror state on disk 0, thus both Side A and Side B (EMS & CA) are fully prepared to do fallback if needed on disk 1(bts10200_FALLBACK boot environment).

Appendix K

Caveats and solutions

- Internal Oracle Error (ORA-00600) during DataBase Copy

Symptom: The upgrade script may exit with the following error during DataBase copy.

<Mon Jan 29 17:08:22 EST 2007> ERROR: Fail to restore Referential Constraints

==========================================================

ERROR: Database copy failed

==========================================================

secems02# echo $?

1

secems02# ************************************************************

Error: secems02: failed to start platform

Work around:

Login to the EMS platform on which this issue was encountered and issue the following commands

  - su – oracle

  - optical1:priems02: /opt/orahome$ sqlplus / as sysdba

SQL*Plus: Release 10.1.0.4.0 - Production on Tue Jan 30 19:40:56 2007

Copyright (c) 1982, 2005, Oracle. All rights reserved.

Connected to: Oracle Database 10g Enterprise Edition Release 10.1.0.4.0 - 64bit Production With the Partitioning and Data Mining options

  - SQL> shutdown immediate
ORA-00600: internal error code, arguments: [2141], [2642672802], [2637346301], [], [], [], [], []

  - SQL> shutdown abort
ORACLE instance shut down.

- SQL> startup 
ORACLE instance started.

Total System Global Area 289406976 bytes
Fixed Size 1302088 bytes
Variable Size 182198712 bytes
Database Buffers 104857600 bytes
Redo Buffers 1048576 bytes
Database mounted.
Database opened.

- SQL> exit
Disconnected from Oracle Database 10g Enterprise Edition Release 10.1.0.4.0 - 64bit Production
With the Partitioning and Data Mining options

- Access Disk 0 to get traces, after Fallback to Disk 1

Following steps can be executed to access Disk 0, after performing Appendix A and the system is running on Disk 1.

- mount /dev/dsk/c1t0d0s5 /mnt

- That should mount the /opt partition of disk 0 on /mnt

- # mount |grep opt

- This will show /opt/mounted either on /dev/dsk/cxtyd0s5, or on /dev/md/dsk/d11. If the former, just flip the target (t) from 1 to zero or vice versa. If the latter do:

- # metastat

- Identify the submirrors of d11. They should be d9 and d10. Which ever one has a *not* Okay state is the one you want to mount, using the cxtxd0s5 asssociated with that submirror.

Appendix L

# Sync Data from Active EMS to Active CA/FS

In case there are errors indicating DB mismatches, execute the following steps to sync data from Active EMS to Active CA/FS.

# Task 1: Sync Data from Active EMS to Active CA/FS

# From Active EMS

Follow the command syntax provided below to correct the mismatches:

Step 1 Login as “btsadmin”

<hostname># su - btsadmin

Step 2 btsadmin>unblock session user=%;

Step 3 btsadmin>exit

Step 4 Login as “ciscouser”

<hostname># su - ciscouser

Step 5 CLI> sync <table name> master=EMS; target=<CAxxx/FSPTCyyy/FSAINzzz>;

Step 6 CLI> exit

Example:

- CLI> sync language master=EMS; target=CAxxx;

- CLI> sync language master=EMS; target=FSPTCyyy;

- CLI> sync policy_profile master=EMS; target=CAxxx;

- CLI> sync policy_profile master=EMS; target=FSAINzzz;

- CLI> sync sip_element master=EMS; target=CAxxx;

- CLI> sync dn2subscriber master=EMS; target=FSPTCyyy;

- CLI> sync isdn_dchan master=EMS; target=CAxxx;

- CLI> sync pop master=EMS; target=FSAINzzz;

# Task 2: Execute DB Audit (Row Count)

Once the data sync between Active EMS and Active CA/FS is complete, a row count audit MUST be performed before restarting the upgrade script.

Step 1   Login as “ciscouser”

<hostname># su - ciscouser

Step 2   CLI>audit database type=row-count

Step 3    CLI> exit

Note: If there is no database mismatch found then CLI session must be block again before continuing the upgrade

Step 4  Login as “btsadmin”

<hostname># su - btsadmin

Step 5 btsadmin>block session user=%;

Step 6 btsadmin>exit

Appendix M

Opticall.cfg parameters

Caution: The values provided by the user for the following parameters will be written into /etc/opticall.cfg and transported to all 4 BTS nodes.

- The following parameters are associated to Log Archive Facility (LAF) process. If they are left blank, the LAF process for a particular platform (ie CA, FSPTC, FSAIN) will be turned off.

If the user wants to use this feature, the user  must provision the following parameters with the external archive system target directory as well as the disk quota (in Gega Bytes) for each platform.

For example (Note xxx must be replaced with each platform instance number)

- CAxxx_LAF_PARAMETER:

- FSPTCxxx_LAF_PARAMETER:

- FSAINxxx_LAF_PARAMETER:

# Example: CA146_LAF_PARAMETER="yensid /CA146_trace_log 20"

# Example: FSPTC235_LAF_PARAMETER="yensid /FSPTC235_trace_log 20"

# Example: FSAIN205_LAF_PARAMETER="yensid /FSAIN205_trace_log 20"

Note: In order to enable Log Archive Facility (LAF) process, refer to BTS http://lbj/push_targets1/ucdit/cc/td/doc/product/voice/bts10200/bts5_0/install/index.htm (Application Installation Procedure)

- This parameter specifies the billing record filenaming convention. Default value is Default. Possible values are Default and PacketCable.

- BILLING_FILENAME_TYPE:

- This parameter specifies the delimiter used to separate the fields within a record in a billing file. Default value is semicolon. Possible values are semicolon, semi-colon, verticalbar, vertical-bar, linefeed, comma, caret.

For example:

- BILLING_FD_TYPE: semicolon

4.  This parameter specifies the delimiter used to separate the records within a     billing file. Default value is verticalbar. Possible values are semicolon, semi-colon, verticalbar, vertical-bar, linefeed, comma, caret

For example:

- BILLING_RD_TYPE: verticalbar

5. The following parameter should be populated with qualified domain names used by MGA process in the Call agents for external communication. Each domain name should return two logical external IP addresses. For example.

- DNS_FOR_CA146_MGCP_COM: mga-SYS76CA146.ipclab.cisco.com

6. The following parameter should be populated with qualified domain names used by H3A process in the Call agents for external communication. Each domain name should return two logical external IP addresses. For example.

- DNS_FOR_CA146_H323_COM: h3a-SYS76CA146.ipclab.cisco.com

7. The following parameter should be populated with qualified domain names used by IUA process in the Call agents for external communication. Each domain name should return two physical IP addresses. For example.

- DNS_FOR_CA_SIDE_A_IUA_COM: iua-asysCA.domainname

- DNS_FOR_CA_SIDE_B_IUA_COM: iua-bsysCA.domainname

8. These are qualified domain names used by MDII process in the EMS agents for internal communication. Each domain name should return two internal logical IP addresses. For example.

- DNS_FOR_EMS_SIDE_A_MDII_COM: mdii-asysEMS.domainname

- DNS_FOR_EMS_SIDE_B_MDII_COM: mdii-bsysEMS.domainname

9. This is the qualified DNS names used by HDM to communicate to HSS      through diameter protocol.

This parameter should return one logical IP address, in the same subnet as either first or second subnet of EMS host (i.e. management network). If user choose NOT to enable diameter protocol, (i.e. set DIA_ENABLED=n) user do NOT need to fill in this FQDN.

- DNS_FOR_EM01_DIA_COM=

- EMS_DIA_ORIGIN_HOST=

Appendix N

Verifying the Disk mirror

Step 1 The following command determines if the system has finished the disk mirror setup.

<hostname>#  metastat |grep %

If no output is returned as a result of the above command then the system is syncing disks and the systems are up to date. Note however that this does not guarantee the disks are properly mirrored.

Step 2 The following command determines status of all the metadb slices on the disk.

<hostname>#  metadb |grep c1

The output should look very similar to the following

a m  p  luo        16              8192            /dev/dsk/c1t0d0s4

a      p  luo        8208          8192            /dev/dsk/c1t0d0s4

a      p  luo        16400        8192            /dev/dsk/c1t0d0s4

a     p   luo        16              8192            /dev/dsk/c1t1d0s4

a     p   luo        8208          8192            /dev/dsk/c1t1d0s4

a     p   luo        16400        8192            /dev/dsk/c1t1d0s4

Step 3 The following command determines the status of all the disk slices under mirrored control.

<hostname>#  metastat |grep c1

The output of the above command should look similar to the following:

c1t0d0s1          0     No            Okay   Yes

c1t1d0s1          0     No            Okay   Yes

c1t0d0s5          0     No            Okay   Yes

c1t1d0s5          0     No            Okay   Yes

c1t0d0s6          0     No            Okay   Yes

c1t1d0s6          0     No            Okay   Yes

c1t0d0s0          0     No            Okay   Yes

c1t1d0s0          0     No            Okay   Yes

c1t0d0s3          0     No            Okay   Yes

c1t1d0s3          0     No            Okay   Yes

c1t1d0   Yes    id1,sd@SFUJITSU_MAP3735N_SUN72G_00Q09UHU____

c1t0d0   Yes    id1,sd@SFUJITSU_MAP3735N_SUN72G_00Q09ULA____

Caution: Verify all 10 above slices are displayed. Also if an Okay is not seen on each of the slices for disk 0 and disk 1, then the disks are not properly mirrored.

Appendix O

Verifying index id for aor2sub, user-auth and SiaRegContact Tables

# From Active CA

Step 1 Login to Active CA and execute following commands

<hostname> cd /opt/OptiCall/CA146/bin

<hostname> ./ca_repl_verify.CA146 data  1 aor2sub 0 >aor2sub.log

<hostname> ./ca_repl_verify.CA146 data  1 user_auth 0 >userauth.log

<hostname> ./ca_repl_verify.CA146 data  1 siaregcontact 0 >siareg.log

Step 2 Issue the following grep commands to above log files.

<hostname> grep "idx=[2-9][0-9][0-9][0-9] " aor2sub.log | sort | grep -v "idx=2000 "

<hostname> grep "idx=[2-9][0-9][0-9][0-9] " userauth.log | sort | grep -v "idx=2000 "

<hostname> grep "idx=[2-9][0-9][0-9][0-9] " siareg.log | sort | grep -v "idx=2000 "

Step 3 If any of the above grep commands return with results then please proceed to step 4 & 5 below, otherwise Appendix O complete.

Step 4 Use CLI commands from active EMS and delete all user-auth and aor2sub records from the system.

Step 5 Issue the following CLI command to verify SIP Subscriber records has been deleted from the system.

CLI> show aor2sub sub_id=%;

Example Output:

Reply : Success: at 2008-02-06 10:05:58 by optiuser
    Database is void of entries.

Note: SIP Subscribers can be provisioned back again, after the system upgraded to 6.0 Release successfully.

Appendix P

Midpoint Patch Install Procedure for CA

# From CA

Step 1  Locate CD-ROM Disc labeled as "BTS_06_00_01_V02_PXX"

- Log in as root

- Put Disc labeled as "BTS_06_00_01_V02_PXX” in the CD-ROM drive

- Mount CD-ROM drive

<hostname># cd /

<hostname># mkdir -p /cdrom

<hostname># mount -o ro -F hsfs /dev/dsk/c0t0d0s0 /cdrom

- Copy file from the CD-ROM to the /opt directory.

<hostname># cp –f /cdrom/BTS_06_00_01_V02_PXX.tar  /opt

<hostname># umount /cdrom

- Manually eject the CD-ROM and take out Disc from drive

Step 2	Untar the file on CA platform.

<hostname># cd /opt

<hostname># tar -xvf /opt/BTS_06_00_01_V02_PXX.tar

Step 3 Delete the PatchLevel file if present.

<hostname># cd /opt/ems/utils/

- Check for PatchLevel file

<hostname># ls PatchLevel

- If PatchLevel file exists then delete the file.

<hostname># \rm -rf PatchLevel

Step 4 Run the CA Patch script.

<hostname># cd /opt/BTS_06_00_01_V02_PXX/

<hostname># chmod +x patchman

<hostname># ./patchman BTS_06_00_01_V02_PXX

==========================================================

Install BTS_06_00_01_V02_PXX   - Press 1

Uninstall BTS_06_00_01_V02_PXX - Press 2

EXIT                           - Press 0

==========================================================

Make a choice (0/1/2):

- Select 1 to install.

==========================================================

INSTALLING [BTS_06_00_01_V02_PXX] PATCH on CA-SIDE

==========================================================

Are You Sure?   [y,n,?,q]

- Select "y"

==========================================================

MEM.CFG files have been changed

==========================================================

Do you want to save copy of OLD DATA DIR?  [y,n,?,q]

- Select "y"

Step 5 Please follow the remaining patch installation procedure given in the BTS_06_00_01_V02_PXX.procedure present in /opt/BTS_06_00_01_V02_PXX.

Appendix Q

Midpoint Patch Install Procedure for EMS

# From EMS

Step 1  Locate CD-ROM Disc labeled as "BTS_06_00_01_V02_PXX"

- Log in as root

- Put Disc labeled as "BTS_06_00_01_V02_PXX” in the CD-ROM drive

- Mount CD-ROM drive

<hostname># cd /

<hostname># mkdir -p /cdrom

<hostname># mount -o ro -F hsfs /dev/dsk/c0t0d0s0 /cdrom

- Copy file from the CD-ROM to the /opt directory.

<hostname># cp –f /cdrom/BTS_06_00_01_V02_PXX.tar  /opt

<hostname># umount /cdrom

- Manually eject the CD-ROM and take out Disc from drive

Step 2	Untar the file on CA platform.

<hostname># cd /opt

<hostname># tar -xvf /opt/BTS_06_00_01_V02_PXX.tar

Step 3 Delete the PatchLevel file if present.

<hostname># cd /opt/ems/utils/

- Check for PatchLevel file

<hostname># ls PatchLevel

- If PatchLevel file exists then delete the file.

<hostname># \rm -rf PatchLevel

Step 4 Run the EMS Patch script.

<hostname># cd /opt/BTS_06_00_01_V02_PXX/

<hostname># chmod +x patchman

<hostname># ./patchman BTS_06_00_01_V02_PXX

==========================================================

Install BTS_06_00_01_V02_PXX   - Press 1

Uninstall BTS_06_00_01_V02_PXX - Press 2

EXIT                           - Press 0

==========================================================

Make a choice (0/1/2):

- Select 1 to install.

==========================================================

INSTALLING [BTS_06_00_01_V02_PXX] PATCH on EMS-SIDE

==========================================================

Are You Sure?  [y,n,?,q]

- Select "y"

Step 5 Please follow the remaining patch installation procedure given in the BTS_06_00_01_V02_PXX.procedure present in /opt/BTS_06_00_01_V02_PXX

Appendix R

Midpoint Patch Uninstall Procedure for CA

# From CA

Step 1 Run the CA Patch script.

<hostname># cd /opt/BTS_06_00_01_V02_PXX/

<hostname># chmod +x patchman

<hostname># ./patchman BTS_06_00_01_V02_PXX

==========================================================

Install BTS_06_00_01_V02_PXX   - Press 1

Uninstall BTS_06_00_01_V02_PXX - Press 2

EXIT                           - Press 0

==========================================================

Make a choice (0/1/2):

- Select 2 to uninstall.

==========================================================

UN-INSTALLING [BTS_06_00_01_V02_PXX] PATCH on CA-SIDE

==========================================================

Are You Sure?  [y,n,?,q]

- Select "y"

Step 2 Please follow the remaining patch un-installation procedure given in the BTS_06_00_01_V02_PXX.procedure present in /opt/BTS_06_00_01_V02_PXX

Appendix S

Midpoint Patch Uninstall Procedure for EMS

# From EMS

Step 1 Run the EMS Patch script.

<hostname># cd /opt/BTS_06_00_01_V02_PXX/

<hostname># chmod +x patchman

<hostname># ./patchman BTS_06_00_01_V02_PXX

==========================================================

Install BTS_06_00_01_V02_PXX   - Press 1

Uninstall BTS_06_00_01_V02_PXX - Press 2

EXIT                           - Press 0

==========================================================

Make a choice (0/1/2):

- Select 2 to uninstall.

==========================================================

UN-INSTALLING [BTS_06_00_01_V02_PXX] PATCH on CA-SIDE

==========================================================

Are You Sure?  [y,n,?,q]

- Select "y"

Step 2 Please follow the remaining patch un-installation procedure given in the BTS_06_00_01_V02_PXX.procedure present in /opt/BTS_06_00_01_V02_PXX

Appendix T

Patch Procedure for 5.0.3 to 6.0.1V02PXX before starting Upgrade Control Program.

Execute the steps from 1-3 on all 4 BTS nodes

Step 1 Locate CD-ROM Disc labeled as "BTS_06_00_01_V02_PXX"

- Log in as root

- Put Disc labeled as "BTS_06_00_01_V02_PXX” in the CD-ROM drive

- Mount CD-ROM drive

<hostname># cd /

<hostname># mkdir -p /cdrom

<hostname># mount -o ro -F hsfs /dev/dsk/c0t0d0s0 /cdrom

- Copy file from the CD-ROM to the /opt directory.

<hostname># cp –f /cdrom/BTS_06_00_01_V02_PXX.tar  /opt

<hostname># umount /cdrom

- Manually eject the CD-ROM and take out Disc from drive

Step 2	Untar the file.

<hostname># cd /opt

<hostname># tar -xvf /opt/BTS_06_00_01_V02_PXX.tar

Step 3 Take the backup of the existing install.sh and bts_upgrade.exp

<hostname> # cd /opt/Build

<hostname> #cp install.sh install.sh.orig

<hostname> #cp bts_upgrade.exp bts_upgrade.exp.orig

Step 4 Copy the file install.sh and bts_upgrade.exp and make them executable.

<hostname># cd /opt/BTS_06_00_01_V02_PXX

<hostname># cp –p install.sh.BTS_06_00_01_V02_PXX /opt/Build/install.sh

<hostname>#cp –p bts_upgrade.exp.BTS_06_00_01_V02_PXX /opt/Build/bts_upgrade.exp

<hostname># cd /opt/Build

<hostname># chmod +x install.sh

<hostname># chmod +x bts_upgrade.exp

Appendix U

Procedure to verify the entries in ACG table

This procedure is used to verify if there are any entries in the ACG table on Call Agent side. If there are any entries found in ACG table they should be removed before proceeding to next step in Upgrade.

# From Active CA

Step 1 Login as root to Active Call Agent.

Step 2 <hostname># cd /opt/OptiCall/FSAIN*/bin/

Step 3 <hostname># ./ain_repl_verify.FSAIN* ./data

Step 4 At the prompt Enter the following.

1 acg 0

Caution: If you DO NOT see the following response. Please follow the steps below to clean the ACG table on this node else continue from Step 11 in Task 6 in Chapter 5.

printing acg table, record size=52

********** NO OF RECORDS= 0 * Filtered  Out= 0 * Selected= 0 **********

Step 5 At the prompt Enter the following.

0

Step 6 <hostname># platform stop –i FSAINXXX

Step 7 <hostname># cd /opt/OptiCall/FSAIN*/bin/data/

Step 8 <hostname># \rm acg.dat acg.hdr

Step 9 <hostname># platform start –i FSAINXXX

Caution: Wait till the platform becomes Active before following the Step 11 in Task 6 in Chapter 5.