Document Number 	EDCS-594383

Revision		9.0

Cisco BTS 10200 Softswitch Software Upgrade for Release

4.5.13 to 4.5.1 load

Dec 13, 2007

Corporate Headquarters

Cisco Systems, Inc.

170 West Tasman Drive

San Jose, CA 95134-1706

USA

http://www.cisco.com

Tel: 	408 526-4000

800 553-NETS (6387)

Fax:	408 526-4100

THE SPECIFICATIONS AND INFORMATION REGARDING THE PRODUCTS IN THIS MANUAL ARE SUBJECT TO CHANGE WITHOUT NOTICE. ALL STATEMENTS, INFORMATION, AND RECOMMENDATIONS IN THIS MANUAL ARE BELIEVED TO BE ACCURATE BUT ARE PRESENTED WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED. USERS MUST TAKE FULL RESPONSIBILITY FOR THEIR APPLICATION OF ANY PRODUCTS.

THE SOFTWARE LICENSE AND LIMITED WARRANTY FOR THE ACCOMPANYING PRODUCT ARE SET FORTH IN THE INFORMATION PACKET THAT SHIPPED WITH THE PRODUCT AND ARE INCORPORATED HEREIN BY THIS REFERENCE. IF YOU ARE UNABLE TO LOCATE THE SOFTWARE LICENSE OR LIMITED WARRANTY, CONTACT YOUR CISCO REPRESENTATIVE FOR A COPY.

The Cisco implementation of TCP header compression is an adaptation of a program developed by the University of California, Berkeley (UCB) as part of UCB’s public domain version of the UNIX operating system. All rights reserved. Copyright © 1981, Regents of the University of California.

NOTWITHSTANDING ANY OTHER WARRANTY HEREIN, ALL DOCUMENT FILES AND SOFTWARE OF THESE SUPPLIERS ARE PROVIDED “AS IS” WITH ALL FAULTS. CISCO AND THE ABOVE-NAMED SUPPLIERS DISCLAIM ALL WARRANTIES, EXPRESSED OR IMPLIED, INCLUDING, WITHOUT LIMITATION, THOSE OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT OR ARISING FROM A COURSE OF DEALING, USAGE, OR TRADE PRACTICE.

IN NO EVENT SHALL CISCO OR ITS SUPPLIERS BE LIABLE FOR ANY INDIRECT, SPECIAL, CONSEQUENTIAL, OR INCIDENTAL DAMAGES, INCLUDING, WITHOUT LIMITATION, LOST PROFITS OR LOSS OR DAMAGE TO DATA ARISING OUT OF THE USE OR INABILITY TO USE THIS MANUAL, EVEN IF CISCO OR ITS SUPPLIERS HAVE BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES.

CCIP, CCSP, the Cisco Arrow logo, the Cisco Powered Network mark, the Cisco Systems Verified logo, Cisco Unity, Follow Me Browsing, FormShare, iQ Breakthrough, iQ FastTrack, the iQ Logo, iQ Net Readiness Scorecard, Networking Academy, ScriptShare, SMARTnet, TransPath, and Voice LAN are trademarks of Cisco Systems, Inc.; Changing the Way We Work, Live, Play, and Learn, The Fastest Way to Increase Your Internet Quotient, and iQuick Study are service marks of Cisco Systems, Inc.; and Aironet, ASIST, BPX, Catalyst, CCDA, CCDP, CCIE, CCNA, CCNP, Cisco, the Cisco Certified Internetwork Expert logo, Cisco IOS, the Cisco IOS logo, Cisco Press, Cisco Systems, Cisco Systems Capital, the Cisco Systems logo, Empowering the Internet Generation, Enterprise/Solver, EtherChannel, EtherSwitch, Fast Step, GigaStack, Internet Quotient, IOS, IP/TV, iQ Expertise, LightStream, MGX, MICA, the Networkers logo, Network Registrar, Packet, PIX, Post-Routing, Pre-Routing, RateMUX, Registrar, SlideCast, StrataView Plus, Stratm, SwitchProbe, TeleRouter, and VCO are registered trademarks of Cisco Systems, Inc. and/or its affiliates in the U.S. and certain other countries.

All other trademarks mentioned in this document or Web site are the property of their respective owners. The use of the word partner does not imply a partnership relationship between Cisco and any other company. (0301R)

Cisco BTS 10200 Softswitch Software Upgrade

Copyright © 2008, Cisco Systems, Inc.

All rights reserved.

| Revision History |  |  |
| --- | --- | --- |
| Date | Version | Description |
| 05/10/2007 | 1.0 | - Initial Version |
| 06/11/2007 | 2.0 | - Updated to resolve CSCsj11359 regarding OS reboot procedure. Also updated the procedure per review meeting comments. |
| 06/18/2007 | 3.0 | - Updated on Chapter 3 to resolve CSCsj27430. Also incorporated Option2 disk mirroring procedure. |
| 07/01/2007 | 4.0 | - Added LAF check on Chapter 3 to resolve CSCsj23808. - Added Dial Plan ID check on Chapter 3 to resolve CSCsi09503. - Updated Appendix M, caution note to refer Appendix K |
| 08/01/2007 | 5.0 | - Modified Appendix A, B, D, E, K & M based on live upgrade disk mirroring procedure. Also removed Appendix N and O. - Also Added task #5 in Chapter 3 to Change NAMED_ENABLED value |
| 08/02/2007 | 6.0 | - Updated Step #7 in Appendix K |
| 08/07/2007 | 7.0 | - Updated Procedure to resolve CSCsj97105 |
| 08/21/2007 | 8.0 | - Added Task # 6 in Chapter 3 to Install SUN OS 126546-01 patch. - Also updated Appendix K to verify booting from disk 1. |
| 12/13/2007 | 9.0 | - Updated Appendix I per Juann’s comments for Audit Oracle Database, also added Task#8 in Chapter 6 to refer to Appendix I |

Table of Contents

Table of Contents	4

Preface	7

Obtaining Documentation	7

World Wide Web	7

Documentation CD-ROM	7

Ordering Documentation	7

Documentation Feedback	7

Obtaining Technical Assistance	8

Cisco.com	8

Technical Assistance Center	9

Cisco TAC Web Site	9

Cisco TAC Escalation Center	10

Chapter 1	11

Upgrade Requirements	11

Introduction	11

Assumptions	11

Requirements	12

Important notes about this procedure	12

Chapter 2	14

Preparation	14

Task 1: Requirements and Prerequisites	14

Task 2: Preparation	14

Task 3: Stage the load to the system	14

Task 4: Save the current opticall.cfg	15

Task 5: Check for HW errors	16

Task 6: CDR delimiter customization	16

Chapter 3	17

17

Complete the following tasks 24-48 hours before the scheduled upgrade	17

Task 1: Check CA-CONFIG Table	17

From Active EMS	17

Task 2: Check SUBSCRIBER Table	18

From Active EMS	18

Task 3: Check DIAL_PLAN_ID Table	19

From Active EMS	19

20

Task 4: Check LAF Configuration	20

From Active EMS	20

Task 5: Change NAMED_ENABLED value	20

Task 6: Install SUN OS Patch 126546-01 on All Four Nodes	21

Chapter 4	23

23

Complete the following tasks the night before the scheduled upgrade	23

Task 1: Perform full database audit	23

Chapter 5	24

24

Upgrade the System	24

Task 1: Verify system in normal operating status	25

From Active EMS	25

Task 2: Alarms	26

Refer to Appendix H to verify that there are no outstanding major or critical alarms. 	26

Task 3: Audit Oracle Database and Replication	26

26

Refer to Appendix I to verify Oracle database and replication functionality.	26

Task 4: Creation of Backup Disks	26

Task 5: Verify Task 1, 2 & 3	26

Task 6: Start Upgrade Process by Starting the Upgrade Control Program	27

On all 4 BTS nodes	27

From EMS side B	27

Task 7: Validate New Release operation	30

Task 8: Upgrade Side A	30

Chapter 6	32

Finalizing Upgrade	32

Task 1: Restore cron jobs for EMS	32

From EMS side A	32

From EMS side B	32

Task 2: CDR delimiter customization	33

Task 3: Add Feature Config for LOCAL-NOD-ACCT-CODE-COLLECT	34

From Active EMS	34

Task 4: Add Feature Config for TOLLFREE-NOD-ACCT-CODE-COLLECT	35

From Active EMS	35

Task 5: Add ISDN-DCHAN Licensed-record-count	35

From Active EMS	35

Task 6: Enable LAF on CA & EMS nodes	36

36

36

Task 7: To install CORBA on EMS, please follow Appendix C.	36

37

Task 8: Audit Oracle Database and Replication	37

37

Refer to Appendix I to verify Oracle database and replication functionality.	37

Task 9: Initiate disk mirroring by using Appendix D.	37

Appendix A	38

Backout Procedure for Side B Systems	38

Appendix B	48

Full System Backout Procedure	48

Appendix C	59

CORBA Installation	59

Task 1: Install OpenORB CORBA Application	59

Remove Installed OpenORB Application	59

Install OpenORB Packages	60

Appendix D	62

Full System Successful Upgrade Procedure	62

Appendix E	65

Emergency Fallback Procedure Using the Backup Disks	65

Appendix F	72

Staging the 4.5.1 load to the system	72

From EMS Side A	74

From CA/FS Side B	76

Appendix G	77

Check database	77

Perform database audit	77

Appendix H	79

Check Alarm Status	79

From EMS side A	79

Appendix I	81

Audit Oracle Database and Replication	81

Check Oracle DB replication status	81

From STANDBY EMS	81

Correct replication error	83

From EMS Side B	83

From EMS Side A	83

Appendix J	85

Caveats and solutions	85

Appendix K	87

Creation Of Backup Disks	87

87

Task 1: Creating a Bootable Backup Disk	87

Task 2: Restore the BTS Platforms	92

93

Task 3: Perform Switchover to prepare Side A CA and EMS Bootable Backup Disk	93

94

Task 4: Repeat tasks 1 and 2 on the Side A EMS and CA Nodes	94

Appendix L	95

Mirroring the Disks	95

Appendix M	98

Verifying the Disk mirror	98

Preface

# Obtaining Documentation

These sections explain how to obtain documentation from Cisco Systems.

### World Wide Web

You can access the most current Cisco documentation on the World Wide Web at this URL: http://www.cisco.com/

Translated documentation is available at this URL: http://www.cisco.com/public/countries_languages.shtml

### Documentation CD-ROM

Cisco documentation and additional literature are available in a Cisco Documentation CD-ROM package, which is shipped with your product. The Documentation CD-ROM is updated monthly and may be more current than printed documentation. The CD-ROM package is available as a single unit or through an annual subscription.

### Ordering Documentation

You can order Cisco documentation in these ways:

Registered Cisco.com users (Cisco direct customers) can order Cisco product documentation from the Networking Products MarketPlace: http://www.cisco.com/cgi-bin/order/order_root.pl

Registered Cisco.com users can order the Documentation CD-ROM through the online Subscription Store: http://www.cisco.com/go/subscription

Nonregistered Cisco.com users can order documentation through a local account representative by calling Cisco Systems Corporate Headquarters (California, U.S.A.) at 408 526-7208 or, elsewhere in North America, by calling 800 553-NETS (6387).

### Documentation Feedback

You can submit comments electronically on Cisco.com. In the Cisco Documentation home page, click the Fax or Email option in the “Leave Feedback” section at the bottom of the page.

You can e-mail your comments to mailto:bug-doc@cisco.com.

You can submit your comments by mail by using the response card behind the front cover of your document or by writing to the following address:

Cisco Systems, INC.

Attn: Document Resource Connection

170 West Tasman Drive

San Jose, CA 95134-9883

# Obtaining Technical Assistance

Cisco provides Cisco.com as a starting point for all technical assistance. Customers and partners can obtain online documentation, troubleshooting tips, and sample configurations from online tools by using the Cisco Technical Assistance Center (TAC) Web Site. Cisco.com registered users have complete access to the technical support resources on the Cisco TAC Web Site: http://www.cisco.com/tac

### Cisco.com

Cisco.com is the foundation of a suite of interactive, networked services that provides immediate, open access to Cisco information, networking solutions, services, programs, and resources at any time, from anywhere in the world.

Cisco.com is a highly integrated Internet application and a powerful, easy-to-use tool that provides a broad range of features and services to help you with these tasks:

Streamline business processes and improve productivity

Resolve technical issues with online support

Download and test software packages

Order Cisco learning materials and merchandise

Register for online skill assessment, training, and certification programs

If you want to obtain customized information and service, you can self-register on Cisco.com. To access Cisco.com, go to this URL: http://www.cisco.com/

### Technical Assistance Center

The Cisco Technical Assistance Center (TAC) is available to all customers who need technical assistance with a Cisco product, technology, or solution. Two levels of support are available: the Cisco TAC Web Site and the Cisco TAC Escalation Center.

Cisco TAC inquiries are categorized according to the urgency of the issue:

Priority level 4 (P4)—You need information or assistance concerning Cisco product capabilities, product installation, or basic product configuration.

Priority level 3 (P3)—Your network performance is degraded. Network functionality is noticeably impaired, but most business operations continue.

Priority level 2 (P2)—Your production network is severely degraded, affecting significant aspects of business operations. No workaround is available.

Priority level 1 (P1)—Your production network is down, and a critical impact to business operations will occur if service is not restored quickly. No workaround is available.

The Cisco TAC resource that you choose is based on the priority of the problem and the conditions of service contracts, when applicable.

### Cisco TAC Web Site

You can use the Cisco TAC Web Site to resolve P3 and P4 issues yourself, saving both cost and time. The site provides around-the-clock access to online tools, knowledge bases, and software. To access the Cisco TAC Web Site, go to this URL: http://www.cisco.com/tac

All customers, partners, and resellers who have a valid Cisco service contract have complete access to the technical support resources on the Cisco TAC Web Site. The Cisco TAC Web Site requires a Cisco.com Log in ID and password. If you have a valid service contract but do not have a Log in ID or password, go to this URL to register: http://www.cisco.com/register/

If you are a Cisco.com registered user, and you cannot resolve your technical issues by using the Cisco TAC Web Site, you can open a case online by using the TAC Case Open tool at this URL: http://www.cisco.com/tac/caseopen

If you have Internet access, we recommend that you open P3 and P4 cases through the Cisco TAC Web Site: http://www.cisco.com/tac

### Cisco TAC Escalation Center

The Cisco TAC Escalation Center addresses priority level 1 or priority level 2 issues. These classifications are assigned when severe network degradation significantly impacts business operations. When you contact the TAC Escalation Center with a P1 or P2 problem, a Cisco TAC engineer automatically opens a case.

To obtain a directory of toll-free Cisco TAC telephone numbers for your country, go to this URL: http://www.cisco.com/warp/public/687/Directory/DirTAC.shtml

Before calling, please check with your network operations center to determine the level of Cisco support services to which your company is entitled: for example, SMARTnet, SMARTnet Onsite, or Network Supported Accounts (NSA). When you call the center, please have available your service agreement number and your product serial number.

Chapter 1

Upgrade Requirements

# Introduction

Application software loads are designated as Release 900-aa.bb.cc.Vxx, where

- aa=major release number, for example, 01

- bb=minor release number, for example, 03

- cc=maintenance release, for example, 00

- Vxx=Version number, for example V04

This procedure can be used on an in-service system, but the steps must be followed as shown in this document in order to avoid traffic interruptions.

|  | Caution Performing the steps in this procedure will bring down and restart individual platforms in a specific sequence. Do not perform the steps out of sequence, as it could affect traffic. If you have questions, contact Cisco TAC. |
| --- | --- |

This procedure should be performed during a maintenance window.

|  | Note In this document, the following designations are used: EMS = Element Management System; CA/FS = Call Agent / Feature Server "Primary" is also referred to as "Side A", and "Secondary" is also referred to as "Side B". |
| --- | --- |

# Assumptions

The following assumptions are made.

- The installer has a basic understanding of UNIX and Oracle commands.

- The installer has the appropriate user name(s) and password(s) to log on to each EMS/CA/FS platform as root user, and as Command Line Interface (CLI) user on the EMS.

- The installer has a NETWORK INFORMATION DATA SHEET (NIDS) with the IP addresses of each EMS/CA/FS to be upgraded, and all the data for the opticall.cfg file.

- Confirm that all names in opticall.cfg are in the DNS server

- The CD-ROM for the correct software version is available to the installer, and is readable.

|  | Note Contact Cisco TAC before you start if you have any questions. |
| --- | --- |

# Requirements

Verify that opticall.cfg has the correct information for each of the following machines.

- Side A EMS

- Side B EMS

- Side A CA/FS

- Side B CA/FS

Determine the oracle and root passwords for the systems you are upgrading. If you do not know these passwords, ask your system administrator.

Refer to local documentation to determine if CORBA installation is required on this system. If unsure, ask your system administrator.

# Important notes about this procedure

Throughout this procedure, each command is shown with the appropriate system prompt, followed by the command to be entered in bold. The prompt is generally one of the following:

- Host system prompt (<hostname>#)

- Oracle prompt (<hostname>$)

- SQL prompt (SQL>)

- CLI prompt (CLI>)

- SFTP prompt (sftp>)

Note the following conventions used throughout the steps in this procedure:

- Enter commands as shown, as they are case sensitive (except for CLI commands).

- Press the Return (or Enter) key at the end of each command, as indicated by "<Return>".

It is recommended that you read through the entire procedure before performing any steps.

There will be no CLI provisioning allowed during entire upgrade process.  Close all CLI sessions before upgrade or fallback.

Upgrade process overview.

Meeting

Upgrade

Requirements

Preparing

1 Week

Before

Upgrade

Preparing

24-48 Hours

Before

Upgrade

Preparing

the Night

Before

Upgrade

Upgrading

Finalizing

the

Upgrade

Chapter 2

Preparation

This section describes the steps a user must complete in the week prior to upgrade.

# Task 1: Requirements and Prerequisites

- For 4.5.1 load

  - One CD-ROM disc labeled as Release 4.5.1 Vxx BTS 10200 Application Disk

    - Where x is 00 -99

  - One CD-ROM disc labeled as Release 4.5.1 Vxx BTS 10200 Database Disk

    - Where x is 00 -99

  - One CD-ROM disc labeled as Release 4.5.1 Vxx BTS 10200 Oracle Disk

    - Where x is 00 -99

# Task 2: Preparation

A week before the upgrade, you must perform the following list of tasks:

- Make sure all old tar files and/or any large data files on the systems are removed from the system before the upgrade.

- Verify the CD ROM drive is in working order by using the mount command and a valid CD ROM.

# Task 3: Stage the load to the system

From EMS Side A

Step 1   Log in as root.

Step 2   If /opt/Build contains the currently running load, please save it in case fallback is needed.  Use the following commands to save /opt/Build.

<hostname># cat /opt/Build/Version

- Assume the above command returns the following output

900-04.05.00.V13

- Use “04.05.00.V13” as part of the new directory name

<hostname># mv /opt/Build /opt/Build.04.05.00.V13

Step 3   Repeat Step 1 and Step 2 for EMS Side B.

Step 4   Repeat Step 1 and Step 2 for CA/FS Side A.

Step 5   Repeat Step 1 and Step 2 for CA/FS side B.

Step 6   Use the appropriate Appendix to stage the load

- To stage 4.5.1 Vxx load using CD-ROM, use Appendix F.

# Task 4: Save the current opticall.cfg

Please perform the following steps on all 4 BTS nodes

Step 1   Log in as root.

Step 2   Save the current /etc/opticall.cfg before it’s updated

<hostname># cp  –fp  /etc/opticall.cfg  /opt/.upgrade/opticall.cfg.fallback

- If the above command returns error, /opt/.upgrade directory may not exist yet.  Please do the following commands.

  - <hostname># mkdir –p /opt/.upgrade

  - run “cp –fp /etc/opticall.cfg /opt/.upgrade/opticall.cfg.fallback” command again

# Task 5: Check for HW errors

On all four systems, check /var/adm/messages file for any hardware related errors conditions. Rectify the error conditions before proceeding with the upgrade

# Task 6: CDR delimiter customization

CDR delimiter customization is not retained after software upgrade. If the system has been customized, then the operator must manually recustomize the system after the upgrade.

The following steps must be excuted on both EMS side A and side B

Step 1 <hostname># cd /opt/bdms/bin

Step 2 <hostname># vi platform.cfg

Step 3 Locate the section for the command argument list for the BMG process

Args=-port 15260 -h localhost -u optiuser -p optiuser -fmt default_formatter -UpdIntvl 3300 -ems_local_dn blg-aSYS14EMS.cisco.com -FD semicolon -RD linefeed

Step 4   Record the customized values. These values will be used for CDR customization in the post upgrade steps.

Chapter 3

Complete the following tasks 24-48 hours before the scheduled upgrade

This chapter describes the tasks a user must complete 24-48 hours before the scheduled upgrade.

# Task 1: Check CA-CONFIG Table

# From Active EMS

Step 1   Login to active EMS side and execute the following commands.

Step 2   <hostname># su – btsadmin

Step 3     CLI> show ca-config type=MGCP-MAX-UNREACH-COUNT;

Please check following three possible return results:

- If the result returns with no entry, then execute following command.

CLI>add ca-config type=MGCP-MAX-UNREACH-COUNT; datatype=INTEGER   ; value=1

- If the result return one entry with value=0, then execute following command.

For Example:

TYPE=MGCP-MAX-UNREACH-COUNT

DATATYPE=INTEGER

VALUE=0

CLI>change ca-config type=MGCP-MAX-UNREACH-COUNT; datatype=INTEGER   ; value=1

- If the result return one entry with value=1 or any number other then 0, then leave it as is.

For Example:

TYPE=MGCP-MAX-UNREACH-COUNT

DATATYPE=INTEGER

VALUE=1 or any number other then 0

# Task 2: Check SUBSCRIBER Table

# From Active EMS

Step 1   Log in the active EMS as “root” user

Step 2   <hostname># su – oracle

Step 3    <hostname>$ sqlplus optiuser/optiuser

Step 4    SQL> col id for a30

Step 5    SQL> col term_id for a7

Step 6    SQL> col mgw_id for a30

Step 7    SQL> select id, term_id, mgw_id from subscriber where (term_id, mgw_id ) in   (select term_id,mgw_id from subscriber group by term_id, mgw_id having count(*) > 1) order by term_id,mgw_id;

- If the above query returns a result, a list of subscribers with same term_id and mgw_id. For example,

ID                                 TERM_ID     MGW_ID
------------------------------         -------     ------------------------------
tb67stimgen1-1                 aaln/1      tb67stimgen1
tb67stimgen4-1                 aaln/1      tb67stimgen1
tb67stimgen2-1                 aaln/1      tb67stimgen2
tb67stimgen5-1                 aaln/1      tb67stimgen2

- Please use CLI commands to delete or update one of the duplicated subscribers.

Failure to do so, you will have two subscribers with same term_id and mgw_id. This will result in an upgrade failure.

# Task 3: Check DIAL_PLAN_ID Table

# From Active EMS

Step 1    SQL> col direction for a9;

Step 2    SQL> col TG_TYPE for a7;

Step 3    SQL> select id,dial_plan_id,tg_type,direction,main_sub_id from trunk_grp where TG_TYPE != 'ANNC' and DIRECTION != 'OUT'and MAIN_SUB_ID is null and dial_plan_id is null;

- If the above query returns a result with any row, then use CLI command to assign the missing DIAL_PLAN_ID. Failure to do so will result in an upgrade failure.

For example:

ID          DIAL_PLAN_ID         TG_TYPE     DIRECTION     MAIN_SUB_ID
-------------     -----------------------          --------------     -----------------     ----------------------80033                                          SOFTSW         BOTH

CLI example to fix> change trunk-group; ID=80033; dial-plan-id=<some entry>;

Step 4 Exit from Oracle:

SQL> quit;

<hostname>$ exit

# Task 4: Check LAF Configuration

# From Active EMS

Step 1   Login to active EMS side and execute the following command.

Step 2  <hostname>#cat /etc/opticall.cfg | grep –i LAF

Step 3   Verify LAF configured by displaying the entry as below example.

For example:

CA146_LAF_PARAMETER= <some entry>

FSPTC235_LAF_PARAMETER= <some entry>

FSAIN205_LAF_PARAMETER= <some entry>

EMS_LAF_PARAMETER= <some entry>

BDMS_LAF_PARAMETER= <some entry>

Note: If the LAF parameters are present, it means LAF is configured for that particular process, thus record the LAF configured values to be applied during the post upgrade section on Chapter 6 Task #6. Otherwise LAF is not configured and no post upgrade activity for LAF is needed.

# Task 5: Change NAMED_ENABLED value

Step 1 Execute following steps on all four nodes.

Step 2 Login to the system and execute following command.

<hostname># grep ‘^NAMED_ENABLED’ /etc/opticall.cfg

EXAMPLE OUTPUT:

<hostname># grep ‘^NAMED_ENABLED’ /etc/opticall.cfg

NAMED_ENABLED= < y or n>

Step 3 If the above displayed value is “y” then change the value to “cache_only”; otherwise leave it as “n”.

# Task 6: Install SUN OS Patch 126546-01 on All Four Nodes

Step 1 Download SUN OS patch 126546-01 from http://www.sun.com/ site

Step 2 Copy 126546-01.zip file in to /opt dir on all four nodes.

Step 3 Unzip and install the Patch by executing following commands.

<hostname># cd /opt

<hostname># unzip 126546-01.zip

<hostname># patchadd 126546-01

Example Output:

<hostname># patchadd 126546-01

Validating patches...

Loading patches installed on the system...

Done!

Loading patches requested to install.

Package SUNWbashS from patch 126546-01 is not installed on the system.

Done!

Checking patches that you specified for installation.

Done!

Approved patches will be installed in this order:

126546-01

Checking installed patches...

The original package SUNWbashS that 126546-01 is attempting to install to does not exist on this system.

Verifying sufficient filesystem capacity (dry run method)...

Installing patch packages...

Patch 126546-01 has been successfully installed.

See /var/sadm/patch/126546-01/log for details

Patch packages installed:

SUNWbash

Chapter 4

# Complete the following tasks the night before the scheduled upgrade

This chapter describes the tasks a user must complete the night before the scheduled upgrade.

# Task 1: Perform full database audit

All provisioning activity MUST be suspended before executing the following pre-upgrade DB integrity checks.

In this task a full database audit is performed and errors if any are corrected. Refer to Appendix G to perform full data base Audit.

|  | Caution: It is recommended that a full-data base audit be executed 24 hours prior to performing the upgrade. Execution of full database audit within this time period will provide the ability to bypass a full database audit during the upgrade. In deployments with large databases the full database audit can take several hours which may cause the upgrade to extend beyond the maintenance window. |
| --- | --- |

Chapter 5

Upgrade the System

- Caution: Suspend all CLI provisioning activity during the entire upgrade process.  Close all the CLI provisioning sessions.

2.   Caution: Refer to Appendix J for known caveats and corresponding solutions

1. Note: In the event of the following conditions, use Appendix A to fallback side B systems to the old release.

- Failure to bring up the side B systems to standby state with the new release

- Failure to switch over from Side A systems to side B systems

2.  Note: In the event of the following conditions, use Appendix B to fallback the entire system to the old release.

- Failure to bring up the side A systems to standby state with the new release

- Failure to switch over from Side B systems to side A systems

3.  Note: If the upgrade of the entire systems is successful but it is still required to rollback the entire system to the old release then use Appendix B to fallback the entire system.

4.  Note: If the upgrade of the entire system needs to abandon due to call processing failure or the upgrade performance is so degraded that it is not possible to continue operations with the upgrade release, to restore service as quickly as possible to the old release then use Appendix E.

# Task 1: Verify system in normal operating status

Make sure the Primary systems are in ACTIVE state, and Secondary systems are in STANDBY state.

# From Active EMS

Step 1   Log in as CLI user.

Step 2   CLI> status system;

- Verify the Primary systems are in ACTIVE state and the Secondary systems are in STANDBY state. If not, please use the control command to bring the system to the desired state.

Step 3   CLI> exit

# Task 2: Alarms

# Refer to Appendix H to verify that there are no outstanding major or critical alarms.

# Task 3: Audit Oracle Database and Replication

# Refer to Appendix I to verify Oracle database and replication functionality.

|  | Caution Do NOT continue until all data base mismatches and errors have been completely rectified. |
| --- | --- |

# Task 4: Creation of Backup Disks

Refer to Appendix K for creation of backup disks. It will take 12-15 minutes to complete the task.

Caution: Appendix K must be executed before starting the upgrade process. Creation of backup disks procedure (Appendix K) will split the mirror between the disk set and create two identical and bootable drives on each of the platforms for fallback purpose.

# Task 5: Verify Task 1, 2 & 3

Repeat Task 1, 2 & 3 again to verify that system is in normal operating state.

Note: The upgrade script must be executed from the Console port

Note: If the upgrade script exits as a result of any errors or otherwise, the operator can continue the upgrade process by restarting the upgrade script after rectifying the error that caused the script execution failure. The script will restart at the last recorded successful checkpoint.

# Task 6: Start Upgrade Process by Starting the Upgrade Control Program

# On all 4 BTS nodes

Step 1   Log in as root user.

Step 2   Execute the following commands on all 4 BTS nodes and remove the install.lock file if it is present. .

<hostname># ls /tmp/install.lock

- If the lock file is present, please do the following command to remove it.

<hostname># \rm -f /tmp/install.lock

# From EMS side B

Step 1   Log in as root user.

Step 2   Log all upgrade activities and output to a file

<hostname># script /opt/.upgrade/upgrade.log

- If you get an error from the above command, “/opt/.upgrade” may not exist yet.

  - Please do the following command to create this directory.

<hostname># mkdir –p /opt/.upgrade

  - Run the command“script /opt/.upgrade/upgrade.log”again.

Step 3   Execute the BTS software upgrade script.

- <hostname># /opt/Build/bts_upgrade.exp -stopBeforeStartApps

Step 4   If this BTS system does not use the default root password, you will be prompted for the root password.  The root password must be identical on all the 4 BTS nodes.  Enter the root password when you get following message:

root@[Side A EMS hostname]'s password:

Step 5	The upgrade procedure prompts the user to populate the values of certain parameters in opticall.cfg file. Be prepared to populate the values when prompted.

Caution: If any of the below LAF parameters prompted then press enter for each LAF parameter values (leave it blank). Enter the correct value for Billing Filename Type from /etc/opticall.cfg file.

- Please provide a value for CA146_LAF_PARAMETER:

- Please provide a value for FSPTC235_LAF_PARAMETER:

- Please provide a value for FSAIN205_LAF_PARAMETER:

- Please provide a value for BILLING_FILENAME_TYPE:

.

Step 6   Answer “n” to the following prompt.

- Would you like to perform a full DB audit again?? (y/n) [n] n

Step 7   Caution: It is not recommended to continue the upgrade with outstanding major/critical alarms. Refer to appendix H to mitigate outstanding alarms.

- Question: Do you want to continue (y/n)? [n] y

Step 8    Caution: It is not recommended to continue the upgrade with outstanding major/critical alarms. Refer to appendix H to mitigate outstanding alarms.

- Question: Are you sure you want to continue (y/n)? [n] y

Step 9   Answer “y” to the following prompts.

- <hostname> # About to change platform to standby-active.  Continue? [y/n] y

- <hostname> # About to change platform to active-standby.  Continue? [y/n] y

- <hostname># About to stop platforms on secemsxx and seccaxx.Continue? [y/n] y

Note: At this point if the OS patches are required the upgrade script will install the patches and reboot the system. After the reboot, login to the system again and follow the following steps to continue the upgrade process.

- Run the command “script /opt/.upgrade/upgrade_continue.log”

- <hostname># /opt/Build/bts_upgrade.exp -stopBeforeStartApps

- After running the upgrade.exp above the system will display the following prompt

  - The Solaris patch 110208-21 requires rebooting ..

  - Connection to seccaxx closed by remote host.

  - Connection to seccaxx closed.

  - ========================================================

  - OS Patches were applied, and seccaxx was rebooted.

  - Enter y when the system is back up

  - ========================================================

Are you ready to reconnect (y/n)? y

- About to start platform on secondary side, continue? [y/n] y

- <hostname># About to change platform to standby-active.  Continue? [y/n] y

- The following NOTE will be displayed once the Side B EMS and Side B CA/FS have been upgraded to the new release. After the following NOTE is displayed proceed to Task 5,

***********************************************************************

NOTE: The mid-upgrade point has been reached successfully. Now is the time to verify functionality by making calls, if desired, before proceeding with the upgrade of side A of the BTS.

***********************************************************************

# Task 7: Validate New Release operation

Step 1   Once the side B systems are upgraded and are in ACTIVE state, validate the new release software operation.  If the validation is successful, continue to next step, otherwise refer to Appendix A , Backout Procedure for Side B Systems.

- Verify existing calls are still active

- Verify new calls can be placed

- Verify billing records generated for the new calls just made are correct

  - Log in as CLI user

  - CLI> report billing-record tail=1;

  - Verify that the attributes in the CDR match the call just made.

# Task 8: Upgrade Side A

Note: These prompts are displayed on EMS Side B.

Step 1   Answer “y” to the following prompts.

- <hostname># About to stop platforms on priemsxx and pricaaxx. Continue? [y] y

Note: At this point if the OS patches are required the upgrade script will install the patches and display the following prompt

  - The Solaris patch 110208-21 requires rebooting ..

  - Connection to priemsxx closed by remote host.

  - Connection to priemsxx closed.

  - ========================================================

  - OS Patches were applied, and priemsxx was rebooted.

  - Enter y when the system is back up

  - ========================================================

  - Are you ready to reconnect (y/n)? y

  - ========================================================

  - OS Patches were applied, and pricaxx was rebooted.

  - Enter y when the system is back up

  - ========================================================

  - Are you ready to reconnect (y/n)? y

- About to start platform on primary side, continue? [y/n] y

- <hostname># About to change platform to active-standby.  Continue? [y] y

*** CHECKPOINT syncHandsetData ***

Handset table sync may take long time.  Would you like to do it now?

Please enter “Y” if you would like to run handset table sync, otherwise enter “N”.

- <hostname> # About to change platform to standby-active.  Continue (y/n)?y

- <hostname> # About to change platform to active-standby.  Continue (y/n)?y

==================================================

===============Upgrade is complete==================

==================================================

Chapter 6

Finalizing Upgrade

# Task 1: Restore cron jobs for EMS

Restoration of root cron jobs for the system is not necessary as the upgrade procedure does not overwrite the previous root cron jobs, however a backup was taken for safety purposes and if needed can be found on each system in the /opt/.upgrade directory.

# From EMS side A

Step 1   Log in as root.

Step 2   <hostname># cp -p /opt/.upgrade/root   /var/spool/cron/crontabs

Step 3   <hostname># cp -p /opt/.upgrade/oracle   /var/spool/cron/crontabs

Step 4   Update any customized cron jobs using “crontab –e” command.

Step 5   <hostname># svcadm disable system/cron

Step 6   <hostname># svcadm enable system/cron

# From EMS side B

Step 1   Log in as root.

Step 2   <hostname># cp -p /opt/.upgrade/root   /var/spool/cron/crontabs

Step 3   <hostname># cp -p /opt/.upgrade/oracle   /var/spool/cron/crontabs

Step 4   Update any customized cron jobs using “crontab –e” command.

Step 5   <hostname># svcadm disable system/cron

Step 6   <hostname># svcadm enable system/cron

Step 7   Exit the “script /opt/.upgrade/upgrade.log” session:

<hostname># exit

# Task 2: CDR delimiter customization

CDR delimiter customization is not retained after software upgrade. The system must be  manually recustomized the system after the upgrade.

The following steps must be excuted on both EMS side A and side B

Step 1 <hostname># cd /opt/bdms/bin

Step 2 <hostname># vi platform.cfg

Step 3 Locate the section for the command argument list for the BMG process

Args=-port 15260 -h localhost -u optiuser -p optiuser -fmt default_formatter -UpdIntvl 3300 -ems_local_dn blg-aSYS14EMS.cisco.com -FD semicolon -RD linefeed

Step 4 Modify the customized values. These values were recorded in Chapter 2 Task 7. Customize the CDR delimiters in the “Args=” line according to customer specific requirement. For Example:

Note:These values were recorded in pre-upgrade steps in Chapter 2 Task 6.

Args=-port 15260 -h localhost -u optiuser -p optiuser -fmt default_formatter -UpdIntvl 3300 -ems_local_dn blg-aSYS14EMS.cisco.com -FD semicolon -RD linefeed

Step 5 Restart BDMS on side B EMS

<hostname># platform stop –i BDMS01

<hostname># platform start –i BDMS01

Step 6 Login into the EMS side A and execute the following commands.

<hostname># su – btsadmin

CLI>control bdms id=BDMSxx; target-state= STANDBY_ACTIVE;

Exit from cli and perform below commands on EMS side A.

<hostname># platform stop –i BDMS01

<hostname># platform start –i BDMS01

Step 7 Login into the EMS side A again, and execute the following commands.

<hostname># su – btsadmin

CLI>control bdms id=BDMSxx; target-state= ACTIVE_STANDBY;

Exit from cli.

# Task 3: Add Feature Config for LOCAL-NOD-ACCT-CODE-COLLECT

# From Active EMS

Step 1 Login to CLI as “btsuser”.

<hostname> su – btsuser

Step 2 Issue the following CLI command.

CLI> add feature_config fname=COS;type=LOCAL-NOD-ACCT-CODE-COLLECT;value=ALLOW;

CLI > exit

# Task 4: Add Feature Config for TOLLFREE-NOD-ACCT-CODE-COLLECT

# From Active EMS

Step 1 Login to CLI as “btsuser”.

<hostname> su – btsuser

Step 2 Issue the following CLI command.

CLI> add feature_config fname=COS;type=TOLLFREE-NOD-ACCT-CODE-COLLECT;value=ALLOW;

CLI > exit

# Task 5: Add ISDN-DCHAN Licensed-record-count

# From Active EMS

Step 1 Login to CLI as “btsuser”.

<hostname> su – btsuser

Step 2 Issue the following CLI command.

CLI> add db-license table-name=ISDN-DCHAN;licensed-record-count=2000;

Step 3 Show db-usage to verify licensed-record-count changed.

CLI> show db-usage table-name=ISDN-DCHAN;

CLI > exit

# Task 6: Enable LAF on CA & EMS nodes

Note: Following steps are required to execute if LAF were configured at pre upgrade check. Verify LAF configuration from Chapter 3 Task# 4 before executing following steps to enable LAF.

Step1: Execute following scripts on both CA nodes to enable LAF.

Step 2:   <hostname># cd /opt/OptiCall/CA146/bin

And run  <hostname>#./enableLAF

Step 3:   <hostname># cd /opt/OptiCall/FSAIN205/bin

And run <hostname># ./enableLAF

Step 4:   <hostname># cd /opt/OptiCall/FSPTC235/bin

And run <hostname>#./enableLAF

Step5: Execute following scripts on both EMS nodes to enable LAF.

Step 6:   <hostname># cd /opt/EMS/bin

And run  <hostname>#./enableLAF

Step 7:   <hostname># cd /opt/bdms/bin

And run <hostname># ./enableLAF

# Task 7: To install CORBA on EMS, please follow Appendix C.

# Task 8: Audit Oracle Database and Replication

# Refer to Appendix I to verify Oracle database and replication functionality.

# Task 9: Initiate disk mirroring by using Appendix D.

Refer to Appendix D for initiating disk mirroring. It will take about 2.5 hours for each side to complete the mirroring process.

Warning: It is strongly recommended to wait for next maintenance window for initiating disk mirroring process. After disk mirroring is completed by using Appendix D, the system will no longer have the ability to fallback to the previous release. Make sure the entire software upgrade process is completed successfully and the system does not experience any call processing issue before executing Appendix D.

You have completed the entire software upgrade process.

Appendix A

Backout Procedure for Side B Systems

This procedure allows you to back out of the upgrade procedure if any verification checks (in "Verify system status" section) failed. This procedure is intended for the scenario in which the side B system has been upgraded to the new load and in active state, or side B system failed to upgrade to the new release, while the side A system is still at the previous load and in standby state. The procedure will back out the side B system to the previous load.

This backout procedure will:

- Restore the side A system to active mode without making any changes to it

- Revert to the previous application load on the side B system

- Restart the side B system in standby mode

- Verify that the system is functioning properly with the previous load

This procedure is used to restore the previous version of the release on Side B using a fallback release on disk 1.

The system must be in split mode so that the Side B EMS and CA can be reverted back to the previous release using the fallback release on disk 1.

Step 1 Verify that oracle is in simplex mode and Hub is in split state on EMS Side A

<hostname># nodestat

- Verify ORACLE DB REPLICATION should be IN SIMPLEX SERVICE

- Verify OMSHub mate port status: No communication between EMS

- Verify OMSHub slave port status: should not contain Side B CA IP address

Note: If the above verification is not correct then follow following bullets, otherwise go to step 2

- On the EMS Side A place oracle in the simplex mode and split the Hub.

  - <hostname> su – oracle

  - <hostname>$ cd /opt/oracle/admin/utl

  - <hostname>$ rep_toggle -s optical1 -t set_simplex

  - <hostname> /opt/ems/utils/updMgr.sh -split_hub

- On the EMS Side A

  - <hostname> platform stop all

  - <hostname> platform start all

- Verify that the EMS Side A is in STANDBY state.

  - <hostname> btsstat

- Control Side A EMS to ACTIVE state.

- On EMS Side B execute the following commands.

  - <hostname> su - btsuser

  - CLI> control bdms id=BDMSxx; target-state=active-standby;

  - CLI> control element-manager id=EMyy; target-state=active-standby;

  - CLI> exit

Step 2 Verify that the Side A EMS and CA are ACTIVE and Side B EMS and CA are in OOS-FAULTY or STANDBY state. If side A EMS and CA are in STANDBY state, the following “platform stop all” command will switchover.

<hostname> btsstat

Step 3 Stop Side B EMS and CA platforms.  Issue the following command on Side B EMS and CA.

<hostname> platform stop all

Note: At this point, Side B system is getting prepared to boot from fallback release on disk 1.

Step 4 To boot from disk1 (bts10200_FALLBACK release), do the following commands

<hostname># eeprom boot-device=”disk1 disk0”

<hostname># shutdown –y –g0 –i6

Step 5 After logging in as root, execute following commands to verify system booted on disk1 (bts10200_FALLBACK release) and that the platform on the Secondary side is not started.

<hostname> nodestat

<hostname># lustatus   (Verification for Boot Environment)

Example Output

<hostname># lustatus

Boot Environment             Is       	Active 	Active    	Can    	Copy

Name                       	   Complete 	Now    On Reboot 	Delete 	Status

--------------------------     -------- 	------ 	---------		 ------ 	----------

d2                         	     yes      	no   	 no       	yes     	-

bts10200_FALLBACK   yes      	yes     	yes       	no   	 -

Step 6 On the EMS and CA Side B

<hostname> platform start all

Step 7 Verify that the Side A EMS and CA are ACTIVE and Side B EMS and CA are in STANDBY state.

<hostname> btsstat

Step 8 Restore hub on the Side A EMS.

<hostname>  /opt/ems/utils/updMgr.sh -restore_hub

Step 9 On Side A EMS set mode to Duplex

<hostname>  su - oracle

<hostname> $ cd /opt/oracle/admin/utl

<hostname> $ rep_toggle -s optical1 -t set_duplex

<hostname> $ exit

Step 10 Restart Side A EMS

<hostname> platform stop all

<hostname> platform start all

Step 11 Verify HUB and EMS communication restored on Side B EMS.

<hostname> nodestat

- Verify  HUB communication is restored.

- Verify OMS Hub mate port status: communication between EMS nodes is restored

Step 12 Control the Side A EMS to active state. Login to Side B EMS and execute following commands.

<hostname> su - btsuser

CLI> control bdms id=BDMSxx; target-state=active-standby;

CLI> control element-manager id=EMyy; target-state=active-standby;

Step 13 Verify call processing is working normally with new call completion.

Step 14 Perform an EMS database audit on Side A EMS and verify that there are no mismatch between side A EMS and Side B EMS.

<hostname> su - oracle

<hostname> dbadm -C db

<hostname> exit;

Note: If there are any mismatch errors found, please refer to Appendix I on correcting replication error section.

Step 15 Perform an EMS/CA database audit and verify that there are no mismatches.

<hostname> su - btsadmin

CLI>audit database type=full;

CLI> exit

The backup version is now fully restored and running on non-mirrored disk.

Step 16 Restore the /etc/rc3.d/S99platform feature for auto platform start on Side B nodes using the following commands.

<hostname> cd /etc/rc3.d

<hostname> mv _S99platform S99platform

Step 17 Verify that phone calls are processed correctly.

Note: At this point, Side B is running on disk 1 (bts10200_FALLBACK release) and Side A is running on disk 0. Also both systems Side A and Side B are running on non-mirrored disk. To get back to state prior to upgrade on Side B, execute following steps on Side B

Step 18 Prepare Side B (EMS & CA) for disk mirroring process by using following commands.

<hostname># metaclear –r d2

Example output

<hostname># metaclear -r d2

d2: Mirror is cleared

d0: Concat/Stripe is cleared

<hostname># metaclear –r d5

Example output

<hostname># metaclear -r d5

d5: Mirror is cleared

d3: Concat/Stripe is cleared

<hostname># metaclear –r d11

Example output

<hostname># metaclear -r d11

d11: Mirror is cleared

d9: Concat/Stripe is cleared

<hostname># metaclear –r d14

Example output

<hostname># metaclear -r d14

d14: Mirror is cleared

d12: Concat/Stripe is cleared

<hostname># metainit –f d0 1 1 c1t0d0s0

Example output

<hostname># metainit -f d0 1 1 c1t0d0s0

d0: Concat/Stripe is setup

<hostname># metainit –f d1 1 1 c1t1d0s0

Example output

<hostname># metainit -f d1 1 1 c1t1d0s0

d1: Concat/Stripe is setup

<hostname># metainit d2 –m d1

Example output

<hostname># metainit d2 -m d1

d2: Mirror is setup

<hostname># metaroot d2

Example output

<hostname># metaroot d2

<hostname># lockfs -fa

Example output

<hostname># lockfs -fa

<hostname># metainit –f d12 1 1 c1t0d0s6

Example output

<hostname># metainit -f d12 1 1 c1t0d0s6

d12: Concat/Stripe is setup

<hostname># metainit –f d13 1 1 c1t1d0s6

Example output

<hostname># metainit -f d13 1 1 c1t1d0s6

d13: Concat/Stripe is setup

<hostname># metainit d14 –m d13

Example output

<hostname># metainit d14 -m d13

d14: Mirror is setup

<hostname># metainit –f d3 1 1 c1t0d0s1

Example output

<hostname># metainit -f d3 1 1 c1t0d0s1

d3: Concat/Stripe is setup

<hostname># metainit –f d4 1 1 c1t1d0s1

Example output

<hostname># metainit -f d4 1 1 c1t1d0s1

d4: Concat/Stripe is setup

<hostname># metainit d5 –m d4

Example output

<hostname># metainit d5 -m d4

d5: Mirror is setup

<hostname># metainit –f d9 1 1 c1t0d0s5

Example output

<hostname># metainit -f d9 1 1 c1t0d0s5

d9: Concat/Stripe is setup

<hostname># metainit –f d10 1 1 c1t1d0s5

Example output

<hostname># metainit -f d10 1 1 c1t1d0s5

d10: Concat/Stripe is setup

<hostname># metainit d11 –m d10

Example output

<hostname># metainit d11 -m d10

d11: Mirror is setup

Step 19 Copy vfstab file by using following commands.

<hostname># cp /etc/vfstab /etc/vfstab.org.mirror.upgrade

<hostname># cp /opt/setup/vfstab_mirror /etc/vfstab

<hostname># dumpadm -d /dev/md/dsk/d8

Step 20 Reboot the system on Side B (EMS & CA)

<hostname># shutdown –y –g0 –i6

Step 21 After logging in as root, run following command to install boot block on disk 0.

<hostname>#  installboot /usr/platform/`uname -i`/lib/fs/ufs/bootblk /dev/rdsk/c1t0d0s0

Example Output

<hostname>#  installboot /usr/platform/`uname -i`/lib/fs/ufs/bootblk /dev/rdsk/c1t0d0s0

Step 22 Initiate disks mirroring from disk 1 to disk 0 by using following commands.

<hostname># metattach d2 d0

Example Output

<hostname># metattach d2 d0

d2: submirror d0 is attached

<hostname># metattach d14 d12

Example Output

<hostname>#  metattach d14 d12

d14: submirror d12 is attached

<hostname># metattach d11 d9

Example Output

<hostname>#  metattach d11 d9

d11: submirror d9 is attached

<hostname># metattach d5 d3

Example Output

<hostname>#  metattach d5 d3

d5: submirror d3 is attached

Step 23 Verify that disk mirroring process is in progress by using following command.

<hostname># metastat |grep %

Example Output

<hostname># metastat | grep %

Resync in progress: 0 % done

Resync in progress: 4 % done

Resync in progress: 6 % done

Resync in progress: 47 % done

Note: It will take about 2.5 hours to complete the disk mirroring process on each node. Following steps can be executed while disk mirroring is in progress.

Step 24 Execute following command on Side B to set the system to boot on disk0.

<hostname># eeprom boot-device=”disk0 disk1”

Step 25 Cleanup the boot environment database on Side B by using following command.

<hostname># \rm /etc/lutab

Example Output

<hostname># \rm /etc/lutab

Step 26 Verify that the boot environment on Side B is cleaned by using following command.

<hostname># lustatus

Example Output

<hostname># lustatus

ERROR: No boot environments are configured on this system

ERROR: cannot determine list of all boot environment names

Step 27 Verify that the platforms on the Side B EMS and CA have started and are in standby state.

<hostname># nodestat

Step 28 Verify that phone calls are processed correctly.

Appendix B

Full System Backout Procedure

CAUTION: This procedure is recommended only when full system upgrade has been completed and the system is experiencing unrecoverable problems for which the only solution is to take a full system service outage and restore the systems to the previous release as quickly as possible.

This procedure is used to restore the previous version of the release using a fallback release on disk 1.

The system must be in split mode so that the Side B EMS and CA can be reverted back to the previous release using the fallback release on disk 1.

Step 1 On the EMS Side A place oracle in the simplex mode and split the Hub.

<hostname> su – oracle

<hostname>$ cd /opt/oracle/admin/utl

<hostname>$ rep_toggle -s optical1 -t set_simplex

<hostname>$ exit

<hostname> /opt/ems/utils/updMgr.sh -split_hub

Step 2 On the EMS Side A

<hostname> platform stop all

<hostname> platform start all

Step 3 Verify that the EMS Side A is in STANDBY state.

<hostname> btsstat

Step 4 Control Side A EMS to ACTIVE state.

On EMS Side B execute the following commands.

<hostname> su - btsuser

CLI> control bdms id=BDMSxx; target-state=active-standby;

CLI> control element-manager id=EMyy; target-state=active-standby;

Step 5 Verify that the Side A EMS and CA are ACTIVE and Side B EMS and CA are in STANDBY state.

<hostname> btsstat

Step 6 Stop Side B EMS and CA platforms.  Issue the following command on Side B EMS and CA.

<hostname> platform stop all

Note: At this point, Side B system is getting prepared to boot from fallback release on disk 1.

Step 7 To boot from disk1 (bts10200_FALLBACK release) on Side B EMS & CA, do the following command.

<hostname># eeprom boot-device=”disk1 disk0”

<hostname># shutdown –y –g0 –i6

Step 8 After logging in as root, execute following commands to verify Side B system booted on disk 1 (bts10200_FALLBACK release) and that the platform on Secondary side is not started.

<hostname> nodestat

<hostname># lustatus   (Verification for Boot Environment)

Example Output

<hostname># lustatus

Boot Environment             Is       	Active 	Active    	Can    	Copy

Name                       	   Complete 	Now    On Reboot 	Delete 	Status

--------------------------     -------- 	------ 	---------		 ------ 	----------

d2                         	     yes      	no   	 no       	yes     	-

bts10200_FALLBACK   yes      	yes     	yes       	no   	 -

Step 9 Log into the Side B EMS as root

<hostname> /opt/ems/utils/updMgr.sh -split_hub

<hostname> platform start -i oracle

<hostname> su – oracle

<hostname>$ cd /opt/oracle/admin/utl

<hostname>$ rep_toggle -s optical2 -t set_simplex

<hostname>$ exit

The next steps will cause FULL system outage

Step 10 Stop Side A EMS and CA nodes.

Note: Wait for Side A EMS and CA nodes to stop completely before executing Step 11 below.

<hostname> platform stop all

Step 11 Start Side B EMS and CA nodes.

<hostname> platform start all

Step 12 Verify that Side B EMS and CA are ACTIVE on the “fallback release” and calls are being processed.

<hostname> btsstat

Note: At this point, Side A system is getting prepared to boot from fallback release on disk 1.

Step 13 To boot from disk1 (bts10200_FALLBACK release) on Side A EMS and CA, do the following command.

<hostname># eeprom boot-device=”disk1 disk0”

<hostname># shutdown –y –g0 –i6

Step 14 After logging in as root, execute following commands to verify Side A system booted on disk 1 (bts10200_FALLBACK release) and that the platform on Primary side is not started.

<hostname> nodestat

<hostname># lustatus   (Verification for Boot Environment)

Example Output

<hostname># lustatus

Boot Environment             Is       	Active 	Active    	Can    	Copy

Name                       	   Complete 	Now    On Reboot 	Delete 	Status

--------------------------     -------- 	------ 	---------		 ------ 	----------

d2                         	     yes      	no   	 no       	yes     	-

bts10200_FALLBACK   yes      	yes     	yes       	no   	 -

Step 15 Issue the platform start command to start up the Side A EMS and CA nodes.

<hostname> platform start all

Step 16 Verify that Side A EMS and CA platforms are in standby state.

<hostname>  btsstat

Step 17 Restore hub on Side B EMS.

<hostname>  /opt/ems/utils/updMgr.sh -restore_hub

Step 18 On Side B EMS set mode to Duplex

<hostname>  su - oracle

<hostname> $ cd /opt/oracle/admin/utl

<hostname> $ rep_toggle -s optical2 -t set_duplex

<hostname> $ exit

Step 19 Restart Side B EMS and CA

<hostname> platform stop all

<hostname> platform start all

Step 20 Verify that the Side A EMS and CA are in active state.

<hostname> nodestat

* Verify  HUB communication is restored.
* Verify OMS Hub mate port status: communication between EMS nodes is restored

Step 21 Verify call processing is working normally with new call completion.

Step 22 Perform an EMS database audit on Side A EMS and verify that there are no mismatch between side A EMS and Side B EMS.

<hostname> su - oracle

<hostname> dbadm -C db

<hostname> exit;

Step 23 Perform an EMS/CA database audit and verify that there are no mismatches.

<hostname> su - btsadmin

CLI>audit database type=full;

CLI> exit

The backup version is now fully restored and running on non-mirrored disk.

Step 24 Restore the /etc/rc3.d/S99platform feature for auto platform start on all four nodes using the following commands.

<hostname> cd /etc/rc3.d

<hostname> mv _S99platform S99platform

Step 25 Verify that phone calls are processed correctly.

Note: At this point, Side A and Side B are running on disk 1 (bts10200_FALLBACK release). Also both systems Side A and Side B are running on non-mirrored disk. To get back to state prior to upgrade on Side A and Side B, execute following steps on Side A and Side B.

Step 26 Prepare Side A & Side B (EMS & CA) for disk mirroring process by using following commands.

<hostname># metaclear –r d2

Example output

<hostname># metaclear -r d2

d2: Mirror is cleared

d0: Concat/Stripe is cleared

<hostname># metaclear –r d5

Example output

<hostname># metaclear -r d5

d5: Mirror is cleared

d3: Concat/Stripe is cleared

<hostname># metaclear –r d11

Example output

<hostname># metaclear -r d11

d11: Mirror is cleared

d9: Concat/Stripe is cleared

<hostname># metaclear –r d14

Example output

<hostname># metaclear -r d14

d14: Mirror is cleared

d12: Concat/Stripe is cleared

<hostname># metainit –f d0 1 1 c1t0d0s0

Example output

<hostname># metainit -f d0 1 1 c1t0d0s0

d0: Concat/Stripe is setup

<hostname># metainit –f d1 1 1 c1t1d0s0

Example output

<hostname># metainit -f d1 1 1 c1t1d0s0

d1: Concat/Stripe is setup

<hostname># metainit d2 –m d1

Example output

<hostname># metainit d2 -m d1

d2: Mirror is setup

<hostname># metaroot d2

Example output

<hostname># metaroot d2

<hostname># lockfs -fa

Example output

<hostname># lockfs -fa

<hostname># metainit –f d12 1 1 c1t0d0s6

Example output

<hostname># metainit -f d12 1 1 c1t0d0s6

d12: Concat/Stripe is setup

<hostname># metainit –f d13 1 1 c1t1d0s6

Example output

<hostname># metainit -f d13 1 1 c1t1d0s6

d13: Concat/Stripe is setup

<hostname># metainit d14 –m d13

Example output

<hostname># metainit d14 -m d13

d14: Mirror is setup

<hostname># metainit –f d3 1 1 c1t0d0s1

Example output

<hostname># metainit -f d3 1 1 c1t0d0s1

d3: Concat/Stripe is setup

<hostname># metainit –f d4 1 1 c1t1d0s1

Example output

<hostname># metainit -f d4 1 1 c1t1d0s1

d4: Concat/Stripe is setup

<hostname># metainit d5 –m d4

Example output

<hostname># metainit d5 -m d4

d5: Mirror is setup

<hostname># metainit –f d9 1 1 c1t0d0s5

Example output

<hostname># metainit -f d9 1 1 c1t0d0s5

d9: Concat/Stripe is setup

<hostname># metainit –f d10 1 1 c1t1d0s5

Example output

<hostname># metainit -f d10 1 1 c1t1d0s5

d10: Concat/Stripe is setup

<hostname># metainit d11 –m d10

Example output

<hostname># metainit d11 -m d10

d11: Mirror is setup

Step 27 Copy vfstab file on all four nodes by using following commands.

<hostname># cp /etc/vfstab /etc/vfstab.org.mirror.upgrade

<hostname># cp /opt/setup/vfstab_mirror /etc/vfstab

<hostname># dumpadm -d /dev/md/dsk/d8

Step 28 Reboot the Side A (EMS & CA) system first.

<hostname># shutdown –y –g0 –i6

Step 29 After logging in as root on Side A, run following command to install boot block on disk 0.

<hostname>#  installboot /usr/platform/`uname -i`/lib/fs/ufs/bootblk /dev/rdsk/c1t0d0s0

Example Output

<hostname>#  installboot /usr/platform/`uname -i`/lib/fs/ufs/bootblk /dev/rdsk/c1t0d0s0

Step 30 Reboot the Side B (EMS & CA) system.

<hostname># shutdown –y –g0 –i6

Step 31 After logging in as root on Side B, run following command to install boot block on disk 0.

<hostname>#  installboot /usr/platform/`uname -i`/lib/fs/ufs/bootblk /dev/rdsk/c1t0d0s0

Example Output

<hostname>#  installboot /usr/platform/`uname -i`/lib/fs/ufs/bootblk /dev/rdsk/c1t0d0s0

Step 32 Initiate disks mirroring from disk 1 to disk 0 on all four nodes by using following commands.

<hostname># metattach d2 d0

Example Output

<hostname># metattach d2 d0

d2: submirror d0 is attached

<hostname># metattach d14 d12

Example Output

<hostname>#  metattach d14 d12

d14: submirror d12 is attached

<hostname># metattach d11 d9

Example Output

<hostname>#  metattach d11 d9

d11: submirror d9 is attached

<hostname># metattach d5 d3

Example Output

<hostname>#  metattach d5 d3

d5: submirror d3 is attached

Step 33 Verify that disk mirroring process is in progress on all four nodes by using following command.

<hostname># metastat |grep %

Example Output

<hostname># metastat | grep %

Resync in progress: 0 % done

Resync in progress: 4 % done

Resync in progress: 6 % done

Resync in progress: 47 % done

Note: It will take about 2.5 hours to complete the disk mirroring process on each node. Following steps can be executed while disk mirroring is in progress.

Step 34 Execute following command on all four nodes to set the system to boot on disk0.

<hostname># eeprom boot-device=”disk0 disk1”

Step 35 Cleanup the boot environment database on all four nodes by using following command.

<hostname># \rm /etc/lutab

Example Output

<hostname># \rm /etc/lutab

Step 36 Verify that the boot environment is cleaned on all four nodes by using following command.

<hostname># lustatus

Example Output

<hostname># lustatus

ERROR: No boot environments are configured on this system

ERROR: cannot determine list of all boot environment names

Step 37 Verify that Side A (EMS & CA) is in Active state and Side B (EMS & CA) is in Standby state.

<hostname># btsstat

Step 38 Verify that phone calls are processed correctly.

This completes the entire system fallback

Appendix C

CORBA Installation

This procedure describes how to install the OpenORB Common Object Request Broker Architecture (CORBA) application on Element Management System (EMS) of the Cisco BTS 10200 Softswitch.

Note This installation process is to be used for both EMS side A and EMS side B.

Caution This CORBA installation will remove existing CORBA application on EMS machines. Once you have executed this procedure, there is no backout. Do not start this procedure until you have proper authorization. If you have questions, contact Cisco TAC.

# Task 1: Install OpenORB CORBA Application

### Remove Installed OpenORB Application

Step 1   Log in as root to EMS.

Step 2   Remove the OpenORB CORBA packages if they are installed, other wise go to next step.

<hostname># pkginfo | grep BTScis

- If the output of the above command indicates that BTScis package is installed, then follow the next step to remove the BTScis package.

<hostname># pkgrm BTScis

  - Answer “y” when prompted

<hostname># pkginfo | grep BTSoorb

- If the output of the above command indicates that BTSoorb package is installed, then follow the next step to remove the BTSoorb package.

<hostname># pkgrm BTSoorb

  - Answer “y” when prompted

Step 3   Enter the following command to verify that the CORBA application is removed:

<hostname># pgrep cis3

The system will respond by displaying no data, or by displaying an error message. This verifies that the CORBA application is removed.

### Install OpenORB Packages

The CORBA application files are available for installation once the Cisco BTS 10200 Softswitch is installed.

Step 1   Log in as root to EMS

Step 2   <hostname># cd /opt/Build

Step 3   <hostname># ./cis-install.sh

System responds:

  - Answer “y” when prompt

Step 5   It will take about 2-4 minutes for the installation to complete.

Step 6   Verify CORBA Application is running On EMS:

<hostname># init q

<hostname># pgrep ins3

Note System will respond by displaying the Name Service process ID, which is a number between 2 and 32,000 assigned by the system during CORBA installation. By displaying this ID, the system confirms that the ins3 process was found and is running.

<hostname># pgrep cis3

Note The system will respond by displaying the cis3 process ID, which is a number between 2 and 32,000 assigned by the system during CORBA installation. By displaying this ID, the system confirms that the cis3 process was found and is running.

Step 7   If you do not receive both of the responses described in Step 6, or if you experience any verification problems, do not continue. Contact your system administrator. If necessary, call Cisco TAC for additional technical assistance.

Appendix D

Full System Successful Upgrade Procedure

Note: This procedure is recommended only when full system upgrade has been completed successfully and the system is not experiencing any issues.

This procedure is used to initiate the disk mirroring from disk 0 to disk 1, once Side A and Side B have been successfully upgraded. It will take about 2.5 hours on each side to complete the disk mirroring process.

The system must be in split mode and both Side A and Side B (EMS and CA) have been upgraded successfully on disk 0, with disk 1 remains as fallback release. Both Side A and Side B (EMS and CA) disk 1 can be mirrored to disk0, so that both disks will have the upgrade release.

Step 1 Initiate disks mirroring from disk 0 to disk 1 on all four nodes by using following commands.

<hostname># metainit d1 1 1 c1t1d0s0

Example Output

<hostname># metainit d1 1 1 c1t1d0s0

d1: Concat/Stripe is setup

<hostname># metainit d4 1 1 c1t1d0s1

Example Output

<hostname># metainit d4 1 1 c1t1d0s1

d4: Concat/Stripe is setup

<hostname># metainit d10 1 1 c1t1d0s5

Example Output

<hostname># metainit d10 1 1 c1t1d0s5

d10: Concat/Stripe is setup

<hostname># metainit d13 1 1 c1t1d0s6

Example Output

<hostname># metainit d13 1 1 c1t1d0s6

d13: Concat/Stripe is setup

<hostname># metattach d2 d1

Example Output

<hostname># metattach d2 d1

d2: submirror d1 is attached

<hostname># metattach d14 d13

Example Output

<hostname>#  metattach d14 d13

d14: submirror d13 is attached

<hostname># metattach d11 d10

Example Output

<hostname>#  metattach d11 d10

d11: submirror d10 is attached

<hostname># metattach d5 d4

Example Output

<hostname>#  metattach d5 d4

d5: submirror d4 is attached

Step 2 Verify that disk mirroring process is in progress on all four nodes by using following command.

<hostname># metastat |grep %

Example Output

<hostname># metastat | grep %

Resync in progress: 0 % done

Resync in progress: 4 % done

Resync in progress: 6 % done

Resync in progress: 47 % done

Note: It will take about 2.5 hours to complete the disk mirroring process on each node. Following steps can be executed while disk mirroring is in progress.

Step 3 Execute following command on all four nodes to set the system to boot on disk0.

<hostname># eeprom boot-device=”disk0 disk1”

Step 4 Cleanup the boot environment database on all four nodes by using following command.

<hostname># \rm /etc/lutab

Example Output

<hostname># \rm /etc/lutab

Step 5 Verify that the boot environment is cleaned on all four nodes by using following command.

<hostname># lustatus

Example Output

<hostname># lustatus

ERROR: No boot environments are configured on this system

ERROR: cannot determine list of all boot environment names

Step 6 Verify that Side A (EMS & CA) is in Active state and Side B (EMS & CA) is in Standby state.

<hostname># btsstat

Step 7 Verify that phone calls are processed correctly.

Appendix E

Emergency Fallback Procedure Using the Backup Disks

This procedure should be used to restore service as quickly as possible in the event that there is a need to abandon the upgrade version due to call processing failure.

This procedure will be used when there is either no successful call processing, or the upgrade performance is so degraded that it is not possible to continue operations with the upgrade release.

Step 1 To boot on disk 1 (bts10200_FALLBACK release), execute following commands on all four nodes.

<hostname># eeprom boot-device=”disk1 disk0”

<hostname># shutdown –y –g0 –i6

Step 2 After logging in as root, execute following commands to verify system booted on disk1 (bts10200_FALLBACK release) and that the platforms on Side A & Side B are not started.

<hostname> nodestat

<hostname># lustatus   (Verification for Boot Environment)

Example Output

<hostname># lustatus

Boot Environment             Is       	Active 	Active    	Can    	Copy

Name                       	   Complete 	Now    On Reboot 	Delete 	Status

--------------------------     -------- 	------ 	---------		 ------ 	----------

d2                         	     yes      	no   	 no       	yes     	-

bts10200_FALLBACK   yes      	yes     	yes       	no   	 -

Step 3 Start platform on all four nodes

<hostname># platform start all

Step 4 	 Verify that the Side A EMS and CA node platforms and hub are in active mode and that the Side B EMS and CA nodes are in standby mode.

<hostname># nodestat

Step 5 	Enable platform auto start at boot-up with the following commands on all four nodes.

<hostname>#  cd /etc/rc3.d

<hostname>#  mv _S99platform S99platform

Step 6 Verify that phone calls are processed correctly.

Note: At this point, Side A and Side B are running on disk 1 (bts10200_FALLBACK release). Also both systems Side A and Side B are running on non-mirrored disk. To get back to state prior to upgrade on Side A and Side B, execute following steps on Side A and Side B.

Step 7 Prepare Side A & Side B (EMS & CA) for disk mirroring process by using following commands.

<hostname># metaclear –r d2

Example output

<hostname># metaclear -r d2

d2: Mirror is cleared

d0: Concat/Stripe is cleared

<hostname># metaclear –r d5

Example output

<hostname># metaclear -r d5

d5: Mirror is cleared

d3: Concat/Stripe is cleared

<hostname># metaclear –r d11

Example output

<hostname># metaclear -r d11

d11: Mirror is cleared

d9: Concat/Stripe is cleared

<hostname># metaclear –r d14

Example output

<hostname># metaclear -r d14

d14: Mirror is cleared

d12: Concat/Stripe is cleared

<hostname># metainit –f d0 1 1 c1t0d0s0

Example output

<hostname># metainit -f d0 1 1 c1t0d0s0

d0: Concat/Stripe is setup

<hostname># metainit –f d1 1 1 c1t1d0s0

Example output

<hostname># metainit -f d1 1 1 c1t1d0s0

d1: Concat/Stripe is setup

<hostname># metainit d2 –m d1

Example output

<hostname># metainit d2 -m d1

d2: Mirror is setup

<hostname># metaroot d2

Example output

<hostname># metaroot d2

<hostname># lockfs -fa

Example output

<hostname># lockfs -fa

<hostname># metainit –f d12 1 1 c1t0d0s6

Example output

<hostname># metainit -f d12 1 1 c1t0d0s6

d12: Concat/Stripe is setup

<hostname># metainit –f d13 1 1 c1t1d0s6

Example output

<hostname># metainit -f d13 1 1 c1t1d0s6

d13: Concat/Stripe is setup

<hostname># metainit d14 –m d13

Example output

<hostname># metainit d14 -m d13

d14: Mirror is setup

<hostname># metainit –f d3 1 1 c1t0d0s1

Example output

<hostname># metainit -f d3 1 1 c1t0d0s1

d3: Concat/Stripe is setup

<hostname># metainit –f d4 1 1 c1t1d0s1

Example output

<hostname># metainit -f d4 1 1 c1t1d0s1

d4: Concat/Stripe is setup

<hostname># metainit d5 –m d4

Example output

<hostname># metainit d5 -m d4

d5: Mirror is setup

<hostname># metainit –f d9 1 1 c1t0d0s5

Example output

<hostname># metainit -f d9 1 1 c1t0d0s5

d9: Concat/Stripe is setup

<hostname># metainit –f d10 1 1 c1t1d0s5

Example output

<hostname># metainit -f d10 1 1 c1t1d0s5

d10: Concat/Stripe is setup

<hostname># metainit d11 –m d10

Example output

<hostname># metainit d11 -m d10

d11: Mirror is setup

Step 8 Copy vfstab file on all four nodes by using following commands.

<hostname># cp /etc/vfstab /etc/vfstab.org.mirror.upgrade

<hostname># cp /opt/setup/vfstab_mirror /etc/vfstab

<hostname># dumpadm -d /dev/md/dsk/d8

Step 9 Reboot the Side A (EMS & CA) system first.

<hostname># shutdown –y –g0 –i6

Step 10 After logging in as root on Side A, run following command to install boot block on disk 0.

<hostname>#  installboot /usr/platform/`uname -i`/lib/fs/ufs/bootblk /dev/rdsk/c1t0d0s0

Example Output

<hostname>#  installboot /usr/platform/`uname -i`/lib/fs/ufs/bootblk /dev/rdsk/c1t0d0s0

Step 11 Reboot the Side B (EMS & CA) system.

<hostname># shutdown –y –g0 –i6

Step 12 After logging in as root on Side B, run following command to install boot block on disk 0.

<hostname>#  installboot /usr/platform/`uname -i`/lib/fs/ufs/bootblk /dev/rdsk/c1t0d0s0

Example Output

<hostname>#  installboot /usr/platform/`uname -i`/lib/fs/ufs/bootblk /dev/rdsk/c1t0d0s0

Step 13 Initiate disks mirroring from disk 1 to disk 0 on all four nodes by using following commands.

<hostname># metattach d2 d0

Example Output

<hostname># metattach d2 d0

d2: submirror d0 is attached

<hostname># metattach d14 d12

Example Output

<hostname>#  metattach d14 d12

d14: submirror d12 is attached

<hostname># metattach d11 d9

Example Output

<hostname>#  metattach d11 d9

d11: submirror d9 is attached

<hostname># metattach d5 d3

Example Output

<hostname>#  metattach d5 d3

d5: submirror d3 is attached

Step 14 Verify that disk mirroring process is in progress on all four nodes by using following command.

<hostname># metastat |grep %

Example Output

<hostname># metastat | grep %

Resync in progress: 0 % done

Resync in progress: 4 % done

Resync in progress: 6 % done

Resync in progress: 47 % done

Note: It will take about 2.5 hours to complete the disk mirroring process on each node. Following steps can be executed while disk mirroring is in progress.

Step 15 Execute following command on all four nodes to set the system to boot on disk0.

<hostname># eeprom boot-device=”disk0 disk1”

Step 16 Cleanup the boot environment database on all four nodes by using following command.

<hostname># \rm /etc/lutab

Example Output

<hostname># \rm /etc/lutab

Step 17 Verify that the boot environment is cleaned on all four nodes by using following command.

<hostname># lustatus

Example Output

<hostname># lustatus

ERROR: No boot environments are configured on this system

ERROR: cannot determine list of all boot environment names

Step 18 Verify that Side A (EMS & CA) is in Active state and Side B (EMS & CA) is in Standby state.

<hostname># btsstat

Step 19 Verify that phone calls are processed correctly.

Appendix F

Staging the 4.5.1 load to the system

This procedure describes how to stage the 4.5.1 load to the system using CD-ROM.

From EMS Side B

Step 1   Log in as root.

Step 2   Put BTS 10200 Application Disk CD-ROM in the CD-ROM drive.

Step 3   Remove old files.

<hostname># cd /

<hostname># \rm –rf /opt/Build

Step 4   Create /cdrom directory and mount the directory.

<hostname># mkdir -p /cdrom

- A system with Continuous Computing hardware, please run:

<hostname># mount -o ro -F hsfs /dev/dsk/c0t6d0s0 /cdrom

- Other hardware platform, please run:

<hostname># mount -o ro -F hsfs /dev/dsk/c0t0d0s0 /cdrom

Step 5   Use the following commands to copy file from the CD-ROM to the /opt directory.

<hostname># cp –f /cdrom/K9-opticall.tar.gz /opt

Step 6   Verify that the check sum values match with the values located in the “checksum.txt” file located on Application CD-ROM.

<hostname># cat /cdrom/checksum.txt

<hostname># cksum /opt/K9-opticall.tar.gz

Step 7   Unmount the CD-ROM.

<hostname># umount /cdrom

Step 8   Manually eject the CD-ROM and take out BTS 10200 Application Disk CD-ROM from CD-ROM drive.

Step 9   Put BTS 10200 Database Disk CD-ROM in the CD-ROM drive of EMS Side B.

Step 10   Mount the /cdrom directory.

- A system with Continuous Computing hardware, please run:

<hostname># mount -o ro -F hsfs /dev/dsk/c0t6d0s0 /cdrom

- Other hardware platform, please run:

<hostname># mount -o ro -F hsfs /dev/dsk/c0t0d0s0 /cdrom

Step 11   Use the following commands to copy file from the CD-ROM to the /opt directory.

<hostname># cp –f /cdrom/K9-btsdb.tar.gz /opt

<hostname># cp –f /cdrom/K9-extora.tar.gz /opt

Step 12   Verify that the check sum values match with the values located in the “checksum.txt” file located on BTS 10200 Database Disk CD-ROM.

<hostname># cat /cdrom/checksum.txt

<hostname># cksum /opt/K9-btsdb.tar.gz

<hostname># cksum /opt/K9-extora.tar.gz

Step 13   Unmount the CD-ROM.

<hostname># umount /cdrom

Step 14   Manually eject the CD-ROM and take out BTS 10200 Database Disk CD-ROM from CD-ROM drive.

Step 15   Put BTS 10200 Oracle Engine Disk CD-ROM in the CD-ROM drive of EMS Side B.

Step 16   Mount the /cdrom directory.

- A system with Continuous Computing hardware, please run:

<hostname># mount -o ro -F hsfs /dev/dsk/c0t6d0s0 /cdrom

- Other hardware platform, please run:

<hostname># mount -o ro -F hsfs /dev/dsk/c0t0d0s0 /cdrom

Step 17   Use the following commands to copy file from the CD-ROM to the /opt directory.

<hostname># cp –f /cdrom/K9-oraengine.tar.gz /opt

Step 18   Verify that the check sum values match with the values located in the “checksum.txt” file located on Oracle Engine CD-ROM.

<hostname># cat /cdrom/checksum.txt

<hostname># cksum /opt/K9-oraengine.tar.gz

Step 19   Unmount the CD-ROM.

<hostname># umount /cdrom

Step 20   Manually eject the CD-ROM and take out BTS 10200 Oracle Engine Disk CD-ROM from CD-ROM drive.

Step 21   Extract tar files.

<hostname># cd /opt

<hostname># gzip -cd K9-opticall.tar.gz | tar -xvf -

<hostname># gzip -cd K9-btsdb.tar.gz | tar -xvf -

<hostname># gzip -cd K9-extora.tar.gz | tar -xvf -

<hostname># gzip -cd K9-oraengine.tar.gz | tar -xvf -

|  | Note The files will take up 30 minutes to extract. |
| --- | --- |

# From EMS Side A

Step 1   <hostname># cd /opt

Step 2   <hostname># \rm –rf /opt/Build

Step 3   <hostname># sftp <hostname of EMS Side B>

Step 4   sftp> cd /opt

Step 5   sftp> get K9-opticall.tar.gz

Step 6   sftp> get K9-btsdb.tar.gz

Step 7   sftp> get K9-extora.tar.gz

Step 8   sftp> get K9-oraengine.tar.gz

Step 9   sftp> exit

Step 10   <hostname># gzip -cd K9-opticall.tar.gz | tar -xvf -

Step 11   <hostname># gzip -cd K9-btsdb.tar.gz | tar -xvf -

Step 12   <hostname># gzip -cd K9-extora.tar.gz | tar -xvf -

Step 13   <hostname># gzip -cd K9-oraengine.tar.gz | tar -xvf -

|  | Note The files will take up 30 minutes to extract. |
| --- | --- |

From CA/FS Side A

Step 1   <hostname># cd /opt

Step 2   <hostname># \rm –rf /opt/Build

Step 3   <hostname># sftp <hostname of EMS Side B>

Step 4   sftp> cd /opt

Step 5   sftp> get K9-opticall.tar.gz

Step 6   sftp> exit

Step 7   <hostname># gzip -cd K9-opticall.tar.gz | tar -xvf -

|  | Note The files will take up 5-10 minutes to extract. |
| --- | --- |

# From CA/FS Side B

Step 1   <hostname># cd /opt

Step 2   <hostname># \rm –rf /opt/Build

Step 3   <hostname># sftp <hostname of EMS side B>

Step 4   sftp> cd /opt

Step 5   sftp> get K9-opticall.tar.gz

Step 6   sftp> exit

Step 7   <hostname># gzip -cd K9-opticall.tar.gz | tar -xvf -

|  | Note The files will take up 5-10 minutes to extract. |
| --- | --- |

Appendix G

Check database

This procedure describes how to perform database audit and correct database mismatch as a result of the DB audit.

# Perform database audit

In this task, you will perform a full database audit and correct any errors, if necessary.  The results of the audit can be found on the active EMS via the following Web location. For example ….https://priems108/report/

Step 1 Login as “ciscouser”

Step 2   CLI> audit database type=full;

Step 3   Check the audit report and verify there is no discrepancy or error. If errors are found, please try to correct them. If you are unable to correct, please contact Cisco Support.

Please follow the sample command provided below to correct the mismatches:

CLI> sync <table name> master=EMS; target=<CAxxx/FSPTCyyy/FSAINzzz>;

CLI> audit <table name>

Step 4   CLI> exit

Use the following command to clear data base mismatches for the following tables.

- SLE

- SC1D

- SC2D

- SUBSCRIBER-FEATURE-DATA

Step 1 CLI> sync <table name> master=FSPTC; target=<EMS>;

Step 2 CLI> audit <table name>

Step 3 CLI> exit

Appendix H

Check Alarm Status

The purpose of this procedure is to verify that there are no outstanding major/critical alarms.

# From EMS side A

Step 1   Log in as “btsuser” user.

Step 2   CLI> show alarm

- The system responds with all current alarms, which must be verified or cleared before proceeding with next step.

|  | Tip Use the following command information for reference material ONLY. |
| --- | --- |

Step 3   To monitor system alarm continuously.

CLI> subscribe alarm-report severity=all; type=all;

|  | Valid severity: MINOR, MAJOR, CRITICAL, ALL Valid types: CALLP, CONFIG, DATABASE, MAINTENANCE, OSS, SECURITY, SIGNALING, STATISTICS, BILLING, ALL, SYSTEM, AUDIT |
| --- | --- |

Step 4   System will display alarms if alarm is reported.

<------------------- START ALARM REPORT --------------------> TIMESTAMP: 20040503174759 DESCRIPTION: General MGCP Signaling Error between MGW and CA. TYPE & NUMBER: SIGNALING (79) SEVERITY: MAJOR ALARM-STATUS: OFF ORIGIN: MGA.PRIMARY.CA146 COMPONENT-ID: null ENTITY NAME: S0/DS1-0/1@64.101.150.181:5555 GENERAL CONTEXT: MGW_TGW SPECIFC CONTEXT: NA FAILURE CONTEXT: NA <------------------- END ALARM REPORT ---------------------->

Step 5   To stop monitoring system alarm.

CLI> unsubscribe alarm-report severity=all; type=all;

Step 6   CLI> exit

Appendix I

Audit Oracle Database and Replication

Perform the following steps on the Standby EMS side to check the Oracle database and replication status.

# Check Oracle DB replication status

# From STANDBY EMS

Step 1   Log in as root.

Step 2   Log in as oracle.

<hostname># su – oracle

Step 3   Enter the command to compare contents of tables on the side A and side B EMS databases:

Note: This may take 5-20 minutes time, depending on the size of the database.

<hostname>$ dbadm –C db

Step 4 Please check following two possible return results:

A) If all tables are in sync, output will be as follows:

Number of tables to be checked:  234

Number of tables checked OK:    234

Number of tables out-of-sync:      0

Step 5 If the tables are in sync as above, then Continue on Step 7 and skip Step 6.

B) If tables are out of sync, output will be as follows:

Number of tables to be checked: 157
Number of tables checked OK:    154
Number of tables out-of-sync:   3

Below is a list of out-of-sync tables:

OAMP.SECURITYLEVELS   				=> 1/0

OPTICALL.SUBSCRIBER_FEATURE_DATA		=> 1/2
OPTICALL.MGW                   				=> 2/2

Step 6 If the tables are out of sync as above, then Continue on Step C to sync the tables.

- For each table that is out of sync, please run the following step:

Note: Execute below “dbadm –A copy” command from the EMS side that has *BAD* data.

<hostname>$ dbadm -A copy -o <OWNER> -t <TABEL NAME>

Example:    dbadm –A copy –o opticall –t subscriber_feature_data

- Enter “y” to continue

- Please contact Cisco Support if the above command fails.

Step 7   Enter the command to check replication status:

<hostname>$ dbadm –C rep

Verify that “Deferror is empty?” is “YES”.

OPTICAL2::Deftrandest is empty?           YES

OPTICAL2::dba_repcatlog is empty?       YES

OPTICAL2::Deferror is empty?              YES Make sure it is “YES”

OPTICAL2::Deftran is empty?                  YES

OPTICAL2::Has no broken job?               YES

OPTICAL2::JQ Lock is empty?                YES

OPTICAL1::Deftrandest is empty?          YES

OPTICAL1::dba_repcatlog is empty?      YES

OPTICAL1::Deferror is empty?             YES Make sure it is “YES”

OPTICAL1::Deftran is empty?                YES

OPTICAL1::Has no broken job?              YES

OPTICAL1::JQ Lock is empty?               YES

If the “Deferror is empty?” is “NO”, please try to correct the error using steps in “Correct replication error” below. If you are unable to clear the error or if any of the individual steps fails, please contact Cisco Support. If the “Deferror is empty?” is “YES”, then proceed to step 8.

Step 8 <hostname> $ exit

# Correct replication error

|  | Note You must run the following steps on standby EMS side B first, then on active EMS side A. |
| --- | --- |

# From EMS Side B

Step 1   Log in as root

Step 2   <hostname># su – oracle

Step 5   <hostname>$ dbadm –A truncate_deferror

- Enter “y” to continue

Step 6 <hostname> $ exit

# From EMS Side A

Step 1   Login in as root.

Step 2   <hostname> # su – oracle

Step 3   <hostname>$ dbadm –A truncate_deferror

- Enter “y” to continue

Step 4   Re-verify that “Deferror is empty?” is “YES” and none of tables is out of sync.

<hostname>$dbadm –C rep

OPTICAL1::Deftrandest is empty?           YES

OPTICAL1::dba_repcatlog is empty?       YES

OPTICAL1::Deferror is empty?              YES  Make sure it is “YES”

OPTICAL1::Deftran is empty?                  YES

OPTICAL1::Has no broken job?               YES

OPTICAL1::JQ Lock is empty?                YES

OPTICAL2::Deftrandest is empty?          YES

OPTICAL2::dba_repcatlog is empty?      YES

OPTICAL2::Deferror is empty?             YES  Make sure it is “YES”

OPTICAL2::Deftran is empty?                YES

OPTICAL2::Has no broken job?              YES

OPTICAL2::JQ Lock is empty?               YES

Step 5   <hostname> # exit

Appendix J

Caveats and solutions

- Internal Oracle Error (ORA-00600) during DataBase Copy

Symptom: The upgrade script may exit with the following error during DataBase copy.

<Mon Jan 29 17:08:22 EST 2007> ERROR: Fail to restore Referential Constraints

==========================================================

ERROR: Database copy failed

==========================================================

secems02# echo $?

1

secems02# ************************************************************

Error: secems02: failed to start platform

Work around:

Login to the EMS platform on which this issue was encountered and issue the following commands

  - su – oracle

  - optical1:priems02: /opt/orahome$ sqlplus / as sysdba

SQL*Plus: Release 10.1.0.4.0 - Production on Tue Jan 30 19:40:56 2007

Copyright (c) 1982, 2005, Oracle. All rights reserved.

Connected to: Oracle Database 10g Enterprise Edition Release 10.1.0.4.0 - 64bit Production With the Partitioning and Data Mining options

  - SQL> shutdown immediate
ORA-00600: internal error code, arguments: [2141], [2642672802], [2637346301], [], [], [], [], []

  - SQL> shutdown abort
ORACLE instance shut down.

- SQL> startup 
ORACLE instance started.

Total System Global Area 289406976 bytes
Fixed Size 1302088 bytes
Variable Size 182198712 bytes
Database Buffers 104857600 bytes
Redo Buffers 1048576 bytes
Database mounted.
Database opened.

SQL> exit

Appendix K

Creation Of Backup Disks

The following instructions split the mirror between the disk set and create two identical and bootable drives on each of the platforms.

Before continuing with the following procedure, the procedure in Appendix L “Mirroring the disks” must be executed to mirror the disk 0 and disk 1.

It is possible that the mirror process for a node may have been previously started but may not have completed properly. Refer to Appendix M “Verifying the disk mirror” to verify if the mirror process was completed properly.

Caution: If the mirror process was not completed properly the creation of backup disks procedure will not work and the disks will be left in an indeterminate state.

# Task 1: Creating a Bootable Backup Disk

The following steps can be executed in parallel on both the CA and EMS nodes.

Note: This procedure has to be executed on Side B EMS and CA nodes while side A is active and processing calls.  Subsequently, it has to be executed on Side A EMS and CA nodes.

Step 1   Shutdown the platform on the EMS and CA nodes.

<hostname># platform stop all

Step 2   Verify that the application is not running.

<hostname># nodestat

Step 3 Rename the startup files on the EMS and CA nodes to prevent the platform from starting up after a reboot

<hostname># cd /etc/rc3.d

<hostname># mv S99platform _S99platform

Step 4 Break the mirror from disk 1 by using following commands.

<hostname># metadetach d2 d1	====== / (root) partition

Example output

<hostname># metadetach d2 d1

d2: submirror d1 is detached

<hostname># metadetach d14 d13    ====== reserved partition

Example output

<hostname># metadetach d14 d13

d14: submirror d13 is detached

<hostname># metadetach d11 d10     ====== /opt partition

Example output

<hostname>#  metadetach d11 d10

d11: submirror d10 is detached

<hostname># metadetach d5 d4        ====== /var partition

Example output

<hostname>#  metadetach d5 d4

d5: submirror d4 is detached

Step 5 Perform the following commands to clear submirror metadevices.

<hostname># metaclear d1

Example output

<hostname># metaclear d1

d1: Concat/Stripe is cleared

<hostname># metaclear d13

Example output

<hostname># metaclear d13

d13: Concat/Stripe is cleared

<hostname># metaclear d10

Example output

<hostname>#      metaclear d10

d10: Concat/Stripe is cleared

<hostname># metaclear d4

Example output

<hostname>#metaclear d4

d4: Concat/Stripe is cleared

Step 6 Verify that the system has following metastat devices after the split.

<hostname># metastat –p

Note: The output should be similar as below

Example Output

<hostname># metastat –p

d5 -m d3 1

d3 1 1 c1t0d0s1

d11 -m d9 1

d9 1 1 c1t0d0s5

d14 -m d12 1

d12 1 1 c1t0d0s6

d2 -m d0 1

d0 1 1 c1t0d0s0

d8 -m d6 1

d6 1 1 c1t0d0s3

d7 1 1 c1t1d0s3

Step 7 Create new Alternate Boot Environment for Fallback purpose by using following command.

<hostname>#  lucreate -C /dev/dsk/c1t0d0s0 -m /:/dev/dsk/c1t1d0s0:ufs -m /var:/dev/dsk/c1t1d0s1:ufs -m /opt:/dev/dsk/c1t1d0s5:ufs  -n bts10200_FALLBACK

Note: It will take about 10-12 minutes to complete the above command successfully.

Example Output

<hostname>#  lucreate -C /dev/dsk/c1t0d0s0 -m /:/dev/dsk/c1t1d0s0:ufs -m /var:/dev/dsk/c1t1d0s1:ufs -m /opt:/dev/dsk/c1t1d0s5:ufs  -n bts10200_FALLBACK

Discovering physical storage devices

Discovering logical storage devices

Cross referencing storage devices with boot environment configurations

Determining types of file systems supported

Validating file system requests

Preparing logical storage devices

Preparing physical storage devices

Configuring physical storage devices

Configuring logical storage devices

Analyzing system configuration.

Comparing source boot environment <d2> file systems with the file

system(s) you specified for the new boot environment. Determining which

file systems should be in the new boot environment.

Updating boot environment description database on all BEs.

Searching /dev for possible boot environment filesystem devices

Updating system configuration files.

Creating configuration for boot environment <bts10200_FALLBACK>.

Creating boot environment <bts10200_FALLBACK>.

Creating file systems on boot environment <bts10200_FALLBACK>.

Creating <ufs> file system for </> on </dev/dsk/c1t1d0s0>.

Creating <ufs> file system for </opt> on </dev/dsk/c1t1d0s5>.

Creating <ufs> file system for </var> on </dev/dsk/c1t1d0s1>.

Mounting file systems for boot environment <bts10200_FALLBACK>.

Calculating required sizes of file systems for boot environment <bts10200_FALLBACK>.

Populating file systems on boot environment <bts10200_FALLBACK>.

Checking selection integrity.

Integrity check OK.

Populating contents of mount point </>.

Populating contents of mount point </opt>.

Populating contents of mount point </var>.

Copying.

Creating shared file system mount points.

Creating compare databases for boot environment <bts10200_FALLBACK>.

Creating compare database for file system </var>.

Creating compare database for file system </opt>.

Creating compare database for file system </>.

Updating compare databases on boot environment <bts10200_FALLBACK>.

Making boot environment <bts10200_FALLBACK> bootable.

Setting root slice to </dev/dsk/c1t1d0s0>.

Population of boot environment <bts10200_FALLBACK> successful.

Creation of boot environment <bts10200_FALLBACK> successful.

Step 8 Verify that the new Alternate Boot Environment being created for Fallback purpose by using following command.

<hostname># lustatus

Example Output

<hostname># lustatus

Boot Environment             Is       	Active 	Active    	Can    	Copy

Name                       	   Complete 	Now    On Reboot 	Delete 	Status

--------------------------     -------- 	------ 	---------		 ------ 	----------

d2                         	     yes      	yes   	 yes       	no     	-

bts10200_FALLBACK  yes      	no     	no        	yes   	 -

Note: At this point the system has two bootable disks (disk 0 & disk 1), and currently the system is in a spilt mirror state running on disk 0 (d2 Boot Environment).

Step 9 Verify system can boot from disk1 (bts10200_FALLBACK release), do the following commands

<hostname># eeprom boot-device=”disk1 disk0”

<hostname># shutdown –y –g0 –i6

Step 10 After logging in as root, execute following commands to verify system booted on disk1 (bts10200_FALLBACK release) and that the platform is not started.

<hostname> nodestat

<hostname># lustatus   (Verification for Boot Environment)

Example Output

<hostname># lustatus

Boot Environment             Is       	Active 	Active    	Can    	Copy

Name                       	   Complete 	Now    On Reboot 	Delete 	Status

--------------------------     -------- 	------ 	---------		 ------ 	----------

d2                         	     yes      	no   	 no       	yes     	-

bts10200_FALLBACK   yes      	yes     	yes       	no   	 -

Step 11 Start all BTS platforms on the EMS and CA nodes

<hostname>#   platform start -nocopy

<hostname># nodestat

Step 12 login to CLI on EMS node and verify there are no errors and warnings for accessing CLI, also verify basic commands can be performed through CLI (Show, Add, Delete & Change etc.).

<hostname>#   su - btsadmin

Step 13 To boot from disk 0 (d2 release), do the following commands

<hostname># eeprom boot-device=”disk0 disk1”

<hostname># shutdown –y –g0 –i6

Step 14 After logging in as root, execute following commands to verify system booted on disk0 (d2 release) and that the platform is not started.

<hostname> nodestat

<hostname># lustatus   (Verification for Boot Environment)

Example Output

<hostname># lustatus

Boot Environment             Is       	Active 	Active    	Can    	Copy

Name                       	   Complete 	Now    On Reboot 	Delete 	Status

--------------------------     -------- 	------ 	---------		 ------ 	----------

d2                         	     yes      	yes   	 yes       	no     	-

bts10200_FALLBACK  yes      	no     	no        	yes   	 -

# Task 2: Restore the BTS Platforms

Step 1 Start all BTS platforms on the EMS and CA nodes

<hostname>#   platform start -nocopy

Step 2 Verify that the platform elements are all in standby state.

<hostname>#  nodestat

Step 3 Restore the auto platform start on bootup capability

<hostname>#   cd /etc/rc3.d

<hostname>#   mv _S99platform S99platform

# Task 3: Perform Switchover to prepare Side A CA and EMS Bootable Backup Disk

Step 1   Control all the platforms to standby-active. Login into the EMS side A and execute the following commands

<hostname>#   su - btsadmin

CLI> control call-agent id=CAxxx; target-state=STANDBY_ACTIVE;

CLI>control feature-server id=FSPTCyyy; target-state= STANDBY_ACTIVE;

CLI>control feature-server id=FSAINzzz; target-state= STANDBY_ACTIVE;

CLI>control bdms id=BDMSxx; target-state= STANDBY_ACTIVE;

CLI>control element_manager id=EMyy; target-state= STANDBY_ACTIVE;

CLI>Exit

Note:  It is possible that the mirror process for a node was previously started and not completed.  If this is the case, the Backup Disk Creation procedure will not work and the disks will be left in an indeterminate state.

Refer to Appendix M to verify if the disks are properly mirrored for Side A.

# Task 4: Repeat tasks 1 and 2 on the Side A EMS and CA Nodes

Note: At this point both Side A and Side B are running in a split mirror state on disk 0, thus both Side A and Side B (EMS & CA) are fully prepared to do fallback if needed on disk 1(bts10200_FALLBACK boot environment).

Appendix L

Mirroring the Disks

The following procedure is necessary and must be executed for mirroring the disks for field installations.

Step 1 	<hostname>#  cd /opt/setup

Step 2 	Execute the following command on EMS to set up the mirror for an EMS node.

<hostname>#  ./setup_mirror_ems

Expected Output:

Warning: Current Disk has mounted partitions.

/dev/dsk/c1t0d0s0 is currently mounted on /. Please see umount(1M).

/dev/dsk/c1t0d0s1 is currently mounted on /var. Please see umount(1M).

/dev/dsk/c1t0d0s3 is currently used by swap. Please see swap(1M).

/dev/dsk/c1t0d0s5 is currently mounted on /opt. Please see umount(1M).

partioning the 2nd disk for mirroring

fmthard:  New volume table of contents now in place.

checking disk partition

Disk partition match, continue with mirroring

If you see any error at all from this script, please stop

and don't reboot !!!

metainit: waiting on /etc/lvm/lock

d0: Concat/Stripe is setup

d1: Concat/Stripe is setup

d2: Mirror is setup

d12: Concat/Stripe is setup

d13: Concat/Stripe is setup

d14: Mirror is setup

d9: Concat/Stripe is setup

d10: Concat/Stripe is setup

d11: Mirror is setup

d3: Concat/Stripe is setup

d4: Concat/Stripe is setup

d5: Mirror is setup

d6: Concat/Stripe is setup

d7: Concat/Stripe is setup

d8: Mirror is setup

Dump content: kernel pages

Dump device: /dev/md/dsk/d8 (dedicated)

Savecore directory: /var/crash/secems76

Savecore enabled: yes

Step 3 	Execute the following command on CA to set up the mirror on the CA.

<hostname>#  cd /opt/setup

<hostname># ./setup_mirror_ca

Expected Results:

Warning: Current Disk has mounted partitions.

partioning the 2nd disk for mirroring

fmthard:  New volume table of contents now in place.

checking disk partition

Disk partition match, continue with mirroring

If you see any error at all from this script, please stop

and don't reboot !!!

d0: Concat/Stripe is setup

d1: Concat/Stripe is setup

d2: Mirror is setup

d12: Concat/Stripe is setup

d13: Concat/Stripe is setup

d14: Mirror is setup

d9: Concat/Stripe is setup

d10: Concat/Stripe is setup

d11: Mirror is setup

d3: Concat/Stripe is setup

d4: Concat/Stripe is setup

d5: Mirror is setup

d6: Concat/Stripe is setup

d7: Concat/Stripe is setup

d8: Mirror is setup

Dump content: kernel pages

Dump device: /dev/md/dsk/d8 (dedicated)

Savecore directory: /var/crash/secca76

Savecore enabled: yes

NOTE: Do not reboot your system if an error occurs. You must fix the error before moving to the next step.

Step 4 After the mirror setup completes successfully, reboot the system.

<hostname># reboot -- -r

Step 5 Once the system boots up, login as root and issue the following command

<hostname># cd  /opt/setup

Step 6 Synchronize the disk

<hostname>#	nohup ./sync_mirror &

Step 7 Wait for the disks to synchronize. Synchronization can be verified by executing the following commands

<hostname># cd /opt/utils

<hostname># Resync_status

Step 8 	Execute the following command to check the “real time” status of the disk sync,

<hostname># tail -f /opt/setup/nohup.out

NOTE: The disk syncing time will vary depending on the disk size. For a 72 gig disk, it can take approximately 3 hours.

Step 9 	Execute the following command to find out the percentage completion of this process. (Note that once the disk sync is complete no output will be returned as a result of the following command.)

<hostname># metastat | grep %

Step 10 The following message will be displayed once the disk syncing process completes.

Resync of disks has completed

Tue Feb 27 17:13:45 CST 2007

Step 11 Once the disk mirroring is completed, refer to Appendix M to verify Disk Mirroring.

Appendix M

Verifying the Disk mirror

Step 1 The following command determines if the system has finished the disk mirror setup.

<hostname>#  metastat |grep %

If no output is returned as a result of the above command then the system is syncing disks and the systems are up to date. Note however that this does not guarantee the disks are properly mirrored.

Step 2 The following command determines status of all the metadb slices on the disk.

<hostname>#  metadb |grep c1

The output should look very similar to the following

a m  p  luo        16              8192            /dev/dsk/c1t0d0s4

a      p  luo        8208          8192            /dev/dsk/c1t0d0s4

a      p  luo        16400        8192            /dev/dsk/c1t0d0s4

a     p   luo        16              8192            /dev/dsk/c1t1d0s4

a     p   luo        8208          8192            /dev/dsk/c1t1d0s4

a     p   luo        16400        8192            /dev/dsk/c1t1d0s4

Step 3 The following command determines the status of all the disk slices under mirrored control.

<hostname>#  metastat |grep c1

The output of the above command should look similar to the following:

c1t0d0s1          0     No            Okay   Yes

c1t1d0s1          0     No            Okay   Yes

c1t0d0s5          0     No            Okay   Yes

c1t1d0s5          0     No            Okay   Yes

c1t0d0s6          0     No            Okay   Yes

c1t1d0s6          0     No            Okay   Yes

c1t0d0s0          0     No            Okay   Yes

c1t1d0s0          0     No            Okay   Yes

c1t0d0s3          0     No            Okay   Yes

c1t1d0s3          0     No            Okay   Yes

c1t1d0   Yes    id1,sd@SFUJITSU_MAP3735N_SUN72G_00Q09UHU____

c1t0d0   Yes    id1,sd@SFUJITSU_MAP3735N_SUN72G_00Q09ULA____

Caution: Verify all 10 above slices are displayed. Also if an Okay is not seen on each of the slices for disk 0 and disk 1, then the disks are not properly mirrored. You must execute steps 1 through 6 in Task 1 of Appendix K to correct this. Steps 1 through 6 will break any established mirror on both disk 0 and disk 1. After completion, verify that disk 0 is bootable and proceed with mirroring disk 0 to disk 1 according to procedure in Appendix L.

Next, run the steps 1 – 3 above and verify that the disks are properly mirrored before running the procedure Creation of Backup Disk (Appendix K).