Cisco BTS 10200 Softswitch Software Upgrade for Release

4.2.0.V04-V10 to 4.2.0.V11

April 1, 2005

Corporate Headquarters

Cisco Systems, Inc.

170 West Tasman Drive

San Jose, CA 95134-1706

USA

http://www.cisco.com

Tel: 	408 526-4000

800 553-NETS (6387)

Fax:	408 526-4100

THE SPECIFICATIONS AND INFORMATION REGARDING THE PRODUCTS IN THIS MANUAL ARE SUBJECT TO CHANGE WITHOUT NOTICE. ALL STATEMENTS, INFORMATION, AND RECOMMENDATIONS IN THIS MANUAL ARE BELIEVED TO BE ACCURATE BUT ARE PRESENTED WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED. USERS MUST TAKE FULL RESPONSIBILITY FOR THEIR APPLICATION OF ANY PRODUCTS.

THE SOFTWARE LICENSE AND LIMITED WARRANTY FOR THE ACCOMPANYING PRODUCT ARE SET FORTH IN THE INFORMATION PACKET THAT SHIPPED WITH THE PRODUCT AND ARE INCORPORATED HEREIN BY THIS REFERENCE. IF YOU ARE UNABLE TO LOCATE THE SOFTWARE LICENSE OR LIMITED WARRANTY, CONTACT YOUR CISCO REPRESENTATIVE FOR A COPY.

The Cisco implementation of TCP header compression is an adaptation of a program developed by the University of California, Berkeley (UCB) as part of UCB’s public domain version of the UNIX operating system. All rights reserved. Copyright © 1981, Regents of the University of California.

NOTWITHSTANDING ANY OTHER WARRANTY HEREIN, ALL DOCUMENT FILES AND SOFTWARE OF THESE SUPPLIERS ARE PROVIDED “AS IS” WITH ALL FAULTS. CISCO AND THE ABOVE-NAMED SUPPLIERS DISCLAIM ALL WARRANTIES, EXPRESSED OR IMPLIED, INCLUDING, WITHOUT LIMITATION, THOSE OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT OR ARISING FROM A COURSE OF DEALING, USAGE, OR TRADE PRACTICE.

IN NO EVENT SHALL CISCO OR ITS SUPPLIERS BE LIABLE FOR ANY INDIRECT, SPECIAL, CONSEQUENTIAL, OR INCIDENTAL DAMAGES, INCLUDING, WITHOUT LIMITATION, LOST PROFITS OR LOSS OR DAMAGE TO DATA ARISING OUT OF THE USE OR INABILITY TO USE THIS MANUAL, EVEN IF CISCO OR ITS SUPPLIERS HAVE BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES.

CCIP, CCSP, the Cisco Arrow logo, the Cisco Powered Network mark, the Cisco Systems Verified logo, Cisco Unity, Follow Me Browsing, FormShare, iQ Breakthrough, iQ FastTrack, the iQ Logo, iQ Net Readiness Scorecard, Networking Academy, ScriptShare, SMARTnet, TransPath, and Voice LAN are trademarks of Cisco Systems, Inc.; Changing the Way We Work, Live, Play, and Learn, The Fastest Way to Increase Your Internet Quotient, and iQuick Study are service marks of Cisco Systems, Inc.; and Aironet, ASIST, BPX, Catalyst, CCDA, CCDP, CCIE, CCNA, CCNP, Cisco, the Cisco Certified Internetwork Expert logo, Cisco IOS, the Cisco IOS logo, Cisco Press, Cisco Systems, Cisco Systems Capital, the Cisco Systems logo, Empowering the Internet Generation, Enterprise/Solver, EtherChannel, EtherSwitch, Fast Step, GigaStack, Internet Quotient, IOS, IP/TV, iQ Expertise, LightStream, MGX, MICA, the Networkers logo, Network Registrar, Packet, PIX, Post-Routing, Pre-Routing, RateMUX, Registrar, SlideCast, StrataView Plus, Stratm, SwitchProbe, TeleRouter, and VCO are registered trademarks of Cisco Systems, Inc. and/or its affiliates in the U.S. and certain other countries.

All other trademarks mentioned in this document or Web site are the property of their respective owners. The use of the word partner does not imply a partnership relationship between Cisco and any other company. (0301R)

Cisco BTS 10200 Softswitch Software Upgrade

Copyright © 2005, Cisco Systems, Inc.

All rights reserved.

| Revision History |  |  |  |
| --- | --- | --- | --- |
| Date | Version | Revised By | Description |
| 1/04/2004 | 1.0 | Jack Daih | - Initial Version |
| 2/1/2005 | 2.0 | Jack Daih | Added reboot steps after completing install.sh -upgrade |
| 2/14/2005 | 3.0 | Jack Daih | - Added a step to kill mysql DB before running install.sh and uninstall.sh on EMS machines |
| 3/31/2005 | 4.0 | Jack Daih | - Added V10 to the upgrade path in the title |

Table of Contents

Table of Contents	4

Preface	9

Obtaining Documentation	9

World Wide Web	9

Documentation CD-ROM	9

Ordering Documentation	9

Documentation Feedback	10

Obtaining Technical Assistance	10

Cisco.com	10

Technical Assistance Center	11

Cisco TAC Web Site	11

Cisco TAC Escalation Center	12

Chapter 1	13

Upgrade Requirements	13

Introduction	13

Assumptions	14

Requirements	15

Important notes about this procedure	15

Upgrade Time Requirement	16

Chapter 2	17

Preparation	17

Task 1: Requirements and Prerequisites	17

Task 2: Preparation	17

Task 3: Add new domain names to DNS	17

Task 4: Copy Files from CD-ROM to Hard Drive and Extract tar Files	18

From EMS Side B	18

From EMS Side A	19

From CA/FS Side B	20

From CA/FS Side A	20

Chapter 3	22

Perform System Backup and Prepare System for Upgrade	22

Task 1: Backup Files	22

From EMS Side B	22

From EMS Side A	22

From CA/FS Side B	23

From CA/FS Side A	23

Task 2: Verify system status	23

Chapter 4	24

Upgrade Side B Systems	24

Task 1: Force side A system to active	24

From Active EMS	24

Task 2: Stop applications and cron daemon on Side B system	24

From EMS side B	24

From CA/FS side B	25

Task 3: Upgrade CA/FS Side B to the new release	25

From CA/FS side B	25

Task 4: Upgrade EMS side B to the new release	26

From EMS side B	26

Task 5: To install CORBA on EMS side B, please follow Appendix I.	27

Chapter 5	28

Upgrade Side A Systems	28

Task 1: Force side A system to standby	28

From EMS side A	28

Task 2: FTP Billing records to a mediation device	29

From EMS side A	29

Task 3: Verify system status	29

Task 4: Verify database state	29

From EMS side B	29

Task 5: Validate new release software operation	29

Task 6: Stop applications and cron daemon on side A system	30

From EMS Side A	30

From CA/FS Side A	30

Task 7: Upgrade CA/FS side A to the new release	30

From CA/FS side A	31

Task 8: Upgrade EMS side A to the new release	31

From EMS side A	31

Task 9 To install CORBA on EMS side A, please follow Appendix I.	32

Chapter 6	33

Finalizing Upgrade	33

Task 1: Restore EMS mate communication	33

From EMS side B	33

Task 2: Switchover activity from side B to side A	33

From EMS side B	33

Task 3: Restore cron jobs for EMS	34

From EMS side A	34

From EMS side B	35

Task 4: Restore the system to normal mode	36

From EMS side A	36

Task 5: Verify system status	36

Appendix A	38

Check System Status	38

From Active EMS side A	38

Appendix B	40

Check Call Processing	40

From EMS side A	40

Appendix C	43

Check Provisioning and Database	43

From EMS side A	43

Perform database audits	43

Check transaction queue	43

Appendix D	45

Check Alarm Status	45

From EMS side A	45

Appendix E	47

Check Oracle Database Replication and Error Correction	47

Check Oracle DB replication status	47

From EMS side A	47

Correct replication error	48

From EMS Side B	48

From EMS Side A	48

Appendix F	50

Flash Archive Steps	50

Task 1: Ensure side A system is ACTIVE	50

Task 2: Perform a full database audit	50

From EMS Side A	51

Task 3: Perform shared memory integrity check	51

From CA/FS side A	51

From CA/FS side B	52

Task 4: Perform flash archive on EMS side B	53

From EMS side B	53

Task 5: Perform flash archive on CA/FS side B	55

From CA/FS side B	55

Task 6: Switch activity from side A to side B	57

From EMS side A	57

Task 7: Perform flash archive on EMS side A	58

From EMS side A	58

Task 8: Perform flash archive on CA/FS side A	60

From CA/FS side A	60

Task 9: Release forced switch	61

From EMS side B	62

From EMS side A	62

This completes the flash archive process.	62

Appendix G	64

Backout Procedure for Side B System	64

Introduction	64

Task 1: Force side A system to active	65

From Active EMS side B	66

Task 2: FTP Billing records to a mediation device	66

From EMS side B	66

Task 3: Stop applications on EMS side B and CA/FS side B	66

From EMS side B	66

From CA/FS side B	67

Task 4: Remove installed applications on EMS side B and CA/FS side B	67

From EMS side B	67

From CA/FS side B	67

Task 5: Copy files from CD-ROM to hard drive and extract tar files	68

From EMS Side B	68

From CA/FS Side B	69

Task 6: Restore side B to the old release	69

From CA/FS Side B	69

From EMS side B	70

Task 7: Restore cron job	71

From EMS side B	71

Task 8: To install CORBA on EMS side B, please follow Appendix I.	71

Task 9: Restore the system to the normal mode	72

From EMS side A	72

Task 10: Verify system status	72

This completes the side B system fallback.	73

Appendix H	74

System Backout Procedure	74

Introduction	74

Task 1: Inhibit EMS mate communication	74

From EMS side B	74

Task 2: Force side B system to active	75

From EMS side A	75

Task 3: Stop applications and cron daemon on side A system	75

From EMS side A	75

From CA/FS side A	76

Task 4: FTP Billing records to a mediation device	76

From EMS side A	76

Task 5: Remove installed applications on EMS side A and CA/FS side A	76

From EMS side A	76

From CA/FS side A	77

Task 6: Copy files from CD-ROM to hard drive and extract tar files	77

From EMS Side A	77

From CA/FS Side A	78

Task 7: Restore CA/FS side A to the old release	79

From CA/FS side A	79

From EMS side A	79

Task 8: Restore cron jobs for EMS side A	80

From EMS side A	80

Task 9: To install CORBA on EMS side A, please follow Appendix I.	81

Task 10: To continue fallback process, please follow Appendix G.	81

This completes the entire system fallback	81

Appendix I	82

CORBA Installation	82

Task 1: Remove Installed VisiBroker	82

Remove Installed VisiBroker CORBA Application	82

Task 2: Install OpenORB CORBA Application	83

Remove Installed OpenORB Application	83

Install OpenORB Packages	83

Appendix J	85

Check and Sync System Clock	85

Task 1: Check system clock	85

From each machine in a BTS system	85

Task 2: Sync system clock	85

From each machine in a BTS system	85

Preface

# Obtaining Documentation

These sections explain how to obtain documentation from Cisco Systems.

### World Wide Web

You can access the most current Cisco documentation on the World Wide Web at this URL: http://www.cisco.com/

Translated documentation is available at this URL: http://www.cisco.com/public/countries_languages.shtml

### Documentation CD-ROM

Cisco documentation and additional literature are available in a Cisco Documentation CD-ROM package, which is shipped with your product. The Documentation CD-ROM is updated monthly and may be more current than printed documentation. The CD-ROM package is available as a single unit or through an annual subscription.

### Ordering Documentation

You can order Cisco documentation in these ways:

Registered Cisco.com users (Cisco direct customers) can order Cisco product documentation from the Networking Products MarketPlace: http://www.cisco.com/cgi-bin/order/order_root.pl

Registered Cisco.com users can order the Documentation CD-ROM through the online Subscription Store: http://www.cisco.com/go/subscription

Nonregistered Cisco.com users can order documentation through a local account representative by calling Cisco Systems Corporate Headquarters (California, U.S.A.) at 408 526-7208 or, elsewhere in North America, by calling 800 553-NETS (6387).

### Documentation Feedback

You can submit comments electronically on Cisco.com. In the Cisco Documentation home page, click the Fax or Email option in the “Leave Feedback” section at the bottom of the page.

You can e-mail your comments to mailto:bug-doc@cisco.com.

You can submit your comments by mail by using the response card behind the front cover of your document or by writing to the following address:

Cisco Systems, INC.

Attn: Document Resource Connection

170 West Tasman Drive

San Jose, CA 95134-9883

# Obtaining Technical Assistance

Cisco provides Cisco.com as a starting point for all technical assistance. Customers and partners can obtain online documentation, troubleshooting tips, and sample configurations from online tools by using the Cisco Technical Assistance Center (TAC) Web Site. Cisco.com registered users have complete access to the technical support resources on the Cisco TAC Web Site: http://www.cisco.com/tac

### Cisco.com

Cisco.com is the foundation of a suite of interactive, networked services that provides immediate, open access to Cisco information, networking solutions, services, programs, and resources at any time, from anywhere in the world.

Cisco.com is a highly integrated Internet application and a powerful, easy-to-use tool that provides a broad range of features and services to help you with these tasks:

Streamline business processes and improve productivity

Resolve technical issues with online support

Download and test software packages

Order Cisco learning materials and merchandise

Register for online skill assessment, training, and certification programs

If you want to obtain customized information and service, you can self-register on Cisco.com. To access Cisco.com, go to this URL: http://www.cisco.com/

### Technical Assistance Center

The Cisco Technical Assistance Center (TAC) is available to all customers who need technical assistance with a Cisco product, technology, or solution. Two levels of support are available: the Cisco TAC Web Site and the Cisco TAC Escalation Center.

Cisco TAC inquiries are categorized according to the urgency of the issue:

Priority level 4 (P4)—you need information or assistance concerning Cisco product capabilities, product installation, or basic product configuration.

Priority level 3 (P3)—your network performance is degraded. Network functionality is noticeably impaired, but most business operations continue.

Priority level 2 (P2)—your production network is severely degraded, affecting significant aspects of business operations. No workaround is available.

Priority level 1 (P1)—your production network is down, and a critical impact to business operations will occur if service is not restored quickly. No workaround is available.

The Cisco TAC resource that you choose is based on the priority of the problem and the conditions of service contracts, when applicable.

### Cisco TAC Web Site

You can use the Cisco TAC Web Site to resolve P3 and P4 issues yourself, saving both cost and time. The site provides around-the-clock access to online tools, knowledge bases, and software. To access the Cisco TAC Web Site, go to this URL: http://www.cisco.com/tac

All customers, partners, and resellers who have a valid Cisco service contract have complete access to the technical support resources on the Cisco TAC Web Site. The Cisco TAC Web Site requires a Cisco.com Log in ID and password. If you have a valid service contract but do not have a Log in ID or password, go to this URL to register: http://www.cisco.com/register/

If you are a Cisco.com registered user, and you cannot resolve your technical issues by using the Cisco TAC Web Site, you can open a case online by using the TAC Case Open tool at this URL: http://www.cisco.com/tac/caseopen

If you have Internet access, we recommend that you open P3 and P4 cases through the Cisco TAC Web Site: http://www.cisco.com/tac

### Cisco TAC Escalation Center

The Cisco TAC Escalation Center addresses priority level 1 or priority level 2 issues. These classifications are assigned when severe network degradation significantly impacts business operations. When you contact the TAC Escalation Center with a P1 or P2 problem, a Cisco TAC engineer automatically opens a case.

To obtain a directory of toll-free Cisco TAC telephone numbers for your country, go to this URL: http://www.cisco.com/warp/public/687/Directory/DirTAC.shtml

Before calling, please check with your network operations center to determine the level of Cisco support services to which your company is entitled: for example, SMARTnet, SMARTnet Onsite, or Network Supported Accounts (NSA). When you call the center, please have available your service agreement number and your product serial number.

Chapter 1

Upgrade Requirements

# Introduction

Application software loads are designated as Release 900-aa.bb.cc.Vxx, where

- aa=major release number, for example, 01

- bb=minor release number, for example, 03

- cc=maintenance release, for example, 00

- Vxx=Version number, for example V04

This procedure can be used on an in-service system, but the steps must be followed as shown in this document in order to avoid traffic interruptions.

|  | Caution Performing the steps in this procedure will bring down and restart individual platforms in a specific sequence. Do not perform the steps out of sequence, as it could affect traffic. If you have questions, contact Cisco TAC. |
| --- | --- |

This procedure should be performed during a maintenance window.

|  | Note In this document, the following designations are used: EMS = Element Management System; CA/FS = Call Agent / Feature Server "Primary" is also referred to as "Side A", and "Secondary" is also referred to as "Side B". See Figure 1-1 for a front view of the Softswitch rack. |
| --- | --- |

Figure 1-1   Cisco BTS 10200 Softswitch—Rack Configuration

# Assumptions

The following assumptions are made.

- The installer has a basic understanding of UNIX and Oracle commands.

- The installer has the appropriate user name(s) and password(s) to log on to each EMS/CA/FS platform as root user, and as Command Line Interface (CLI) user on the EMS.

- The installer has a NETWORK INFORMATION DATA SHEET (NIDS) with the IP addresses of each EMS/CA/FS to be upgraded, and all the data for the opticall.cfg file.

- Confirm that all names in opticall.cfg are in the DNS server

- The CD-ROM for the correct software version is available to the installer, and is readable.

|  | Note Contact Cisco TAC before you start if you have any questions. |
| --- | --- |

# Requirements

Verify that opticall.cfg has the correct information for each of the following machines.

- Side A EMS

- Side B EMS

- Side A CA/FS

- Side B CA/FS

Determine the oracle and root passwords for the systems you are upgrading. If you do not know these passwords, ask your system administrator.

Refer to local documentation to determine if CORBA installation is required on this system. If unsure, ask your system administrator.

# Important notes about this procedure

Throughout this procedure, each command is shown with the appropriate system prompt, followed by the command to be entered in bold. The prompt is generally one of the following:

- Host system prompt (<hostname>#)

- Oracle prompt (<hostname>$)

- SQL prompt (SQL>)

- CLI prompt (CLI>)

- FTP prompt (ftp>)

Note the following conventions used throughout the steps in this procedure:

- Enter commands as shown, as they are case sensitive (except for CLI commands).

- Press the Return (or Enter) key at the end of each command, as indicated by "".

It is recommended that you read through the entire procedure before performing any steps.

# Upgrade Time Requirement

This upgrade procedure will take about 2 hours to complete including pre- and post - check of the system health. This upgrade process will preserve Oracle provisioned data, shared memory data, and live traffic.

The total upgrade time does not account for the time it takes to do a complete system wide flash archive. The flash archive process is considered one of the normal operational activities.

In the event when fallback is needed, the process will take additional 2 hours to fallback the entire system, and 1 hour and 20 minutes to fallback one half of the system.

Chapter 2

Preparation

|  | Note CDR delimiter customization is not retained after software upgrade. The customer or Cisco engineer must manually customize again to keep the same customization. |
| --- | --- |

This section describes the steps a user must complete a week prior to the scheduled upgrade.

# Task 1: Requirements and Prerequisites

- One release 900-04.02.00.V11 CD labeled BTS 10200 System Disk.

- Host names for the system

- Console access

- Secure shell access

# Task 2: Preparation

- Verify the CD ROM drive is in working order by using the mount command and a valid CD ROM.

# Task 3: Add new domain names to DNS

This task must be performed on Domain Name Servers that are serving the Cisco BTS 10200 system.

Step 1   Log in to Domain Name Servers for Cisco BTS 10200

Step 2   Add domain names for the following opticall.cfg parameters to Domain Name Server database:

- BROKER_DNS_FOR_PMG_IN_CA

Note This is a qualified domain name used by a pmg process in Call Agent/Feature Servers. This domain name is used for redundancy and switchover operations. They are set to the IP addresses of the two routers between the CA/FS and the Media Gateways.

- BROKER_DNS_FOR_PMG_IN_EMS

Note This is a qualified domain name used by a pmg process in Element Manager. This domain name is used for redundancy and switchover operations. They are set to the IP addresses of the two routers between the Element Manager and the Media Gateways.

- DNS_FOR_EMS_SIDE_A_OMS_SLAVE_HUB

Note This is a qualified domain name used by master OMSHub in both EMS Side A and B to talk to a slave OMSHub in CA/FS Side A host. Each name should return two IP addresses of a CA/FS Side A host.

- DNS_FOR_EMS_SIDE_B_OMS_SLAVE_HUB

Note This is a qualified domain name used by master OMSHub in both EMS Side A and B to talk to a slave OMSHub in CA/FS Side B host. Each name should return two IP addresses of a CA/FS Side B host.

## Task 4: Copy Files from CD-ROM to Hard Drive and Extract tar Files

# From EMS Side B

Step 1   Log in as root.

Step 2   Put the release 900-04.02.00.V11 BTS 10200 System Disk CD-ROM in the CD-ROM drive.

Step 3   Remove old files.

<hostname># cd /

<hostname># \rm –rf /opt/Build

Step 4   Create /cdrom directory and mount the directory.

<hostname># mkdir -p /cdrom

<hostname># mount -o ro -F hsfs /dev/dsk/c0t6d0s0 /cdrom

Step 5   Use the following commands to copy files from the CD-ROM to the local disk.

<hostname># cp –f /cdrom/K9-opticall.tar /opt

Step 6   Verify that the check sum values match with the values located in the “checksum.txt” file located on Application CD-ROM.

<hostname># cat /cdrom/checksum.txt

<hostname># cksum /opt/K9-opticall.tar

Step 7   Unmount the CD-ROM.

<hostname># umount cdrom

Step 8   Manually eject the CD-ROM and take out the release 900-04.02.00.V11 BTS 10200 System Disk CD-ROM from CD-ROM drive.

Step 9   Extract tar files.

<hostname># cd /opt

<hostname># tar -xvf K9-opticall.tar

|  | Note The files will take up 10 minutes to extract. |
| --- | --- |

# From EMS Side A

Step 1   Log in as root.

Step 2   <hostname># cd /opt

Step 3   <hostname># \rm –rf /opt/Build

Step 4   <hostname># sftp <hostname of EMS Side B>

Step 5   sftp> cd /opt

Step 6   sftp> get K9-opticall.tar

Step 7   sftp> exit

Step 8   <hostname># tar -xvf K9-opticall.tar

|  | Note The files will take up 10 minutes to extract. |
| --- | --- |

# From CA/FS Side B

Step 1   Log in as root.

Step 2   <hostname># cd /opt

Step 3   <hostname># \rm –rf /opt/Build

Step 4   <hostname># sftp <hostname of EMS Side B>

Step 5   sftp> cd /opt

Step 6   sftp> get K9-opticall.tar

Step 7   sftp> exit

Step 8   <hostname># tar -xvf K9-opticall.tar

|  | Note The files will take up 10 minutes to extract. |
| --- | --- |

# From CA/FS Side A

Step 1   <hostname># cd /opt

Step 2   <hostname># \rm –rf /opt/Build

Step 3   <hostname># sftp <hostname of EMS side B>

Step 4   sftp> cd /opt

Step 5   sftp> get K9-opticall.tar

Step 6   sftp> exit

Step 7   <hostname># tar -xvf K9-opticall.tar

|  | Note The files will take up 10 minutes to extract. |
| --- | --- |

Chapter 3

Perform System Backup and Prepare System for Upgrade

# Task 1: Backup Files

# From EMS Side B

Step 1   Log in as root.

Step 2   <hostname># mkdir –p /opt/.upgrade

Step 3   <hostname># cp -fp /var/spool/cron/crontabs/* /opt/.upgrade

Step 4   <hostname># tar -cvf /opt/.upgrade/users.tar /opt/ems/users

# From EMS Side A

Step 1   Log in as root.

Step 2   <hostname># mkdir –p /opt/.upgrade

Step 3   <hostname># cp -fp /var/spool/cron/crontabs/* /opt/.upgrade

Step 4   <hostname># tar -cvf /opt/.upgrade/users.tar /opt/ems/users

Step 5   <hostname># /opt/ems/utils/updMgr.sh –split_hub

Step 6   <hostname># nodestat

- Verify there is no HUB communication from EMS side A to CA/FS side B

- Verify OMS Hub mate port status: No communication between EMS

# From CA/FS Side B

Step 1   Log in as root.

Step 2   <hostname># mkdir –p /opt/.upgrade

Step 3   <hostname># cp -fp /var/spool/cron/crontabs/root /opt/.upgrade

# From CA/FS Side A

Step 1   Log in as root.

Step 2   <hostname># mkdir –p /opt/.upgrade

Step 3   <hostname># cp -fp /var/spool/cron/crontabs/root /opt/.upgrade

# Task 2: Verify system status

Step 1   Verify that the side A system is in the active state. Use Appendix A for this procedure.

Step 2   Verify that call processing is working without error. Use Appendix B for this procedure.

Step 3   Verify that provisioning is operational from CLI command line, and verify database. Use Appendix C for this procedure.

Step 4   Verify that there are no outstanding major or critical alarms. Use Appendix D for this procedure.

Step 5   Use Appendix E to verify that Oracle database and replication functions are working properly.

Step 6   Use Appendix J to verify that the system clock is in sync.

|  | Caution Do not continue until the above verifications have been made. Call Cisco TAC if you need assistance. |
| --- | --- |

Chapter 4

Upgrade Side B Systems

# Task 1: Force side A system to active

This procedure will force the side A system to remain active.

|  | Note In the commands below, "xxx", "yyy" or "zzz" is the instance for the process on your system. |
| --- | --- |

# From Active EMS

Step 1   Log in to Active EMS as CLI user.

Step 2   CLI> control feature-server id=FSPTCyyy; target-state=forced-active-standby;

Step 3   CLI> control feature-server id=FSAINzzz; target-state=forced-active-standby;

Step 4   CLI> control call-agent id=CAxxx; target-state=forced-active-standby;

Step 5   CLI> control bdms id=BDMS01; target-state=forced-active-standby;

Step 6   CLI> control element-manager id=EM01; target-state=forced-active-standby;

# Task 2: Stop applications and cron daemon on Side B system

# From EMS side B

Step 1   Log in as root.

Step 2   <hostname># /etc/rc2.d/S75cron stop

Step 3   <hostname># platform stop all

# From CA/FS side B

Step 1   Log in as root.

Step 2   <hostname># /etc/rc2.d/S75cron stop

Step 3   <hostname># platform stop all

|  | Note To speed up the upgrade process, you can execute Task 3 and Task 4 in parallel. |
| --- | --- |

# Task 3: Upgrade CA/FS Side B to the new release

# From CA/FS side B

|  | Note In the commands below, "xxx", "yyy" or "zzz" is the instance for the process on your system. |
| --- | --- |

Step 1   Log in as root.

Step 2   <hostname># /opt/Build/install.sh –update_cfg

- <hostname># vi /etc/opticall.cfg

- Define the value for parameter: BROKER_DNS_FOR_PMG_CA

- Define the value for parameter: BROKER_DNS_FOR_PMG_EMS

- Define the value for parameter: DNS_FOR_EMS_SIDE_A_OMS_SLAVE_HUB

- Define the value for parameter: DNS_FOR_EMS_SIDE_B_OMS_SLAVE_HUB

- Save the opticall.cfg file

Step 3   <hostname># /opt/Build/install.sh -aponly

Step 4   Answer "y" when prompted. This process will take up to 15 minutes to complete.

Step 5   Answer "y” when prompt for “reboot”

Step 6   Wait for the system to boot up. Then Log in as root

Step 7   Verify applications are in service and in standby state.

<hostname># nodestat

# Task 4: Upgrade EMS side B to the new release

# From EMS side B

Step 1   Log in as root.

Step 2   sftp /etc/opticall.cfg from CA/FS side B

Step 3   <hostname># pkill mysql

Step 4   <hostname># /opt/Build/install.sh -aponly

Step 5   Answer "y" when prompt. This process will take up to 15 minutes to complete.

Step 6   Answer "y” when prompt for “reboot”

Step 7   Wait for the system to boot up. Then Log in as root

Step 8   Verify applications are in service and in standby state.

<hostname># nodestat

Step 9   Re-load static CLI data for provisioning.

<hostname># su - oracle

<hostname>$ cd /opt/oracle/opticall/create/data

<hostname>$ update_cmdtab.sh optical2 upgrade

<hostname>$ exit

Step 10   <hostname># /opt/ems/utils/updMgr.sh –split_hub

Step 11   <hostname># nodestat

- Verify there is no HUB communication from EMS side B to CA/FS side A

Step 12   Restore user accounts:

<hostname># cd /opt/.upgrade

<hostname># tar -xvf users.tar

# Task 5: To install CORBA on EMS side B, please follow Appendix I.

Chapter 5

Upgrade Side A Systems

# Task 1: Force side A system to standby

This procedure will force the side A system to standby and force the side B system to active.

|  | Note In the commands below, "xxx", "yyy" or "zzz" is the instance for the process on your system. |
| --- | --- |

# From EMS side A

Step 1   Log in as CLI user.

Step 2   CLI> control feature-server id=FSPTCzzz; target-state=forced-standby-active;

Step 3   CLI> control feature-server id=FSAINyyy; target-state=forced-standby-active;

Step 4   CLI> control call-agent id=CAxxx; target-state=forced-standby-active;

Step 5   CLI> control bdms id=BDMS01; target-state=forced-standby-active;

Step 6   CLI> control element-manager id=EM01; target-state=forced-standby-active;

Step 7   CLI session will terminate when the last CLI command completes.

|  | Note If the system failed to switchover from side A to side B, please contact Cisco TAC to determine whether the system should fallback. If fallback is needed, please following Appendix G. |
| --- | --- |

# Task 2: FTP Billing records to a mediation device

# From EMS side A

Step 1   Log in as root

Step 2   <hostname># cd /opt/bms/ftp/billing

Step 3   <hostname># ls

Step 4   If there are files listed, then FTP the files to a mediation device on the network.

# Task 3: Verify system status

Step 1   Verify that call processing is working without error. Use Appendix B for this procedure.

# Task 4: Verify database state

# From EMS side B

Step 1   Log in as CLI user.

Step 2   CLI> audit database type=row-count;

- Verify there is no error in the report and the database is not empty.

Step 3   CLI> exit

|  | Caution Do not continue until the above verifications have been made. Call Cisco TAC if you need assistance. |
| --- | --- |

# Task 5: Validate new release software operation

To verify the stability of the newly installed release, let CA/FS side B carry live traffic for period of time. Monitor the Cisco BTS 10200 Softswitch and the network; if there are any problems, please investigate and contact Cisco TAC if necessary.

|  | Note Once the system proves stable and you decide to move ahead with the upgrade, then you must execute subsequent tasks. If fallback is needed at this stage, please follow the fallback procedure in Appendix G. |
| --- | --- |

# Task 6: Stop applications and cron daemon on side A system

# From EMS Side A

Step 1   Log in as root.

Step 2   <hostname># /etc/rc2.d/S75cron stop

Step 3   <hostname># platform stop all

# From CA/FS Side A

Step 1   Log in as root.

Step 2   <hostname># /etc/rc2.d/S75cron stop

Step 3   <hostname># platform stop all

|  | Note To speed up the upgrade process, you can execute Task 7 and Task 8 in parallel. |
| --- | --- |

# Task 7: Upgrade CA/FS side A to the new release

# From CA/FS side A

Step 1   Log in as root.

Step 2   sftp /etc/opticall.cfg from CA/FS side B

Step 3   <hostname># /opt/Build/install.sh -aponly

Step 4   Answer "y" when prompt. This process will take up to 15 minutes to complete.

Step 5   Answer "y” when prompt for “reboot”

Step 6   Wait for the system to boot up. Then Log in as root

Step 7   Verify applications are in service and in standby state.

<hostname># nodestat

# Task 8: Upgrade EMS side A to the new release

# From EMS side A

Step 1   Log in as root.

Step 2   sftp /etc/opticall.cfg from CA/FS side B

Step 3   <hostname># pkill mysql

Step 4   <hostname># /opt/Build/install.sh -aponly

Step 5   Answer "y" when prompt. This process will take up to 15 minutes to complete.

Step 6   Answer "y” when prompt for “reboot”

Step 7   Wait for the system to boot up. Then Log in as root

Step 8   Verify applications are in service and in standby state.

<hostname># nodestat

Step 9   Re-load static CLI data for provisioning.

<hostname># su - oracle

<hostname>$ cd /opt/oracle/opticall/create/data

<hostname>$ update_cmdtab.sh optical1 upgrade

  - optical1 – optical followed by numeric 1

<hostname>$ exit

Step 10   <hostname># /opt/ems/utils/updMgr.sh –restore_hub

Step 11   <hostname># nodestat

- Verify there is HUB communication from EMS side B to CA/FS side A

Step 12   Restore user accounts:

<hostname># cd /opt/.upgrade

<hostname># tar -xvf users.tar

# Task 9 To install CORBA on EMS side A, please follow Appendix I.

Chapter 6

Finalizing Upgrade

# Task 1: Restore EMS mate communication

In this task, you will restore the OMS Hub communication from EMS side B to side A.

# From EMS side B

Step 1   Log in as root

Step 2   <hostname># /opt/ems/utils/updMgr.sh –restore_hub

Step 3   <hostname># nodestat

- Verify OMS Hub mate port status is established

- Verify HUB communication from EMS side B to CA/FS side A is established

# Task 2: Switchover activity from side B to side A

This procedure will force the active system activity from side B to side A.

# From EMS side B

|  | Note In the commands below, "xxx", "yyy" or "zzz" is the instance for the process on your system. |
| --- | --- |

Step 1   Log in to EMS side B as CLI user.

Step 2   CLI> control feature-server id=FSPTCyyy; target-state=forced-active-standby;

Step 3   CLI> control feature-server id=FSAINzzz; target-state=forced-active-standby;

Step 4   CLI> control call-agent id=CAxxx; target-state=forced-active-standby;

Step 5   CLI> control bdms id=BDMS01; target-state=forced-active-standby;

Step 6   CLI> control element-manager id=EM01; target-state=forced-active-standby;

Step 7   CLI shell session should be terminated when last CLI commands completes.

# Task 3: Restore cron jobs for EMS

Restoration of root cron jobs for the system is not necessary as the upgrade procedure does not overwrite the previous root cron jobs, however a backup was taken for safety purposes and if needed can be found on each system in the /opt/.upgrade directory.

# From EMS side A

Step 1   Log in as root.

Step 2   <hostname># cd /opt/.upgrade

Step 3   <hostname># more oracle

Step 4   <hostname># cd /var/spool/cron/crontabs

###### Step 5   <hostname># more oracle

Step 6   Compare the backed up version of the cron jobs to the new cron and restore previous settings.

Note Do not simply copy the old cron over the new. You must edit the new and restore the settings manually.

- For example backup version has the following

# Get optical1 DB statistics

#

0 11,17 * * * /opt/oracle/admin/stat/db_tune/get_all_stats.sh optical1 > /opt/oracle/admin/stat/db_tune/report/get_all_stats.log 2>&1

#

New Version has:

# Get optical1 DB statistics

#

#0 11,17 * * * /opt/oracle/admin/stat/db_tune/get_all_stats.sh optical1 > /opt/oracle/admin/stat/db_tune/report/get_all_stats.log 2>&1

#

###### Step 7   To change the setting, run:

- <hostname># crontab –e oracle

- Navigate to the line to be changed, Remove the “#” to match the backup version and then save the file. So the line is changed:

From:

#0 11,17 * * * /opt/oracle/admin/stat/db_tune/get_all_stats.sh optical1 > /opt/oracle/admin/stat/db_tune/report/get_all_stats.log 2>&1

To:

0 11,17 * * * /opt/oracle/admin/stat/db_tune/get_all_stats.sh optical1 > /opt/oracle/admin/stat/db_tune/report/get_all_stats.log 2>&1

# From EMS side B

Step 1   Log in as root.

Step 2   <hostname># cd /var/spool/cron/crontabs

Step 3   <hostname># sftp <EMS side a hostname>

Step 4   sftp> cd /var/spool/cron/crontabs

Step 5   sftp> get oracle

Step 6   sftp> exit

Step 7   <hostname># sed s/optical1/optical2/g oracle > temp

Step 8   <hostname># mv temp oracle

Step 9   <hostname># /etc/rc2.d/S75cron stop

Step 10   <hostname># /etc/rc2.d/S75cron start

# Task 4: Restore the system to normal mode

This procedure will remove the forced switch and restore the system to NORMAL state.

# From EMS side A

|  | Note In the commands below, "xxx", "yyy" or "zzz" is the instance for the process on your system. |
| --- | --- |

Step 1   Log in as CLI user.

Step 2   CLI> control feature-server id=FSPTCyyy; target-state=normal;

Step 3   CLI> control feature-server id=FSAINzzz; target-state=normal;

Step 4   CLI> control call-agent id=CAxxx; target-state=normal;

Step 5   CLI> control bdms id=BDMS01; target-state=normal;

Step 6   CLI> control element-manager id=EM01; target-state=normal;

Step 7   CLI> exit

# Task 5: Verify system status

Verify that the system is operating properly before you leave the site.

Step 1   Verify that the side A system is in the active state. Use Appendix A for this procedure.

Step 2   Verify that call processing is working without error. Use Appendix B for this procedure.

Step 3   Verify that provisioning is operational from CLI command line, and verify database. Use Appendix C for this procedure.

Step 4   Verify that there are no outstanding major or critical alarms. Use Appendix D for this procedure.

Step 5   Use Appendix E to verify that Oracle database and replication functions are working properly.

Step 6   Use Appendix J to verify that the system clock is in sync.

Step 7   If you answered NO to any of the above questions (Step 1 through Step 6), do not proceed. Instead, use the backout procedure in Appendix H . Contact Cisco TAC if you need assistance.

Once site has verified that all critical call-thru testing has successfully completed and the upgrade is complete Appendix F should be executed to gather an up to date archive file of the system.

Appendix A

Check System Status

The purpose of this procedure is to verify the system is running in NORMAL mode, with the side A system in ACTIVE state and the side B system in STANDBY state. This condition is illustrated in Figure A-1.

Figure A-1   Side A ACTIVE_NORMAL and Side B STANDBY_NORMAL

|  | Note In the commands below, "xxx", "yyy" or "zzz" is the instance for the process on your system, and DomainName is your system domain name. |
| --- | --- |

# From Active EMS side A

Step 1   Log in as CLI user.

Step 2   CLI> status call-agent id=CAxxx;

System response:

APPLICATION INSTANCE -> Call Agent [CAxxx] PRIMARY STATUS -> ACTIVE_NORMAL SECONDARY STATUS -> STANDBY_NORMAL

Step 3   CLI> status feature-server id=FSAINyyy;

System response:

APPLICATION INSTANCE -> Feature Server [FSAIN205] PRIMARY STATUS -> ACTIVE_NORMAL SECONDARY STATUS -> STANDBY_NORMAL

Step 4   CLI> status feature-server id=FSPTCzzz;

System response:

APPLICATION INSTANCE -> Feature Server [FSPTC235] PRIMARY STATUS -> ACTIVE_NORMAL SECONDARY STATUS -> STANDBY_NORMAL

Step 5   CLI> status bdms id=BDMS01;

System response:

APPLICATION INSTANCE -> Bulk Data Management Server [BDMS01] PRIMARY STATUS -> ACTIVE_NORMAL SECONDARY STATUS -> STANDBY_NORMAL BILLING ORACLE STATUS IS... -> Daemon is running!

Step 6   CLI> status element-manager id=EM01;

System response:

APPLICATION INSTANCE -> Element Manager [EM01] PRIMARY STATUS -> ACTIVE_NORMAL SECONDARY STATUS -> STANDBY_NORMAL EMS MYSQL STATUS IS ... -> Daemon is running! ORACLE STATUS IS... -> Daemon is running!

Appendix B

Check Call Processing

This procedure verifies that call processing is functioning without error. The billing record verification is accomplished by making a sample phone call and verify the billing record is collected correctly.

# From EMS side A

Step 1   Log in as CLI user.

Step 2   Make a new phone call on the system. Verify that you have two-way voice communication. Then hang up both phones.

Step 3   CLI>report billing-record tail=1;

... CALLTYPE=LOCAL SIGSTARTTIME=2004-02-18 18:36:56 SIGSTOPTIME=2004-02-18 18:38:37 ICSTARTTIME=2004-02-18 18:36:56 ICSTOPTIME=2004-02-18 18:38:37 CALLCONNECTTIME=2004-02-18 18:37:01 CALLANSWERTIME=2004-02-18 18:37:01 CALLDISCONNECTTIME=2004-02-18 18:38:37 CALLELAPSEDTIME=00:01:36 INTERCONNECTELAPSEDTIME=00:01:41 ORIGNUMBER=9722550010 TERMNUMBER=8505801234 CHARGENUMBER=9722550010 DIALEDDIGITS=8505801234 OFFHOOKINDICATOR=1 SHORTOFFHOOKINDICATOR=0 CALLTERMINATIONCAUSE=NORMAL_CALL_CLEARING OPERATORACTION=0 ORIGSIGNALINGTYPE=0 TERMSIGNALINGTYPE=1 ORIGTRUNKNUMBER=0 TERMTRUNKNUMBER=1501 OUTGOINGTRUNKNUMBER=0 ORIGCIRCUITID=0 TERMCIRCUITID=1 PICSOURCE=2 ICINCIND=1 ICINCEVENTSTATUSIND=20 ICINCRTIND=0 ORIGQOSTIME=2004-02-18 18:38:37 ORIGQOSPACKETSSENT=2223 ORIGQOSPACKETSRECD=1687 ORIGQOSOCTETSSENT=175154 ORIGQOSOCTETSRECD=132906 ORIGQOSPACKETSLOST=0 ORIGQOSJITTER=520 ORIGQOSAVGLATENCY=0 TERMQOSTIME=2004-02-18 18:38:37 TERMQOSPACKETSSENT=1687 TERMQOSPACKETSRECD=2223 TERMQOSOCTETSSENT=132906 TERMQOSOCTETSRECD=175154 TERMQOSPACKETSLOST=0 TERMQOSJITTER=120 TERMQOSAVGLATENCY=1 PACKETIZATIONTIME=0 SILENCESUPPRESSION=1 ECHOCANCELLATION=0 CODERTYPE=PCMU CONNECTIONTYPE=IP OPERATORINVOLVED=0 CASUALCALL=0 INTERSTATEINDICATOR=0 OVERALLCORRELATIONID=CA1469 TIMERINDICATOR=0 RECORDTYPE=NORMAL RECORD TERMCLLI=HERNVANSDS1 CALLAGENTID=CA146 ORIGPOPTIMEZONE=CST ORIGTYPE=ON NET TERMTYPE=OFF NET NASERRORCODE=0 NASDLCXREASON=0 ORIGPOPID=1 TERMPOPTIMEZONE=GMT Reply : Success: Entry 1 of 1 returned from host: priems08

Step 4   Verify that the attributes in the CDR match the call just made.

Appendix C

Check Provisioning and Database

# From EMS side A

The purpose of this procedure is to verify that provisioning is functioning without error. The following commands will add a "dummy" carrier then delete it.

Step 1   Log in as CLI user.

Step 2   CLI>add carrier id=8080;

Step 3   CLI>show carrier id=8080;

Step 4   CLI>delete carrier id=8080; 

Step 5   CLI>show carrier id=8080;

- Verify message is: Database is void of entries.

# Perform database audits

In this task, you will perform a full database audit and correct any errors, if necessary.

Step 1   CLI>audit database type=full;

Step 2   Check the audit report and verify there is no discrepancy or errors. If errors are found, please try to correct them. If you are unable to correct, please contact Cisco TAC.

# Check transaction queue

In this task, you will verify that the OAMP transaction queue status. The queue should be empty.

Step 1   CLI>show transaction-queue;

- Verify there is no entry shown. You should get following reply back:

Reply : Success: Database is void of entries.

- If the queue is not empty, wait for the queue to empty. If the problem persists, contact Cisco TAC.

Step 2   CLI>exit

Appendix D

Check Alarm Status

The purpose of this procedure is to verify that there are no outstanding major/critical alarms.

# From EMS side A

Step 1   Log in as CLI user.

Step 2   CLI>show alarm

- The system responds with all current alarms, which must be verified or cleared before executing this upgrade procedure.

|  | Tip Use the following command information for reference material ONLY. |
| --- | --- |

Step 3   To monitor system alarm continuously.

CLI>subscribe alarm-report severity=all; type=all;

|  | Valid severity: MINOR, MAJOR, CRITICAL, ALL Valid types: CALLP, CONFIG, DATABASE, MAINTENANCE, OSS, SECURITY, SIGNALING, STATISTICS, BILLING, ALL, SYSTEM, AUDIT |
| --- | --- |

Step 4   System will display alarms if alarm is reported.

Sample alarm:

<------------------- START ALARM REPORT --------------------> TIMESTAMP: 20040219162436 DESCRIPTION: Disk Partition Moderately Consumed TYPE & NUMBER: MAINTENANCE (90) SEVERITY: MINOR ALARM-STATUS: ON ORIGIN: priems08 COMPONENT-ID: null DIRECTORY: /opt DEVICE: /dev/dsk/c0t0d0s5 PERCENTAGE USED: 58.81 <------------------- END ALARM REPORT ---------------------->

Step 5   To stop monitoring system alarm.

CLI>unsubscribe alarm-report severity=all; type=all;

Step 6   Exit CLI.

CLI>exit

Appendix E

Check Oracle Database Replication and Error Correction

Perform the following steps on the Active EMS side A to check the Oracle database and replication status.

# Check Oracle DB replication status

# From EMS side A

Step 1   Log in as root.

Step 2   Log in as oracle.

<hostname># su – oracle

Step 3   Enter the command to check replication status and compare contents of tables on the side A and side B EMS databases:

<hostname>$dbadm –C rep

Step 4   Verify that “Deferror is empty?” is “YES”.

OPTICAL1::Deftrandest is empty?           YES

OPTICAL1::dba_repcatlog is empty?       YES

OPTICAL1::Deferror is empty?              YES Make sure it is “YES”

OPTICAL1::Deftran is empty?                  YES

OPTICAL1::Has no broken job?               YES

OPTICAL1::JQ Lock is empty?                YES

OPTICAL2::Deftrandest is empty?          YES

OPTICAL2::dba_repcatlog is empty?      YES

OPTICAL2::Deferror is empty?             YES Make sure it is “YES”

OPTICAL2::Deftran is empty?                YES

OPTICAL2::Has no broken job?              YES

OPTICAL2::JQ Lock is empty?               YES

Step 5   If the “Deferror is empty?” is “NO”, please try to correct the error using steps in “Correct replication error” below. If you are unable to clear the error or if any of the individual steps fails, please contact Cisco support.

# Correct replication error

|  | Note You must run the following steps on standby EMS side B first, then on active EMS side A. |
| --- | --- |

# From EMS Side B

Step 1   Log in as root

Step 2   <hostname># su – oracle

Step 3   <hostname>$ dbadm –C db

Step 4   For each table that is out of sync, please run the following step:

<hostname>$ dbadm -A copy -o <OWNER> -t <TABEL NAME>

- Enter “y” to continue

- Please contact Cisco support if the above command fails.

Step 5   <hostname>$ dbadm –A truncate_deferror

- Enter “y” to continue

# From EMS Side A

Step 1   <hostname>$ dbadm –A truncate_deferror

- Enter “y” to continue

Step 2   Re-verify that “Deferror is empty?” is “YES” and none of tables is out of sync.

<hostname>$dbadm –C db

OPTICAL1::Deftrandest is empty?           YES

OPTICAL1::dba_repcatlog is empty?       YES

OPTICAL1::Deferror is empty?              YES  Make sure it is “YES”

OPTICAL1::Deftran is empty?                  YES

OPTICAL1::Has no broken job?               YES

OPTICAL1::JQ Lock is empty?                YES

OPTICAL2::Deftrandest is empty?          YES

OPTICAL2::dba_repcatlog is empty?      YES

OPTICAL2::Deferror is empty?             YES  Make sure it is “YES”

OPTICAL2::Deftran is empty?                YES

OPTICAL2::Has no broken job?              YES

OPTICAL2::JQ Lock is empty?               YES

Appendix F

Flash Archive Steps

# Task 1: Ensure side A system is ACTIVE

In this task, you will ensure that the EMS side A applications are active.

Step 1   Log in as root to ACTIVE EMS

Step 2   Log in as CLI user

Step 3   CLI> control feature-server id=FSPTCzzz; target-state=forced-active-standby;

Step 4   CLI> control feature-server id=FSAINyyy; target-state=forced-active-standby;

Step 5   CLI> control call-agent id=CAxxx; target-state=forced-active-standby;

Step 6   CLI> control bdms id=BDMS01; target-state=forced-active-standby;

Step 7   CLI> control element-manager id=EM01; target-state=forced-active-standby;

Step 8   CLI> status system;

- Verify CAxxx on CA/FS side A is in forced ACTIVE state.

- Verify FSAINyyy on CA/FS side A is in forced ACTIVE state.

- Verify FSPTCzzz on CA/FS side A is in forced ACTIVE state.

- Verify BDMS01 on EMS side A is in forced ACTIVE state.

- Verify EM01 on EMS side A is in forced ACTIVE state.

- Verify Oracle DB is in service

Step 6   CLI> exit

# Task 2: Perform a full database audit

In this task, you will go to EMS side A and perform a full database audit and correct errors, if there are any.  Contact Cisco TAC if errors cannot be fixed.

# From EMS Side A

Step 1   Log in as CLI user

Step 2   CLI>audit database type=full;

Step 3   Check the audit report and verify there is no discrepancy or errors found. If errors are found, try to correct the errors. If you are unable to make the correction, contact Cisco TAC.

# Task 3: Perform shared memory integrity check

In this task, you will perform shared memory integrity check to detect any potential data problems.

# From CA/FS side A

Step 1   Log in as root

Step 2   <hostname># cd /opt/OptiCall/CAxxx/bin

Step 3   <hostname># ca_tiat data

Step 4   Press “Enter” to continue

The result should be identical to the following:

All tables are OK.

For detail, see ca_tiat.out

If the result does NOT show “All tables are OK”, Stop! Contact Cisco TAC.

Step 5   <hostname># cd /opt/OptiCall/FSPTCzzz/bin

Step 6   <hostname># potsctx_tiat data

Step 7   Press “Enter” to continue

The result should be identical to the following:

All tables are OK.

For detail, see potsctx_tiat.out

If the result does NOT show “All tables are OK”, Stop! Contact Cisco TAC.

Step 8   <hostname># cd /opt/OptiCall/FSAINyyy/bin

Step 9   <hostname># ain_tiat data

Step 10   Press “Enter” to continue

The result should be identical to the following:

All tables are OK.

For detail, see ain_tiat.out

If the result does NOT show “All tables are OK”, Stop! Contact Cisco TAC.

# From CA/FS side B

Step 1   Log in as root

Step 2   <hostname># cd /opt/OptiCall/CAxxx/bin

Step 3   <hostname># ca_tiat data

Step 4   Press “Enter” to continue

The result should be identical to the following:

All tables are OK.

For detail, see ca_tiat.out

If the result does NOT show “All tables are OK”, Stop! Contact Cisco TAC.

Step 5   <hostname># cd /opt/OptiCall/FSPTCzzz/bin

Step 6   <hostname># potsctx_tiat data

Step 7   Press “Enter” to continue

The result should be identical to the following:

All tables are OK.

For detail, see potsctx_tiat.out

If the result does NOT show “All tables are OK”, Stop! Contact Cisco TAC.

Step 8   <hostname># cd /opt/OptiCall/FSAINyyy/bin

Step 9   <hostname># ain_tiat data

Step 10   Press “Enter” to continue

The result should be identical to the following:

All tables are OK.

For detail, see ain_tiat.out

If the result does NOT show “All tables are OK”, Stop! Contact Cisco TAC.

# Task 4: Perform flash archive on EMS side B

In this task, you will perform a flash archive on EMS side B to save a copy of OS and applications to a remote server. This process takes about 1 hour.

|  | Note Perform Task 4: Perform Flash Archive on EMS Side B and Task 5: Perform Flash Archive on CA/FS Side B in parallel. |
| --- | --- |

# From EMS side B

Step 1   Log in as root

Step 2   <hostname># /etc/rc2.d/S75cron stop

Step 3   <hostname># ps -ef | grep cron

- Verify no result is returned, which means cron daemon is no longer running.

Step 4   <hostname># cd /etc/rc3.d

Step 5   <hostname># mv S99platform _S99platform

Step 6   <hostname>#platform stop all

Step 7   <hostname># nodestat

- Verify applications are out of service.

Step 8   <hostname># \rm –rf /opt/Build

Step 9   <hostname># \rm –rf /opt/8_rec

Step 10   <hostname># \rm –rf /opt/.upgrade

Step 11   Remove all directories and files that are no longer needed such as core files, patch directories.

Step 12   <hostname># mv /bin/date /bin/date.orig

Step 13   <hostname># mv /bin/.date /bin/date

Step 14   <hostname># tar –cvf - /opt/* | gzip –c > /opt/<hostname_release>.tar.gz

Where: : hostname_release is the tar file name.

Example: tar –cvf - /opt/* | gzip –c > /opt/secems10_4.2.0.V04.tar.gz

Step 15   <hostname># flarcreate -n <archive name> -x /opt -c /opt/<file name>

Where: archive name is the archive identification.

Example: flarcreate -n CCPU-EMS –x /opt -c /opt/secems10_4.2.0.V04.archive

Step 16   FTP the archive to an NFS server to be used later.

- <hostname>#cd /opt

- <hostname>#ftp <nfs server name>

- ftp>bin

- ftp>cd <target directory>

- ftp>put <archive file name>

- ftp>put <opt tar file name>

- ftp>bye

Step 17   <hostname># mv /bin/date /bin/.date

Step 18   <hostname># mv /bin/date.orig /bin/date

Step 19   <hostname># /etc/rc2.d/S75cron start

Step 20   <hostname># ps -ef | grep cron

- Verify cron daemon is running.

Step 21   <hostname># cd /etc/rc3.d

Step 22   <hostname># mv _S99platform S99platform

Step 23   <hostname># platform start

Step 24   <hostname># nodestat

- Verify EM01 is in forced STANDBY.

- Verify BS01 is in forced STANDBY.

- Verify Oracle and Billing DB are in service.

# Task 5: Perform flash archive on CA/FS side B

In this task, you will perform a flash archive on CA/FS side B to save a copy of OS and applications to a remote server. This process takes about 1 hour.

|  | Note Perform this task in parallel with Task 4: Perform Flash Archive on EMS Side B. |
| --- | --- |

# From CA/FS side B

Step 1   Log in as root

Step 2   <hostname># /etc/rc2.d/S75cron stop

Step 3   <hostname>#ps -ef | grep cron

- Verify no result is returned, which means cron daemon is no longer running

Step 4   <hostname># cd /etc/rc3.d

Step 5   <hostname># mv S99platform _S99platform

Step 6   <hostname># platform stop all

Step 7   <hostname># nodestat

- Verify applications are out of service.

Step 8   <hostname># \rm –rf /opt/Build

Step 9   <hostname># \rm –rf /opt/8_rec

Step 10   <hostname># \rm –rf /opt/.upgrade

Step 11   Remove all directories and files that are no longer needed such as core files, patch directories.

Step 12   <hostname># mv /bin/date /bin/date.orig

Step 13   <hostname># mv /bin/.date /bin/date

Step 14   <hostname># tar –cvf - /opt/* | gzip –c > /opt/<hostname_release>.tar.gz

Where: : hostname_release is the tar file name.

Example: tar –cvf - /opt/* | gzip –c > /opt/secca10_4.2.0.V04.tar.gz

Step 15   <hostname># flarcreate -n <archive name> -x /opt -c /opt/<file name>

Where: archive name is the archive identification.

Example: flarcreate -n CCPU-CA –x /opt -c /opt/secca10_4.2.0.V04.archive

Step 16   FTP the archive to an NFS server to be used later.

- <hostname># cd /opt

- <hostname># ftp <nfs server name>

- ftp> bin

- ftp> cd <target directory>

- ftp> put <archive file name>

- ftp> put <opt tar file name>

- ftp> bye

Step 17   <hostname># mv /bin/date /bin/.date

Step 18   <hostname># mv /bin/date.orig /bin/date

Step 19   <hostname># /etc/rc2.d/S75cron start

Step 20   <hostname># ps -ef | grep cron

- Verify cron daemon is running.

Step 21   <hostname># cd /etc/rc3.d

Step 22   <hostname># mv _S99platform S99platform

Step 23   <hostname># platform start

Step 24   <hostname># nodestat

- Verify CAxxx is in forced STANDBY.

- Verify FSAINyyy is in forced STANDBY.

- Verify FSPTCzzz is in forced STANDBY.

# Task 6: Switch activity from side A to side B

In this task, you will switch activity from the side A to the side B.

# From EMS side A

Step 1   Log in as CLI user

Step 2   CLI> control feature-server id=FSPTCzzz; target-state=forced-standby-active;

Step 3   CLI> control feature-server id=FSAINyyy; target-state=forced-standby-active;

Step 4   CLI> control call-agent id=CAxxx; target-state=forced-standby-active;

Step 5   CLI> control bdms id=BDMS01; target-state=forced-standby-active;

Step 6   CLI> control element-manager id=EM01; target-state=forced-standby-active;

Step 7   CLI session will terminate when EM01 switchover is successful.

# Task 7: Perform flash archive on EMS side A

In this task, you will perform a flash archive on EMS side A to save a copy of the OS and applications to a remote server. This process takes about 1 hour.

|  | Note Perform Task 7: Perform Flash Archive on EMS Side A and Task 8: Perform Flash Archive on CA/FS Side A in parallel. |
| --- | --- |

# From EMS side A

Step 1   Log in as root

Step 2   <hostname># /etc/rc2.d/S75cron stop

Step 3   <hostname># ps -ef | grep cron

- Verify no result is returned, which means cron daemon is no longer running.

Step 4   <hostname># cd /etc/rc3.d

Step 5   <hostname># mv S99platform _S99platform

Step 6   <hostname># platform stop all

Step 7   <hostname># nodestat

- Verify applications are out of service.

Step 8   <hostname># \rm –rf /opt/Build

Step 9   <hostname># \rm –rf /opt/8_rec

Step 10 <hostname># \rm –rf /opt/.upgrade

Step 11   Remove all directories and files that are no longer needed such as core files, patch directories.

Step 12   <hostname>#mv /bin/date /bin/date.orig

Step 13   <hostname># mv /bin/.date /bin/date

Step 14   <hostname># tar –cvf - /opt/* | gzip –c > /opt/<hostname_release>.tar.gz

Where: : hostname_release is the tar file name.

Example: tar –cvf - /opt/* | gzip –c > /opt/priems10_4.2.0.V04.tar.gz

Step 15   <hostname># flarcreate -n <archive name> -x /opt -c /opt/<file name>

Where: archive name is the archive identification.

Example: flarcreate -n CCPU-EMS –x /opt -c /opt/priems10_4.2.0.V04.archive

Step 16   FTP the archive to an NFS server to be used later.

- <hostname># cd /opt

- <hostname># ftp <nfs server name>

- ftp> bin

- ftp> cd <target directory>

- ftp> put <archive file name>

- ftp> put <opt tar file name>

- ftp> bye

Step 17   <hostname># mv /bin/date /bin/.date

Step 18   <hostname># mv /bin/date.orig /bin/date

Step 19   <hostname># /etc/rc2.d/S75cron start

Step 20   <hostname># ps -ef | grep cron

- Verify cron daemon is running.

Step 17   <hostname># cd /etc/rc3.d

Step 18   <hostname># mv _S99platform S99platform

Step 19   <hostname># platform start

Step 20   <hostname># nodestat

- Verify EM01 is in forced STANDBY.

- Verify BS01 is in forced STANDBY.

- Verify Oracle and Billing DB are in service.

# Task 8: Perform flash archive on CA/FS side A

In this task, you will perform flash archive on CA/FS side A to save a copy of OS and applications to a remote server. This process takes about 1 hour.

|  | Note Perform this task in parallel with Task 7: Perform Flash Archive on EMS Side A. |
| --- | --- |

# From CA/FS side A

Step 1   Log in as root

Step 2   <hostname># /etc/rc2.d/S75cron stop

Step 3   <hostname># ps -ef | grep cron

- Verify no result is returned, which means cron daemon is no longer running

Step 4 <hostname># cd /etc/rc3.d

Step 5 <hostname># mv S99platform _S99platform

Step 6   <hostname># platform stop all

Step 7   <hostname># nodestat

- Verify applications are out of service.

Step 9  <hostname># \rm –rf /opt/Build

Step 9  <hostname># \rm –rf /opt/8_rec

Step 10  <hostname># \rm –rf /opt/.upgrade

Step 11   Remove all directories and files that are no longer needed such as core files, patch directories.

Step 12   <hostname># mv /bin/date /bin/date.orig

Step 13   <hostname># mv /bin/.date /bin/date

Step 14   <hostname># tar –cvf - /opt/* | gzip –c > /opt/<hostname_release>.tar.gz

Where: : hostname_release is the tar file name.

Example: tar –cvf - /opt/* | gzip –c > /opt/prica10_4.2.0.V04.tar.gz

Step 15   <hostname># flarcreate -n <archive name> -x /opt -c /opt/<file name>

Where: archive name is the archive identification.

Example: flarcreate -n CCPU-CA –x /opt -c /opt/prica10_4.2.0.V04.archive

Step 16   FTP the archive to an NFS server to be used later.

- <hostname># cd /opt

- <hostname># ftp <nfs server name>

- ftp> bin

- ftp> cd <target directory>

- ftp> put <archive file name>

- ftp> put <opt tar file name>

- ftp> bye

Step 17   <hostname># mv /bin/date /bin/.date

Step 18   <hostname># mv /bin/date.orig /bin/date

Step 19  <hostname># /etc/rc2.d/S75cron start

Step 20  <hostname># ps -ef | grep cron

- Verify cron daemon is running.

Step 21  <hostname># cd /etc/rc3.d

Step 22  <hostname># mv _S99platform S99platform

Step 23  <hostname># platform start

Step 24  <hostname># nodestat

- Verify CAxxx is in forced STANDBY.

- Verify FSAINyyy is in forced STANDBY.

- Verify FSPTCzzz is in forced STANDBY.

# Task 9: Release forced switch

In this task, you will release the forced switch.

# From EMS side B

Step 1   Log in as CLI user

Step 2   CLI> control feature-server id=FSPTCyyy; target-state=forced-active-standby;

Step 3   CLI> control feature-server id=FSAINzzz; target-state=forced-active-standby;

Step 4   CLI> control call-agent id=CAxxx; target-state=forced-active-standby;

Step 5   CLI> control bdms id=BDMS01; target-state =forced-active-standby;

Step 6   CLI> control element-manager id=EM01; target-state=forced-active-standby;

Step 7   CLI session will terminate when the EM01 switchover is successful.

# From EMS side A

Step 1   Log in as CLI user

Step 2   CLI> control feature-server id=FSPTCyyy; target-state=normal;

Step 3   CLI> control feature-server id=FSAINzzz; target-state=normal;

Step 4   CLI> control call-agent id=CAxxx; target-state=normal;

Step 5   CLI> control bdms id=BDMS01; target-state =normal;

Step 6   CLI> control element-manager id=EM01; target-state=normal;

Step 7   CLI> exit

# This completes the flash archive process.

Appendix G

Backout Procedure for Side B System

# Introduction

This procedure allows you to back out of the upgrade procedure if any verification checks (in "Verify system status" section) failed. This procedure is intended for the scenario in which the side B system has been upgraded to the new load and in forced active state, while the side A system is still at the previous load and in forced standby state. The procedure will back out the side B system to the previous load.

This backout procedure will:

- Restore the side A system to active mode without making any changes to it

- Revert to the previous application load on the side B system

- Restart the side B system in standby mode

- Verify that the system is functioning properly with the previous load

|  | Note In addition to performing this backout procedure, you should contact Cisco TAC when you are ready to retry the upgrade procedure. |
| --- | --- |

The flow for this procedure is shown in Figure F-1.

Figure F-1   Flow of Backout Procedure— Side B Only

# Task 1: Force side A system to active

This procedure will force the side A system to forced active state, and the side B system to forced standby state.

|  | Note In the commands below, "xxx", "yyy" or "zzz" is the instance for the process on your system. |
| --- | --- |

# From Active EMS side B

Step 1   Log in as CLI user.

Step 2   CLI> control feature-server id=FSPTCzzz; target-state=forced-active-standby;

Step 3   CLI> control feature-server id=FSAINyyy; target-state=forced-active-standby;

Step 4   CLI> control call-agent id=CAxxx; target-state=forced-active-standby;

Step 5   CLI> control bdms id=BDMS01; target-state=forced-active-standby;

Step 6   CLI> control element-manager id=EM01; target-state=forced-active-standby;

Step 7   CLI session will terminate when the last CLI command completes.

# Task 2: FTP Billing records to a mediation device

# From EMS side B

Step 1   Log in as root

Step 2   <hostname># cd /opt/bms/ftp/billing

Step 3   <hostname># ls

Step 4   If there are files listed, then FTP the files to a mediation device on the network

# Task 3: Stop applications on EMS side B and CA/FS side B

# From EMS side B

Step 1   Log in as root.

Step 2   <hostname># /etc/rc2.d/S75cron stop

Step 3   <hostname># platform stop all

# From CA/FS side B

Step 1   Log in to CA/FS Side B as root.

Step 2   <hostname># /etc/rc2.d/S75cron stop

Step 3   <hostname># platform stop all

# Task 4: Remove installed applications on EMS side B and CA/FS side B

|  | Note To speed up the upgrade process, you can execute EMS side B and CA/FS side B in parallel. |
| --- | --- |

# From EMS side B

Step 1   Log in as root.

Step 2   <hostname># pkill mysql

Step 3   <hostname># /opt/ems/utils/uninstall.sh –aponly

- Answer “y” when prompted

- Answer “y” when prompted for “Reboot”

# From CA/FS side B

Step 1   Log in as root.

Step 2   <hostname># /opt/ems/utils/uninstall.sh –aponly

- Answer “y” when prompted

- Answer “y” when prompted for “Reboot”

# Task 5: Copy files from CD-ROM to hard drive and extract tar files

# From EMS Side B

Step 1   Log in as root.

Step 2   Put the release 900-04.02.00.V04 BTS 10200 System Disk CD-ROM in the CD-ROM drive.

Step 3   Remove old files.

<hostname># cd /

<hostname># \rm –rf /opt/Build

Step 4   Create /cdrom directory and mount the directory.

<hostname># mkdir -p /cdrom

<hostname># mount -o ro -F hsfs /dev/dsk/c0t6d0s0 /cdrom

Step 5   Use the following commands to copy file from the CD-ROM to the /opt directory.

<hostname># cp –f /cdrom/K9-opticall.tar /opt

Step 6   Verify that the check sum values match with the values located in the “checksum.txt” file located on Application CD-ROM.

<hostname># cat /cdrom/checksum.txt

<hostname># cksum /opt/K9-opticall.tar

Step 7   Unmount the CD-ROM.

<hostname># umount cdrom

Step 8   Manually eject the CD-ROM and take out the release 900-04.02.00.V04 BTS 10200 System Disk CD-ROM from CD-ROM drive.

Step 9   Extract tar files.

<hostname># cd /opt

<hostname># tar -xvf K9-opticall.tar

|  | Note Each file will take up 10 minutes to extract. |
| --- | --- |

# From CA/FS Side B

Step 1   <hostname># cd /opt

Step 2   <hostname># \rm –rf /opt/Build

Step 3   <hostname># sftp <hostname of EMS side B>

Step 4   sftp> cd /opt

Step 5   sftp> get K9-opticall.tar

Step 6   sftp> exit

Step 7   <hostname># tar -xvf K9-opticall.tar

|  | Note The file will take up 10 minutes to extract. |
| --- | --- |

# Task 6: Restore side B to the old release

|  | Note To speed up the upgrade process, you can execute EMS side B and CA/FS side B in parallel. |
| --- | --- |

# From CA/FS Side B

Step 1   Log in as root.

Step 2   <hostname># /opt/Build/install.sh –update_cfg

- <hostname># vi /etc/opticall.cfg

- Define the value for parameter: BROKER_DNS_FOR_PMG

- Save the opticall.cfg file

Step 3   <hostname># /opt/Build/install.sh -aponly

Step 4   Answer "y" when prompt. This process will take up to 15 minutes to complete.

Step 5   Answer "y” when prompt for “reboot”

Step 6   Wait for the system to boot up. Then Log in as root

Step 7   Verify applications are in service and in standby state.

<hostname># nodestat

# From EMS side B

Step 1   Log in as root.

Step 2   <hostname># pkill mysql

Step 3   <hostname># /opt/Build/install.sh –update_cfg

- <hostname># vi /etc/opticall.cfg

- Define the value for parameter: BROKER_DNS_FOR_PMG

- Save the opticall.cfg file

Step 4   <hostname># /opt/Build/install.sh -aponly

Step 5   Answer "y" when prompt. This process will take up to 15 minutes to complete.

Step 6   Answer "y” when prompt for “reboot”

Step 7   Wait for the system to boot up. Then Log in as root

Step 8   Verify applications are in service and in standby state.

<hostname># nodestat

Step 9   Re-load static CLI data for provisioning.

<hostname># su - oracle

<hostname>$ cd /opt/oracle/opticall/create/data

<hostname>$ update_cmdtab.sh optical2 upgrade

<hostname>$ exit

Step 10   <hostname># /opt/ems/utils/updMgr.sh –restore_hub

Step 11   <hostname># nodestat

- Verify there is HUB communication from EMS side B to CA/FS side A

Step 12   Restore user accounts:

<hostname># cd /opt/.upgrade

<hostname># tar -xvf users.tar

# Task 7: Restore cron job

# From EMS side B

Step 1   Log in as root.

Step 2   <hostname># cd /var/spool/cron/crontabs

Step 3   <hostname># cp /opt/.upgrade/oracle .

Step 4   <hostname># /etc/rc2.d/S75cron stop

Step 5   <hostname># /etc/rc2.d/S75cron start

# Task 8: To install CORBA on EMS side B, please follow Appendix I.

# Task 9: Restore the system to the normal mode

# From EMS side A

Step 1   Log in as CLI user.

Step 2   CLI> control feature-server id=FSPTCzzz; target-state=normal;

Step 3   CLI> control feature-server id=FSAINyyy; target-state=normal;

Step 4   CLI> control call_agent id=CAxxx; target-state=normal;

Step 5   CLI> control bdms id=BDMS01; target-state=normal;

Step 6   CLI> control element-manager id=EM01; target-state=normal;

Step 7   CLI> exit

# Task 10: Verify system status

Verify that the system is operating properly before you leave the site.

Step 1   Verify that the side A system is in the active state. Use Appendix A for this procedure.

Step 2   Verify that call processing is working without error. Use Appendix B for this procedure.

Step 3   Verify that provisioning is operational from CLI command line, and verify database. Use Appendix C for this procedure.

Step 4   Verify that there are no outstanding major or critical alarms. Use Appendix D for this procedure.

Step 5   Use Appendix E to verify that Oracle database and replication functions are working properly.

Step 6   Use Appendix J to verify that the system clock is in sync.

Step 7   If you answered NO to any of the above questions (Step 1 through Step 6), Contact Cisco TAC for assistance.

# This completes the side B system fallback.

Appendix H

System Backout Procedure

# Introduction

This procedure allows you to back out of the upgrade procedure if any verification checks (in "Verify system status" section) failed. This procedure is intended for the scenario in which both the side A and side B systems have been upgraded to the new load. The procedure will back out the entire system to the previous load.

This backout procedure will:

- Revert to the previous application load on the side A system

- Restart the side A system and place it in active mode

- Revert to the previous application load on the side B system

- Restart the side B system and place it in active mode

- Verify that the system is functioning properly with the previous load

|  | Note In addition to performing this backout procedure, you should contact Cisco TAC when you are ready to retry the upgrade procedure. |
| --- | --- |

# Task 1: Inhibit EMS mate communication

In this task, you will isolate the OMS Hub on EMS side B from talking to CA/FS side A.

# From EMS side B

Step 1   Log in as root

Step 2   <hostname># /opt/ems/utils/updMgr.sh –split_hub

Step 3   <hostname># nodestat

- Verify there is no HUB communication from EMS side B to CA/FS side A

# Task 2: Force side B system to active

This procedure will force the side B system to go active.

|  | Note In the commands below, "xxx", "yyy" or "zzz" is the instance for the process on your system. |
| --- | --- |

# From EMS side A

Step 1   Log in as CLI user.

Step 2   CLI> control feature-server id=FSPTCzzz; target-state=forced-standby-active;

Step 3   CLI> control feature-server id=FSAINyyy; target-state=forced-standby-active;

Step 4   CLI> control call-agent id=CAxxx; target-state=forced-standby-active;

Step 5   CLI> control bdms id=BDMS01; target-state=forced-standby-active;

Step 6   CLI> control element-manager id=EM01; target-state=forced-standby-active;

Step 7   CLI session will terminate when the last CLI command completes.

# Task 3: Stop applications and cron daemon on side A system

# From EMS side A

Step 1   Log in as root.

Step 2   <hostname># /etc/rc2.d/S75cron stop

Step 3   <hostname># platform stop all

# From CA/FS side A

Step 1   Log in as root.

Step 2   <hostname># /etc/rc2.d/S75cron stop

Step 3   <hostname># platform stop all

# Task 4: FTP Billing records to a mediation device

# From EMS side A

Step 1   Log in as root

Step 2   <hostname># cd /opt/bms/ftp/billing

Step 3   <hostname># ls

Step 4   If there are files listed, then FTP the files to a mediation device on the network.

# Task 5: Remove installed applications on EMS side A and CA/FS side A

|  | Note To speed up the upgrade process, you can execute EMS side A and CA/FS side A in parallel. |
| --- | --- |

# From EMS side A

Step 1   Log in as root.

Step 2   <hostname># /opt/ems/utils/uninstall.sh –aponly

- Answer “y” when prompted

- Answer “y” when prompted for “Reboot”

# From CA/FS side A

Step 1   Log in as root.

Step 2   <hostname># pkill mysql

Step 3   <hostname># /opt/ems/utils/uninstall.sh -aponly

- Answer “y” when prompted

- Answer “y” when prompted for “Reboot”

# Task 6: Copy files from CD-ROM to hard drive and extract tar files

# From EMS Side A

Step 1   Log in as root.

Step 2   Put the release 900-04.02.00.V04 BTS 10200 System Disk CD-ROM in the CD-ROM drive.

Step 3   Remove old files.

<hostname># cd /

<hostname># \rm –rf /opt/Build

Step 4   Create /cdrom directory and mount the directory.

<hostname># mkdir -p /cdrom

<hostname># mount -o ro -F hsfs /dev/dsk/c0t6d0s0 /cdrom

Step 5   Use the following commands to copy file from the CD-ROM to the /opt directory.

<hostname># cp –f /cdrom/K9-opticall.tar /opt

Step 6   Verify that the check sum values match with the values located in the “checksum.txt” file located on Application CD-ROM.

<hostname># cat /cdrom/checksum.txt

<hostname># cksum /opt/K9-opticall.tar

Step 7   Unmount the CD-ROM.

<hostname># umount cdrom

Step 8   Manually eject the CD-ROM and take out the release 900-04.02.00.V04 BTS 10200 System Disk CD-ROM from CD-ROM drive.

Step 9   Extract tar files.

<hostname># cd /opt

<hostname># tar -xvf K9-opticall.tar

|  | Note Each file will take up 10 minutes to extract. |
| --- | --- |

# From CA/FS Side A

Step 1  <hostname># cd /opt

Step 2   <hostname># \rm –rf /opt/Build

Step 3   <hostname># sftp <hostname of EMS Side A>

Step 4   sftp> cd /opt

Step 5   sftp> get K9-opticall.tar

Step 6   sftp> exit

Step 7   <hostname># tar -xvf K9-opticall.tar

|  | Note The file will take up 10 minutes to extract. |
| --- | --- |

# Task 7: Restore CA/FS side A to the old release

|  | Note To speed up the upgrade process, you can execute EMS side A and CA/FS side A in parallel. |
| --- | --- |

# From CA/FS side A

Step 1   Log in as root.

Step 2   <hostname># /opt/Build/install.sh –update_cfg

- <hostname># vi /etc/opticall.cfg

- Define the value for parameter: BROKER_DNS_FOR_PMG

- Save the opticall.cfg file

Step 3   <hostname># /opt/Build/install.sh -aponly

Step 4   Answer "y" when prompt. This process will take up to 15 minutes to complete.

Step 5   Answer "y” when prompt for “reboot”

Step 6   Wait for the system to boot up. Then Log in as root

Step 7   Verify applications are in service and in standby state.

<hostname># nodestat

# From EMS side A

Step 1   Log in as root.

Step 2   <hostname># /opt/Build/install.sh –update_cfg

- <hostname># vi /etc/opticall.cfg

- Define the value for parameter: BROKER_DNS_FOR_PMG

- Save the opticall.cfg file

Step 3   <hostname># pkill mysql

Step 4   <hostname># /opt/Build/install.sh -aponly

Step 5   Answer "y" when prompt. This process will take up to 15 minutes to complete.

Step 6   Answer "y” when prompt for “reboot”

Step 7   Wait for the system to boot up. Then Log in as root

Step 8   Verify applications are in service and in standby state.

<hostname># nodestat

Step 9   Re-load static CLI data for provisioning.

<hostname># su - oracle

<hostname>$ cd /opt/oracle/opticall/create/data

<hostname>$ update_cmdtab.sh optical1 upgrade

  - optical1 – optical followed by numeric 1

<hostname>$ exit

Step 10   <hostname># /opt/ems/utils/updMgr.sh –split_hub

Step 11   <hostname># nodestat

- Verify there is no HUB communication from EMS side B to CA/FS side A

Step 12   Restore user accounts:

<hostname># cd /opt/.upgrade

<hostname># tar -xvf users.tar

# Task 8: Restore cron jobs for EMS side A

# From EMS side A

Step 1   Log in as root.

Step 2   <hostname># cd /var/spool/cron/crontabs

Step 3   <hostname># cp /opt/.upgrade/oracle .

Step 4   <hostname># /etc/rc2.d/S75cron stop

Step 5   <hostname># /etc/rc2.d/S75cron start

# Task 9: To install CORBA on EMS side A, please follow Appendix I.

# Task 10: To continue fallback process, please follow Appendix G.

# This completes the entire system fallback

Appendix I

CORBA Installation

This procedure describes how to install the OpenORB Common Object Request Broker Architecture (CORBA) application on Element Management System (EMS) of the Cisco BTS 10200 Softswitch.

Note This installation process is to be used for both EMS side A and EMS side B.

Caution This CORBA installation will remove existing CORBA application on EMS machines. Once you have executed this procedure, there is no backout. Do not start this procedure until you have proper authorization. If you have questions, contact Cisco TAC.

# Task 1: Remove Installed VisiBroker

This version of CORBA is no longer supported. It must be removed from the system.

### Remove Installed VisiBroker CORBA Application

Step 1   Log in as root to EMS

Step 2   Checking VisiBroker CORBA installation

<hostname># pkgchk -q BTSvbcis

- If  the system responds without a message, this means the package exists and must be removed.

<hostname># pkgrm BTSvbcis

Step 3   Verify VisiBroker application is removed:

<hostname># pgrep cis3

The system will respond by displaying no data, or by displaying an error message. This verifies that the CORBA application is removed.

# Task 2: Install OpenORB CORBA Application

### Remove Installed OpenORB Application

Step 1   Log in as root to EMS.

Step 2   Enter the following command to remove the existing OpenORB CORBA

<hostname># pkgrm BTScis

  - Answer “y” when prompt

<hostname># pkgrm BTSoorb

  - Answer “y” when prompt

Step 3   Enter the following command to verify that the CORBA application is removed:

<hostname># pgrep cis3

The system will respond by displaying no data, or by displaying an error message. This verifies that the CORBA application is removed.

### Install OpenORB Packages

The CORBA application files are available for installation once the Cisco BTS 10200 Softswitch is installed.

Step 1   Log in as root to EMS

Step 2   <hostname># cd /opt/Build

Step 3   <hostname># cis-install.sh

System responds:

The NameService & CIS modules listen on a specific host interface. ***WARNING*** This host name or IP address MUST resolve on the CORBA client machine in the OSS. Otherwise, communication failures may occur. Enter the host name or IP address [ local hostname ]:

Step 4   Confirm the “local hostname” is the machine you are on, then press return:

Enter the host name or IP address [ local hostname ]:

  - Answer “y” when prompt

Step 5   It will take about 5-8 minutes for the installation to complete.

Step 6   Verify CORBA Application is running On EMS:

<hostname># pgrep ins3

Note System will respond by displaying the Name Service process ID, which is a number between 2 and 32,000 assigned by the system during CORBA installation. By displaying this ID, the system confirms that the ins3 process was found and is running.

<hostname># pgrep cis3

Note The system will respond by displaying the cis3 process ID, which is a number between 2 and 32,000 assigned by the system during CORBA installation. By displaying this ID, the system confirms that the cis3 process was found and is running.

Step 7   If you do not receive both of the responses described in Step 6, or if you experience any verification problems, do not continue. Contact your system administrator. If necessary, call Cisco TAC for additional technical assistance.

Appendix J

Check and Sync System Clock

This section describes steps to verify the system clock among machines in a BTS system is in sync. Otherwise, correctional steps are provided to sync up the clock in the system.

# Task 1: Check system clock

# From each machine in a BTS system

Step 1   Log in as root.

Step 2   <hostname># date

- Check and verify the date and time is in agreement with other machines in the system.

- If the date and time shown on one machine does not agree with others, please follow the steps in the Task 2 to sync up the clock.

# Task 2: Sync system clock

# From each machine in a BTS system

Step 1   <hostname># /etc/rc2.d/S79xntp stop

Step 2   <hostname># cd /opt/BTSxntp/bin

Step 3   <hostname># ntpdate <NTP server IP address>

Step 4   <hostname># /etc/rc2.d/S79xntp start <Return